3GPP TSG RAN WG1#103e	        	R1-2008350

E-meeting, Oct.26th-Nov.13th, 2020

Agenda Item:					    8.1.2.4

Source:	Sony

Title:	Considerations on HST-SFN operation for multi-TRP

Document for:	Discussion and decision

# Introduction

In RANP#86 meeting, the WID on multi-TRP operation of Rel.17 was established as below

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we present our considerations on the highlighted part, i.e. HST-SFN for multi-TRP scenario. Note that this tdoc is revised from R1-2005564.

# Discussions on HST-SFN

In some regions/countries, the NR network deployment for high speed train (HST) scenario takes significant role for user experience and vertical usage. One common operation to provide robust DL coverage is the so-called single frequency network (SFN). Taking the case in Figure 1 as an example, one UE located in a HST can be served by multiple TRPs simultaneously in a SFN manner. Specifically, two TRPs connecting to one BBU from Cell B transmits to the UE with same content occupying exactly the same time-frequency resource.

- Paradigm of high-speed train and single frequency network (HST-SFN)

For UEs with ordinary velocity, e.g. 3km/h, it doesn’t need to know whether it is served in a SFN mode or not. In other words, the SFN operation normally would be carried out in a spec-transparent manner. However, as for UEs with extremely high velocity, e.g. 500km/h, there are some reasons which call for standard effort to enable the HST-SFN operation. In following sections, we will bring next-level details for consideration and discussion.

## DL SFN transmission schemes

In RAN1#102e, two enhanced schemes for DL SFN transmission were agreed to be studied

Agreement For the discussion purpose consider the following categorization of the enhanced DL transmission schemes - Scheme 1: - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - Scheme 2: - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner - PDSCH from TRPs is transmitted in SFN manner

Assuming two TRPs serving a UE, we have repeated these two schemes in Figure 2 where the red ellipse and blue ellipse represent non-SFN and SFN transmission manner respectively. From the description of both schemes, one may easily find out that the common part of two schemes is that TRS is always transmitted in TRP-specific fashion, i.e. non-SFN manner. It therefore facilitates UE to carry out time/frequency tracking from each transmitting TRP. The differential part of two schemes is whether DMRS associated with PDCCH/PDSCH can be transmitted in SFN manner or not.

- Illustration of scheme 1 (left) and scheme 2 (right)

## Multiple QCL assumptions

Generally speaking, one may expect that the SFN operation normally works in a spec-transparent manner. However, considering the multi-TRP operation at this particular HST scenario (potentially at FR2) as depicted in Figure 2, there are two QCL related issues we do have to take into account and consider the standard impact of RAN1.

The 1st issue is from time tracking at UE side, i.e. QCL-TypeA and QCL-TypeC. Specifically, the delay spread and average delay from TRP#0 to UE could be largely different from that of the link between TRP#1 to UE. Assuming SFN-based transmission, at least the QCL type related time-domain character should be conducted to UE, meaning that one is from TRP#0 to UE and another is from TRP#1 to UE.

The 2nd issue is from spatial Rx filter determination at UE side. Even though the HST-SFN may mainly deployed at FR1 (considering the coverage performance), it is still necessary for us to consider the future proof of HST-SFN at FR2. As illustrated in Figure 2, a UE located in-between two TRPs may need to apply two different Rx beams to receive SFN-based transmission from these two TRPs. In current spec, any DL DMRS port (either associated with PDCCH or PDSCH) could only have one QCL assumption in QCL-TypeD. But as for SFN, since multiple TRPs may send the same channel/signal to the UE in different spatial directions, it is necessary to allow multiple QCL-TypeD which can be referred by UE. Surely, it also depends on UE capability on whether or not to support the SFN operation. In addition, according to the WID, literally multiple QCL assumptions should be supported for DL DMRS port(s).

- In order to enable the HST-SFN operation, RAN1 should support that multiple QCL assumptions can be applied for the same PDCCH or PDSCH DMRS port(s).

In addition, it could be beneficial for a UE located in a HST served not only by TRPs in its serving cell(s), but also TRPs associated with nearby non-serving cell(s). This operation aligns with the spirit of SFN operation, because in SFN mode, a UE doesn’t need to be aware of targets cell(s) nearby. With this being said, UE doesn’t need to know whether it receives the DL transmission from TRP(s) in non-serving cell(s) and what it needs to do is to combine the total received wireless signal.

But from NW configuration point of view, the QCL assumption from TRPs in non-serving cell should be allowable to UE. In our accompany paper [2], the concept of the so-called “universal” TCI states has been discussed and proposed. It extends the QCL assumption from the TCI states configured in serving cell(s) to TCI states configured in non-serving cell(s) with PCI either explicitly configured or implicitly associated. Hence, we have following

- To further enhance the HST-SFN operation, the QCL assumption for a UE to receive PDCCH/PDSCH could be extended from its serving cell(s) to non-serving cell(s).

## Doppler offset pre-compensation

In HST scenario, a UE served by multiple TRPs in a SFN manner may experience the effect of Doppler shift. When the UE approaches toward a TRP, the Doppler effect brings positive frequency offset. At the same time, the UE may also leave away from another TRP, so the Doppler effect introduces negative frequency offset. Combining above two Doppler effect, the total frequency offsets could be comparable to the applied SCS. Let’s take a FR2 example here. Assume train speed up to 500km/h and operating @39GHz with SCS 120kHz or 240kHz, the Doppler offset in extreme cases could be up to 180kHz. So there is surely a strong motivation to pre-compensate the Doppler offset at NW side. In RAN1#102e, the frequency offset pre-compensation steps were discussed and finally agreed as follows

Agreement For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme: - 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation - 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step - 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step Note: A second set of TRS resource(s) may be transmitted at 3rd step.

### QCL-Type of TRS

The specification in Rel.15/16 allows TRS resource serves as QCL sources in DL. For DL channels, normally TRS provides QCL information in either QCL-TypeA or QCL-TypeA + QCL-TypeD (when QCL-TypeD applicable). Take the case of Doppler offset pre-compensation in Figure 3, NW selects TRP#0 as an anchor TRP and allows TRP#1 to pre-compensate its DL RF to the anchor frequency. The anchor frequency is the frequency where the UE receives the un-compensated TRS#0.

For the conduction of QCL information, since TRS#0 is not pre-compensated and PDCCH/PDSCH are received at the anchor frequency at UE which is aligned with that of TRS#0. So TRS#0 is applicable to serve as source RS for QCL-TypeA + QCL-TypeD. But as for TRS#1, since it is not pre-compensated to the anchor frequency, the UE cannot assume TRS#1 and DMRS of PDCCH/PDSCH have the same characters of Doppler offset in frequency domain. So TRS#1 from TRP#1 (non-anchor TRP) cannot serve as source RS in certain QCL-types, i.e. QCL-TypeA, QCL-TypeB or QCL-TypeC. Note that QCL-TypeA and QCL-TypeB contain {Doppler shift and Doppler spread}, and QCL-TypeC contains {Doppler shift}. So we have

- : For TRS transmitted by non-anchor TRP, it is no longer applicable to serve as source RS for conducting QCL-TypeA, QCL-TypeB or QCL-TypeC for PDCCH/PDSCH DMRS transmitted in SFN manner.

- Multi-TRP Doppler offset pre-compensation to an anchor frequency

However, for UE receiving PDCCH/PDSCH from non-anchor TRP(s), the QCL information in time domain i.e. delay spread and average delay and spatial domain, i.e. spatial Rx filter are still valuable and necessary. To solve this problem, one alternative is to define a new QCL type, for instance QCL-TypeE which contains {delay spread, average delay}. Another alternative is to let UE ignore the frequency related part of conducted QCL information, i.e. {Doppler shift, Doppler spread} from the TRS transmitted from non-anchor TRP and only to take the remaining part as QCL assumption.

- For TRS transmitted by non-anchor TRP, RAN1 either support to define a new QCL-TypeE which only contains {delay spread, average delay} or specify UE behavior that UE shall ignore the frequency related QCL assumption when TCI state configured or indicated to UE.

Moreover, from UE perspective, if the UE ignores the frequency part of QCL assumption from non-anchor TRP, then the UE should know which TRP is the anchor TRP with an anchor frequency and which TRP(s) is/are not. This deployment can be done by NW planning and the signaling can be designed as well

- NW should configure or indicate which TRS is used for frequency anchor, when the Doppler offset pre-compensation is carried out at NW side.

### Doppler offset reporting

In RAN1#102e, as for Doppler offset pre-compensation, the following agreement was obtained. It seems that after the 1st step, a UE either implicitly or explicitly reports the measured Doppler shift(s) from TRS.

Agreement Study TRP-based frequency offset pre-compensation including the following aspects: - Aspects related to indication of the carrier frequency determined based on the received TRS resource(s) in the 1st step - Option 1: Implicit indication of the Doppler shift(s) using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step - Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - Type of the uplink reference signals / physical channel used in the 2nd step, necessity of new configuration and corresponding signaling details - Option 2: Explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework - FFS: Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - CSI reporting aspects, configuration, quantization, signalling details, etc. - New QCL types/assumption for TRS with other RS (e.g., SS/PBCH), when TRS resource(s) is used as target RS in TCI state - New QCL types/assumptions for TRS with other RS (e.g., DM-RS), when TRS resource(s) is used as source RS in the TCI state - Target physical channels (e.g., PDSCH only or PDSCH/PDCCH) and reference signals that should be supported for pre-compensation - Signaling/procedural details on whether/how the pre-compensation is applied to target channels - Whether multiple sets of TRS and pre-compensation on TRS is needed in 3rd step. - Note: Other aspects/schemes are not precluded

Due to UE hardware implementation, there could be frequency offset introduced by oscillator error at UE side. This frequency offset can only be observed at UE side. So if UE transmits SRS in the frequency in which it receives TRS in 1st step (anchor frequency), there could be 2 times of Doppler frequency offset plus UE oscillator error observed by TRP. Hence, NW is not able to accurately estimate the Doppler offset and carry out its pre-compensation. But if UE could explicitly report what it observed via CSI framework, NW can then estimate not only Doppler frequency offset, but also the oscillator error at UE side. As a result, the frequency offset pre-compensation could handle the frequency offset introduced by Doppler shift and oscillator error.

- From signaling perspective, support explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework.

In 2nd step of Doppler offset pre-compensation, a UE transmits SRS at the anchor frequency where the UE receive TRS. At least at FR2, the UE should transmit the SRS with spatial relation which was used for reception of its corresponding TRS. To reduce unnecessary signaling, NW could kick off the whole Doppler frequency offset procedure by triggering SP/AP-TRS followed by SP/AP-SRS within a single DCI. Since both DL and UL beam indication are involved, it is straight forward to apply the unified TCI. This operation also aligns well with 2nd sub-bullet of the WID pasted in above section.

- For Doppler frequency offset, support one unified TCI state in DCI to trigger SP/AP-TRS followed by SP/AP-SRS.

- Note that the UL spatial transmission filter of SRS corresponds to the DL spatial reception filter of TRS.

# Conclusions

Finally, allow us to repeat our proposals to draw attention.

- In order to enable the HST-SFN operation, RAN1 should support that multiple QCL assumptions can be applied for the same PDCCH or PDSCH DMRS port(s).

- To further enhance the HST-SFN operation, the QCL assumption for a UE to receive PDCCH/PDSCH could be extended from its serving cell(s) to non-serving cell(s).

- For TRS transmitted by non-anchor TRP, RAN1 either support to define a new QCL-TypeE which only contains {delay spread, average delay} or specify UE behavior that UE shall ignore the frequency related QCL assumption when TCI state configured or indicated to UE.

- NW should configure or indicate which TRS is used for frequency anchor, when the Doppler offset pre-compensation is carried out at NW side.

- From signaling perspective, support explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework.

- For Doppler frequency offset, support one unified TCI state in DCI to trigger SP/AP-TRS followed by SP/AP-SRS.

- Note that the UL spatial transmission filter of SRS corresponds to the DL spatial reception filter of TRS.

# Reference

- RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung

- R1-2005560, Considerations on the enhancement of multi-beam operation, Sony