| 3GPP TSG-RAN WG1 Meeting #103 e-Meeting, October 26th – November 13th, 2020 | R1-2008625 |
| --- | --- |

Agenda item:	8.8.1.2

Source: 	Qualcomm Incorporated

Title: 	Baseline FR2 coverage performance

Document for:	Discussion/Decision

# Introduction

In this document we present our results on coverage of 5G NR using link level simulations, followed by a link budget analysis. For this study, basic parameters necessary for link level evaluations are chosen to be in line with the agreements in the previous RAN1 meetings. Based on link budget analysis, we provide a set of channels and signals that should be enhanced with corresponding enhancement techniques discussed in [1]-[3].

# LLS Assumption for Baseline Coverage Performance

A list of parameters common to all link-level simulations for each test environment is given below. These parameters are applied to all uplink/downlink channels. Most of these common parameters are based on the assumptions agreed in the last RAN1 meeting.

Common parameters

Table 1. General simulation assumptions for Outdoor (Urban and Sub-urban)

| Parameter | Outdoor |
| --- | --- |
| Carrier Frequency | 28GHz |
| Multiplexing | TDD (DDSU, DDDSU) |
| System BW | 100 MHz |
| gNB Tx power | 40 dBm (equivalently 67dBm EIRP) |
| UE Tx power | 23 dBm, 12 dBm |
| Numerology | 120kHz SCS |
| BS antenna gain (dB) | 6 dB |
| UE antenna gain (dB) | 5 dB |
| Thermal noise level | -174 dBm/Hz |
| Channel | NLOS (CDL-A 100ns), |
| UE speed | 30km/h |
| # of gNB antennas | 128 per polarization |
| # of UE antennas | 4 per polarization |
| gNB array gain | 15dB for broadcast, 21dB for unicast |
| # of gNB beams | 32 for broadcast (SSB beams), 128 for unicast |
| # of cells | 1 |
| UL data rate target | 50kbps, 5Mbps |
| DL data rate target | 1Mbps, 25Mbps |

Table 2. General simulation assumptions for Indoor

| Parameter | Indoor |
| --- | --- |
| Carrier Frequency | 28GHz |
| Multiplexing | TDD (DDSU, DDDSU) |
| System BW | 100MHz |
| gNB Tx power | 23 dBm (equivalently 23+18+6=47dBm EIRP) |
| UE Tx power | 23 dBm, 12 dBm (equivalently 34dBm and 23dBm ) |
| Numerology | 120kHz SCS |
| BS antenna gain (dB) | 6 dB |
| UE antenna gain (dB) | 5 dB |
| Thermal noise level | -174 dBm/Hz |
| Channel | NLOS (CDL-A 30ns), |
| UE speed | 3km/h |
| # of gNB antennas | 64 per polarization |
| # of UE antennas | 4 per polarization |
| gNB array gain | 12dB for broadcast, 18dB for unicast |
| # of gNB beams | 16 for broadcast, 64 for unicast |
| # of cells | 1 |
| UL data rate target | 5Mbps |
| DL data rate target | 25Mbps |

Channel-specific parameters

Channels-specific parameters are chosen based on agreements from previous RAN1 meetings (please see Appendix).

# Baseline FR2 Coverage

Link budget analysis for UL and DL channels is presented in Table 5. Note that the effects of shadowing, body and head loss are also incorporated in the link budget analysis. Hardware link budget (also known as MIL) is used for the link budget methodology.

Indoor Performance

Table 3. Link budget for different channels based on the link-level simulation results for Indoor (23 dBm UE Tx power unless otherwise specified)

|  | Channel | Bandwidth (MHz) | Link budget (dB) |
| --- | --- | --- | --- |
| DL | Unicast PDSCH, 25Mbps | 100 | 133.9 |
|  | Msg4 PDSCH, 130 bytes payload | 100 | 129.8 |
|  | Msg2 PDCCH (also RMSI PDCCH) 2-symbol Coreset, AL16 | 69.12 | 132.6 |
|  | Unicast PDCCH 2-symbol Coreset, AL16 | 69.12 | 138.6 |
|  | Unicast PUSCH, 5Mbps, DDDSU, 23dBm Tx power | 100 (64 RBs) | 144.3 |
|  | Unicast PUSCH, 5Mbps, DDDSU, 12dBm Tx power |  | 133.3 |
|  | Unicast PUSCH, 50kbps 23dBm Tx power | 5.76 (4 RBs) | 164.5 |
|  | Msg3 PUSCH, 56 bits payload, 23dBm | 5.76 | 154.1 |
|  | Msg3 PUSCH, 56 bits payload, 12dBm |  | 143.1 |
|  | PRACH, 23dBm | 16.68 | 156.9 |
|  | PRACH, 12 dBm |  | 145.9 |
|  | Msg4 PUCCH (1 bit) Based on 0.01 error rate of ACK to DTX, as Msg4 PUCCH is ACK-only | 1.44 |  |
|  | PUCCH (1 bit) for unicast PDSCH ACK/NACK | 1.44 | 171 |
|  | Unicast PUCCH (11 bits) L1-report for one beam | 1.44 | 161 |
|  | Unicast PUCCH (22 bits, total 28 bits with CRC) L1-report for two beams | 1.44 | 158.5 |
|  | Unicast PUCCH (22 bits, total 28 bits with CRC) L1-report for two beams 12dBm Tx power | 1.44 | 147.5 |

Table 4. Summary of the main link budget bottlenecks and vulnerable channels, for indoor

| Link budget (in dB) | Msg2/RMSI PDCCH 2-symbol | Msg4 PDSCH 130 bytes | 25Mbps unicast PDSCH | 5Mbps PUSCH, DDDSU 12dBm UE Tx power |
| --- | --- | --- | --- | --- |
| Link budget (MIL) | 132.6 | 129.8 | 133.9 | 133.3 |
| Deficiency compared to the target of unicast PDCCH link budget | -6 | -8.8 | -4.7 | -5.3 |

As shown in the simulation results, for indoor, downlink broadcast channels have a relative weakness coverage in FR2, because of lower gain of wide broadcast beams (i.e. SSB beams) and also lower gNB transmit power for indoor. Among broadcast channels and RACH, Msg4 PDSCH and broadcast PDCCH are the main bottlenecks. Among unicast channels, 25 Mbps PDSCH and 5Mbps PUSCH are the bottlenecks. About the performance of Msg4 PDSCH, it should be noted that its vulnerability is not only because of large payload size and using wide broadcast beams, but also because of using open-loop precoder (instead of more efficient SVD-based precoder that is used for unicast PDSCH). The reason for difference in choice of precoder is the lack of CSI feedback and SRS during RACH procedure.

Observation 1: Msg4 PDSCH, Broadcast PDCCH, and 25Mbps PDSCH are the main bottlenecks in FR2 in indoor. Also, 5Mbps PUSCH can be a bottleneck depending on the UE Tx power.

Outdoor performance (Urban)

Table 5. Link budget for different channels based on the link-level simulation results for outdoor (23 dBm UE Tx power unless otherwise specified)

|  | Channel | Bandwidth (MHz) | Link budget (dB) |
| --- | --- | --- | --- |
| DL | Unicast PDSCH, 25Mbps | 100 | 151.9 |
|  | Msg4 PDSCH, 130 bytes payload | 100 | 148.2 |
|  | Msg2 PDCCH (also RMSI PDCCH) 2-symbol Coreset, AL16 | 69.12 | 152.1 |
|  | Unicast PDCCH 2-symbol Coreset, AL16 | 69.12 | 158.1 |
|  | Unicast PUSCH, 5Mbps, DDDSU, 23dBm Tx power | 100 (64 RBs) | 147.3 |
|  | Unicast PUSCH, 5Mbps, DDDSU, 12dBm Tx power |  | 136.3 |
|  | Unicast PUSCH, 50kbps 23dBm Tx power | 5.76 (4 RBs) | 167.5 |
|  | Msg3 PUSCH, 56 bits payload, 23dBm | 5.76 | 157.1 |
|  | Msg3 PUSCH, 56 bits payload, 12dBm |  | 146.1 |
|  | PRACH, 23dBm | 16.68 | 159.9 |
|  | PRACH, 12 dBm |  | 148.9 |
|  | Msg4 PUCCH (1 bit) Based on 0.01 error rate of ACK to DTX, as Msg4 PUCCH is ACK-only | 1.44 |  |
|  | PUCCH (1 bit) for unicast PDSCH ACK/NACK | 1.44 | 174 |
|  | Unicast PUCCH (11 bits) L1-report for one beam | 1.44 | 162.5 |
|  | Unicast PUCCH (22 bits, total 28 bits with CRC) L1-report for two beams | 1.44 | 160 |
|  | Unicast PUCCH (22 bits, total 28 bits with CRC) L1-report for two beams 12dBm Tx power | 1.44 | 149 |

A summary of potential bottlenecks for outdoor FR2 is shown in Table 6 and Table 7:

Table 6. Summary of the main link budget bottlenecks and vulnerable channels for outdoor for 23dBm UE Tx power

| Link budget (in dB) | Msg2/RMSI PDCCH 2-symbol | Msg4 PDSCH | 25Mbps PDSCH | 5Mbps PUSCH, DDDSU |
| --- | --- | --- | --- | --- |
| Link budget (MIL) | 152.1 | 148.2 | 151.9 | 147.3 |
| Deficiency compared to the target of unicast PDCCH link budget | -6 | -9.9 | -6.2 | -10.8 |

Table 7. Summary of the main link budget bottlenecks and vulnerable channels for outdoor for 12dBm UE Tx power

| Link budget (in dB) | Msg2/RMSI PDCCH 2-symbol | Msg4 PDSCH | 25 Mbps PDSCH | Msg3 PUSCH | PRACH | PUCCH for L1-report for two beams | 5Mbps PUSCH, DDDSU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| link budget (MIL) | 152.1 | 148.2 | 151.9 | 146.1 | 148.9 | 149 | 136.3 |
| Deficiency compared to the target of unicast PDCCH link budget | -6 | -9.9 | -6.2 | -12 | -9.2 | -9.1 | -21.8 |

As shown in the simulation results, for outdoor, downlink broadcast channels and Msg1/2/3 during RACH procedure have a relative weakness coverage in FR2, because of lower gain of wide broadcast beams (i.e. SSB beams). Among broadcast channels, RMSI PDCCH is the main bottleneck.

For RACH, for outdoor, the whole procedure can be benefited from coverage enhancement. In particular, Msg2 PDCCH is the clear bottlenecks, and PRACH and Msg3 PUSCH are also vulnerable.

Among the unicast channels, 5Mbps PUSCH is the clear bottleneck. However, it should be noted that the link budget for PUSCH is very much dependent on the rate and targeting a lower UL rate for coverage may be more reasonable for outdoor.

Observation 2: 5Mbps unicast PUSCH, 25 Mbps PDSCH, PRACH, Msg3 PUSCH, broadcast PDCCH, and high-payload PUCCH are the main bottlenecks in FR2, for outdoor.

Proposal 1: Methods for coverage enhancement of broadcast PDCCH, e.g., RMSI PDCCH, should be studied.

Proposal 2: Methods for coverage enhancement of RACH procedure, especially Msg2 PDCCH, PRACH and Msg3 PUSCH, and Msg4 PDSCH should be studied.

It should be noted that the link budget analysis that is presented for unicast channels assumes reliable assignment of the best beam. In case that the current serving beam is becoming weaker, the gNB needs to rely on L1 report (L1-RSRP) transmitted over PUCCH (which is at least 11 bits for one beam), while the beam gain is much lower than the nominal value. Therefore, in practice, PUCCH carrying L1-RSRP report may need coverage enhancement, even though the link budget analysis does not show it. It should be also noted that when two or more beams are reported, the PUCCH payload becomes larger. For example, for the L1-report of two beams, assuming 5bits for the index of each beam, the payload becomes 5+5+7+4=21 bits which can have more coverage vulnerability (as shown in the link budget analysis).

Observation 3: L1-report can have coverage vulnerability, especially for the case of reporting more than one beam.

Proposal 3: Methods for coverage enhancement of L1 report should be studied.

# Conclusion

In this document, a preliminary link-level evaluation of FR2 baseline coverage was presented. Based on the simulation results, the following are the observation and proposals:

Observation 1: Broadcast PDCCH is the main bottlenecks in FR2 in indoor. Also, 5Mbps PUSCH can be a bottleneck depending on the UE Tx power.

Observation 2: 5Mbps unicast PUSCH, PRACH, Msg3 PUSCH, and broadcast PDCCH are the main bottlenecks in FR2, for outdoor.

Proposal 1: Methods for coverage enhancement of RMSI PDCCH should be studied.

Proposal 2: Methods for coverage enhancement of RACH procedure, especially Msg2 PDCCH, PRACH and Msg3 PUSCH, should be studied.

Observation 3: L1-report can have coverage vulnerability, especially for the case of reporting more than one beam.

Proposal 3: Methods for coverage enhancement of L1 report should be studied.

# Reference

- R1-2008626, “Potential coverage enhancement techniques for PUSCH”, Qualcomm, RAN1 103e.

- R1-2008627, “Potential coverage enhancement techniques for PUCCH”, Qualcomm, RAN1 103e.

- R1-2008628, “Potential coverage enhancement techniques for other channels”, Qualcomm, RAN1 103e.

# Appendix (channel-specific simulation assumptions)

| Transmission Rank | - Rank 1 |
| --- | --- |
| PDSCH/PUSCH SLIV | - PDSCH: (S=2, L=12), - PUSCH: (S=0, L=14), intra-slot hopping is optional - Note: Starting symbol, S, (indexed from 0) and length, L. |
| DMRS Configuration | - For DL and UL w/o freq hopping: 1 DMRS symbol (front loaded) for 3km.h, 2 DMRS symbols at (2,11) for 3km/h and 30km/h, 3DMRS symbols at (2, 7, 11) for 30km/h - For UL w/ freq hopping: 2 DMRS symbols for each hop (mapping type B) - Note: no data multiplexing is assumed in DMRS symbols |
| PUCCH | - Format and payload - Format 1: 2bits UCI - Format 3: 11/22 bits UCI, 14symbols with [4] DMRS symbols - FFS: Format 0, 2 - BLER for format 1 - DTX to ACK: 1% - NACK to ACK: 0.1% - ACK misdetection: 1% - BLER for format 3 - BLER for ACK/NACK/SR: 1% - FFS: for CSI 10% or 1% - Number of RBs: 1 RB - Number of Tx chains: 1 |
| PDCCH | - AL: 16 - Payload 40bits - CORESET: 2 symbols, 48RBs - BLER: 1% |
| PRACH | - Format B4 - Target performance: 0.1% false alarm, 1% misdetection |