3GPP TSG RAN WG1 #103-e			R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Title:	[draft] Reply LS to RAN2 on beamSwitchTiming

Response to:	R2-2008318

Release:	Rel-16

Work Item:	TEI16

Source:	vivo [RAN1]

To:	RAN2

Cc:

- Contact Person:

- Name:	                Rakesh Tamrakar

E-mail Address:	 rakesh@vivo.com

- 1. Overall Description:

RAN1 thanks RAN2 for the LS on beamSwicthTiming.

On Rel-15 UE capability of beamSwitchTiming,

- For Rel-15 UEs,

- if the UE indicates a value from the set {14, 28, 48} in Rel-15 capability signaling, the reported value is used for all cases, including CSI-RS with repetition is configured and set to “ON” or “OFF”.

- if the UE indicates a value from the set {224, 336} in Rel-15 capability signaling, there is no specified beahviour for value of 224 and 336.

- If the Rel-15 capability signaling (beamSwitchTiming) is not included, the beam switch timing is up to 48 OFDM symbols for each supported sub-carrier spacing.

from RAN1’s understanding, descriptions in the first, second and third bullets are correct.

Observation from first round of discussion, majority of companies agree with RAN2 understanding.

| company | comments |
| --- | --- |
| Huawei, HiSilicon | We do not agree with the draft reply above, as some of the answers are misaligned with the latest R15 RAN1 specifications. As mentioned in R1-2008791: 1. The UE behavior for receiving AP-CSI-RS configured with repetition (set as ‘ON’ or ‘OFF’) with a scheduling offset smaller than reported beamSwitchTiming was unspecified in RAN1 in R15. This is one of the reasons that the Rel-16 TEI was agreed. 2. In R15, when UE reports {224, 336}, it will be used to determine UE expectation/behavior for aperiodic CSI-RS for tracking and latency requirements for L1-RSRP reporting. 3. In R15, RAN1 did not agree on using 48 as default value for beamSwitchTiming. Instead, in R15, RAN1 listed ‘No recommendation on the desired beam switching timing’ as ‘Consequences if the feature is not supported by the UE’ in UE feature list R1-1907862. We suggest accurately describing RAN1 intention and specification support in the reply LS (if any). |

For Rel-16 UEs, it is correct understanding that Rel-16 capability signaling BeamSwitchTiming-r16 from the set {224, 336} is optional. It is also correct that the UE behaves according to Rel-15 specification in all respects if the UE doesn’t include Rel-16 capability or doesn’t receive parameter enableBeamSwitchTiming-r16. In addition, the following answers are irrelevant to aperiodic TRS. Below please find RAN1’s Answers to Questions A, B and C.

- For Rel-16 UEs, the UE signals beamSwitchTiming and beamSwitchTiming-r16. The first one, i.e., the Rel-15 capability signaling from the set {14, 28, 48}, is always signaled (if this capability needs to be reported). The second one, i.e., the newly introduced Rel-16 capability signaling from the set {224, 336}, is optional.

- If the UE doesn’t include the Rel-16 capability, it behaves in all respects as the Rel-15 UEs above.

- If the UE does include the Rel-16 capability, then, the UE may or may not receive configured parameter enableBeamSwitchTiming-r16 in the RRC configuration:

  - If the UE doesn’t receive parameter enableBeamSwitchTiming-r16, it behaves in all respect as the Rel-15 UE above.

  - If the UE receives parameter enableBeamSwitchTiming-r16, then, the UE does the following:

    - Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48?

    - Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

    - Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

Answer A: for CSI-RS configured with repetition and set to “ON”, when the Rel-16 UE capability signaling indicates one of the values {224, 336}, the beam switch time is equal to [48 or the same as signaled value from the set {224, 336}].

Answer B: for CSI-RS configured with repetition and set to “OFF”, the beam switch time is equal to [48 or the signaled value in Rel-15 capability signaling], if reported.

Answer C: for CSI-RS configured without repetition, the beam switch time is equal to [48 or the signaled value in Rel-15 capability signaling,] if reported.

Observation from first round of discussion,

6 companies are in favor of (Option 1): ZTE, Samsung, QCM, HW, HiSi, Apple

Answer A: the same as signaled value from the set {224, 336}

Answer B: 48

Answer C: 48

3 companies are in favor of (Option 2): CATT, OPPO, vivo

Answer A: 48

Answer B: the signaled value in Rel-15 capability signaling

Answer C: the signaled value in Rel-15 capability signaling

2 companies are in favor of (Option 3): LGE, Ericsson

Answer A: 48

Answer B: 48

Answer C: 48

| company | comments |
| --- | --- |

- 2. Actions:

- To RAN WG2

- RAN1 respectfully requests RAN2 to take above answers into account.

- 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #104-e		25 Jan – 5 Feb 2021				e-Meeting

TSG RAN WG1 Meeting #104-bis-e		12 – 20 Apr 2021					e-Meeting