3GPP TSG-RAN WG1 Meeting #103-e	R1-200xxxx

# e-Meeting, October 26 – November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment CR for TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following updates and corrections are required: - Alignment of higher layer parameter names with ASN.1 - Other editorial corrections (e.g. to not list all DCI format names and allow DCI formats 0_2/1_2 when applicable) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align higher layer parameter names with ASN.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect references to ASN.1 parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2, 7.1.1, 7.2.1, 7.3.1, 7.4, 7.5, 7.6.1, 7.6.2, 7.7, 7.7.1, 8.1, 8.1A, 8.3, 9, 9.1, 9.1.2, 9.1.2.1, 9.1.3, 9.1.3.1, 9.1.3.2, 9.1.3.3, 9.2.1, 9.2.2, 9.2.3, 9.2.5, 9.2.5.0, 9.2.5.1, 9.2.5.2, 10, 10.1, 10.2, 10.3, 10.4, 11.1, 11.1.1, 11.2A, 12, 16, 16.1, 16.2.0, 16.2.1, 16.2.3, 16.2.4.3, 16.3, 16.4, 16.5, 16.5.1, 16.5.1.1, 16.5.1.2, 16.5.2.1, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1	Scope

The present document specifies and establishes the characteristics of the physical layer procedures for control operations in 5G-NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.201: "NR; Physical Layer – General Description"

[3]	3GPP TS 38.202: "NR; Services provided by the physical layer"

[4]	3GPP TS 38.211: "NR; Physical channels and modulation"

[5]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[6]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[7]	3GPP TS 38.215: "NR; Physical layer measurements"

[8-1]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[8-2]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[8-3]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[9]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[10]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[11]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification"

[13]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures"

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification"

[15]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[16]	3GPP TS 38.473: "F1 application protocol (F1AP)"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in [1, TR 21.905] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in [1, TR 21.905]. A parameter referenced in italics is provided by higher layers.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in [1, TR 21.905].

BPRE	Bits per resource element

BWP	Bandwidth part

CB	Code block

CBG	Code block group

CBR	Channel busy ratio

CCE	Control channel element

CORESET	Control resource set

CP	Cyclic prefix

CRC	Cyclic redundancy check

CSI	Channel state information

CSS	Common search space

DAI	Downlink assignment index

DC	Dual connectivity

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

EPRE	Energy per resource element

EN-DC	E-UTRA NR dual connectivity with MCG using E-UTRA and SCG using NR

FR1	Frequency range 1

FR2	Frequency range 2

GSCN	Global synchronization channel number

HARQ-ACK	Hybrid automatic repeat request acknowledgement

MCG	Master cell group

MCS	Modulation and coding scheme

NDI	New Data Indicator

NE-DC	E-UTRA NR dual connectivity with MCG using NR and SCG using E-UTRA

NR-DC	NR NR dual connectivity

PBCH	Physical broadcast channel

PCell	Primary cell

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PRACH	Physical random access channel

PRB	Physical resource block

PRG	Physical resource block group

PSCell	Primary secondary cell

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSS	Primary synchronization signal

PSSCH	Physical sidelink shared channel

PUCCH	Physical uplink control channel

PUCCH-SCell	PUCCH SCell

PUSCH	Physical uplink shared channel

QCL	Quasi co-location

RB	Resource block

RE	Resource element

RLM	Radio link monitoring

RRM	Radio resource management

RS	Reference signal

RSRP	Reference signal received power

SCG	Secondary cell group

SCI	Sidelink control information

SCS	Subcarrier spacing

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SLIV	Start and length indicator value

SPS	Semi-persistent scheduling

SR	Scheduling request

SRI	SRS resource indicator

SRS	Sounding reference signal

SSS	Secondary synchronization signal

TA	Timing advance

TAG	Timing advance group

TCI	Transmission Configuration Indicator

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

USS	UE-specific search space

# 4	Synchronization procedures

## 4.1	Cell search

Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell.

A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211].

A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI.

For a half frame with SS/PBCH blocks, the first symbol indexes for candidate SS/PBCH blocks are determined according to the SCS of SS/PBCH blocks as follows, where index 0 corresponds to the first symbol of the first slot in a half-frame.

-	Case A - 15 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes of .

-	For operation without shared spectrum channel access:

-	For carrier frequencies smaller than or equal to 3 GHz, .

-	For carrier frequencies within FR1 larger than 3 GHz, .

-	For operation with shared spectrum channel access, as described in [15, TS 37.213], n=0, 1, 2, 3, 4.

-	Case B - 30 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes . For carrier frequencies smaller than or equal to 3 GHz, . For carrier frequencies within FR1 larger than 3 GHz, .

-	Case C - 30 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes .

-	For operation without shared spectrum channel access

-	For paired spectrum operation

-	For carrier frequencies smaller than or equal to 3 GHz, . For carrier frequencies within FR1 larger than 3 GHz, .

-	For unpaired spectrum operation

-	For carrier frequencies smaller than 1.88 GHz, . For carrier frequencies within FR1 equal to or larger than 1.88 GHz, .

-	For operation with shared spectrum channel access, n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9.

-	Case D - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes . For carrier frequencies within FR2, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

-	Case E - 240 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes . For carrier frequencies within FR2, n=0, 1, 2, 3, 5, 6, 7, 8.

From the above cases, if the SCS of SS/PBCH blocks is not provided by ssbSubcarrierSpacing, the applicable cases for a cell depend on a respective frequency band, as provided in [8-1, TS 38.101-1] and [8-2, TS 38.101-2]. A same case applies for all SS/PBCH blocks on the cell. If a 30 kHz SS/PBCH block SCS is indicated by ssbSubcarrierSpacing, Case B applies for frequency bands with only 15 kHz SS/PBCH block SCS as specified in [8-1, TS 38.101-1], and the case specified for 30 kHz SS/PBCH block SCS in [8-1, TS 38.101-1] applies for frequency bands with 30 kHz SS/PBCH block SCS or both 15 kHz and 30 kHz SS/PBCH block SCS as specified in [8-1, TS 38.101-1]. For a UE configured to operate with carrier aggregation over a set of cells in a frequency band of FR2 or with frequency-contiguous carrier aggregation over a set of cells in a frequency band of FR1, if the UE is provided SCS values by ssbSubcarrierSpacing for receptions of SS/PBCH blocks on any cells from the set of cells, the UE expects the SCS values to be same.

The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to Lmax-1, where Lmax is determined according to SS/PBCH block patterns for Cases A through E. Lmax is a maximum number of SS/PBCH block indexes in a cell, and the maximum number of transmitted SS/PBCH blocks within a half frame is Lmax.

-	For operation without shared spectrum channel access, Lmax=Lmax

-	For operation with shared spectrum channel access, Lmax=8 for Lmax=10 and 15 kHz SCS of SS/PBCH blocks and for Lmax=20 and 30 kHz SCS of SS/PBCH blocks

For Lmax=4, a UE determines the 2 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211].

For Lmax>4, a UE determines the 3 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211]

-	for Lmax=10, the UE determines the 1 MSB bit of the candidate SS/PBCH block index from PBCH payload bit aA+7 as described in [5, TS 38.212]

-	for Lmax=20, the UE determines the 2 MSB bits of the candidate SS/PBCH block index from PBCH payload bits aA+6,aA+7 as described in [5, TS 38.212]

-	for Lmax=64, the UE determines the 3 MSB bits of the candidate SS/PBCH block index from PBCH payload bits aA+5,aA+6,aA+7 as described in [5, TS 38.212]

A UE can be provided per serving cell by ssb-periodicityServingCell a periodicity of the half frames for reception of the SS/PBCH blocks for the serving cell. If the UE is not configured a periodicity of the half frames for receptions of the SS/PBCH blocks, the UE assumes a periodicity of a half frame. A UE assumes that the periodicity is same for all SS/PBCH blocks in the serving cell.

For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index.

For operation with shared spectrum channel access, a UE assumes that transmission of SS/PBCH blocks in a half frame is within a discovery burst transmission window that starts from the first symbol of the first slot in a half-frame. The UE can be provided per serving cell by DiscoveryBurst-WindowLength a duration of the discovery burst transmission window. If DiscoveryBurst-WindowLength is not provided, the UE assumes that the duration of the discovery burst transmission window is a half frame. For a serving cell, the UE assumes that a periodicity of the discovery burst transmission window is same as a periodicity of half frames for receptions of SS/PBCH blocks in the serving cell. The UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst. If MSB k, k≥1, of ssb-PositionsInBurst is set to 1, the UE assumes that SS/PBCH block(s) within the discovery burst transmission window with candidate SS/PBCH block index(es) corresponding to SS/PBCH block index equal to k-1 may be transmitted; if MSB k is set to 0, the UE assumes that the SS/PBCH block(s) are not transmitted.

For operation with shared spectrum channel access, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location ‘typeA’, and ‘typeD’ properties, when applicable [6, TS 38.214], if a value of NDM-RSPBCHmodNSSBQCL is same among the SS/PBCH blocks. NDM-RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block, and NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-1 with kSSB<24 [4, TS 38.211]. subCarrierSpacingCommon indicates SCS of RMSI only for the case of operation without shared spectrum channel access. The UE can determine an SS/PBCH block index according to NDM-RSPBCHmodNSSBQCL, or according to i modNSSBQCL where i is the candidate SS/PBCH block index. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one.

Table 4.1-1: Mapping between the combination of subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset to NSSBQCL

| subCarrierSpacingCommon | LSB of ssb-SubcarrierOffset | NSSBQCL |
| --- | --- | --- |
| scs15or60 | 0 | 1 |
| scs15or60 | 1 | 2 |
| scs30or120 | 0 | 4 |
| scs30or120 | 1 | 8 |

Upon detection of a SS/PBCH block, the UE determines from MIB that a CORESET for Type0-PDCCH CSS set, as described in Clause 13, is present if kSSB<24 [4, TS 38.211] for FR1 or if kSSB<12 for FR2. The UE determines from MIB that a CORESET for Type0-PDCCH CSS set is not present if kSSB>23 for FR1 or if kSSB>11 for FR2; the CORESET for Type0-PDCCH CSS set may be provided by PDCCH-ConfigCommon.

For a serving cell without transmission of SS/PBCH blocks, a UE acquires time and frequency synchronization with the serving cell based on receptions of SS/PBCH blocks on the PCell, or on the PSCell, or on an SCell if applicable as described in [10, TS 38.133], of the cell group for the serving cell.

## 4.2	Transmission timing adjustments

A UE can be provided a value  of a timing advance offset for a serving cell by n-TimingAdvanceOffset for the serving cell. If the UE is not provided n-TimingAdvanceOffset for a serving cell, the UE determines a default value  of the timing advance offset for the serving cell as described in [10, TS 38.133].

If a UE is configured with two UL carriers for a serving cell, a same timing advance offset value  applies to both carriers.

Upon reception of a timing advance command for a TAG, the UE adjusts uplink timing for PUSCH/SRS/PUCCH transmission on all the serving cells in the TAG based on a value  that the UE expects to be same for all the serving cells in the TAG and based on the received timing advance command where the uplink timing for PUSCH/SRS/PUCCH transmissions is the same for all the serving cells in the TAG.

For a band with synchronous contiguous intra-band EN-DC in a band combination with non-applicable maximum transmit timing difference requirements as described in Note 1 of Table 7.5.3-1 of [10, TS 38.133], if the UE indicates ul-TimingAlignmentEUTRA-NR as 'required' and uplink transmission timing based on timing adjustment indication for a TAG from MCG and a TAG from SCG are determined to be different by the UE, the UE adjusts the transmission timing for PUSCH/SRS/PUCCH transmission on all serving cells part of the band with the synchronous contiguous intra-band EN-DC based on timing adjustment indication for a TAG from a serving cell in MCG in the band. The UE is not expected to transmit a PUSCH/SRS/PUCCH in one CG when the PUSCH/SRS/PUCCH is overlapping in time, even partially, with random access preamble transmitted in another CG.

For a SCS of  kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of . The start timing of the random access preamble is described in [4, TS 38.211].

In case of random access response, a timing advance command [11, TS 38.321], , for a TAG indicates  values by index values of  = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of  kHz is .  is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response.

In other cases, a timing advance command [11, TS 38.321], , for a TAG indicates adjustment of a current  value, , to the new  value, , by index values of  = 0, 1, 2,..., 63, where for a SCS of  kHz, .

If a UE has multiple active UL BWPs, as described in Clause 12, in a same TAG, including UL BWPs in two UL carriers of a serving cell, the timing advance command value is relative to the largest SCS of the multiple active UL BWPs. The applicable  value for an UL BWP with lower SCS may be rounded to align with the timing advance granularity for the UL BWP with the lower SCS while satisfying the timing advance accuracy requirements in [10, TS 38.133].

Adjustment of an  value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing for the TAG by a corresponding amount, respectively.

For a timing advance command received on uplink slot  and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in Clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in Clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot  where ,  is a time duration in msec of  symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured,  is a time duration in msec of  symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214],  is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits,  is the number of slots per subframe, and  is the subframe duration of 1 msec.  and  are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For , the UE assumes  [6, TS 38.214]. Slot  and  are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG.  is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot  is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].

If a UE changes an active UL BWP between a time of a timing advance command reception and a time of applying a corresponding adjustment for the uplink transmission timing, the UE determines the timing advance command value based on the SCS of the new active UL BWP. If the UE changes an active UL BWP after applying an adjustment for the uplink transmission timing, the UE assumes a same absolute timing advance command value before and after the active UL BWP change.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as described in [10, TS 38.133], the UE changes  accordingly.

If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot.

## 4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is  where  is a number of slots for a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot  where  is the SCS configuration for PDSCH reception on the secondary cell.

# 5	Radio link monitoring

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in Clause 12, on the primary cell. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in Clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS.

If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell.

A UE can be configured for each DL BWP of a SpCell [11, TS 38.321] with a set of resource indexes, through a corresponding set of RadioLinkMonitoringRS, for radio link monitoring by failureDetectionResources. The UE is provided either a CSI-RS resource configuration index, by csi-RS-Index, or a SS/PBCH block index, by ssb-Index. The UE can be configured with up to  RadioLinkMonitoringRS for link recovery procedures, as described in Clause 6, and for radio link monitoring. From the  RadioLinkMonitoringRS, up to  RadioLinkMonitoringRS can be used for radio link monitoring depending on Lmax  as described in Table 5-1, wherein Lmax is as defined in Clause 4.1, and up to two RadioLinkMonitoringRS can be used for link recovery procedures.

For operation with shared spectrum channel access, when a UE is provided a SS/PBCH block index by ssb-Index, the UE is expected to perform radio link monitoring using SS/PBCH block(s) in the discovery burst transmission window as described in Clause 4.1, where the SS/PBCH block(s) have candidate SS/PBCH block index(es) corresponding to SS/PBCH block index provided by ssb-Index.

If the UE is not provided RadioLinkMonitoringRS and the UE is provided for PDCCH receptions TCI states that include one or more of a CSI-RS

-	the UE uses for radio link monitoring the RS provided for the active TCI state for PDCCH reception if the active TCI state for PDCCH reception includes only one RS

-	if the active TCI state for PDCCH reception includes two RS, the UE expects that one RS is configured with qcl-Type set to ‘typeD’ [6, TS 38.214] and the UE uses the RS configured with qcl-Type set to ‘typeD’ for radio link monitoring; the UE does not expect both RS to be configured with qcl-Type set to ‘typeD’.

-	the UE is not required to use for radio link monitoring an aperiodic or semi-persistent RS

-	For , the UE selects the  RS provided for active TCI states for PDCCH receptions in CORESETs associated with the search space sets in an order from the shortest monitoring periodicity. If more than one CORESETs are associated with search space sets having same monitoring periodicity, the UE determines the order of the CORESET from the highest CORESET index as described in Clause 10.1.

A UE does not expect to use more than  RadioLinkMonitoringRS for radio link monitoring when the UE is not provided RadioLinkMonitoringRS.

Values of  and  for different values of  are given in Table 5-1.

Table 5-1:  and  as a function of maximum number  of SS/PBCH blocks per half frame

| 4 | 2 | 2 |
| --- | --- | --- |
| 8 | 6 | 4 |
| 64 | 8 | 8 |

For a CSI-RS resource configuration, powerControlOffsetSS is not applicable and a UE expects to be provided only 'noCDM' from cdm-Type, only 'one' and 'three' from density, and only '1 port' from nrofPorts [6, TS 38.214].

If a UE is configured with multiple DL BWPs for a serving cell, the UE performs RLM using the RS(s) corresponding to resource indexes provided by RadioLinkMonitoringRS for the active DL BWP or, if RadioLinkMonitoringRS is not provided for the active DL BWP, using the RS(s) provided for the active TCI state for PDCCH receptions in CORESETs on the active DL BWP.

In non-DRX mode operation, the physical layer in the UE assesses once per indication period the radio link quality, evaluated over the previous time period defined in [10, TS 38.133] against thresholds (Qout and Qin) configured by rlmInSyncOutOfSyncThreshold. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec.

In DRX mode operation, the physical layer in the UE assesses once per indication period the radio link quality, evaluated over the previous time period defined in [10, TS 38.133], against thresholds (Qout and Qin) provided by rlmInSyncOutOfSyncThreshold. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and the DRX period.

The physical layer in the UE indicates, in frames where the radio link quality is assessed, out-of-sync to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the physical layer in the UE indicates, in frames where the radio link quality is assessed, in-sync to higher layers.

# 6	Link recovery procedures

A UE can be provided, for each BWP of a serving cell, a set  of periodic CSI-RS resource configuration indexes by failureDetectionResources and a set  of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. If the UE is not provided  by failureDetectionResources for a BWP of the serving cell, the UE determines the set  to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set  includes RS indexes configured with qcl-Type set to ‘typeD’ for the corresponding TCI states. The UE expects the set  to include up to two RS indexes. The UE expects single port RS in the set . The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set .

The thresholds Qout,LR and Qin,LR correspond to the default value of rlmInSyncOutOfSyncThreshold, as described in [10, TS 38.133] for Qout, and to the value provided by rsrp-ThresholdSSB or rsrp-ThresholdBFR, respectively.

The physical layer in the UE assesses the radio link quality according to the set  of resource configurations against the threshold Qout,LR. For the set , the UE assesses the radio link quality only according to periodic CSI-RS resource configurations, or SS/PBCH blocks on the PCell or the PSCell, that are quasi co-located, as described in [6, TS 38.214], with the DM-RS of PDCCH receptions monitored by the UE. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained from a SS/PBCH block. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by powerControlOffsetSS.

In non-DRX mode operation, the physical layer in the UE provides an indication to higher layers when the radio link quality for all corresponding resource configurations in the set  that the UE uses to assess the radio link quality is worse than the threshold Qout,LR. The physical layer informs the higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined by the maximum between the shortest periodicity among the periodic CSI-RS configurations, and/or SS/PBCH blocks on the PCell or the PSCell, in the set  that the UE uses to assess the radio link quality and 2 msec. In DRX mode operation, the physical layer provides an indication to higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined as described in [10, TS 38.133].

For the PCell or the PSCell, upon request from higher layers, the UE provides to higher layers the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set  and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold.

For the SCell, upon request from higher layers, the UE indicates to higher layers whether there is at least one periodic CSI-RS configuration index and/or SS/PBCH block index from the set  with corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, and provides the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set  and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, if any.

For the PCell or the PSCell, a UE can be provided a CORESET through a link to a search space set provided by recoverySearchSpaceId, as described in Clause 10.1, for monitoring PDCCH in the CORESET. If the UE is provided recoverySearchSpaceId, the UE does not expect to be provided another search space set for monitoring PDCCH in the CORESET associated with the search space set provided by recoverySearchSpaceId.

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Clause 8.1. For PRACH transmission in slot  and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index  provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot  within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index  until the UE receives by higher layers an activation for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList.

For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId for which the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI and until the UE receives an activation command for PUCCH-SpatialRelationInfo [11, TS 38.321] or is provided PUCCH-SpatialRelationInfo for PUCCH resource(s), the UE transmits a PUCCH on a same cell as the PRACH transmission using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in Clause 7.2.1 with , , and

For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where a UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE assumes same antenna port quasi-collocation parameters as the ones associated with index  for PDCCH monitoring in a CORESET with index 0.

A UE can be provided, by schedulingRequestID-BFR-SCell, a configuration for PUCCH transmission with a link recovery request (LRR) as described in Clause 9.2.4. The UE can transmit in a first PUSCH MAC CE providing index(es) for at least corresponding SCell(s) with radio link quality worse than Qout,LR, indication(s) of presence of qnew for corresponding SCell(s), and index(es) qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for corresponding SCell(s). After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	monitors PDCCH in all CORESETs on the SCell(s) indicated by the MAC CE using the same antenna port quasi co-location parameters as the ones associated with the corresponding index(es) qnew, if any

-	transmits PUCCH on a PUCCH-SCell using a same spatial domain filter as the one corresponding to qnew for periodic CSI-RS or SS/PBCH block reception, as described in Clause 9.2.2, and using a power determined as described in Clause 7.2.1 with qu=0, qd=qnew, and l=0, if

-	the UE is provided PUCCH-SpatialRelationInfo for the PUCCH,

-	a PUCCH with the LRR was either not transmitted or was transmitted on the PCell or the PSCell, and

-	the PUCCH-SCell is included in the SCell(s) indicated by the MAC-CE

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the at least one SCell.

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in Clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in Clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in Clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE and μ  is the SCS configuration for the PUCCH or PUSCH, respectively.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

## 7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP  b, as described in Clause 12, of carrier f of serving cell c, a UE first calculates a linear value PPUSCH,b,f,c(i,j,qd,l) of the transmit power PPUSCH,b,f,c(i,j,qd,l), with parameters as defined in Clause 7.1.1. For a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook',

-	if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where:

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2,

-	s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook',

-	s=1, if an SRS resource with a single port is indicated by an SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1

-	else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource.

The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP  of carrier  of serving cell  using parameter set configuration with index  and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power  in PUSCH transmission occasion  as

[dBm]

where,

-	is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of serving cell  in PUSCH transmission occasion .

-	 is a parameter composed of the sum of a component  and a component  where .

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in Clause 8.3,

, , and ,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or  dB if msg3-DeltaPreamble is not provided, for carrier  of serving cell

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in Clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, ,  is provided by p0-NominalWithoutGrant, or  if p0-NominalWithoutGrant is not provided, and  is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a  value, applicable for all , is provided by p0-NominalWithGrant, or  if p0-NominalWithGrant is not provided, for each carrier  of serving cell  and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the value of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value. If the DCI format also includes an open-loop power control parameter set indication field and a value of the open-loop power control parameter set indication field is '1', the UE determines a value of  from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value.

-	If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, ,

-	If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of  from

-	a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00'

-	a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '1' or '01'

-	a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '10'

-	else, the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	For

-	For ,

-	if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha

-	elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided,  is the value of msg3-Alpha

-	else,

-	For ,  is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a set of  values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	 is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  and  is a SCS configuration defined in [4, TS 38.211]

-	is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in Clause 12, of carrier f of serving cell c

-	If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB

-	If the UE is configured with a number of RS resource indexes, up to the value of maxNrofPUSCH-PathlossReferenceRSs, and a respective set of RS configurations for the number of RS resource indexes by PUSCH-PathlossReferenceRS, the set of RS resource indexes can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource index qd in the set of RS resource indexes to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pusch-PathlossReferenceRS-Id in PUSCH-PathlossReferenceRS

-	If the PUSCH transmission is scheduled by a RAR UL grant as described in Clause 8.3, or for a PUSCH transmission for Type-2 random access procedure as described in Clause 8.1A, the UE uses the same RS resource index qd as for a corresponding PRACH transmission

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and a set of PUSCH-PathlossReferenceRS-Id values and determines the RS resource index qd from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the PUSCH transmission is scheduled by DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in Clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index

-	If the PUSCH transmission is not scheduled by DCI format 0_0, and if the UE is provided enableDefaultBeamPL-ForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for an SRS resource set with an SRS resource associated with the PUSCH transmission

-	If

-	the PUSCH transmission is scheduled by DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or

-	the PUSCH transmission is not scheduled by DCI format 0_0 that does not include an SRI field, or

-	SRI-PUSCH-PowerControl is not provided to the UE,

the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If

-	the PUSCH transmission is scheduled by DCI format 0_0 on serving cell c,

-	the UE is not provided PUCCH resources for the active UL BWP of serving cell c, and

-	the UE is provided enableDefaultBeamPL-ForPUSCH0-0

the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to ‘typeD’ in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the serving cell c

-	If

-	the PUSCH transmission is scheduled by DCI format 0_0 on serving cell c,

-	the UE is not provided a spatial setting for PUCCH resources on the active UL BWP of the primary cell [11, TS 38.321], and

-	the UE is provided enableDefaultBeamPL-ForPUSCH0-0

the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to ‘typeD’ in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the serving cell c

-	For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, the UE determines a RS resource index  from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission. If the DCI format activating the PUSCH transmission does not include an SRI field, the UE determines a RS resource index  with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a mapping between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id values can be updated by a MAC CE as described in [11, TS38.321]

-		For a PUSCH transmission scheduled by a DCI format that does not include an SRI field, or for a PUSCH transmission configured by ConfiguredGrantConfig and activated, as described in Clause 10.2, by a DCI format that does not include an SRI field, a RS resource index  is determined from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0

= referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

If the UE is not configured periodic CSI-RS reception, referenceSignalPower is provided by ss-PBCH-BlockPower. If the UE is configured periodic CSI-RS reception, referenceSignalPower is provided either by ss-PBCH-BlockPower or by powerControlOffsetSS providing an offset of the CSI-RS transmission power relative to the SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB.

-	 for  and  for  where  is provided by deltaMCS for each UL BWP  of each carrier  and serving cell . If the PUSCH transmission is over more than one layer [6, TS 38.214], .  and , for active UL BWP  of each carrier  and each serving cell , are computed as below

-	 for PUSCH with UL-SCH data and  for CSI transmission in a PUSCH without UL-SCH data, where

-	 is a number of transmitted code blocks,  is a size for code block , and  is a number of resource elements determined as , where  is a number of symbols for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell ,  is a number of subcarriers excluding DM-RS subcarriers and phase-tracking RS samples [4, TS 38.211] in PUSCH symbol  and assuming no segmentation for a nominal repetition in case the PUSCH transmission is with repetition Type B, , and ,  are defined in [5, TS 38.212]

-	 when the PUSCH includes UL-SCH data and , as described in Clause 9.3, when the PUSCH includes CSI and does not include UL-SCH data

-	 is the modulation order and  is the target code rate, as described in [6, TS 38.214], provided by the DCI format scheduling the PUSCH transmission that includes CSI and does not include UL-SCH data

-	For the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  in PUSCH transmission occasion

-	 is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in Clause 11.3

-	 if the UE is configured with twoPUSCH-PC-AdjustmentStates and  if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in Clause 8.3

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of  is provided to the UE by powerControlLoopToUse

-	If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the  value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the  value that is mapped to the SRI field value

-	If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE,

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is not provided tpc-Accumulation, where

-	The  values are given in Table 7.1.1-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUSCH transmission occasion  and  symbols before PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  for PUSCH power control adjustment state , where  is the smallest integer for which  symbols before PUSCH transmission occasion  is earlier than  symbols before PUSCH transmission occasion

-	If a PUSCH transmission is scheduled by a DCI format,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission

-	If a PUSCH transmission is configured by ConfiguredGrantConfig,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	A UE resets accumulation of a PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  to

-	If a configuration for a corresponding  value is provided by higher layers

-	If a configuration for a corresponding  value is provided by higher layers

where  is determined from the value of  as

-	If  and the UE is provided higher SRI-PUSCH-PowerControl,  is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to

-	If  and the UE is not provided SRI-PUSCH-PowerControl or j=0,

-	If ,  is provided by the value of powerControlLoopToUse

-	 is the PUSCH power control adjustment state for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is provided tpc-Accumulation, where

-	 absolute values are given in Table 7.1.1-1

-	If the UE receives a random access response message in response to a PRACH transmission or a MsgA transmission on active UL BWP  of carrier  of serving cell  as described in Clause 8

-	, where  and

-	 is a TPC command value indicated in a random access response grant of the random access response message corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant of the random access response message corresponding to a MsgA transmission according to Type-2 random access procedure with RAR message(s) for fallbackRAR, on active UL BWP  of carrier  in the serving cell , and

-	 and  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last random access preamble for carrier  in the serving cell ,  is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP  of carrier  of serving cell , and  is the power adjustment of first PUSCH transmission on active UL BWP  of carrier  of serving cell .

-	If the UE transmits the PUSCH in PUSCH transmission occasion i on active UL BWP b  of carrier f of serving cell c as described in Clause 8.1A, fb,f,c(0,l)=ΔPrampup,b,f,c, where

-	l=0, and

-	 and ΔPrampuprequested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers, MRB,b,f,cPUSCH(i) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks, and ΔTF,b,f,c(i) is the power adjustment of the PUSCH transmission in PUSCH transmission occasion i

Table 7.1.1-1: Mapping of TPC Command Field in a DCI format scheduling a PUSCH transmission, or in DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, or in DCI format 2_3, to absolute and accumulated  values or  values

| TPC Command Field | Accumulated or [dB] | Absolute or [dB] |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

## 7.2	Physical uplink control channel

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG.

-	When the procedures are applied for MCG, the term 'serving cell' in this clause refers to serving cell belonging to the MCG.

-	When the procedures are applied for SCG, the term 'serving cell' in this clause refers to serving cell belonging to the SCG. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group.

-	When the procedures are applied for the primary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the primary PUCCH group.

-	When the procedures are applied for the secondary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the secondary PUCCH group. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

### 7.2.1	UE behaviour

If a UE transmits a PUCCH on active UL BWP  of carrier  in the primary cell  using PUCCH power control adjustment state with index , the UE determines the PUCCH transmission power  in PUCCH transmission occasion  as

[dBm]

where

-	 is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of primary cell  in PUCCH transmission occasion

-	 is a parameter composed of the sum of a component , provided by p0-nominal, or  dBm if p0-nominal is not provided, for carrier  of primary cell  and, if provided, a component  provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP  of carrier  of primary cell , where .  is a size for a set of  values provided by maxNrofPUCCH-P0-PerSet. The set of  values is provided by p0-Set. If p0-Set is not provided to the UE, ,

-	If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If the UE is not provided PUCCH-SpatialRelationInfo, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to the minimum p0-PUCCH-Id value in p0-Set

-	 is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell  and  is a SCS configuration defined in [4, TS 38.211]

-	is a downlink pathloss estimate in dB calculated by the UE using RS resource index  as described in Clause 7.1.1 for the active DL BWP  of carrier  of the primary cell  as described in Clause 12

-	If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB

-	If the UE is provided a number of RS resource indexes, the UE calculates  using RS resource with index , where .  is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS

-	If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referenceSignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referenceSignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index  on the active DL BWP of the serving cell

-	If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS where the RS resource is either on the primary cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE

-	is not provided pathlossReferenceRSs, and

-	is not provided PUCCH-SpatialRelationInfo, and

-	is provided enableDefaultBeamPL-ForPUCCH, and

-	is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

the UE determines a RS resource index qd providing a periodic RS resource configured with with qcl-Type set to ‘typeD’ in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots.

-	The parameter  is a value of deltaF-PUCCH-f0 for PUCCH format 0, deltaF-PUCCH-f1 for PUCCH format 1, deltaF-PUCCH-f2 for PUCCH format 2, deltaF-PUCCH-f3 for PUCCH format 3, and deltaF-PUCCH-f4 for PUCCH format 4, if provided; otherwise ΔF_PUCCHF=0.

-	 is a PUCCH transmission power adjustment component on active UL BWP  of carrier  of primary cell

-	For a PUCCH transmission using PUCCH format 0 or PUCCH format 1,  where

-	 is a number of PUCCH format 0 symbols or PUCCH format 1 symbols for the PUCCH transmission as described in Clause 9.2.

-	 for PUCCH format 0

-	 for PUCCH format 1

-	 for PUCCH format 0

-	 for PUCCH format 1, where  is a number of UCI bits in PUCCH transmission occasion

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits smaller than or equal to 11, , where

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Clause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Clause 9.2.5.2

-	 is a number of resource elements determined as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Clause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, , where

-

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Clause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Clause 9.2.5.2

-	 is a number of CRC bits that the UE determines as described in Clause 9.2

-	 is a number of resource elements that the UE determines as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Clause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell .

-	For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value included in a DCI format scheduling a PDSCH reception for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion , or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 36.212], as described in Clause 11.3

-	 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

-	If the UE obtains a TPC command value from a DCI format scheduling a PDSCH reception and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the  value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of  through the link to a corresponding p0-PUCCH-Id index

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the current PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion , where

-	The  values are given in Table 7.1.2-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUCCH transmission occasion  and  symbols before PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell  for PUCCH power control adjustment state, where  is the smallest integer for which  symbols before PUCCH transmission occasion  is earlier than  symbols before PUCCH transmission occasion

-	If the PUCCH transmission is in response to a detection by the UE of a DCI format,  is a number of symbols for active UL BWP  of carrier  of primary cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission

-	If the PUCCH transmission is not in response to a detection by the UE of a DCI format,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of primary cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

-	Else,

-	, where , and  is

-	the TPC command value indicated in a random access response grant corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant corresponding to MsgA transmissions according to Type-2 random access procedure with RAR message(s) for fallbackRAR, or

-	the TPC command value indicated in a successRAR corresponding to MsgA transmissions for Type-2 random access procedure, or

-	the TPC command value in a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that the UE detects in a first PDCCH reception in a search space set provided by recoverySearchSpaceId if the PUCCH transmission is a first PUCCH transmission after 28 symbols from a last symbol of the first PDCCH reception,

and, if the UE transmits PUCCH on active UL BWP  of carrier  of primary cell ,

;

otherwise,

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of primary cell , and  corresponds to PUCCH format 0 or PUCCH format 1

Table 7.2.1-1: Mapping of TPC Command Field in a DCI format to accumulated  values

| TPC Command Field | Accumulated [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 0 |
| 2 | 1 |
| 3 | 3 |

## 7.3	Sounding reference signals

For SRS, a UE splits a linear value  of the transmit power  on active UL BWP  of carrier  of serving cell  equally across the configured antenna ports for SRS.

### 7.3.1	UE behaviour

If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP  of carrier  of serving cell  using SRS power control adjustment state with index , the UE determines the SRS transmission power  in SRS transmission occasion  as

[dBm]

where,

-	 is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS38.101-2] and [TS 38.101-3] for carrier  of serving cell  in SRS transmission occasion

-	 is provided by p0 for active UL BWP  of carrier  of serving cell  and SRS resource set  provided by SRS-ResourceSet and SRS-ResourceSetId

-	 is a SRS bandwidth expressed in number of resource blocks for SRS transmission occasion  on active UL BWP  of carrier  of serving cell  and  is a SCS configuration defined in [4, TS 38.211]

-	 is provided by alpha for active UL BWP  of carrier  of serving cell  and SRS resource set

-	 is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in Clause 7.1.1 for the active DL BWP of serving cell c and SRS resource set qs [6, TS 38.214]. The RS resource index qd is provided by pathlossReferenceRS associated with the SRS resource set qs and is either an ssb-Index providing a SS/PBCH block index or a csi-RS-Index providing a CSI-RS resource index If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a MAC CE [11, TS 38.321] can provide by SRS-PathlossReferenceRS-Id a corresponding RS resource index qd for aperiodic or semi-persistent SRS resource set qs

-	If the UE is not provided pathlossReferenceRS or SRS-PathlossReferenceRS-Id, or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB

-	If the UE is provided pathlossReferenceLinking, the RS resource is on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE

-	is not provided pathlossReferenceRS or SRS-PathlossReferenceRS-Id,

-	is not provided spatialRelationInfo, and

-	is provided enableDefaultBeamPL-ForSRS, and

-	is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

the UE determines a RS resource index  providing a periodic RS resource configured with qcl-Type set to ‘typeD’ in

-	the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP, if CORESETs are provided in the active DL BWP of serving cell c

-	the active PDSCH TCI state with lowest ID [6, TS 38.214] in the active DL BWP, if CORESETs are not provided in the active DL BWP of serving cell c

-	For the SRS power control adjustment state for active UL BWP  of carrier  of serving cell  and SRS transmission occasion

-	, where  is the current PUSCH power control adjustment state as described in Clause 7.1.1, if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions; or

-	 if the UE is not configured for PUSCH transmissions on active UL BWP  of carrier  of serving cell , or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and if tpc-Accumulation is not provided, where

-	The  values are given in Table 7.1.1-1

-	 is jointly coded with other TPC commands in a PDCCH with DCI format 2_3, as described in Clause 11.4

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before SRS transmission occasion  and  symbols before SRS transmission occasion  on active UL BWP  of carrier  of serving cell  for SRS power control adjustment state, where  is the smallest integer for which  symbols before SRS transmission occasion  is earlier than  symbols before SRS transmission occasion

-	if the SRS transmission is aperiodic,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH triggering the SRS transmission and before a first symbol of the SRS transmission

-	if the SRS transmission is semi-persistent or periodic,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of serving cell  at SRS transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of serving cell  at SRS transmission occasion  and , then

-	If a configuration for a  value or for a  value for a corresponding SRS power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers

-

-	Else

-

where

is the TPC command value indicated in the random access response grant corresponding to the random access preamble that the UE transmitted on active UL BWP  of carrier  of the serving cell , and

;

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of serving cell .

-	 if the UE is not configured for PUSCH transmissions on active UL BWP  of carrier  of serving cell , or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and tpc-Accumulation is provided, and the UE detects a DCI format 2_3  symbols before a first symbol of SRS transmission occasion , where absolute values of  are provided in Table 7.1.1-1

-	if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions, the update of the power control adjustment state for SRS transmission occasion  occurs at the beginning of each SRS resource in the SRS resource set ; otherwise, the update of the power control adjustment state SRS transmission occasion  occurs at the beginning of the first transmitted SRS resource in the SRS resource set .

If a UE transmits SRS based on a configuration by SRS-PosResourceSet on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as

[dBm]

where,

-	PO_SRS,b,f,cqs and αSRS,b,f,cqs are provided by p0-r16 and alpha-r16 respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-PosResourceSetId from SRS-PosResourceSet, and

-	PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in Clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos

-	if a ssb-IndexNcell is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16

-	if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower

If the UE determines that the UE is not able to accurately measure PLb,f,cqd, or the UE is not provided with pathlossReferenceRS-Pos, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB

The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource.

## 7.4	Physical random access channel

A UE determines a transmission power for a physical random access channel (PRACH), , on active UL BWP  of carrier  of serving cell  based on DL RS for serving cell  in transmission occasion  as

[dBm],

where  is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3] for carrier  of serving cell  within transmission occasion ,  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers [11, TS 38.321] for the active UL BWP  of carrier  of serving cell , and  is a pathloss for the active UL BWP  of carrier  based on the DL RS associated with the PRACH transmission on the active DL BWP of serving cell  and calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, where RSRP is defined in [7, TS 38.215] and the higher layer filter configuration is defined in [12, TS 38.331]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in Clause 13, the UE determines  based on the SS/PBCH block associated with the PRACH transmission.

If a PRACH transmission from a UE is not in response to a detection of a PDCCH order by the UE, or is in response to a detection of a PDCCH order by the UE that triggers a contention based random access procedure, or is associated with a link recovery procedure where a corresponding index  is associated with a SS/PBCH block, as described in Clause 6, referenceSignalPower is provided by ss-PBCH-BlockPower.

If a PRACH transmission from a UE is in response to a detection of a PDCCH order by the UE that triggers a contention-free random access procedure and depending on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in Clause 10.1, referenceSignalPower is provided by ss-PBCH-BlockPower or, if the UE is configured resources for a periodic CSI-RS reception or the PRACH transmission is associated with a link recovery procedure where a corresponding index  is associated with a periodic CSI-RS configuration as described in Clause 6, referenceSignalPower is obtained by ss-PBCH-BlockPower and powerControlOffsetSS where powerControlOffsetSS provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB. If the active TCI state for the PDCCH that provides the PDCCH order includes two RS, the UE expects that one RS is configured with qcl-Type set to ‘typeD’ and the UE uses the one RS when applying a value provided by powerControlOffsetSS.

If within a random access response window, as described in Clause 8.2, the UE does not receive a random access response that contains a preamble identifier corresponding to the preamble sequence transmitted by the UE, the UE determines a transmission power for a subsequent PRACH transmission, if any, as described in [11, TS 38.321].

If prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher layers to suspend the power ramping counter as described in [11, TS 38.321].

If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in Clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, or due to slot format determination as described in Clause 11.1, or due to the PUSCH/PUCCH/PRACH/SRS transmission occasions are in the same slot or the gap between a PRACH transmission and PUSCH/PUCCH/SRS transmission is small as described in Clause 8.1, the UE does not transmit a PRACH in a transmission occasion, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in Clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, the UE transmits a PRACH with reduced power in a transmission occasion, Layer 1 may notify higher layers to suspend the corresponding power ramping counter.

## 7.5	Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion  would exceed , where  is the linear value of  in transmission occasion  as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to  for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot.

-	PRACH transmission on the PCell

-	PUCCH or PUSCH transmissions with higher priority index according to Clause 9

-	For PUCCH or PUSCH transmissions with same priority index

-	PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information

-	PUCCH transmission with CSI or PUSCH transmission with CSI

-	PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell

-	SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell

In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell. In case of same priority order and for operation with two UL carriers, the UE prioritizes power allocation for transmissions on the carrier where the UE is configured to transmit PUCCH. If PUCCH is not configured for any of the two UL carriers, the UE prioritizes power allocation for transmissions on the non-supplementary UL carrier.

## 7.6	Dual connectivity

### 7.6.1	EN-DC

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by p-MaxEUTRA and a maximum power  for transmissions in FR1 on the SCG by p-NR-FR1.

The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power for the SCG in FR1 as described Clauses 7.1 through 7.5 using  as the maximum transmission power. The UE determines transmission power for the SCG in FR2 as described Clauses 7.1 through 7.5.

A UE does not expect to be configured for operation with shortened TTI and/or processing time [13, TS 36.213] on a cell that is included in an EN-DC configuration.

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows.

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability uplinkTxSwitching-OptionSupport = dualUL in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and

-	if UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG in FR1, and

-	if  in any portion of slot  of the SCG,

the UE reduces transmission power in any portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG in FR1, respectively. The UE is not required to transmit in any portion of slot  of the SCG if  would need to be reduced by more than the value provided by XSCALE in order for  in any portion of slot  of the SCG. The UE is required to transmit in slot  of the SCG if  would not need to be reduced by more than the value provided by XSCALE in order for  in all portions of slot .

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE expects to be configured with reference TDD configuration for E-UTRA by tdm-PatternConfig-r15 in [13, TS 36.213].

### 7.6.1A	NE-DC

If a UE is configured with an MCG using NR radio access and with a SCG using E-UTRA radio access, the UE is configured a maximum power  for transmissions in FR1 on the MCG by p-NR-FR1 and a maximum power  for transmissions on the SCG by p-MaxEUTRA.

The UE determines transmission power for the MCG in FR1 as described Clauses 7.1 through 7.5 using  as the maximum transmission power for . The UE determines transmission power for the MCG in FR2 as described Clauses 7.1 through 7.5.

If the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG, the UE determines a transmission power for the SCG as described in [13, TS 36.213] using  as the maximum transmission power.

If at least one symbol of slot  of the MCG that is indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated overlaps with subframe  of the SCG

-	for subframe , the UE determines a transmission power for the SCG as described in [13, TS 36.213] using  as the maximum transmission power

otherwise

-	the UE determines a transmission power for the SCG as described in [13, TS 36.213] without considering  as the maximum transmission power

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE does not expect to transmit in a slot on the MCG in FR1 when a corresponding subframe on the SCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for NE-DC and

-	if the UE transmission(s) in slot  of the MCG in FR1 overlap in time with UE transmission(s) in subframe  of the SCG, and

-	if  in any portion of slot  of the MCG,

the UE reduces transmission power in any portion of slot  of the MCG so that  in all portions of slot , where  and  are the linear values of the total UE transmission powers in slot  of the MCG in FR1 and in subframe  of the SCG, respectively.

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]).

### 7.6.2	NR-DC

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, or semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3].

If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively.

If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 only for synchronous NR-DC operation [10, TS 38.133].

If a UE

-	is provided dynamic for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, or are configured by higher layers, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

where

-	Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax},

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates the value of 'long' for the capability,

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates the value of 'short' for the capability, and

-	PMCGactual is the total power for the transmissions on the MCG that overlap with the transmission occasion on the SCG where PMCGactual is determined based on transmissions configured by higher layers and on transmissions scheduled by DCI formats in PDCCH receptions with a last symbol that is at least Toffset before the first symbol of the transmission occasion on the SCG.

The UE does not expect to have PUSCH, PUCCH, or SRS transmissions on the MCG that

-	are scheduled/triggered by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

The UE does not expect to receive a positive TPC command value in a DCI format 2_2 or a DCI format 2_3 in a PDCCH reception with a last symbol that is less than Toffset before the first symbol of the transmission occasion on the SCG, if the transmission on the MCG overlaps with the transmission occasion on the SCG.

The UE is not required to apply a TPC command the UE receives in a DCI format 2_2 or a DCI format 2_3 in a PDCCH reception with a last symbol that is less than Toffset before the first symbol of the transmission occasion on the SCG, if the transmission on the MCG overlaps with the transmission occasion on the SCG.

## 7.7	Power headroom report

The types of UE power headroom reports are the following. A Type 1 UE power headroom  that is valid for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell . A Type 3 UE power headroom that is valid for SRS transmission occasion  on active UL BWP  of carrier  of serving cell .

A UE determines whether a power headroom report for an activated serving cell [11, TS 38.321] is based on an actual transmission or a reference format based on the higher layer signalling of configured grant and periodic/semi-persistent sounding reference signal transmissions and downlink control information the UE received until and including the PDCCH monitoring occasion where the UE detects the first DCI format scheduling an initial transmission of a transport block since a power headroom report was triggered if the power headroom report is reported on a PUSCH triggered by the first DCI format. Otherwise, a UE determines whether a power headroom report is based on an actual transmission or a reference format based on the higher layer signalling of configured grant and periodic/semi-persistent sounding reference signal transmissions and downlink control information the UE received until the first uplink symbol of a configured PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 = 1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the power headroom report is reported on the PUSCH using the configured grant.

If a UE

-	is configured with two UL carriers for a serving cell, and

-	determines a Type 1 power headroom report and a Type 3 power headroom report for the serving cell

the UE

-	provides the Type 1 power headroom report if both the Type 1 and Type 3 power headroom reports are based on respective actual transmissions or on respective reference transmissions

-	provides the power headroom report that is based on a respective actual transmission if either the Type 1 report or the Type 3 report is based on a respective reference transmission

If a UE is configured with a SCG and if phr-ModeOtherCG for a CG indicates 'virtual' then, for power headroom reports transmitted on the CG, the UE computes PH assuming that the UE does not transmit PUSCH/PUCCH on any serving cell of the other CG. For NR-DC when both the MCG and the SCG operate either in FR1 or in FR2 and for a power headroom report transmitted on the MCG or the SCG, the UE computes PH assuming that the UE does not transmit PUSCH/PUCCH on any serving cell of the SCG or the MCG, respectively.

If the UE is configured with a SCG,

-	For computing power headroom for cells belonging to MCG, the term 'serving cell' in this clause refers to serving cell belonging to the MCG.

-	For computing power headroom for cells belonging to SCG, the term 'serving cell' in this clause refers to serving cell belonging to the SCG. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell,

-	For computing power headroom for cells belonging to primary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the primary PUCCH group.

-	For computing power headroom for cells belonging to secondary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the secondary PUCCH group. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

For a UE configured with EN-DC/NE-DC and capable of dynamic power sharing, if E-UTRA Dual Connectivity PHR [14, TS 36.321] is triggered and,

-	if the duration of NR slot on active UL BWP is different from that of E-UTRA subframe carrying the Dual Connectivity PHR, the UE provides power headroom of the first NR slot that fully overlaps with the E-UTRA subframe;

-	if the duration of NR slot on active UL BWP is the same as that of E-UTRA subframe carrying the Dual Connectivity PHR for asynchronous EN-DC/NE-DC [10, TS 38.133], the UE provides power headroom of the first NR slot that overlaps with the E-UTRA subframe.

### 7.7.1	Type 1 PH report

If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell , the UE computes the Type 1 power headroom report as

[dB]

where , , , , ,  and  are defined in Clause 7.1.1.

If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration  on active UL BWP  of carrier  of serving cell  is smaller than a SCS configuration  on active UL BWP  of carrier  of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP  that overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP  that fully overlaps with the slot on active UL BWP . If a UE is configured with multiple cells for PUSCH transmissions, where a same SCS configuration on active UL BWP  of carrier  of serving cell  and active UL BWP  of carrier  of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP  , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the slot on active UL BWP  that overlaps with the slot on active UL BWP .

If a UE is configured with multiple cells for PUSCH transmissions and provides a Type 1 power headroom report in a PUSCH transmission with PUSCH repetition Type B having a nominal repetition that spans multiple slots on active UL BWP  and overlaps with one or more slots on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the one or more slots on active UL BWP  that overlaps with the multiple slots of the nominal repetition on active UL BWP .

For a UE configured with EN-DC/NE-DC and capable of dynamic power sharing, if E-UTRA Dual Connectivity PHR [14, TS 36.321] is triggered, the UE provides power headroom of the first PUSCH, if any, on the determined NR slot as described in clause 7.7.

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP  of carrier  of serving cell , a second PUSCH transmission on active UL BWP  of carrier  of serving cell  that overlaps with the first PUSCH transmission if

-	the second PUSCH transmission is scheduled by a DCI format in a PDCCH received in a second PDCCH monitoring occasion, and

-	the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format scheduling an initial transmission of a transport block after a power headroom report was triggered

or

-	the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 =1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the first PUSCH transmission is on a configured grant after a power headroom report was triggered.

If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell , the UE computes the Type 1 power headroom report as

[dB]

where  is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in Clause 7.1.1 where  and  are obtained using  and p0-PUSCH-AlphaSetId = 0,  is obtained using pusch-PathlossReferenceRS-Id = 0, and .

If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 1 power headroom report for the serving cell based on a reference PUSCH transmission, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pusch-Config. If the UE is provided pusch-Config for both UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the non-supplementary UL carrier.

### 7.7.2	Type 2 PH report

This clause is reserved.

### 7.7.3	Type 3 PH report

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion  on active UL BWP  of carrier  of serving cell c and if the UE is not configured for PUSCH transmissions on carrier  of serving cell c and the resource for the SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as

[dB]

where , , , ,  and  are defined in Clause 7.3.1 with corresponding values provided by SRS-ResourceSet.

If the UE determines that a Type 3 power headroom report for an activated serving cell is based on a reference SRS transmission then, for SRS transmission occasion  on UL BWP  of carrier  of serving cell c, and if the UE is not configured for PUSCH transmissions on UL BWP  of carrier  of serving cell c and a resource for the reference SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as

[dB]

where  is an SRS resource set corresponding to SRS-ResourceSetId = 0 for UL BWP  and , ,  and  are defined in Clause 7.3.1 with corresponding values obtained from SRS-ResourceSetId = 0 for UL BWP .  is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB and TC =0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS 38.101-3].

If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 3 power headroom report for the serving cell based on a reference SRS transmission and a resource for the reference SRS is provided by SRS-Resource, the UE computes a Type 3 power headroom report for the serving cell assuming a reference SRS transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 3 power headroom report for the serving cell assuming a reference SRS transmission on the non-supplementary UL carrier.

# 8	Random access procedure

Prior to initiation of the physical random access procedure, Layer 1 receives from higher layers a set of SS/PBCH block indexes and provides to higher layers a corresponding set of RSRP measurements.

Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a Type-1 random access procedure, as described in Clauses 8.1 through 8.4, or a Type-2 random access procedure as described in Clauses 8.1 through 8.2A.

Prior to initiation of the physical random access procedure, Layer 1 receives the following information from the higher layers:

-	Configuration of physical random access channel (PRACH) transmission parameters (PRACH preamble format, time resources, and frequency resources for PRACH transmission).

-	Parameters for determining the root sequences and their cyclic shifts in the PRACH preamble sequence set (index to logical root sequence table, cyclic shift (), and set type (unrestricted, restricted set A, or restricted set B)).

From the physical layer perspective, the Type-1 L1 random access procedure includes the transmission of random access preamble (Msg1) in a PRACH, random access response (RAR) message with a PDCCH/PDSCH (Msg2), and when applicable, the transmission of a PUSCH scheduled by a RAR UL grant, and PDSCH for contention resolution.

From the physical layer perspective, the Type-2 L1 random access procedure includes the transmission of random access preamble in a PRACH and of a PUSCH (MsgA) and the reception of a RAR message with a PDCCH/PDSCH (MsgB), and when applicable, the transmission of a PUSCH scheduled by a fallback RAR UL grant, and PDSCH for contention resolution.

If a random access procedure is initiated by a PDCCH order to the UE, a PRACH transmission is with a same SCS as a PRACH transmission initiated by higher layers.

If a UE is configured with two UL carriers for a serving cell and the UE detects a PDCCH order, the UE uses the UL/SUL indicator field value from the detected PDCCH order to determine the UL carrier for the corresponding PRACH transmission.

## 8.1	Random access preamble

Physical random access procedure is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission [4, TS 38.211].

-	A preamble index, a preamble SCS, , a corresponding RA-RNTI, and a PRACH resource.

A PRACH is transmitted using the selected PRACH format with transmission power , as described in Clause 7.4, on the indicated PRACH resource.

For Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB-PerSharedRO. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within an SSB-RO mapping cycle for a UE provided with a PRACH mask index by msgA-SSB-SharedRO-MaskIndex according to [11, TS 38.321].

For Type-2 random access procedure with separate configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB when provided; otherwise, by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

For Type-1 random access procedure, or for Type-2 random access procedure with separate configuration of PRACH occasions from Type 1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and R contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index 0. If N≥1, R contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure, or by msgA-TotalNumberOfRA-Preambles for Type-2 random access procedure with separate configuration of PRACH occasions from a Type 1 random access procedure, and is an integer multiple of N.

For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and Q contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index R. If N≥1, Q contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN+R, where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure.

For link recovery, a UE is provided N SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-Occasion in BeamFailureRecoveryConfig. For a dedicated RACH configuration provided by RACH-ConfigDedicated, if cfra is provided, a UE is provided N SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-Occasion in occasions. If N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions. If N≥1, all consecutive N SS/PBCH block indexes are associated with one PRACH occasion.

SS/PBCH block indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211].

-	First, in increasing order of preamble indexes within a single PRACH occasion

-	Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Fourth, in increasing order of indexes for PRACH slots

An association period, starting from frame 0, for mapping SS/PBCH block indexes to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 such that  SS/PBCH block indexes are mapped at least once to the PRACH occasions within the association period, where a UE obtains  from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH block indexes to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to  SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PRACH transmissions.

For a PRACH transmission triggered by a PDCCH order, the PRACH mask index field [5, TS 38.212], if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order.

For a PRACH transmission triggered by higher layers, if ssb-ResourceList is provided, the PRACH mask index is indicated by ra-ssb-OccasionMaskIndex which indicates the PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected SS/PBCH block index.

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

For the indicated preamble index, the ordering of the PRACH occasions is

-	First, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Third, in increasing order of indexes for PRACH slots

For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if csirs-ResourceList is provided, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected CSI-RS index indicated by csi-RS. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period.

Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16} |
| 20 | {1, 2, 4, 8} |
| 40 | {1, 2, 4} |
| 80 | {1, 2} |
| 160 | {1} |

For paired spectrum or supplementary uplink band all PRACH occasions are valid.

For unpaired spectrum,

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in Clause 4.1

-	If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213]

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in Clause 4.1.

For preamble format B4 [4, TS 38.211], Ngap=0.

Table 8.1-2: Ngap values for different preamble SCS μ

| Preamble SCS | Ngap |
| --- | --- |
| 1.25 kHz or 5 kHz | 0 |
| 15 kHz or 30 kHz or 60 kHz or 120 kHz | 2 |

If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec, where

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission

-	∆BWPSwitching=0 if the active UL BWP does not change and ∆BWPSwitching is defined in [10, TS 38.133] otherwise

-	∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2

-	Tswitch is a switching gap duration as defined in [6, TS 38.214]

For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N2 assuming SCS configuration μ=0.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

## 8.1A	PUSCH for Type-2 random access procedure

For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. The PUSCH transmission is after the PRACH transmission by at least N symbols where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP.

A UE does not transmit a PUSCH in a PUSCH occasion if the PUSCH occasion associated with a DMRS resource is not mapped to a preamble of valid PRACH occasions or if the associated PRACH preamble is not transmitted as described in Clause 7.5 or Clause 11.1. A UE can transmit a PRACH preamble in a valid PRACH occasion if the PRACH preamble is not mapped to a valid PUSCH occasion.

A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration.

A UE determines time resources and frequency resources for PUSCH occasions in an active UL BWP from msgA-PUSCH-Config for the active UL BWP. If the active UL BWP is not the initial UL BWP and msgA-PUSCH-Config is not provided for the active UL BWP, the UE uses the msgA-PUSCH-Config provided for the initial UL BWP.

A UE determines a first interlace or first RB for a first PUSCH occasion in an active UL BWP respectively from interlaceIndexFirstPO-MsgA-PUSCH or from frequencyStartMsgA-PUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of interlaces or a number of RBs provided by nrofInterlacesPerMsgA-PO or by nrofPRBs-perMsgA-PO, respectively. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgA-PUSCH. A number Nf of PUSCH occasions in the frequency domain of an UL BWP is provided by nrofMsgA-PO-FDM.

If a UE does not have dedicated RRC configuration, or has an initial UL BWP as an active UL BWP, or is not provided startSymbolAndLengthMsgA-PO, msgA-PUSCH-timeDomainAllocation provides a SLIV and a PUSCH mapping type for a PUSCH transmission by indicating

-	one of the first maxNrofUL-Allocations values from PUSCH-TimeDomainResourceAllocationList, if PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon

-	one of the entries from table 6.1.2.1.1-2 or table 6.1.2.1.1-3 in [6, TS 38.214], if PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon

else, the UE is provided a SLIV by startSymbolAndLengthMsgA-PO, and a PUSCH mapping type by mappingTypeMsgA-PUSCH for a PUSCH transmission.

For mapping one or multiple preambles of a PRACH slot to a PUSCH occasion associated with a DMRS resource, a UE determines a first slot for a first PUSCH occasion in an active UL BWP from msgA-PUSCH-TimeDomainOffset that provides an offset, in number of slots in the active UL BWP, relative to the start of a PUSCH slot including the start of each PRACH slot. The UE does not expect to have a PRACH preamble transmission and a PUSCH transmission with a msgA in a PRACH slot or in a PUSCH slot, or to have overlapping msgA PUSCH occasions for a MsgA PUSCH configuration. The UE expects that a first PUSCH occasion in each slot has a same SLIV for a PUSCH transmission that is provided by startSymbolAndLengthMsgA-PO or msgA-PUSCH-timeDomainAllocation [6, TS 38.214].

Consecutive PUSCH occasions within each slot are separated by guardPeriodMsgA-PUSCH symbols and have same duration. A number Nt of time domain PUSCH occasions in each slot is provided by nrofMsgA-PO-perSlot and a number Ns of consecutive slots that include PUSCH occasions is provided by nrofSlotsMsgA-PUSCH.

A UE is provided a DMRS configuration for a PUSCH transmission in a PUSCH occasion in an active UL BWP by msgA-DMRS-Config.

A UE is provided an MCS for data information in a PUSCH transmission for a PUSCH occasion by msgA-MCS.

For a PUSCH transmission with frequency hopping in a slot, when indicated by msgA-intraSlotFrequencyHopping for the active UL BWP, the frequency offset for the second hop [6, TS 38.214] is determined as described in Clause 8.3, Table 8.3-1 using msgA-HoppingBits instead of NUL,hop. If guardPeriodMsgA-PUSCH is provided, a first symbol of the second hop is separated by guardPeriodMsgA-PUSCH symbols from the end of a last symbol of the first hop; otherwise, there is no time separation of the PUSCH transmission before and after frequency hopping. If a UE is provided with useInterlacePUCCH-PUSCH in BWP-UplinkCommon, the UE shall transmit PUSCH without frequency hopping. A PUSCH transmission uses a same spatial filter as an associated PRACH transmission.

A UE determines whether or not to apply transform precoding for a PUSCH transmission as described in [6, TS 38.214].

A PUSCH occasion for PUSCH transmission is defined by a frequency resource and a time resource, and is associated with a DMRS resource. The DMRS resources are provided by msgA-DMRS-Config.

Each consecutive number of Npreamble preamble indexes from valid PRACH occasions in a PRACH slot

-	first, in increasing order of preamble indexes within a single PRACH occasion

-	second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

are mapped to a valid PUSCH occasion and the associated DMRS resource

-	first, in increasing order of frequency resource indexes fid for frequency multiplexed PUSCH occasions

-	second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	third, in increasing order of time resource indexes tid for time multiplexed PUSCH occasions within a PUSCH slot

-	fourth, in increasing order of indexes for Ns PUSCH slots

where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by rach-ConfigCommonTwoStepRA, and TPUSCH is a total number of valid PUSCH occasions per PUSCH configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by msgA-DMRS-Config.

A PUSCH occasion is valid if it does not overlap in time and frequency with any PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2

-	if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	is within UL symbols, or

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 and, if ChannelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

## 8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in Clause 10.1. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and LSBs of a SFN field in the DCI format 1_0, if included and applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers parse the transport block for a random access preamble identity (RAPID) associated with the PRACH transmission. If the higher layers identify the RAPID in RAR message(s) of the transport block, the higher layers indicate an uplink grant to the physical layer. This is referred to as random access response (RAR) UL grant in the physical layer.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE is expected to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception, where  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming  corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For , the UE assumes  [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines  assuming SCS configuration .

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and LSBs of a SFN field in the DCI format 1_0, if included and applicable, are same as corresponding LSBs of the SFN where the UE transmitted the PRACH, and the UE receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for the SpCell [11, TS 38.321], the UE may assume that the PDCCH that includes the DCI format 1_0 and the PDCCH order have same DM-RS antenna port quasi co-location properties. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for a secondary cell, the UE may assume the DM-RS antenna port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set for receiving the PDCCH that includes the DCI format 1_0.

A RAR UL grant schedules a PUSCH transmission from the UE. The contents of the RAR UL grant, starting with the MSB and ending with the LSB, are given in Table 8.2-1.

If the value of the frequency hopping flag is 0, the UE transmits the PUSCH without frequency hopping; otherwise, the UE transmits the PUSCH with frequency hopping.

The UE determines the MCS of the PUSCH transmission from the first sixteen indexes of the applicable MCS index table for PUSCH as described in [6, TS 38.214].

The TPC command value  is used for setting the power of the PUSCH transmission, as described in Clause 7.1.1, and is interpreted according to Table 8.2-2.

The CSI request field is reserved.

The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213].

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-CPext | 0, for operation without shared spectrum channel access 2, for operation with shared spectrum channel access |

Table 8.2-2: TPC Command  for PUSCH

| TPC Command | Value (in dB) |
| --- | --- |
| 0 | -6 |
| 1 | -4 |
| 2 | -2 |
| 3 | 0 |
| 4 | 2 |
| 5 | 4 |
| 6 | 6 |
| 7 | 8 |

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and the LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

## 8.2A	Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow.

In response to a transmission of a PRACH, if the PRACH preamble is not mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆, where n is a slot of the PDSCH reception and ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214]

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI and receives a transport block within the window in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

## 8.3	PUSCH scheduled by RAR UL grant

An active UL BWP, as described in Clause 12 and in [4, TS 38.211], for a PUSCH transmission scheduled by a RAR UL grant is indicated by higher layers.

If useInterlacePUCCH-PUSCH is not provided by BWP-UplinkCommon and BWP-UplinkDedicated, for determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP

-	if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used

-	else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

The frequency domain resource allocation is by uplink resource allocation type 1 [6, TS 38.214]. For an initial UL BWP size of NBWPsize RBs, a UE processes the frequency domain resource assignment field as follows

-	if NBWPsize≤180, or for operation with shared spectrum channel access if NBWPsize≤90

-	truncate the frequency domain resource assignment field to its log2NBWPsize∙NBWPsize+12 least significant bits and interpret the truncated frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212]

-	else

-	insert log2NBWPsize∙NBWPsize+12-14 most significant bits, or for operation with shared spectrum channel access insert log2NBWPsize∙NBWPsize+12-12 most significant bits, with value set to '0' after the NUL,hop bits to the frequency domain resource assignment field, where NUL,hop=0  if the frequency hopping flag is set to '0' and NUL,hop is provided in Table 8.3-1 if the hopping flag bit is set to '1', and interpret the expanded frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212]

-	end if

If useInterlacePUCCH-PUSCH is provided by BWP-UplinkCommon or BWP-UplinkDedicated, the frequency domain resource allocation is by uplink resource allocation type 2 [6, TS 38.214]. A UE processes the frequency domain resource assignment field as follows

-	truncate the frequency domain resource assignment field to the X=6 LSBs if μ=0, or to the X=5 LSBs if μ=1

-	for interlace allocation of a PUSCH transmission, interpret the X MSBs of the truncated frequency domain resource assignment field for the active UL BWP as for the X MSBs of the frequency domain resource assignment field in DCI format 0_0 [6, TS 38.214]

-	for RB set allocation of a PUSCH transmission, the RB set of the active UL BWP is the RB set of the PRACH transmission associated with the RAR UL grant. The UE assumes that the RB set is defined as when the UE is not provided intraCellGuardBandsUL-List [6, TS 38.214].

A UE determines whether or not to apply transform precoding as described in [6, TS 38.214].

For a PUSCH transmission with frequency hopping scheduled by RAR UL grant or for a Msg3 PUSCH retransmission, the frequency offset for the second hop [6, TS 38.214] is given in Table 8.3-1.

Table 8.3-1: Frequency offset for second hop of PUSCH transmission with frequency hopping scheduled by RAR UL grant or of Msg3 PUSCH retransmission

| Number of PRBs in initial UL BWP | Value of NUL,hop Hopping Bits | Frequency offset for 2nd hop |
| --- | --- | --- |
| NBWPsize<50 | 0 | NBWPsize2 |
|  | 1 | NBWPsize4 |
| NBWPsize≥50 | 00 | NBWPsize2 |
|  | 01 | NBWPsize4 |
|  | 10 | -NBWPsize4 |
|  | 11 | Reserved |

A SCS for the PUSCH transmission is provided by subcarrierSpacing in BWP-UplinkCommon. A UE transmits PRACH and the PUSCH on a same uplink carrier of a same serving cell.

A UE transmits a transport block in a PUSCH scheduled by a RAR UL grant in a corresponding RAR message using redundancy version number 0. If a TC-RNTI is provided by higher layers, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by TC-RNTI. Otherwise, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by C-RNTI. Msg3 PUSCH retransmissions, if any, of the transport block, are scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message [11, TS 38.321]. The UE always transmits the PUSCH scheduled by a RAR UL grant without repetitions.

With reference to slots for a PUSCH transmission scheduled by a RAR UL grant, if a UE receives a PDSCH with a RAR message ending in slot n for a corresponding PRACH transmission from the UE, the UE transmits the PUSCH in slot n+k2+∆, where k2 and ∆ are provided in [6, TS 38.214].

The UE may assume a minimum time between the last symbol of a PDSCH reception conveying a RAR message with a RAR UL grant and the first symbol of a corresponding PUSCH transmission scheduled by the RAR UL grant is equal to NT,1+NT,2+0.5 msec, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] and, for determining the minimum time, the UE considers that N1 and N2 correspond to the smaller of the SCS configurations for the PDSCH and the PUSCH. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

## 8.4	PDSCH with UE contention resolution identity

In response to a PUSCH transmission scheduled by a RAR UL grant when a UE has not been provided a C-RNTI, the UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding TC-RNTI scheduling a PDSCH that includes a UE contention resolution identity [11, TS 38.321]. In response to the PDSCH reception with the UE contention resolution identity, the UE transmits HARQ-ACK information in a PUCCH. The PUCCH transmission is within a same active UL BWP as the PUSCH transmission. A minimum time between the last symbol of the PDSCH reception and the first symbol of the corresponding PUCCH transmission with the HARQ-ACK information is equal to  msec.  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For , the UE assumes  [6, TS 38.214].

When detecting a DCI format in response to a PUSCH transmission scheduled by a RAR UL grant, as described in [11, TS 38.321], or corresponding PUSCH retransmission scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message [11, TS 38.321], the UE may assume the PDCCH carrying the DCI format has the same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format.

# 9	UE procedure for reporting control information

If a UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG.

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If a UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group

-	When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the primary PUCCH group respectively.

-	When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

If a UE is provided pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-CodebookList-r16.

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = separate

the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE.

For NR-DC when both the MCG and the SCG operate either in FR1 or in FR2 and for a power headroom report transmitted on the MCG or the SCG, the UE computes PH assuming that the UE does not transmit PUSCH/PUCCH on any serving cell of the SCG or the MCG, respectively.

If a UE is configured for NR-DC operation, the UE does not expect to be configured with a PUCCH-SCell.

A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from phy-PriorityIndex, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in Clause 9.2.4. For a PUSCH transmission with semi-persistent CSI report, a UE determines a priority index from a priority indicator field, if provided, in a DCI format that activates the semi-persistent CSI report. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0.

If a UE is provided two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the second PUCCH-Config

If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. Then,

-	if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission

-	if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission

where

-	the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH

-	Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs

-	if the overlapping group includes the first PUCCH

-	if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH and for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

-	if the overlapping group includes the first PUSCH

-	if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

If a UE would transmit the following channels that would overlap in time

-	a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or

-	a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or

-	a first PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH and a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI report(s) without a corresponding PDCCH, or

-	a PUSCH of larger priority index with SP-CSI reports(s) without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH, or

-	a configured grant PUSCH of larger priority index and a configured PUSCH of lower priority index on a same serving cell

the UE is expected to cancel the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index.

A UE does not expect to be scheduled to transmit a PUCCH or a PUSCH with smaller priority index that would overlap in time with a PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH. A UE does not expect to be scheduled to transmit a PUCCH of smaller priority index that would overlap in time with a PUSCH of larger priority index with SP-CSI report(s) without a corresponding PDCCH.

In the remaining of this Clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH.

In the remaining of this Clause, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH.

If a UE would transmit on a serving cell a PUSCH without UL-SCH that overlaps with a PUCCH transmission on a serving cell that includes positive SR information, the UE does not transmit the PUSCH.

If a UE would transmit CSI reports on overlapping physical channels, the UE applies the priority rules described in [6, TS 38.214] for the multiplexing of CSI reports.

If a UE has overlapping resources for PUCCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first follows the procedures described in Clause 9.2.6 for resolving the overlapping among the resources for the PUCCH transmissions.

If a UE

-	would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and

-	the PUSCH and PUCCH transmissions fulfill the conditions in Clause 9.2.5 for UCI multiplexing,

the UE

-	multiplexes only HARQ-ACK information, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH;

-	multiplexes only HARQ-ACK information and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH.

A UE does not expect to multiplex in a PUSCH transmission in one slot with SCS configuration μ1 UCI of same type that the UE would transmit in PUCCHs in different slots with SCS configuration μ2 if μ1<μ2.

A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs.

A UE does not expect a PUCCH resource that results from multiplexing overlapped PUCCH resources, if applicable, to overlap with more than one PUSCHs if each of the more than one PUSCHs includes aperiodic CSI reports.

A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in Clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

If a UE transmits multiple PUSCHs in a slot on respective serving cells that include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in Clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs.

If a UE transmits multiple PUSCHs in a slot on respective serving cells and the UE would multiplex UCI in one of the multiple PUSCHs and the UE does not multiplex aperiodic CSI in any of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in Clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in Clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

If a UE transmits a PUSCH over multiple slots and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in Clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent.

If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in Clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol. The UE does not expect that all actual repetitions that would overlap with the PUCCH transmission do not include more than one symbol.

If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information.

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.

## 9.1	HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE can be indicated by pdsch-HARQ-ACK-CodebookList to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks

-	a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1

-	the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-1, PDSCH-CodeBlockGroupTransmissionList} or {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-2, PDSCH-CodeBlockGroupTransmissionList}, respectively, for use with the first and second HARQ-ACK codebooks, respectively

If a UE receives a PDSCH without receiving a corresponding PDCCH, or if the UE receives a PDCCH indicating a SPS PDSCH release, the UE generates one corresponding HARQ-ACK information bit. If the UE generates two HARQ-ACK codebooks, the UE is indicated by harq-CodebookID, per SPS PDSCH configuration, a HARQ-ACK codebook index for multiplexing the corresponding HARQ-ACK information bit.

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback and the UE detects a DCI format in any PDCCH monitoring occasion that includes a One-shot HARQ-ACK request field with value 1

-	the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in Clause 9.1.4

-	the UE does not expect that the PDSCH-to-HARQ_feedback timing indicator field of the DCI format provides an inapplicable value from dl-DataToUL-ACK-r16

In the remaining of this Clause, reference is to one HARQ-ACK codebook and to DCI formats that schedule PDSCH reception, or indicate SPS PDSCH release, or indicate SCell dormancy without scheduling a PDSCH reception and are associated with the HARQ-ACK codebook.

If a UE is configured to receive SPS PDSCHs in a slot for SPS configurations that are indicated to be released by a DCI format, and if the UE receives the PDCCH providing the DCI format in the slot where the end of a last symbol of the PDCCH reception is not after the end of a last symbol of any of the SPS PDSCH receptions, and if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions would be multiplexed in a same PUCCH, the UE does not expect to receive the SPS PDSCHs, does not generate HARQ-ACK information for the SPS PDSCH receptions, and generates a HARQ-ACK information bit for the SPS PDSCH release.

If a UE detects a DCI format 1_1 indicating

-	SCell dormancy without scheduling a PDSCH reception, as described in Clause 10.3, and

-	is provided pdsch-HARQ-ACK-Codebook = dynamic or pdsch-HARQ-ACK-Codebook-r16

the UE generates a HARQ-ACK information bit as described in Clause 9.1.3 for a DCI format 1_1 indicating SCell dormancy and the HARQ-ACK information bit value is ACK.

If a UE is not provided PDSCH-CodeBlockGroupTransmission, the UE generates one HARQ-ACK information bit per transport block.

For a HARQ-ACK information bit, a UE generates a positive acknowledgement (ACK) if the UE detects a DCI format that provides a SPS PDSCH release or correctly decodes a transport block, and generates a negative acknowledgement (NACK) if the UE does not correctly decode the transport block. A HARQ-ACK information bit value of 0 represents a NACK while a HARQ-ACK information bit value of 1 represents an ACK.

In the following, the CRC for a DCI format is scrambled with a C-RNTI, an MCS-C-RNTI, or a CS-RNTI.

### 9.1.1	CBG-based HARQ-ACK codebook determination

If a UE is provided PDSCH-CodeBlockGroupTransmission for a serving cell, the UE receives a PDSCH scheduled by DCI format 1_1, that includes code block groups (CBGs) of a transport block. The UE is also provided maxCodeBlockGroupsPerTransportBlock indicating a maximum number  of CBGs for generating respective HARQ-ACK information bits for a transport block reception for the serving cell.

For a number of  code blocks (CBs) in a transport block, the UE determines a number of CBGs  according to Clause 5.1.7.1 of [6, TS 38.214] and determines a number of HARQ-ACK bits for the transport block as .

The UE generates an ACK for the HARQ-ACK information bit of a CBG if the UE correctly received all code blocks of the CBG and generates a NACK for the HARQ-ACK information bit of a CBG if the UE incorrectly received at least one code block of the CBG. If the UE receives two transport blocks, the UE concatenates the HARQ-ACK information bits for CBGs of the second transport block after the HARQ-ACK information bits for CBGs of the first transport block.

The HARQ-ACK codebook includes the  HARQ-ACK information bits and, if  for a transport block, the UE generates a NACK value for the last  HARQ-ACK information bits for the transport block in the HARQ-ACK codebook.

If the UE generates a HARQ-ACK codebook in response to a retransmission of a transport block, corresponding to a same HARQ process as a previous transmission of the transport block, the UE generates an ACK for each CBG that the UE correctly decoded in a previous transmission of the transport block.

If a UE correctly detects each of the  CBGs and does not correctly detect the transport block for the  CBGs, the UE generates a NACK value for each of the  CBGs.

### 9.1.2	Type-1 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format.

If a UE is not provided pdsch-HARQ-ACK-OneShotFeedback, the UE does not expect to receive a PDSCH scheduled by a DCI format that the UE detects in any PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16.

If the UE is provided pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config; otherwise  NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception

-	from slot n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat is provided by pdsch-AggregationFactor or pdsch-AggregationFactor-r16 [6, TS 38.214], or

-	from slot n-repetitionNumber-r16+1 to slot n, if the time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber, or

-	in slot n, otherwise

only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or

-	a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or

-	SPS PDSCH reception(s)

within the  occasions for candidate PDSCH receptions as determined in Clause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for one SPS PDSCH reception according to corresponding  occasion(s) on respective serving cell(s), where the value of counter DAI in DCI format 1_0 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to more than one SPS PDSCH receptions that the UE is configured to receive are ordered according to the following pseudo-code; otherwise, the procedures in Clause 9.1.2.1 and Clause 9.1.2.2 for a HARQ-ACK codebook determination apply.

Set NcellsDL to the number of serving cells configured to the UE

Set NcSPS to the number of SPS PDSCH configuration configured to the UE for serving cell c

Set NcDL to the number of DL slots for SPS PDSCH reception on serving cell c with HARQ-ACK information multiplexed on the PUCCH

Set j=0 – HARQ-ACK information bit index

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while c<NcellsDL

Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations

while s<NcSPS

Set nD=0 – slot index

while nD<NcDL

if {

a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in sps-Config or, if pdsch-AggregationFactor-r16 is not included in sps-Config, by pdsch-AggregationFactor in pdsch-config, and

HARQ-ACK information for the SPS PDSCH is associated with the PUCCH

}

ojACK = HARQ-ACK information bit for this SPS PDSCH reception

j=j+1;

end if

nD=nD+1;

end while

s=s+1;

end while

c=c+1;

end while

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK

c)	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-ForDCIFormat1_2

d)	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2

b)	on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c

a)	if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for each PDCCH monitoring occasion in a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12  for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

c)	on the ratio 2μDL-μUL between the downlink SCS configuration μDL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively

d)	if provided, on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as described in Clause 11.1

e)	if ca-SlotOffset is provided, on Nslot,offset,cDL and μoffset,DL,c provided by ca-SlotOffset for serving cell c, or on Nslot,offsetUL  and μoffset,UL provided by ca-SlotOffset for the primary cell, as described in [4, TS 38.211].

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = joint

where

-	a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and

-	a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates a Type-1 HARQ-ACK codebook for the set  S0 and the set S1 of serving cells separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1 to obtain a total number of  HARQ-ACK information bits.

For the set of slot timing values , the UE determines a set of  occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases.

Set  - index of occasion for candidate PDSCH reception or SPS PDSCH release

Set

Set

Set  to the cardinality of set

Set k =0 – index of slot timing values , in descending order of the slot timing values, in set  for serving cell

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while

if

Set  – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot nU-K1,k⋅2μDL-μUL+nD-NPDSCHrepeat,max+1 to slot nU-K1,k⋅2μDL-μUL+nD, at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

else

while

if  modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0

Set  – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD

is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD-NPDSCHrepeat,max+1 to slot nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD, at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

end if

If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of , where , the UE does not expect to receive more than one PDSCH in a same DL slot.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format that does not support CBG-based PDSCH receptions and if

-	the UE is configured with one serving cell, and

-	, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE generates HARQ-ACK information only for the transport block in the PDSCH or only for the SPS PDSCH release.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 and if

-	the UE is configured with more than one serving cells, or

-	, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE repeats  times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release.

A UE does not expect to detect a DCI format switching a DL BWP within  symbols prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where  is defined in Clause 9.2.3.

If a UE is provided dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCIFormat1_2, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} and the set of slot timing values provided by K1 for the active DL BWP of a corresponding serving cell.

If an occasion for a candidate PDSCH reception can be in response to a PDCCH with DCI format 1_1 and if maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

A UE determines  HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set  defines a total number  of occasions for PDSCH reception or SPS PDSCH release for serving cell  corresponding to the HARQ-ACK information bits.

Set  – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cells including, when applicable, cells in the set S0 and the set S1

Set - HARQ-ACK information bit index

Set  to the number of serving cells configured by higher layers for the UE

while

Set  – index of occasion for candidate PDSCH reception or SPS PDSCH release

while

if harq-ACK-SpatialBundlingPUCCH is not provided, PDSCH-CodeBlockGroupTransmission is not provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= HARQ-ACK information bit corresponding to a first transport block of this cell;

;

= HARQ-ACK information bit corresponding to a second transport block of this cell;

;

elseif harq-ACK-SpatialBundlingPUCCH is provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of this cell - if the UE receives one transport block, the UE assumes ACK for the second transport block;

;

elseif PDSCH-CodeBlockGroupTransmission is provided, and  CBGs are indicated by maxCodeBlockGroupsPerTransportBlock for serving cell ,

Set - CBG index

while

= HARQ-ACK information bit corresponding to CBG  of the first transport block;

if the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell

= HARQ-ACK information bit corresponding to CBG  of the second transport block;

end if

;

end while

, where  is the value of maxNrofCodeWordsScheduledByDCI for the active DL BWP of serving cell ;

else

= HARQ-ACK information bit of serving cell ;

;

end if

;

end while

;

end while

If , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as  where

-	 is the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release in PDSCH reception occasion  for serving cell  and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	 is the number of CBGs the UE receives in a PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH.

#### 9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_1 or on the value of dl-DataToUL-ACK-ForDCI-Format1-2 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_2, in any of the Mc occasions for candidate PDSCH receptions by a DCI format or SPS PDSCH on any serving cell c, as described in Clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH reception, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission.

A UE does not expect to detect a DCI format switching a DL BWP within N2 symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where N2 is defined in [6, TS 38.214].

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1 when a value of the DAI field is VT-DAIUL=1 except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-DAIUL=0 unless the UE receives only a SPS PDSCH release, or only SPS PDSCH(s), or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2. VT-DAIUL=0 if the PUSCH is scheduled by a DCI format that includes a DAI field and the DAI field is set to '0'; otherwise, VT-DAIUL=1.

### 9.1.3	Type-2 HARQ-ACK codebook determination

This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16. Unless stated otherwise, a PDSCH-to-HARQ_feedback timing indicator field provides an applicable value.

A UE does not expect to multiplex in a Type-2 HARQ-ACK codebook HARQ-ACK information that is in response to a detection of a DCI format that does not include a counter DAI field.

If a UE receives a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16,

-	if the UE detects a second DCI format, the UE multiplexes the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in the second DCI format, where

-	if the UE is not provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one

-	if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in Clause 9.1.3.3

-	if the UE is provided pdsch-HARQ-ACK-OneShotFeedback, the first DCI format does not indicate SPS PDSCH release or SCell dormancy, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in Clause 9.1.4.

-	otherwise, the UE does not multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission.

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release on an active DL BWP of a serving cell c, as described in Clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PDSCH-to-HARQ_feedback timing indicator field values for PUCCH transmission with HARQ-ACK information in slot n in response to PDSCH receptions or SPS PDSCH release

-	slot offsets K0 [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions or SPS PDSCH release and by pdsch-AggregationFactor, or pdsch-AggregationFactor-r16, or repetitionNumber, when provided.

The set of PDCCH monitoring occasions for a DCI format scheduling PDSCH receptions or SPS PDSCH release is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells. PDCCH monitoring occasions are indexed in an ascending order of start time of the search space sets associated with a PDCCH monitoring occasion. The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with the DCI formats is present up to the current serving cell and current PDCCH monitoring occasion,

-	first, if the UE indicates by type2-HARQ-ACK-Codebook support for more than one PDSCH receptions on a serving cell that are scheduled from a same PDCCH monitoring occasion, in increasing order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair,

-	second in ascending order of serving cell index, and

-	third in ascending order of PDCCH monitoring occasion index m, where m0≤m<M.

If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the value of the counter DAI is in the order of the first CORESETs and then the second CORESETs for a same serving cell index and a same PDCCH monitoring occasion index.

The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI formats is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the total DAI value counts the {serving cell, PDCCH monitoring occasion}-pair(s) for both the first CORESETs and the second CORESETs.

Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by VC-DAI,c,mDL the value of the counter DAI in a DCI format scheduling PDSCH reception or SPS PDSCH release on serving cell c in PDCCH monitoring occasion m according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by VT-DAI,mDL the value of the total DAI in a DCI format in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion m. A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field.

If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK  , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code:

Set m=0 – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion

Set j=0

Set Vtemp=0

Set Vtemp2=0

Set Vs=∅

Set NcellsDL to the number of serving cells configured by higher layers for the UE

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNackFeedbackMode = JointFeedback, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs

-	if the UE indicates type2-HARQ-ACK-Codebook, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of type2-HARQ-ACK-Codebook

Set M to the number of PDCCH monitoring occasion(s)

while m<M

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while c<NcellsDL

if PDCCH monitoring occasion m is before an active DL BWP change on serving cell c or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion m

c=c+1;

else

if there is a PDSCH on serving cell c associated with PDCCH in PDCCH monitoring occasion m, or there is a PDCCH indicating SPS PDSCH release on serving cell c

if VC-DAI,c,mDL≤Vtemp

j=j+1

end if

Vtemp=VC-DAI,c,mDL

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL

else

Vtemp=VT-DAI,mDL

end if

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell,

o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell

o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell

Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1

elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell,

oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

else

oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

end if

end if

c=c+1

end if

end while

m=m+1

end while

if Vtemp2<Vtemp

j=j+1

end if

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell,

OACK=2⋅TD⋅j+Vtemp2-1mod TD+1

else

OACK=TD⋅j+Vtemp2-1mod TD+1

end if

oiACK=NACK for any i∈0,1,⋯,OACK-1\Vs

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits.

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in Clause 9.1.2 and appends it to the OACK HARQ-ACK information bits.

For a PDCCH monitoring occasion with DCI format scheduling PDSCH reception or SPS PDSCH release in the active DL BWP of a serving cell, when a UE receives a PDSCH with one transport block and the value of maxNrofCodeWordsScheduledByDCI is 2, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

If a UE is not provided PDSCH-CodeBlockGroupTransmission for each of the NcellsDL serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for SPS PDSCH release, and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell c that the UE detects within the M PDCCH monitoring occasions.

-	if NcellsDL>1

-	if the UE does not detect any DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell c, VDAI,mlastDL is the value of the counter DAI in a last DCI format the UE detects in the last PDCCH monitoring occasion

-	if the UE detects at least one DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell c, VDAI,mlastDL is the value of the total DAI in the at least one DCI format that includes a total DAI field

-	VDAI,mlastDL=0 if the UE does not detect any DCI format scheduling PDSCH reception or indicating SPS PDSCH release for any serving cell c in any of the M PDCCH monitoring occasions.

-	UDAI,c is the total number of a DCI format scheduling PDSCH reception or indicating SPS PDSCH release that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,c=0 if the UE does not detect any DCI format scheduling PDSCH reception or indicating SPS PDSCH release for serving cell c in any of the M PDCCH monitoring occasions.

-	NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1.

-	Nm,creceived is the number of transport blocks the UE receives in a PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI format that the UE detects and indicate SPS PDSCH release in PDCCH monitoring occasion m for serving cell c.

-	NSPS,c is the number of SPS PDSCH receptions by the UE on serving cell c for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the M PDCCH monitoring occasions.

If a UE

-	is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells; and

-	is not provided PDSCH-CodeBlockGroupTransmission, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,CBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH release,

-	SPS PDSCH reception,

-	DCI format 1_1 indicating SCell dormancy, and

-	for TB-based PDSCH receptions on the NcellsDL,CBG serving cells and on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,CBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,CBG serving cells for CBG-based PDSCH receptions, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,CBG serving cells, the UE generates NHARQ-ACK,maxCBG/TB,max HARQ-ACK information bits, where  is the maximum value of NTB,cDL⋅NHARQ-ACK,cCBG/TB,max across all NcellsDL,CBG serving cells and  is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c it is NTB,cDL⋅NHARQ-ACK,cCBG/TB,max<NHARQ-ACK,maxCBG/TB,max, the UE generates NACK for the last NHARQ-ACK,maxCBG/TB,max-NTB,cDL⋅NHARQ-ACK,cCBG/TB,max HARQ-ACK information bits for serving cell c

-	the pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook

If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in Clause 7.2.1, with

nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1,, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=1,  if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions

-	UDAI,cCBG is the total number of DCI formats scheduling CBG-based PDSCH receptions that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cCBG=0 if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH scheduled by a DCI format that supports CBG-based PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH

Table 9.1.3-1: Value of counter DAI for NC-DAIDL=2 and of total DAI

| DAI MSB, LSB | VC-DAIDL or VT-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0,0 | 1 | Y-1modTD+1=1 |
| 0,1 | 2 | Y-1modTD+1=2 |
| 1,0 | 3 | Y-1modTD+1=3 |
| 1,1 | 4 | Y-1modTD+1=4 |

Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1

| DAI | VC-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0 | 1 | Y-1modTD+1=1 |
| 1 | 2 | Y-1modTD+1=2 |

#### 9.1.3.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception, or in response to a detection of a DCI format 1_1 indicating SCell dormancy, to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, with the following modifications:

-	For the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, after the completion of the c and m loops, the UE sets Vtemp2=VDAIUL where VDAIUL is the value of the DAI field according to Table 9.1.3-2

-	For the case of first and second HARQ-ACK sub-codebooks, the DCI format includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook

-	harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission.

If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with first value VT-DAIUL=4 or with second value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy, on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission.

Table 9.1.3-2: Value of DAI

| DAI MSB, LSB | VT-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or DCI format 1_1 indicating SCell dormancy is present, denoted as X and X≥1 |
| --- | --- | --- |
| 0,0 | 1 | X-1mod4+1=1 |
| 0,1 | 2 | X-1mod4+1=2 |
| 1,0 | 3 | X-1mod4+1=3 |
| 1,1 | 4 | X-1mod4+1=4 |

#### 9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

If a UE is provided pdsch-HARQ-ACK-Codebook, the UE determines HARQ-ACK information for multiplexing in a PUCCH transmission occasion according to the following procedure.

Set g to the value of a PDSCH group index field in a last DCI format that provides a value of g and indicates a PUCCH transmission occasion.

Set i(g) to denote a PUCCH transmission occasion for multiplexing HARQ-ACK information

Set k to the value of a PDSCH-to-HARQ_feedback timing field, if any, in a DCI format providing a value of g

-	If the DCI format does not include a PDSCH-to-HARQ_feedback timing field, set k to the value provided by dl-DataToUL-ACK

Set h(g) to the value of a first New feedback indicator field in the last DCI format providing the value of g

Set hg+1mod2(g) to a value of a second New feedback indicator field in the last DCI format providing the value of g

Set VDAIg+1mod2 to the value of a total DAI field for group g+1mod2 in the last DCI format providing the value of g

-	If g=1 and the UE detects a DCI format that does not include a PDSCH group index field in a PDCCH reception that is after the PDCCH reception for the last DCI format detection providing the value of g and indicating a same slot for a PUCCH transmission occasion, set VDAIg+1mod2=∅

Set q to the value of a number of requested PDSCH group(s) field in the last DCI format providing the value of g

A UE does not expect to detect DCI formats with respective

-	Number of requested PDSCH group(s) field values of 0, and

-	PDSCH-to-HARQ_feedback timing field values that indicate a same PUCCH transmission occasion, and

-	PDSCH group index field values that are different

Generate first HARQ-ACK information for PUCCH transmission occasion i(g) in a slot, as described in Clause 9.1.3.1, where

-	the first HARQ-ACK information corresponds only to detections of DCI formats each providing a same value of g, of h(g), and to detections of DCI formats that do not provide a value of g and h(g) and are associated with a same value of g, of h(g), and at least one of the DCI formats providing a value of k indicating the slot

-	at least one of the DCI formats provides a h(g) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g or is associated with a value of g, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g)

The generation of the first HARQ-ACK information for PUCCH transmission occasion ig in a slot, as described in Clause 9.1.3.1, excludes the generation of HARQ-ACK information for SPS PDSCH receptions.

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where

-	the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2) and to detections of DCI formats that do not provide a value of g+1mod2, of h(g+1mod2), but are associated with a same value of g+1mod2, of h(g+1mod2)

-	at least one of the DCI formats provides a h(g+1mod2) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g+1mod2 or that is associated with a value of g+1mod2, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g+1mod2)

-	the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g)

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2. If the UE is provided PDSCH-CodeBlockGroupTransmission for  NcellsDL,CBG serving cells, set Vtemp2=VDAIg+1mod2 for both sub-codebooks before appending the second sub-codebook to the first sub-codebook.

If hg+1mod2g≠∅ and hg+1mod2g≠h(g+1mod2), generate second HARQ-ACK information as described in Clause 9.1.3.1 by setting M=0 and, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, setting Vtemp2=VDAIg+1mod2.

The generation of the second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, excludes the generation of HARQ-ACK information for SPS PDSCH receptions.

If q=0, the UE

includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

elseif q=1

if g = 1

appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

else

append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

end if

end if

The UE appends the HARQ-ACK information corresponding to SPS PDSCH receptions, if any, as described in Clause 9.1.3.1, after the first and second, if any, HARQ-ACK information.

If the HARQ-ACK information is multiplexed in a PUSCH transmission, the HARQ-ACK information is determined as

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DCI field with value VDAIUL

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format without ul-TotalDAI-Included configured except that for PDSCH group g, the DCI field with value VDAIUL in the DCI format is used after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, and when the HARQ-ACK information multiplexed in the PUCCH transmission occasion ig does not include HARQ-ACK information for PDSCH group g+1mod2

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format without ul-TotalDAI-Included configured except that for PDSCH group g=0, the DCI field with value VDAIUL in the DCI format is used after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, and when the HARQ-ACK information multiplexed in the PUCCH transmission occasion ig includes HARQ-ACK information for PDSCH groups g and g+1mod2

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format without ul-TotalDAI-Included configured except that for PDSCH group g=0, the DCI field with value  in the DCI format is used after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, and when the UE has not detected any DCI format scheduling PDSCH receptions, and the UE has not detected any DCI format with a request for HARQ-ACK information for any PDSCH group

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format with ul-TotalDAI-Included configured except that MSBs of the DCI field with value VDAIUL in the DCI format are used for PDSCH group g=0, and LSBs of DCI field with value VDAIUL in the DCI format are used for PDSCH group g=1, after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1.

If a UE is scheduled a PUSCH transmission by DCI format 0_1 having a DAI field value VDAIUL=4 for a PDSCH group index, and the UE has not detected any DCI format scheduling PDSCH receptions for the PDSCH group index, and the UE has not detected any DCI format with a request for HARQ-ACK information for the PDSCH group index, the UE does not multiplex HARQ-ACK information in the PUSCH transmission for the PDSCH group index.

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New feedback indicator field for g=0, and at least one of the DCI formats is DCI format 1_0, the UE generates HARQ-ACK information only for PDSCH receptions scheduled by detections of DCI format 1_0 and SPS PDSCH releases indicated by detections of DCI format 1_0, as described in Clause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion.

If a DCI format indicating a slot for a PUCCH transmission occasion does not include a New feedback indicator field, a PDSCH reception scheduled by the DCI format or a SPS PDSCH release indicated by the DCI format is associated with PDSCH group 0 and a value of h(g) associated with the DCI format is set only if h(g) is provided by another DCI format that provides a value of h(g) for PDSCH group 0 and indicates the slot for the PUCCH transmission occasion.

For PUCCH transmission occasion i(g), the UE determines a PUCCH or a PUSCH transmission to multiplex the HARQ-ACK information according to the procedures in Clauses 9.2.3 and 9.2.5.

If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK,g for group g and a number of HARQ-ACK information bits nHARQ-ACK,(g+1)mod2 for group g+1mod2 as described in Clause 9.1.3.1 where NSPS,c is included in nHARQ-ACK,g and, if q=1 and VDAIg+1mod2≠∅, the UE determines nHARQ-ACK,(g+1)mod2 by setting VDAI,mlastDL=VDAIg+1mod2. For obtaining a PUCCH transmission power, if q=0, nHARQ-ACK = nHARQ-ACK,  g; else, nHARQ-ACK = nHARQ-ACK,g+nHARQ-ACK,  (g+1)mod2.

### 9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines a Type-3 HARQ-ACK codebook according to the following procedure.

Set NcellsDL to the number of configured serving cells

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI is provided; else set NDIHARQ=1

Set c=0 – serving cell index

Set h=0 – HARQ process number

Set t=0 – TB index

Set g=0 – CBG index

Set j=0

while c<NcellsDL

while h<NHARQ,cDL

if NDIHARQ=0

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

g=0

j=j+1

t=t+1

end while

else

while t<NTB,cDL

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

end if

t=0

else

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c, and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

= NACK

j=j+1

t=t+1

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

if harq-ACK-SpatialBundlingPUCCH is not provided

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

else

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process h of serving cell c. If the UE receives one transport block, the UE assumes ACK for the second transport block

end if

j=j+1

t=t+1

end if

end while

end if

t=0

end if

h=h+1

end while

h=0

c=c+1

end while

If a UE receives a SPS PDSCH, or a PDSCH that is scheduled by a DCI format 1_0 or DCI format 1_2 for a serving cell c and if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c, and pdsch-HARQ-ACK-OneShotFeedbackCBG is provided, the UE repeats NHARQ-ACK,cCBG/TB,max times the HARQ-ACK information for the transport block in the PDSCH.

If a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, the UE determines a PUCCH or a PUSCH to multiplex a Type-3 HARQ-ACK codebook for transmission in a slot as described in Clauses 9.2.3 and 9.2.5. The UE multiplexes only the Type-3 HARQ-ACK codebook in the PUCCH or the PUSCH for transmission in the slot.

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in Clause 10.2 by replacing "SPS PDSCH release" with "DCI format".

If a UE multiplexes HARQ-ACK information in a PUSCH transmission, the UE generates the HARQ-ACK codebook as described in this Clause except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

## 9.2	UCI reporting in physical uplink control channel

UCI types reported in a PUCCH include HARQ-ACK information, SR, LRR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bits, if any, LRR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in Clause 9.1. For the remaining of this clause, any reference to SR is applicable for SR and/or for LRR.

A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ackNackFeedbackMode = separate, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2.

If a UE is provided ackNackFeedbackMode = separate, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot.

In Clauses 9.2.3, 9.2.5.1 and 9.2.5.2, a UE assumes 11 CRC bits if a number of respective UCI bits is larger than or equal to 360; otherwise, the UE determines a number of CRC bits based on the number of respective UCI bits as described in [5, TS 38.212].

### 9.2.1	PUCCH Resource Sets

If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs.

The PUCCH resource set includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset RBBWPoffset, and a cyclic shift index set for a PUCCH transmission.

The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping.

An orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1 in Table 9.2.1-1.

The UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3.

If a UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, the UE generates at most one HARQ-ACK information bit.

If the UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a DCI format scheduling a PDSCH reception or a SPS PDSCH release, the UE determines a PUCCH resource with index rPUCCH, 0≤rPUCCH≤15, as rPUCCH=2⋅nCCE,0NCCE+2⋅∆PRI, where NCCE is a number of CCEs in a CORESET of a PDCCH reception with the DCI format, as described in Clause 10.1, nCCE,0 is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format.

If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines the PRB index of the PUCCH transmission in the first hop as RBBWPoffset+rPUCCHNCS and the PRB index of the PUCCH transmission in the second hop as NBWPsize-1-RBBWPoffset-rPUCCHNCS, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS

If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines the PRB index of the PUCCH transmission in the first hop as NBWPsize-1-RBBWPoffset-rPUCCH-8NCS and the PRB index of the PUCCH transmission in the second hop as RBBWPoffset+rPUCCH-8NCS

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

If a UE is provided a PUCCH resource by pucch-ResourceCommon and is provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines for the PUCCH resource an interlace index m as m=m0+rPUCCHNCSmodM where M is a number of interlaces [4, TS 38.211] and m0=RBBWPoffset is an interlace index offset and RBBWPoffset is as given in Table 9.2.1-1

-	the UE determines an initial cyclic shift index in a set of initial cyclic shift indexes as rPUCCHmodNCS, where NCS is the total number of initial cyclic shifts indexes in the set of initial cyclic shift indexes in Table 9.2.1-1

-	if pucch-ResourceCommon indicates

-	index 0: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH≥10

-	index 1 or 2: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH=15

-	index 3, 7, or 11: an orthogonal cover code with index 1 is used for a PUCCH resource with PUCCH format 1 if rPUCCH≥10

-	the UE does not expect pucch-ResourceCommon to indicate index 15

Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

If a UE has dedicated PUCCH resource configuration, the UE is provided by higher layers with one or more PUCCH resources.

A PUCCH resource includes the following parameters:

-	a PUCCH resource index provided by pucch-ResourceId

-	an index of the first PRB prior to frequency hopping or for no frequency hopping by startingPRB, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of the first PRB after frequency hopping by secondHopPRB, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an indication for intra-slot frequency hopping by intraSlotFrequencyHopping, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of a first interlace by interlace0, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	if provided, an index of a second interlace by interlace1, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of an RB set by rb-SetIndex, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	a configuration for a PUCCH format provided by format

The UE expects that useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated are provided either in all UL BWPs or in none of the UL BWPs for a serving cell.

If a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, the UE determines available RBs for PUCCH transmissions within the active UL BWP as the intersection of RBs corresponding to an interlace index provided by interlace0 and, if provided, interlace1, and RBs of an RB set provided by rb-SetIndex. The intersection results in Minterlace,0PUCCH RBs in the first interlace and the UE expects that Minterlace,0PUCCH is either 10 or 11. If interlace1 is provided, the intersection results in Minterlace,1PUCCH RBs in the second interlace and the UE expects that Minterlace,1PUCCH is either 10 or 11.

If the format indicates PUCCH-format0, the PUCCH format configured for a PUCCH resource is PUCCH format 0, where the PUCCH resource also includes an index for an initial cyclic shift provided by initialCyclicShift, a number of symbols for a PUCCH transmission provided by nrofSymbols, a first symbol for the PUCCH transmission provided by startingSymbolIndex.

If the format indicates PUCCH-format1, the PUCCH format configured for a PUCCH resource is PUCCH format 1, where the PUCCH resource also includes an index for an initial cyclic shift provided by initialCyclicShift, a number of symbols for a PUCCH transmission provided by nrofSymbols, a first symbol for the PUCCH transmission provided by startingSymbolIndex, and an index for an orthogonal cover code by timeDomainOCC.

If the format indicates PUCCH-format2 or PUCCH-format3, the PUCCH format configured for a PUCCH resource is PUCCH format 2 or PUCCH format 3, respectively, where the PUCCH resource also includes a number of PRBs provided by nrofPRBs, a number of symbols for a PUCCH transmission provided by nrofSymbols, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. If a UE is provided by useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, and the format indicates PUCCH-format2 or PUCCH-format3 and PUCCH-ResourceExt is provided, the PUCCH resource also includes an index of a second interlace by interlace1, if provided; otherwise, if interlace1 is not provided, the PUCCH resource also includes, if provided, an orthogonal cover code length by occ-Length and an orthogonal cover code index by occ-Index. If the format indicates PUCCH-format3 and PUCCH-ResourceExt is provided, the UE assumes that the MRBPUCCH,3 [4, TS38.211] PRBs with the lowest indexes within the first, and if configured, second interlace are used for PUCCH transmission.

If the format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by nrofSymbols, an orthogonal cover code length by occ-Length, an orthogonal cover code index by occ-Index, and a first symbol for the PUCCH transmission provided by startingSymbolIndex.

If a UE is provided subslotLengthForPUCCH in a PUCCH-Config, the first symbol of a PUCCH resource in PUCCH-Config for multiplexing HARQ-ACK in a PUCCH transmission is relative to the first symbol of the subslotLengthForPUCCH symbols [12, TS 38.331]. For the remaining cases, the first symbol of a PUCCH resource is relative to the first symbol of a slot with Nsymslot symbols [4, TS 38.211].

A UE can be configured up to four sets of PUCCH resources. A PUCCH resource set is provided by PUCCH-ResourceSet and is associated with a PUCCH resource set index provided by pucch-ResourceSetId, with a set of PUCCH resource indexes provided by resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by maxPayloadSize. For the first PUCCH resource set, the maximum number of UCI information bits is 2. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other PUCCH resource sets is 8.

If the UE transmits OUCI UCI information bits, that include HARQ-ACK information bits, the UE determines a PUCCH resource set to be

-	a first set of PUCCH resources with pucch-ResourceSetId = 0 if OUCI≤2 including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or

-	a second set of PUCCH resources with pucch-ResourceSetId = 1, if provided by higher layers, if 2<OUCI≤N2 where N2 is equal to maxPayloadSize if maxPayloadSize is provided for the PUCCH resource set with pucch-ResourceSetId = 1; otherwise N2is equal to 1706, or

-	a third set of PUCCH resources with pucch-ResourceSetId = 2, if provided by higher layers, if N2<OUCI≤N3 where N3 is equal to maxPayloadSize if maxPayloadSize is provided for the PUCCH resource set with pucch-ResourceSetId = 2; otherwise N3is equal to 1706, or

-	a fourth set of PUCCH resources with pucch-ResourceSetId = 3, if provided by higher layers, if N3<OUCI≤1706.

If the UE is provided SPS-PUCCH-AN-List and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions, the UE determines a PUCCH resource to be

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the first entry in sps-PUCCH-AN-List if OUCI≤2, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the second entry in sps-PUCCH-AN-List, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize obtained from the second entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the third entry in sps-PUCCH-AN-List, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize obtained from the third entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the fourth entry in sps-PUCCH-AN-List, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706.

### 9.2.2	PUCCH Formats for UCI transmission

If a UE is not transmitting PUSCH, and the UE is transmitting UCI, the UE transmits UCI in a PUCCH using

-	PUCCH format 0 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of HARQ-ACK information bits with positive or negative SR (HARQ-ACK/SR bits) is 1 or 2

-	PUCCH format 1 if

-	the transmission is over 4 or more symbols,

-	the number of HARQ-ACK/SR bits is 1 or 2

-	PUCCH format 2 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of UCI bits is more than 2

-	PUCCH format 3 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2,

-	the PUCCH resource does not include an orthogonal cover code, or the UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	PUCCH format 4 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2,

-	the PUCCH resource includes an orthogonal cover code and the UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

A spatial setting for a PUCCH transmission is provided by PUCCH-SpatialRelationInfo if the UE is configured with a single value for pucch-SpatialRelationInfoId; otherwise, if the UE is provided multiple values for PUCCH-SpatialRelationInfo, the UE determines a spatial setting for the PUCCH transmission as described in [11, TS 38.321]. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo and  is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo provides ssb-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a SS/PBCH block with index provided by ssb-Index for a same serving cell or, if servingCellId is provided, for a serving cell indicated by servingCellId

-	else if PUCCH-SpatialRelationInfo provides csi-RS-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a CSI-RS with resource index provided by csi-RS-Index for a same serving cell or, if servingCellId is provided, for a serving cell indicated by servingCellId

-	else PUCCH-SpatialRelationInfo provides srs, the UE transmits the PUCCH using a same spatial domain filter as for a transmission of a SRS with resource index provided by resource for a same serving cell and/or active UL BWP or, if servingCellId and/or uplinkBWP are provided, for a serving cell indicated by servingCellId and/or for an UL BWP indicated by uplinkBWP

If a UE

-	is not provided pathlossReferenceRSs in PUCCH-PowerControl,

-	is provided enableDefaultBeamPL-ForPUCCH, and

-	is not provided PUCCH-SpatialRelationInfo, and

-	is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

a spatial setting for a PUCCH transmission from the UE is same as a spatial setting for PDCCH receptions by the UE in the CORESET with the lowest ID on the active DL BWP of the PCell. For a PUCCH transmission over multiple slots, a same spatial setting applies to the PUCCH transmission in each of the multiple slots.

A number of DMRS symbols for a PUCCH transmission using PUCCH format 3 or 4 is provided by additionalDMRS.

Use of /2-BPSK, instead of QPSK, for a PUCCH transmission using PUCCH format 3 or 4 is indicated by pi2BPSK.

### 9.2.3	UE procedure for reporting HARQ-ACK

A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot.

For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0, scheduling a PDSCH reception or a SPS PDSCH release, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, as defined in Table 9.2.3-1.

For a SPS PDSCH reception ending in slot , the UE transmits the PUCCH in slot n+k where k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception.

If the UE detects a DCI format that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in slot n, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot n+k where k is provided by dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2.

With reference to slots for PUCCH transmissions, if the UE detects a DCI format scheduling a PDSCH reception ending in slot n or if the UE detects a DCI format indicating a SPS PDSCH release through a PDCCH reception ending in slot n, or if the UE detects a DCI format that requests Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception through a PDCCH reception ending in slot n, as described in Clause 9.1.4, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2. k=0 corresponds to the last slot of the PUCCH transmission that overlaps with the PDSCH reception or with the PDCCH reception in case of SPS PDSCH release or in case of the DCI format that requests Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception.

A PUCCH transmission with HARQ-ACK information is subject to the limitations for UE transmissions described in Clause 11.1 and Clause 11.1.1.

Table 9.2.3-1: Mapping of PDSCH-to-HARQ_feedback timing indicator field values to numbers of slots

| PDSCH-to-HARQ_feedback timing indicator |  |  | Number of slots |
| --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |  |
| '0' | '00' | '000' | 1st value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
| '1' | '01' | '001' | 2nd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  | '10' | '010' | 3rd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  | '11' | '011' | 4th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '100' | 5th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '101' | 6th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '110' | 7th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '111' | 8th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |

For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources for  HARQ-ACK information bits, as described in Clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For indexing DCI formats within a serving cell for a same PDCCH monitoring occasion, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and with ackNackFeedbackMode = joint for the active UL BWP, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, provided by resourceList for PUCCH resources from a set of PUCCH resources provided by PUCCH-ResourceSet with a maximum of eight PUCCH resources. If the PUCCH resource indicator field includes 1 bit or 2 bits, the values map to the first two values or the first four values, respectively, of Table 9.2.3-2. If the last DCI format does not include a PUCCH resource indicator field, the first value of Table 9.2.3-2 is used.

For the first set of PUCCH resources and when the size  of resourceList is larger than eight, when a UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a last DCI format in a PDCCH reception, among DCI formats with a value of the PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, indicating a same slot for the PUCCH transmission, the UE determines a PUCCH resource with index , , as

where  is a number of CCEs in CORESET  of the PDCCH reception for the DCI format as described in Clause 10.1,  is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. If the DCI format does not include a PUCCH resource indicator field, ∆PRI=0.

Table 9.2.3-2: Mapping of PUCCH resource indication field values to a PUCCH resource in a PUCCH resource set with maximum 8 PUCCH resources

| PUCCH resource indicator |  |  | PUCCH resource |
| --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |  |
| '0' | '00' | '000' | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |  |
| '1' | '01' | '001' | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |  |
|  | '10' | '010' | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |  |
|  | '11' | '011' | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |  |
|  |  | '100' | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |  |
|  |  | '101' | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |  |
|  |  | '110' | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |  |
|  |  | '111' | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |  |

If a UE detects a first DCI format indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than  from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in clause 4.1 of [4, TS 38.211] and  corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, for ,  for ,  for ; otherwise ,  for ,  for ,  for ,  for .

If a UE is not provided SPS-PUCCH-AN-List and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.

If a UE transmits a PUCCH with HARQ-ACK information using PUCCH format 0, the UE determines values  and  for computing a value of cyclic shift  [4, TS 38.211] where  is provided by initialCyclicShift of PUCCH-format0 or, if initialCyclicShift is not provided, by the initial cyclic shift index as described in Clause 9.2.1 and  is determined from the value of one HARQ-ACK information bit or from the values of two HARQ-ACK information bits as in Table 9.2.3-3 and Table 9.2.3-4, respectively.

Table 9.2.3-3: Mapping of values for one HARQ-ACK information bit to sequences for PUCCH format 0

| HARQ-ACK Value | 0 | 1 |
| --- | --- | --- |
| Sequence cyclic shift |  |  |

Table 9.2.3-4: Mapping of values for two HARQ-ACK information bits to sequences for PUCCH format 0

| HARQ-ACK Value | {0, 0} | {0, 1} | {1, 1} | {1, 0} |
| --- | --- | --- | --- | --- |
| Sequence cyclic shift |  |  |  |  |

If a UE transmits a PUCCH with HARQ-ACK information using PUCCH format 1, the UE is provided a value for  by initialCyclicShift of PUCCH-format1 or, if initialCyclicShift is not provided, by the initial cyclic shift index as described in Clause 9.2.1.

If a UE transmits a PUCCH with  HARQ-ACK information bits and  bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs  provided respectively by nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and start from the first PRB from the number of PRBs, that results to  and, if , , where , , , and  are defined in Clause 9.2.5.2. For PUCCH format 3, if  is not equal  according to [4, TS 38.211],  is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 [12, TS 38.331]. If , the UE transmits the PUCCH over  PRBs.

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation and transmits a PUCCH with OACK HARQ-ACK information bits and OCRC bits using PUCCH format 2 or PUCCH format 3, the UE transmits the PUCCH over the first interlace if OACK+OCRC≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r; otherwise, if the UE is provided a second interlace by interlace1 in PUCCH-format2 or PUCCH-format3, the UE transmits the PUCCH over the first and second interlaces.

### 9.2.4	UE procedure for reporting SR

A UE can be configured by SchedulingRequestResourceConfig a set of configurations for SR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. A UE can be configured by schedulingRequestID-BFR-SCell a configuration for LRR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. The UE can be provided, by phy-PriorityIndex in SchedulingRequestResourceConfig, a priority index 0 or a priority index 1 for the SR. If the UE is not provided a priority index for SR, the priority index is 0.

The UE is configured a PUCCH resource by SchedulingRequestResourceId, or by schedulingRequestID-BFR-SCell, providing a PUCCH format 0 resource or a PUCCH format 1 resource as described in Clause 9.2.1. The UE is also configured a periodicity  in symbols or slots and an offset  in slots by periodicityAndOffset for a PUCCH transmission conveying SR. If  is larger than one slot, the UE determines a SR transmission occasion in a PUCCH to be in a slot with number  [4, TS 38.211] in a frame with number  if .

If  is one slot, the UE expects that  and every slot is a SR transmission occasion in a PUCCH.

If  is smaller than one slot, the UE determines a SR transmission occasion in a PUCCH to start in a symbol with index  [4, TS 38.211] if  where  is the value of startingSymbolIndex.

If the UE determines that, for a SR transmission occasion in a PUCCH, the number of symbols available for the PUCCH transmission in a slot is smaller than the value provided by nrofSymbols, the UE does not transmit the PUCCH in the slot.

SR transmission occasions in a PUCCH are subject to the limitations for UE transmissions described in Clause 11.1 and Clause 11.1.1.

The UE transmits a PUCCH in the PUCCH resource for the corresponding SR configuration only when the UE transmits a positive SR. For a positive SR transmission using PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining  as described for HARQ-ACK information in Clause 9.2.3 and by setting . For a positive SR transmission using PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting .

### 9.2.5	UE procedure for reporting multiple UCI types

This Clause is applicable to the case that a UE has resources for PUCCH transmissions or for PUCCH and PUSCH transmissions that overlap in time and each PUCCH transmission is over a single slot without repetitions. Any case that a PUCCH transmission is with repetitions over multiple slots is described in Clause 9.2.6. If a UE is configured with multiple PUCCH resources in a slot to transmit CSI reports

-	if the UE is not provided multi-CSI-PUCCH-ResourceList or if PUCCH resources for transmissions of CSI reports do not overlap in the slot, the UE determines a first resource corresponding to a CSI report with the highest priority [6, TS 38.214]

-	if the first resource includes PUCCH format 2, and if there are remaining resources in the slot that do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting

-	if the first resource includes PUCCH format 3 or PUCCH format 4, and if there are remaining resources in the slot that include PUCCH format 2 and do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting

-	if the UE is provided multi-CSI-PUCCH-ResourceList and if any of the multiple PUCCH resources overlap, the UE multiplexes all CSI reports in a resource from the resources provided by multi-CSI-PUCCH-ResourceList, as described in Clause 9.2.5.2.

A UE multiplexes DL HARQ-ACK information, with or without SR, and CSI report(s) in a same PUCCH if the UE is provided simultaneousHARQ-ACK-CSI; otherwise, the UE drops the CSI report(s) and includes only DL HARQ-ACK information, with or without SR, in the PUCCH. If the UE would transmit multiple PUCCHs in a slot that include DL HARQ-ACK information and CSI report(s), the UE expects to be provided a same configuration for simultaneousHARQ-ACK-CSI each of PUCCH formats 2, 3, and 4.

If a UE would multiplex CSI reports that include Part 2 CSI reports in a PUCCH resource, the UE determines the PUCCH resource and a number of PRBs for the PUCCH resource or a number of Part 2 CSI reports assuming that each of the CSI reports indicates rank 1.

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot and, when applicable as described in Clauses 9.2.5.1 and 9.2.5.2, the UE is configured to multiplex different UCI types in one PUCCH, and at least one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types if the following conditions are met. If one of the PUCCH transmissions or PUSCH transmissions is in response to a DCI format detection by the UE, the UE expects that the first symbol S0 of the earliest PUCCH or PUSCH, among a group overlapping PUCCHs and PUSCHs in the slot, satisfies the following timeline conditions

-	S0 is not before a symbol with CP starting after Tproc,1mux after a last symbol of any corresponding PDSCH, Tproc,1mux is given by maximum of Tproc,1mux,1,⋯,Tproc,1mux,i,⋯ where for the i-th PDSCH with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,1mux,i=N1+d1,1+1⋅2048+144⋅κ⋅2-μ⋅TC, d1,1 is selected for the i-th PDSCH following [6, TS 38.214], N1 is selected based on the UE PDSCH processing capability of the i-th PDSCH and SCS configuration μ, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH scheduling the i-th PDSCH, the i-th PDSCH, the PUCCH with corresponding HARQ-ACK transmission for the i-th PDSCH, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

-	S0 is not before a symbol with CP starting after Tproc,releasemux after a last symbol of any corresponding SPS PDSCH release or of a DCI format 1_1 indicating SCell dormancy as described in Clause 10.3, or of a DCI format 1_1 indicating a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH. Tproc,releasemux is given by maximum of Tproc,releasemux,1,⋯,Tproc,releasemux,i,⋯ where for the i-th PDCCH providing the SPS PDSCH release or the DCI format 1_1 with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,releasemux,i=N+1⋅2048+144⋅κ⋅2-μ⋅TC, N as described in Clause 10.2, or DCI format 1_1 that requests Type-3 HARQ-ACK codebook report as described in Clause 10.2, or the DCI format 1_1 indicating SCell dormancy as described in Clause 10.3, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH providing the i-th SPS PDSCH release or the DCI format 1_1, the PUCCH with corresponding HARQ-ACK transmission for the i-th SPS PDSCH release or the DCI format 1_1, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

-	if there is no aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs, S0 is not before a symbol with CP starting after Tproc,2mux after a last symbol of

-	any PDCCH with the DCI format scheduling an overlapping PUSCH, and

-	any PDCCH scheduling a PDSCH or SPS PDSCH release, or a DCI format 1_1 indicating a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH, with corresponding HARQ-ACK information in an overlapping PUCCH in the slot

If there is at least one PUSCH in the group of overlapping PUCCHs and PUSCHs, Tproc,2mux is given by maximum of Tproc,2mux,1,⋯,Tproc,2mux,i,⋯ where for the i-th PUSCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,2mux,i=maxN2+d2,1+1⋅2048+144⋅κ⋅2-μ⋅TC+Tswitch,d2,2, d2,1, d2,2 and Tswitch are selected for the i-th PUSCH following [6, TS 38.214], N2 is selected based on the UE PUSCH processing capability of the i-th PUSCH and SCS configuration μ, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH scheduling the i-th PUSCH, the PDCCHs scheduling the PDSCHs or providing the SPS PDSCH releases with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs/PUSCHs, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

If there is no PUSCH in the group of overlapping PUCCHs and PUSCHs, Tproc,2muxis given by maximum of Tproc,2mux,1,⋯,Tproc,2mux,i,⋯ where for the i-th PDSCH or the i-th SPS PDSCH release with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs, Tproc,2mux,i=N2+1⋅2048+144⋅κ⋅2-μ⋅TC, N2is selected based on the UE PUSCH processing capability of the PUCCH serving cell if configured.  N2 is selected based on the UE PUSCH processing capability 1, if PUSCH processing capability is not configured for the PUCCH serving cell. μ is selected based on the smallest SCS configuration between the SCS configuration used for the PDCCH scheduling the i-th PDSCH or providing the i-th SPS PDSCH release with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs, and the SCS configuration for the PUCCH serving cell.

-	if there is an aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs, S0 is not before a symbol with CP starting after Tproc,CSImux=maxZ+d⋅2048+144⋅κ⋅2-μ⋅TC+Tswitch,d2,2 after a last symbol of

-	any PDCCH with the DCI format scheduling an overlapping PUSCH, and

-	any PDCCH scheduling a PDSCH, or SPS PDSCH release, or providing a DCI format 1_1 indicating SCell dormancy, or a DCI format 1_1 indicating a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH, with corresponding HARQ-ACK information in an overlapping PUCCH in the slot

where μ corresponds to the smallest SCS configuration among the SCS configuration of the PDCCHs, the smallest SCS configuration for the group of the overlapping PUSCHs, and the smallest SCS configuration of CSI-RS associated with the DCI format scheduling the PUSCH with the multiplexed aperiodic CSI report, and d=2 for μ=0,1 , d=3 for μ=2 and d=4 for μ=3

-	N1, N2, d1,1, d2,1, d2,2, and Z are defined in [6, TS 38.214], and κ and TC are defined in [4, TS 38.211].

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot, one of the PUCCHs includes HARQ-ACK information in response to an SPS PDSCH reception, and any PUSCH is not in response to a DCI format detection, the UE expects that the first symbol  of the earliest PUCCH or PUSCH satisfies the first of the previous timeline conditions with the exception that components associated to a SCS configuration for a PDCCH scheduling a PDSCH or a PUSCH are absent from the timeline conditions.

A UE does not expect a PUCCH or a PUSCH that is in response to a DCI format detection to overlap with any other PUCCH or PUSCH that does not satisfy the above timing conditions.

A UE that

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = separate

does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap in time with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs.

If there is one or more aperiodic CSI reports multiplexed on a PUSCH in the group of overlapping PUCCHs and PUSCHs and if symbol  is before symbol  that is a next uplink symbol with CP starting after  after the end of the last symbol of

-	the last symbol of aperiodic CSI-RS resource for channel measurements, and

-	the last symbol of aperiodic CSI-IM used for interference measurements, and

-	the last symbol of aperiodic NZP CSI-RS for interference measurements, when aperiodic CSI-RS is used for channel measurement for triggered CSI report

the UE is not required to update the CSI report for the triggered CSI report . is defined in [6, TS 38.214] and  corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs scheduling the PUSCHs, the smallest SCS configuration of aperiodic CSI-RSs associated with DCI formats provided by the PDCCHs triggering the aperiodic CSI reports, and the smallest SCS configuration of the overlapping PUCCHs and PUSCHs and  for ,  for  and  for .

If a UE would transmit multiple PUCCHs in a slot that include HARQ-ACK information, and/or SR, and/or CSI reports and any PUCCH with HARQ-ACK information in the slot satisfies the above timing conditions and does not overlap with any other PUCCH or PUSCH in the slot that does not satisfy the above timing conditions, the UE multiplexes the HARQ-ACK information, and/or SR, and/or  CSI reports and determines corresponding PUCCH(s) for transmission in the slot according to the following pseudo-code. If the multiple PUCCHs do not include HARQ-ACK information and do not overlap with any PUSCH transmission by the UE in response to a DCI format detection by the UE, the timing conditions do not apply.

If

-	a UE is not provided multi-CSI-PUCCH-ResourceList, and

-	a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and

-	there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and

-	the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports

the UE

-	multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and

-	does not transmit the PUCCH with the CSI report having the lower priority

Set  to the set of resources for transmission of corresponding PUCCHs in a single slot without repetitions where

-	a resource with earlier first symbol is placed before a resource with later first symbol

-	for two resources with same first symbol, the resource with longer duration is placed before the resource with shorter duration

-	for two resources with same first symbol and same duration, the placement is arbitrary

-	the above three steps for the set  are according to a subsequent pseudo-code for a function

-	a resource for negative SR transmission that does not overlap with a resource for HARQ-ACK or CSI transmission is excluded from set

-	if the UE is not provided simultaneousHARQ-ACK-CSI and resources for transmission of HARQ-ACK information include PUCCH format 0 or PUCCH format 2, resources that include PUCCH format 2, or PUCCH format 3, or PUCCH format 4 for transmission of CSI reports are excluded from the set  if they overlap with any resource from the resources for transmission of HARQ-ACK information

-	if the UE is not provided simultaneousHARQ-ACK-CSI and at least one of the resources for transmission of HARQ-ACK information includes PUCCH format 1, PUCCH format 3, or PUCCH format 4

-	resources that include PUCCH format 3 or PUCCH format 4 for transmission of CSI reports are excluded from the set

-	resources that include PUCCH format 2 for transmission of CSI reports are excluded from the set  if they overlap with any resource from the resources for transmission of HARQ-ACK information

Set  to the cardinality of

Set to be the first symbol of resource  in the slot

Set  to be the number of symbols of resource  in the slot

Set  - index of first resource in set

Set  - counter of overlapped resources

while

if  and resource  overlaps with resource

else

if

determine a single resource for multiplexing UCI associated with resources  as described in Clauses 9.2.5.0, 9.2.5.1 and 9.2.5.2

set the index of the single resource to

% start from the beginning after reordering unmerged resources at next step

% function that re-orders resources in current set

Set  to the cardinality of

else

end if

end if

end while

The function  performs the following pseudo-code

{

while  % the next two while loops are to re-order the unmerged resources

while

if  OR

end if

end while

end while

}

For each PUCCH resource in the set  that satisfies the aforementioned timing conditions, when applicable,

-	the UE transmits a PUCCH using the PUCCH resource if the PUCCH resource does not overlap in time with a PUSCH transmission after multiplexing UCI following the procedures described in Clauses 9.2.5.1 and 9.2.5.2

-	the UE multiplexes HARQ-ACK information and/or CSI reports in a PUSCH if the PUCCH resource overlaps in time with a PUSCH transmission, as described in Clause 9.3, and does not transmit SR. In case the PUCCH resource overlaps in time with multiple PUSCH transmissions, the PUSCH for multiplexing HARQ-ACK information and/or CSI is selected as described in Clause 9. If the PUSCH transmission by the UE is not in response to a DCI format detection and the UE multiplexes only CSI reports, the timing conditions are not applicable

-	the UE does not expect the resource to overlap with a second resource of a PUCCH transmission over multiple slots if the resource is obtained from a group of resources that do not overlap with the second resource.

Clauses 9.2.5.0, 9.2.5.1 and 9.2.5.2 assume the following

-	resources for transmissions of UCI types, prior to multiplexing or dropping, overlap in a slot

-	multiplexing conditions of corresponding UCI types in a single PUCCH are satisfied, and

-	the UE does not transmit any PUSCH time-overlapping with PUCCH in the slot.

#### 9.2.5.0	UE procedure for prioritization between SL HARQ-ACK information in a PUCCH and UCI in a PUCCH

The priority value of a PUCCH transmission is as described in Clause 16.2.4.3.1.

For prioritization between SL HARQ-ACK information in a first PUCCH and UCI in a second PUCCH

-	if the second PUCCH has priority index 1,

-	if sl-PriorityThreshold-UL-URLLC is provided

-	the UE transmits the first PUCCH if a smallest priority value of the first PUCCH is smaller than sl-PriorityThreshold-UL-URLLC; otherwise, the UE transmits the second PUCCH

-	else

-	the UE transmits the second PUCCH

-	else

-	the UE transmits the first PUCCH if the smallest priority value of the first PUCCH is smaller than sl-PriorityThreshold; otherwise, the UE transmits the second PUCCH

When the UE determines to transmit the second PUCCH, the UE determines a single resource for multiplexing UCI in the second PUCCH as described in Clauses 9.2.5.1 and 9.2.5.2.

#### 9.2.5.1	UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH

In the following, a UE is configured to transmit K PUCCHs for respective K SRs in a slot, as determined by a set of schedulingRequestResourceId and a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, with SR transmission occasions that would overlap with a transmission of a PUCCH with HARQ-ACK information from the UE in the slot or with a transmission of a PUCCH with CSI report(s) from the UE in the slot.

If a UE would transmit a PUCCH with positive SR and at most two HARQ-ACK information bits in a resource using PUCCH format 0, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for HARQ-ACK information as described in Clause 9.2.3. The UE determines a value of  and  for computing a value of cyclic shift  [4, TS 38.211] where  is provided by initialCyclicShift of PUCCH-format0, and  is determined from the value of one HARQ-ACK information bit or from the values of two HARQ-ACK information bits as in Table 9.2.5-1 and Table 9.2.5-2, respectively.

If the UE would transmit negative SR and a PUCCH with at most two HARQ-ACK information bits in a resource using PUCCH format 0, the UE transmits the PUCCH in the resource using PUCCH format 0 for HARQ-ACK information as described in Clause 9.2.3.

Table 9.2.5-1: Mapping of values for one HARQ-ACK information bit and positive SR to sequences for PUCCH format 0

| HARQ-ACK Value | 0 | 1 |
| --- | --- | --- |
| Sequence cyclic shift |  |  |

Table 9.2.5-2: Mapping of values for two HARQ-ACK information bits and positive SR to sequences for PUCCH format 0

| HARQ-ACK Value | {0, 0} | {0, 1} | {1, 1} | {1, 0} |
| --- | --- | --- | --- | --- |
| Sequence cyclic shift |  |  |  |  |

If a UE would transmit SR in a resource using PUCCH format 0 and HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the UE transmits only a PUCCH with the HARQ-ACK information bits in the resource using PUCCH format 1.

If the UE would transmit positive SR in a first resource using PUCCH format 1 and at most two HARQ-ACK information bits in a second resource using PUCCH format 1 in a slot, the UE transmits a PUCCH with HARQ-ACK information bits in the first resource using PUCCH format 1 as described in Clause 9.2.3. If a UE would not transmit a positive SR in a resource using PUCCH format 1 and would transmit at most two HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the UE transmits a PUCCH in the resource using PUCCH format 1 for HARQ-ACK information as described in Clause 9.2.3.

If a UE would transmit a PUCCH with  HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in Clauses 9.2.1 and 9.2.3,  bits representing a negative or positive SR, in ascending order of the values of schedulingRequestResourceId and a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, are appended to the HARQ-ACK information bits and the UE transmits the combined  UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in Clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the  bits indicates the positive LRR. An all-zero value for the  bits represents a negative SR value across all K SRs.

If a UE would transmit a PUCCH with  CSI report bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot,  bits representing corresponding negative or positive SR, in ascending order of the values of schedulingRequestResourceId and a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, are prepended to the CSI information bits as described in Clause 9.2.5.2 and the UE transmits a PUCCH with the combined  UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. If one of the SRs is a positive LRR, the value of the  bits indicates the positive LRR. An all-zero value for the  bits represents a negative SR value across all K SRs.

If a UE transmits a PUCCH with  HARQ-ACK information bits,  SR bits, and  CRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs provided respectively by nrofPRBs in PUCCH-format2  or nrofPRBs in PUCCH-format3 and starts from the first PRB from the number of PRBs, that results to  and, if , , where , , , and  are defined in Clause 9.2.5.2. For PUCCH format 3, if  is not equal  according to [4, TS 38.211],  is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 [12, TS 38.331]. If , the UE transmits the PUCCH over the  PRBs.

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation and transmits a PUCCH with OACK HARQ-ACK information bits, OSR=log2K+1 SR bits, and OCRC CRC bits using PUCCH format 2 or PUCCH format 3, the UE transmits the PUCCH over the first interlace if OACK+OSR+OCRC≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r; otherwise, if the UE is provided a second interlace by interlace1 in PUCCH-format2 or PUCCH-format3, the UE transmits the PUCCH over the first and second interlaces.

#### 9.2.5.2	UE procedure for multiplexing HARQ-ACK/SR/CSI in a PUCCH

For a transmission occasion of a single CSI report, a PUCCH resource is provided by pucch-CSI-ResourceList. For a transmission occasion of multiple CSI reports, corresponding PUCCH resources can be provided by multi-CSI-PUCCH-ResourceList. If a UE is provided first and second PUCCH-Config, multi-CSI-PUCCH-ResourceList is provided by the first PUCCH-Config, and PUCCH-ResourceId in pucch-CSI-ResourceList or multi-CSI-PUCCH-ResourceList indicates a corresponding PUCCH resource in PUCCH-Resource provided by the first PUCCH-Config.

If a UE is provided only one PUCCH resource set for transmission of HARQ-ACK information in response to PDSCH reception scheduled by a DCI format or in response to a SPS PDSCH release, the UE does not expect to be provided simultaneousHARQ-ACK-CSI.

A UE is configured by maxCodeRate a code rate for multiplexing HARQ-ACK, SR, and CSI report(s) in a PUCCH transmission using PUCCH format 2, PUCCH format 3, or PUCCH format 4.

If a UE transmits CSI reports using PUCCH format 2, the UE transmits only wideband CSI for each CSI report [6, TS 38.214]. In the following, a Part 1 CSI report refers either to a CSI report with only wideband CSI or to a Part 1 CSI report with wideband CSI and sub-band CSI.

Denote as

-	 a total number of HARQ-ACK information bits, if any

-	 a total number of SR bits.  if there is no scheduling request bit; otherwise,  as described in Clause 9.2.5.1

-	, where  is a number of Part 1 CSI report bits for CSI report with priority value ,  is a number of Part 2 CSI report bits, if any, for CSI report with priority value  [6, TS 38.214], and  is a number of CSI reports that include overlapping CSI reports

-	, where  is a number of CRC bits, if any, for encoding HARQ-ACK, SR and Part 1 CSI report bits and  is a number of CRC bits, if any, for encoding Part 2 CSI report bits

In the following

-	 is a code rate given by maxCodeRate as in Table 9.2.5.2-1.

-	 is a number of PRBs for PUCCH format 2, or PUCCH format 3, or PUCCH format 4, respectively, where  is provided by nrofPRBs in PUCCH-format2 for PUCCH format 2 or by nrofPRBs in PUCCH-format3 for PUCCH format 3, and  for PUCCH format 4

-	 for PUCCH format 2 or, if the PUCCH resource with PUCCH format 2 includes an orthogonal cover code with length  provided by OCC-Length, ,  for PUCCH format 3 or, if the PUCCH resource with PUCCH format 3 includes an orthogonal cover code with length  provided by OCC-Length, , and  for PUCCH format 4, where  is a number of subcarriers per resource block [4, TS 38.211]

-	 is equal to a number of PUCCH symbols  for PUCCH format 2 provided by nrofSymbols in PUCCH-format2. For PUCCH format 3 or for PUCCH format 4,  is equal to a number of PUCCH symbols  for PUCCH format 3 or equal to a number of PUCCH symbols  for PUCCH format 4 provided by nrofSymbols in PUCCH-format3 or nrofSymbols in PUCCH-format4, respectively, after excluding a number of symbols used for DM-RS transmission for PUCCH format 3 or for PUCCH format 4, respectively [4, TS 38.211]

-	 if pi/2-BPSK is the modulation scheme and  if QPSK is the modulation scheme as indicated by pi2BPSK for PUCCH format 3 or PUCCH format 4. For PUCCH format 2,

If a UE has one or more CSI reports and zero or more HARQ-ACK/SR information bits to transmit in a PUCCH where the HARQ-ACK, if any, is in response to a PDSCH reception without a corresponding PDCCH

-	if any of the CSI reports are overlapping and the UE is provided by multi-CSI-PUCCH-ResourceList with  PUCCH resources in a slot, for PUCCH format 2 and/or PUCCH format 3 and/or PUCCH format 4, as described in Clause 9.2.1, where the resources are indexed according to an ascending order for the product of a number of corresponding REs, modulation order , and configured code rate ;

-	if , the UE uses PUCCH format 2 resource , or the PUCCH format 3 resource , or the PUCCH format 4 resource

-	else if  and , , the UE transmits a PUCCH conveying HARQ-ACK information, SR and CSI report(s) in a respective PUCCH where the UE uses the PUCCH format 2 resource , or the PUCCH format 3 resource , or the PUCCH format 4 resource

-	else the UE uses the PUCCH format 2 resource , or the PUCCH format 3 resource , or the PUCCH format 4 resource  and the UE selects  CSI report(s) for transmission together with HARQ-ACK information and SR, when any, in ascending priority value as described in [6, TS 38.214]

-	else, the UE transmits the  bits in a PUCCH resource provided by pucch-CSI-ResourceList and determined as described in Clause 9.2.5

If a UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports [6, TS 38.214] to transmit and the UE determines a PUCCH resource with PUCCH format 3 or PUCCH format 4, where

-	the UE determines the PUCCH resource using the PUCCH resource indicator field [5, TS 38.212] in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and

-	the UE determines the PUCCH resource set as described in Clause 9.2.1 and Clause 9.2.3 for  UCI bits

and

-	if , the UE transmits the HARQ-ACK, SR, and CSI reports bits by selecting the minimum number  of the  PRBs satisfying  as described in Clauses 9.2.3 and 9.2.5.1;

-	else, the UE selects  CSI report(s), from the  CSI reports, for transmission together with HARQ-ACK and SR in ascending priority value [6, TS 38.214], where  the value of  satisfies  and , where  is a number of CRC bits corresponding to  UCI bits, and  is a number of CRC bits corresponding to  UCI bits.

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, where

-	the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and

-	the UE determines the PUCCH resource set as described in Clauses 9.2.1 and 9.2.3 for OUCI UCI bits

and

-	if OACK+OSR+OCSI-part1+OCRC,CSI-part1≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

-	else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if OACK+OSR+OCSI-part1+OCRC,CSI-part1≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r,  the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

-	else, the procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

If a UE has HARQ-ACK, SR and sub-band CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3 or PUCCH format 4, where

-	the UE determines the PUCCH resource using the PUCCH resource indicator field [5, TS 38.212] in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and

-	the UE determines the PUCCH resource set as described in Clause 9.2.1 and Clause 9.2.3 for  UCI bits

and

-	if , the UE transmits the HARQ-ACK, SR and the  CSI report bits by selecting the minimum number  of PRBs from the  PRBs satisfying  as described in Clauses 9.2.3 and 9.2.5.1

-	else,

-	if for  Part 2 CSI report priority value(s), it is

and

,

the UE selects the first  Part 2 CSI reports, according to respective priority value(s) [6, TS 38.214], for transmission together with the HARQ-ACK, SR and  Part 1 CSI reports , where  is the number of Part 1 CSI report bits for the  CSI report and  is the number of Part 2 CSI report bits for the  CSI report priority value,  is a number of CRC bits corresponding to , and  is a number of CRC bits corresponding to

-	else, the UE drops all Part 2 CSI reports and selects  Part 1 CSI report(s), from the  CSI reports in ascending priority value [6, TS 38.214], for transmission together with the HARQ-ACK and SR information bits where the value of  satisfies  and , where is a number of CRC bits corresponding to  UCI bits, and  is a number of CRC bits corresponding to  UCI bits.

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

-	the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and

-	the UE determines the PUCCH resource set as described in Clauses 9.2.1 and 9.2.3 for OUCI UCI bits

and

-	if OACK+OSR+OCSI+OCRC≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the HARQ-ACK, SR and the NCSItotal CSI report bits in a PUCCH over the first interlace

-	else if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if OACK+OSR+OCSI+OCRC≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

-	else, the procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, with MInterlace,0PUCCH+MInterlace,1PUCCH.

Table 9.2.5.2-1: Code rate  corresponding to value of maxCodeRate

| maxCodeRate | Code rate |
| --- | --- |
| 0 | 0.08 |
| 1 | 0.15 |
| 2 | 0.25 |
| 3 | 0.35 |
| 4 | 0.45 |
| 5 | 0.60 |
| 6 | 0.80 |
| 7 | Reserved |

### 9.2.6	PUCCH repetition procedure

For PUCCH formats 1, 3, or 4, a UE can be configured a number of slots, NPUCCHrepeat, for repetitions of a PUCCH transmission by respective nrofSlots. If a UE is provided a PUCCH-config that includes subslotLengthForPUCCH, the UE does not expect the PUCCH-config to include nrofSlots.

For NPUCCHrepeat>1,

-	the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots

-	a PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols in PUCCH-format1, nrofSymbols in PUCCH-format3, or nrofSymbols in PUCCH-format4

-	a PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex in PUCCH-format1, startingSymbolIndex in PUCCH-format3, or startingSymbolIndex in PUCCH-format4

-	the UE is configured by interslotFrequencyHopping whether or not to perform frequency hopping for PUCCH transmissions in different slots

-	if the UE is configured to perform frequency hopping for PUCCH transmissions across different slots

-	the UE performs frequency hopping per slot

-	the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in slots with even number and starting from the second PRB, provided by secondHopPRB, in slots with odd number. The slot indicated to the UE for the first PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in NPUCCHrepeat slots is counted regardless of whether or not the UE transmits the PUCCH in the slot

-	the UE does not expect to be configured to perform frequency hopping for a PUCCH transmission within a slot

-	If the UE is not configured to perform frequency hopping for PUCCH transmissions across different slots and if the UE is configured to perform frequency hopping for a PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot

If the UE determines that, for a PUCCH transmission in a slot, the number of symbols available for the PUCCH transmission is smaller than the value provided by nrofSymbols for the corresponding PUCCH format, the UE does not transmit the PUCCH in the slot.

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, as described in Clause 4.1.

For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having

-	an UL symbol, as described in Clause 11.1, or flexible symbol that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and

-	consecutive UL symbols, as described in Clause 11.1, or flexible symbols that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4

For paired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission as the NPUCCHrepeat consecutive slots starting from a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting.

If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type A over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in Clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots.

If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type B over a second number of slots, and the PUCCH transmission would overlap with actual PUSCH repetitions in one or more slots, and the conditions in Clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied for the overlapping actual PUSCH repetitions, the UE transmits the PUCCH and does not transmit the overlapping actual PUSCH repetitions.

A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority

-	the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority

-	if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot

-	if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

A UE does not expect a PUCCH that is in response to a DCI format detection to overlap with any other PUCCH that does not satisfy the corresponding timing conditions in Clause 9.2.5.

If a UE would transmit a PUCCH over NPUCCHrepeat slots and the UE does not transmit the PUCCH in a slot from the NPUCCHrepeat slots due to overlapping with another PUCCH transmission in the slot, the UE counts the slot in the number of NPUCCHrepeat slots.

## 9.3	UCI reporting in physical uplink shared channel

Offset values are defined for a UE to determine a number of resources for multiplexing HARQ-ACK information and for multiplexing CSI reports in a PUSCH. Offset values are also defined for multiplexing CG-UCI [5, TS 38.212] in a CG-PUSCH. The offset values are signalled to a UE either by a DCI format scheduling the PUSCH transmission or by higher layers.

If a DCI format that does not include a beta_offset indicator field schedules the PUSCH transmission from the UE and the UE is provided betaOffsets = 'semiStatic', the UE applies the , , and  values that are provided by betaOffsets = 'semiStatic' for the corresponding HARQ-ACK information, Part 1 CSI reports and Part 2 CSI reports.

If the PUSCH transmission is with a configured grant and the UE is provided CG-UCI-OnPUSCH= 'semiStatic', the UE applies the , , and  values that are provided by CG-UCI-OnPUSCH = 'semiStatic' for the corresponding HARQ-ACK information, Part 1 CSI reports and Part 2 CSI reports.

If the PUSCH is scheduled by DCI format 0_0 and the UE is provided betaOffsets = 'dynamic', the UE applies the  , , and  values that are determined from the first value of  betaOffsets = 'dynamic'.

If the PUSCH is a configured grant Type 2 PUSCH activated by DCI format 0_0 and the UE is provided CG-UCI-OnPUSCH ='dynamic', the UE applies the , , and  values that are determined from the first value of CG-UCI-OnPUSCH = 'dynamic'.

HARQ-ACK information offsets  are configured to values according to Table 9.3-1. The betaOffsetACK-Index1, betaOffsetACK-Index2, and betaOffsetACK-Index3 respectively provide indexes , , and  for the UE to use if the UE multiplexes up to 2 HARQ-ACK information bits, more than 2 and up to 11 HARQ-ACK information bits, and more than 11 bits in the PUSCH, respectively.

Part 1 CSI report and Part 2 CSI report offsets  and , respectively, are configured to values according to Table 9.3-2. The betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part2-Index1 respectively provide indexes  and  for the UE to use if the UE multiplexes up to 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. The betaOffsetCSI-Part1-Index2 and betaOffsetCSI-Part2-Index2 respectively provide indexes  or  for the UE to use if the UE multiplexes more than 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH.

If a DCI format that includes a beta_offset indicator field with one bit or two bits, as configured by uci-OnPUSCH, schedules the PUSCH transmission from the UE, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3} a set of two or four  indexes, by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of two or four  indexes, and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of two or four  indexes from Tables 9.3-1 and 9.3-2, respectively, for multiplexing HARQ-ACK information, Part 1 CSI reports, and Part 2 CSI reports, respectively, in the PUSCH transmission. The beta_offset indicator field indicates a  value, a  value and a  value from the respective sets of values, with the mapping defined in Table 9.3-3 and in Table 9.3-3A.

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI, the UE multiplexes CG-UCI in the PUSCH transmission if the UE is provided by betaOffsetCG-UCI a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK which provides indexes Ioffset,1HARQ-ACK and Ioffset,2HARQ-ACK for the UE to use if the UE multiplexes up to 11, and more than 11 combined information bits, respectively.

Table 9.3-1: Mapping of beta_offset values for HARQ-ACK information and/or for CG-UCI and the index signalled by higher layers

| or or or IoffsetCG-UCI | βoffsetHARQ-ACK or βoffsetCG-UCI |
| --- | --- |
| 0 | 1.000 |
| 1 | 2.000 |
| 2 | 2.500 |
| 3 | 3.125 |
| 4 | 4.000 |
| 5 | 5.000 |
| 6 | 6.250 |
| 7 | 8.000 |
| 8 | 10.000 |
| 9 | 12.625 |
| 10 | 15.875 |
| 11 | 20.000 |
| 12 | 31.000 |
| 13 | 50.000 |
| 14 | 80.000 |
| 15 | 126.000 |
| 16 | Reserved |
| 17 | Reserved |
| 18 | Reserved |
| 19 | Reserved |
| 20 | Reserved |
| 21 | Reserved |
| 22 | Reserved |
| 23 | Reserved |
| 24 | Reserved |
| 25 | Reserved |
| 26 | Reserved |
| 27 | Reserved |
| 28 | Reserved |
| 29 | Reserved |
| 30 | Reserved |
| 31 | Reserved |

Table 9.3-2: Mapping of beta_offset values for CSI and the index signalled by higher layers

| or or |  |
| --- | --- |
| 0 | 1.125 |
| 1 | 1.250 |
| 2 | 1.375 |
| 3 | 1.625 |
| 4 | 1.750 |
| 5 | 2.000 |
| 6 | 2.250 |
| 7 | 2.500 |
| 8 | 2.875 |
| 9 | 3.125 |
| 10 | 3.500 |
| 11 | 4.000 |
| 12 | 5.000 |
| 13 | 6.250 |
| 14 | 8.000 |
| 15 | 10.000 |
| 16 | 12.625 |
| 17 | 15.875 |
| 18 | 20.000 |
| 19 | Reserved |
| 20 | Reserved |
| 21 | Reserved |
| 22 | Reserved |
| 23 | Reserved |
| 24 | Reserved |
| 25 | Reserved |
| 26 | Reserved |
| 27 | Reserved |
| 28 | Reserved |
| 29 | Reserved |
| 30 | Reserved |
| 31 | Reserved |

Table 9.3-3: Mapping of four beta_offset indicator values to offset indexes

| beta_offset indicator | ( or or ), ( or ), ( or ) |
| --- | --- |
| '00' | 1st offset index provided by higher layers |
| '01' | 2nd offset index provided by higher layers |
| '10' | 3rd offset index provided by higher layers |
| '11' | 4th offset index provided by higher layers |

Table 9.3-3A: Mapping of two beta_offset indicator values to offset indexes

| beta_offset indicator | ( or or ), ( or ), ( or ) |
| --- | --- |
| '0' | 1st offset index provided by higher layers |
| '1' | 2nd offset index provided by higher layers |

# 10	UE procedure for receiving control information

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2-PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

A UE monitors a set of PDCCH candidates in one or more CORESETs on the active DL BWP on each activated serving cell configured with PDCCH monitoring according to corresponding search space sets where monitoring implies decoding each PDCCH candidate according to the monitored DCI formats.

If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs

-	per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or

-	per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability

If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot.

A UE can indicate a capability to monitor PDCCH according to one or more of the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1. A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination X,Y, the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to Y.

If a UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more of the multiple combinations X,Y, the UE monitors PDCCH on the cell according to the combination X,Y, from the one or more combinations X,Y, that is associated with the largest maximum number of MPDCCHmax,X,Y,μ and CPDCCHmax,X,Y,μ defined in Table 10.1-2A and Table 10.1-3A. The UE expects to monitor PDCCH according to the same combination X,Y in every slot on the active DL BWP of a cell.

A UE capability for PDCCH monitoring per slot or per span on an active DL BWP of a serving cell is defined by a maximum number of PDCCH candidates and non-overlapped CCEs the UE can monitor per slot or per span, respectively, on the active DL BWP of the serving cell.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon,

the UE is not required to monitor the PDCCH candidate.

If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in Clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell.

If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate.

If a UE is provided availableRB-SetsPerCell, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets that are indicated as unavailable for receptions by an available RB set indicator field in DCI format 2_0 as described in Clause 11.1.1. If the UE does not obtain the available RB set indicator for a symbol, the UE monitors PDCCH candidates on all RB sets in the symbol.

If a UE can support

-	a first set of Ncells,0DL serving cells where the UE is either not provided coresetPoolIndex or is provided coresetPoolIndex with a single value for all CORESETs on all DL BWPs of each scheduling cell from the first set of serving cells, and

-	a second set of Ncells,1DL serving cells where the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with a value 0 for a first CORESET, and with a value 1 for a second CORESET on any DL BWP of each scheduling cell from the second set of serving cells

the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is a value reported by the UE.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than 4 serving cells and the UE is not provided monitoringCapabilityConfig for any downlink cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and for a maximum number of non-overlapped CCEs the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap downlink cells, where

-	Ncellscap is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCA where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise, Ncellscap is the value of pdcch-BlindDetectionCA

When a UE is configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap=NcellsMCG downlink cells for the MCG where NcellsMCG is provided by pdcch-BlindDetection for the MCG and determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap=NcellsSCG downlink cells for the SCG where NcellsSCG is provided by pdcch-BlindDetection for the SCG. When the UE is configured for carrier aggregation operation over more than 4 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per slot a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap.

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy

-	pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= pdcch-BlindDetectionCA, if the UE reports pdcch-BlindDetectionCA, or

-	pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-BlindDetectionCA.

For NR-DC operation, the UE may indicate, through pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

If the UE reports pdcch-BlindDetectionCA,

-	the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, …, pdcch-BlindDetectionCA-1], and

-	pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA.

Otherwise, if NNR-DC,maxDL,cells is a maximum total number of downlink cells that the UE can be configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability,

-	the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, 2, 3], and

-	pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= NNR-DC,maxDL,cells.

If a UE indicates in UE-NR-Capability-r16 a carrier aggregation capability larger than two downlink cells, the UE includes in UE-NR-Capability-r16 an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that the UE can monitor per span when the UE is configured for carrier aggregation operation over more than two downlink cells with monitoringCapabilityConfig = r16monitoringcapability. When a UE is not configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cell where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to Ncellscap-r16 downlink cells, where

-	Ncellscap-r16 is the number of configured downlink cells if the UE does not provide pdcch-MonitoringCA

-	otherwise, Ncellscap-r16 is the value of pdcch-MonitoringCA

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to

-	Ncellscap-r16= Ncells, r16MCG downlink cells for the MCG where Ncells, r16MCG is provided by pdcch-BlindDetection2 for the MCG, and

-	Ncellscap-r16= Ncells, r16SCG downlink cells for the SCG where Ncells, r16SCG is provided by pdcch-BlindDetection2 for the SCG

When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per span a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r16.

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-MonitoringCA, if the UE reports pdcch-MonitoringCA, or

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-MonitoringCA

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE-r16 and pdcch-BlindDetectionSCG-UE-r16, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

If the UE reports pdcch-MonitoringCA,

-	the value range of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is [1, …, pdcch-MonitoringCA-1], and

-	pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= pdcch-MonitoringCA.

Otherwise, if NNR-DC,  max, r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is 1,

-	pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= NNR-DC,  max, r16DL,cells.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than one downlink cell with monitoringCapabilityConfig = r15monitoringcapability or larger than one downlink cell with monitoringCapabilityConfig = r16monitoringcapability, the UE includes in UE-NR-Capability-r16 an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs the UE can monitor for downlink cells with monitoringCapabilityConfig = r15monitoringcapability or for downlink cells with monitoringCapabilityConfig = r16monitoringcapability when the UE is configured for carrier aggregation operation over more than two downlink cells with at least one downlink cell with monitoringCapabilityConfig = r15monitoringcapability and at least one downlink cell with monitoringCapabilityConfig = r16monitoringcapability. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot or per span that corresponds to Ncells,r15cap-r16 downlink cells or to Ncells,r16cap-r16 downlink cells, respectively, where

-	Ncells,r15cap-r16 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA1

-	otherwise,

-	if the UE reports only one combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2), Ncells,r15cap-r16 is the value of pdcch-BlindDetectionCA1

-	else, Ncells,r15cap-r16 is the value of pdcch-BlindDetectionCA1 from a combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2) that is provided by pdcch-BlindDetectionCA-CombIndicator

and

-	Ncells,r16cap-r16 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA2

-	otherwise,

-	if the UE reports only one combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2), Ncells,r16cap-r16 is the value of pdcch-BlindDetectionCA2

-	else, Ncells,r16cap-r16 is the value of pdcch-BlindDetectionCA2 from a combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2) that is provided by pdcch-BlindDetectionCA-CombIndicator

When a UE is configured for NR-DC operation and is provided monitoringCapabilityConfig = r15monitoringcapability for at least one downlink cell and monitoringCapabilityConfig = r16monitoringcapability for at least one downlink cell where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that corresponds to

-	Ncells,r15cap-r16= Ncells, r15MCG downlink cells for the MCG where Ncells, r15MCG is provided by pdcch-BlindDetection3 for the MCG,

-	Ncells,r15cap-r16= Ncells, r15SCG downlink cells for the SCG where Ncells, r15SCG is provided by pdcch-BlindDetection3 for the SCG, and

-	Ncells,r16cap-r16= Ncells, r16MCG downlink cells for the MCG where Ncells, r16MCG is provided by pdcch-BlindDetection2 for the MCG,

-	Ncells,r16cap-r16= Ncells, r16SCG downlink cells for the SCG where Ncells, r16SCG is provided by pdcch-BlindDetection2 for the SCG

When the UE is configured for carrier aggregation operation over more than two downlink cells with at least one downlink cell with monitoringCapabilityConfig = r15monitoringcapability and at least one downlink cell with monitoringCapabilityConfig = r16monitoringcapability, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to

-	monitor per slot a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncells,r15cap-r16, and

-	monitor per span a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncells,r16cap-r16

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection3 and pdcch-BlindDetection2 for the MCG, and pdcch-BlindDetection3 and pdcch-BlindDetection2 for the SCG with values that satisfy

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-BlindDetectionCA, if the UE reports pdcch-BlindDetectionCA1, or

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= NNR-DC,r15DL,cells, if the UE does not report pdcch-BlindDetectionCA1

and

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-BlindDetectionCA2, if the UE reports pdcch-BlindDetectionCA2, or

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= NNR-DC,r16DL,cells, if the UE does not report pdcch-BlindDetectionCA2

When a UE is configured for NR-DC operation and is provided monitoringCapabilityConfig = r15monitoringcapability for at least one downlink cell and monitoringCapabilityConfig = r16monitoringcapability for at least one downlink cell where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE1 and pdcch-BlindDetectionSCG-UE1, respective maximum values for pdcch-BlindDetection3 for the MCG and pdcch-BlindDetection3 for the SCG, and through pdcch-BlindDetectionMCG-UE2 and pdcch-BlindDetectionSCG-UE2 respective maximum values for pdcch-BlindDetection2 for the MCG and pdcch-BlindDetection2 for the SCG.

If the UE reports pdcch-BlindDetectionCA1,

-	the value range of pdcch-BlindDetectionMCG-UE1 or of pdcch-BlindDetectionSCG-UE1 is [0, 1, …, pdcch-BlindDetectionCA1], and

-	pdcch-BlindDetectionMCG-UE1 + pdcch-BlindDetectionSCG-UE1 >= pdcch-BlindDetectionCA1.

Otherwise, if NNR-DC,  max, r15DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value range of pdcch-BlindDetectionMCG-UE1 or of pdcch-BlindDetectionSCG-UE1 is [0, 1, 2],

-	pdcch-BlindDetectionMCG-UE1 + pdcch-BlindDetectionSCG-UE1 >= NNR-DC,  max, r15DL,cells.

If the UE reports pdcch-BlindDetectionCA2

-	the value range of pdcch-BlindDetectionMCG-UE2 or of pdcch-BlindDetectionSCG-UE2 is [0, 1, …, pdcch-BlindDetectionCA2], and

-	pdcch-BlindDetectionMCG-UE2 + pdcch-BlindDetectionSCG-UE2 >= pdcch-BlindDetectionCA2.

Otherwise, if NNR-DC,  max, r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value range of pdcch-BlindDetectionMCG-UE2 or of pdcch-BlindDetectionSCG-UE2 is [0, 1],

-	pdcch-BlindDetectionMCG-UE2 + pdcch-BlindDetectionSCG-UE2 >= NNR-DC,  max, r16DL,cells.

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL Semi-Persistent Scheduling V-RNTI.

For a DL BWP, if a UE is not provided searchSpaceSIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1. If the active DL BWP and the initial DL BWP have same SCS and same CP length and the active DL BWP includes all RBs of the CORESET with index 0, or the active DL BWP is the initial DL BWP, the CORESET configured for Type0-PDCCH CSS set has CORESET index 0 and the Type0-PDCCH CSS set has search space set index 0.

For a DL BWP, if a UE is not provided searchSpaceOtherSystemInformation for Type0A-PDCCH CSS set, the UE does not monitor PDCCH for Type0A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH CSS set are given in Table 10.1-1.

For a DL BWP, if a UE is not provided ra-SearchSpace for Type1-PDCCH CSS set, the UE does not monitor PDCCH for Type1-PDCCH CSS set on the DL BWP. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI and has been provided a Type1-PDCCH CSS set, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set.

If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in Clause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of

-	a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or

-	a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set based on the search space set associated with the value of searchSpaceID.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB, the DM-RS antenna port associated with corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, quasi co-location ‘typeA’ and ‘typeD’ properties, when applicable [6, TS 38.214], if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. A SCS is provided by subCarrierSpacingCommon in MIB.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to ‘typeD’ properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, a CS-RNTI, a SL-RNTI, a SL-CS-RNTI, or a SL Semi-Persistent Scheduling V-RNTI

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | 1 |

For each DL BWP configured to a UE in a serving cell, the UE can be provided by higher layer signalling with

-	P≤3 CORESETs if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided

-	P≤5 CORESETs if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET

For each CORESET, the UE is provided the following by ControlResourceSet:

-	a CORESET index p, by controlResourceSetId or by controlResourceSetId-v1610, where

-	0<p<12 if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided;

-	0<p<16 if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;

-	a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID;

-	a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity;

-	a number of consecutive symbols provided by duration;

-	a set of resource blocks provided by frequencyDomainResources;

-	CCE-to-REG mapping parameters provided by cce-REG-MappingType;

-	an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a respective CORESET;

-	if the UE is provided by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 up to two  lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by TCI-States with same activated tci-StateID value to CORESETs with index p in all configured DL BWPs of all configured cells in a list determined from a serving cell index provided by a MAC CE command

-	an indication for a presence or absence of a transmission configuration indication (TCI) field for a DCI format, other than DCI format 1_0, that schedules PDSCH receptions or indicates SPS PDSCH release and is transmitted by a PDCCH in CORESET p, by tci-PresentInDCI or tci-PresentDCI-1-2.

When precoderGranularity = allContiguousRBs, a UE does not expect

-	to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency

-	any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList, or with any RE of a SS/PBCH block.

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap

-	if a CORESET is not associated with any search space set configured with freqMonitorLocations, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset.

-	if a CORESET is associated with at least one search space set configured with freqMonitorLocations, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set k in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position  RBs0+k,DLstart,μ  [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RBs0+k,DLstart,μ+NRBoffset and k is indicated by freqMonitorLocations if provided for a search space set; otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. If a UE is provided RB sets in the DL BWP, the UE expects that the RBs of the CORESET are within the union of the PRBs in the RB sets of the DL BWP.

For a CORESET other than a CORESET with index 0,

-	if a UE has not been provided a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has been provided initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure;

-	if a UE has been provided a configuration of more than one TCI states by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET as part of Reconfiguration with sync procedure as described in [12, TS 38.331] but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

For a CORESET with index 0, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure.

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI state. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to ‘typeD’ in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH. The active BWP is defined as the active BWP in the slot when the activation command is applied.

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10  search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace:

-	a search space set index s, 0<s<40 , by searchSpaceId

-	an association between the search space set s  and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610

-	a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot

-	a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration

-	a number of PDCCH candidates Ms(L) per CCE aggregation level L by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

-	an indication that search space set s is either a CSS set or a USS set by searchSpaceType

-	if search space set s is a CSS set

-	an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates, or to monitor one PDCCH candidate per RB set if the UE is provided freqMonitorLocations for the search space set, for DCI format 2_0 and a corresponding CCE aggregation level

-	an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1

-	an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2

-	an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3

-	an indication by dci-Format2-4 to monitor PDCCH candidates for DCI format 2_4

-	an indication by dci-Format2-6 to monitor PDCCH candidates for DCI format 2_6

-	if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt- to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

-	a bitmap by freqMonitorLocations, if provided, to indicate an index of one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by RBs0+k,DLstart,μ+NRBoffset, where RBs0+k,DLstart,μ is the index of first common RB of the RB set k [6, TS 38.214], and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. For each RB set with a corresponding value of 1 in the bitmap, the frequency domain resource allocation pattern for the monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration.

If the monitoringSymbolsWithinSlot indicates to a UE to monitor PDCCH in a subset of up to three consecutive symbols that are same in every slot where the UE monitors PDCCH for all search space sets, the UE does not expect to be configured with a PDCCH SCS other than 15 kHz if the subset includes at least one symbol after the third symbol.

A UE does not expect to be provided a first symbol and a number of consecutive symbols for a CORESET that results to a PDCCH candidate mapping to symbols of different slots.

A UE does not expect any two PDCCH monitoring occasions on an active DL BWP, for a same search space set or for different search space sets, in a same CORESET to be separated by a non-zero number of symbols that is smaller than the CORESET duration.

A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf. Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots.

A USS at CCE aggregation level L∈1, 2, 4, 8, 16 is defined by a set of PDCCH candidates for CCE aggregation level L.

If a UE is configured with CrossCarrierSchedulingConfig for a serving cell the carrier indicator field value corresponds to the value indicated by CrossCarrierSchedulingConfig.

For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if the UE is not configured with a carrier indicator field, the UE monitors the PDCCH candidates without carrier indicator field. For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if a UE is configured with a carrier indicator field, the UE monitors the PDCCH candidates with carrier indicator field.

A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell. For the active DL BWP of a serving cell on which the UE monitors PDCCH candidates, the UE monitors PDCCH candidates at least for the same serving cell.

For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCIare given by

L⋅Yp,ns,fμ+ms,nCI⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

where

for any CSS, Yp,ns,fμ=0;

for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537;

i=0,⋯,L-1;

NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set;

nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0;

ms,nCI=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

for any CSS, Ms,maxL=Ms,0L;

for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ;

the RNTI value used for nRNTI is the C-RNTI.

A UE does not expect to be provided freqMonitorLocations for a search space set s in a serving cell if intraCellGuardBandsDL-List indicates that no intra-cell guard-bands are configured for the serving cell.

A UE that

-	is configured for operation with carrier aggregation, and

-	indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and

-	has a PDCCH candidate with CCE aggregation level L in CORESET p for a first DCI format scheduling PUSCH transmission, other than DCI format 0_0, or for a second DCI format scheduling PDSCH reception or SPS PDSCH release, other than DCI format 1_0, having a first size and associated with serving cell nCI,2,

can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level L in CORESET p for a first DCI format or for a second DCI format, respectively, having a second size and associated with serving cell nCI,1 if the first size and the second size are same.

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP.

A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format with CRC scrambled by a SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, P-RNTI, C-RNTI, CS-RNTI, or MCS-RNTI and a DCI format with CRC scrambled by a SL-RNTI or a SL-CS-RNTI for scheduling respective PDSCH and PSSCH receptions on a same serving cell.

A PDCCH candidate with index msj,nCI for a search space set sj using a set of CCEs in a CORESET p on the active DL BWP for serving cell nCI is not counted for monitoring if there is a PDCCH candidate with index msi,nCI for a search space set si<sj, or if there is a PDCCH candidate with index nsj,nCI and nsj,nCI<msj,nCI, in the CORESET p on the active DL BWP for serving cell nCI using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index msj,nCI is counted for monitoring.

Table 10.1-2 provides the maximum number of monitored PDCCH candidates, MPDCCHmax,slot,μ, per slot for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2: Maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of monitored PDCCH candidates per slot and per serving cell MPDCCHmax,slot,μ |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-2A provides the maximum number of monitored PDCCH candidates, MPDCCHmax,X,Y,μ, per span for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2A: Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 14 | 28 | 44 |
| 1 | 12 | 24 | 36 |

Table 10.1-3 provides the maximum number of non-overlapped CCEs, CPDCCHmax,slot,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell.

CCEs for PDCCH candidates are non-overlapped if they correspond to

-	different CORESET indexes, or

-	different first symbols for the reception of the respective PDCCH candidates.

Table 10.1-3: Maximum number CPDCCHmax,slot,μ of non-overlapped CCEs per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of non-overlapped CCEs per slot and per serving cell CPDCCHmax,slot,μ |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

Table 10.1-3A provides the maximum number of non-overlapped CCEs, CPDCCHmax,X,Y,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell.

Table 10.1-3A: Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 18 | 36 | 56 |
| 1 | 18 | 36 | 56 |

If a UE

-	does not report pdcch-BlindDetectionCA or is not provided BDFactorR, γ=R

-	reports pdcch-BlindDetectionCA, the UE can be indicated by BDFactorR either γ=1 or γ=R

If a UE is configured with  Ncells,0DL,μ+Ncells,1DL,μ downlink cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, on the active DL BWPs of the scheduling cells,

-	more than MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,0DL,μ downlink cells, or

-	more than MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=γ∙CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

-	more than MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHmax,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

If a UE

-	is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig,

-	with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and

-	a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell,

the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ downlink cells.

For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot.

For each scheduled cell from the Ncells,1DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell

-	more than minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot

-	more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value

If a UE is configured with Ncells,r16DL,μ  downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ≤Ncellscap-r16,  the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than MPDCCHtotal,(X,Y),μ=MPDCCHmax,(X,Y),μ PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=CPDCCHmax,(X,Y),μ non-overlapped CCEs per span for each scheduled cell when the scheduling cell is from the Ncells,r16DL,(X,Y),μ downlink cells. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs

-	per set of spans on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells within every X symbols, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and any pair of spans in the set is within Y symbols, where first X symbols start at a first symbol with a PDCCH monitoring occasion and next X symbols start at a first symbol with a PDCCH monitoring occasion that is not included in the first X symbols

-	per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise

where Ncells,r16DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

For each scheduled cell from the Ncells,r16DL,(X,Y),μ downlink cells using combination X,Y, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,(X,Y),μ,MPDCCHtotal,(X,Y),μ PDCCH candidates or more than minCPDCCHmax,(X,Y),μ,CPDCCHtotal,(X,Y),μ non-overlapped CCEs per span.

A UE does not expect to be configured CSS sets that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot or per span that exceed the corresponding maximum numbers per slot or per span, respectively.

For same cell scheduling or for cross-carrier scheduling, a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot or per span on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot or per span, respectively. If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, except the first span of each slot, the UE does not expect a number of PDCCH candidates and a number of corresponding non-overlapped CCEs per span on the primary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the primary cell per span.

For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per span or per slot are separately counted for each scheduled cell.

For all search space sets within a slot n or within a span in slot n, denote by  a set of CSS sets with cardinality of  and by  a set of USS sets with cardinality of . The location of USS sets , , in  is according to an ascending order of the search space set index.

Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set .

For the CSS sets, a UE monitors  PDCCH candidates requiring a total of  non-overlapping CCEs in a slot or in a span.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration μ in a slot if the UE is not provided monitoringCapabilityConfig for the primary cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for the primary cell, or in the first span of each slot if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided coresetPoolIndex for first CORESETs, or is provided coresetPoolIndex with value 0 for first CORESETs, and is provided coresetPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(X,Y),μ and CPDCCHmax,(X,Y),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ respectively.

Denote by  the set of non-overlapping CCEs for search space set  and by  the cardinality of  where the non-overlapping CCEs for search space set  are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , .

Set

Set

Set

while  AND

allocate  PDCCH candidates for monitoring to USS set

;

;

;

end while

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to ‘typeD’ properties on active DL BWP(s) of one or more cells

the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs that have been configured with qcl-Type set to same ‘typeD’ properties as the CORESET, on the active DL BWP of a cell from the one or more cells

-	the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

-	for the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL ‘typeD’ properties than a CSI-RS

-	for the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL ‘typeD’ properties

-	the allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells

-	the number of active TCI states is determined from the multiple CORESETs

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where none of the CORESETs has TCI-states configured with qcl-Type set to ‘typeD’,

the UE is required to monitor PDCCH candidates in overlapping PDCCH monitoring occasions for search space sets associated with different CORESETs.

For a scheduled cell and at any time, a UE expects to have received at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI scheduling 16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol and at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI scheduling 16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol.

If a UE is not provided monitoringCapabilityConfig, and

-	is not configured for NR-DC operation and indicates through pdcch-BlindDetectionCA or pdcch-MonitoringCA a capability to monitor PDCCH candidates for Ncellscap≥4 downlink cells and the UE is configured with NcellsDL>4 downlink cells or NcellsUL>4 uplink cells, or

-	is configured with NR-DC operation and for a cell group with NcellsDL downlink cells or NcellsUL uplink cells

the UE expects to have respectively received at most 16∙Ncellscap PDCCHs for

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all NcellsDL downlink cells

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all NcellsUL uplink cells

If a UE

-	is configured to monitor a first PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a CSS set and a second PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a USS set in a CORESET with index zero on an active DL BWP, and

-	the DCI formats 0_0/1_0 associated with the first PDCCH candidate and the DCI formats 0_0/1_0 associated with the second PDCCH candidate have same size, and

-	the UE receives the first PDCCH candidate and the second PDCCH candidate over a same set of CCEs, and

-	the first PDCCH candidate and the second PDCCH candidate have identical scrambling, and

-	the DCI formats 0_0/1_0 for the first PDCCH candidate and the DCI formats 0_0/1_0 for the second PDCCH candidate have CRC scrambled by either C-RNTI, or MCS-C-RNTI, or CS-RNTI

the UE decodes only the DCI formats 0_0/1_0 associated with the first PDCCH candidate.

If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.

A UE configured with a bandwidth part indicator in a DCI format determines, in case of an active DL BWP or of an active UL BWP change, that the information in the DCI format is applicable to the new active DL BWP or UL BWP, respectively, as described in Clause 12.

For unpaired spectrum operation, if a UE is not configured for PUSCH/PUCCH transmission on serving cell c2, the UE does not expect to monitor PDCCH on serving cell c1 if the PDCCH overlaps in time with SRS transmission (including any interruption due to uplink or downlink RF retuning time [10, TS 38.133]) on serving cell c2 and if the UE is not capable of simultaneous reception and transmission on serving cell c1and serving cell c2.

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception as described in [6, TS 38.214]. If a PDCCH candidate in a slot is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.

## 10.2	PDCCH validation for DL SPS and UL grant Type 2

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field in the DCI format for the enabled transport block is set to '0', and

-	the DFI flag field, if present, in the DCI format is set to '0', and

-	if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK-r16.

If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3.

If a UE is provided more than one configuration for UL grant Type 2 PUSCH or for SPS PDSCH

-	if the UE is provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList, a value of the HARQ process number field in a DCI format indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations

-	if the UE is not provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-4.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2 | DCI format 1_1 |
| --- | --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-4: Special fields for a single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after N symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release, N=5 for μ=0, N=5.5 for μ=1, and N=11 for μ=2, otherwise, N=10 for μ=0, N=12 for μ=1, N=22 for μ=2, and N=25 for μ=3, wherein μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

## 10.2A	PDCCH validation for SL configured grant Type 2

A UE validates, for scheduling activation or scheduling release, a SL configured grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format 3_0 is scrambled with a SL-CS-RNTI provided by sl-CS-RNTI, and

-	the new data indicator field in the DCI format 3_0 for the enabled transport block is set to '0'

Validation of the DCI format 3_0 is achieved if all fields for the DCI format 3_0 are set according to Table 10.2A-1 or Table 10.2A-2.

If validation is achieved, the UE considers the information in the DCI format 3_0 as a valid activation or valid release of SL configured grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format 3_0.

Table 10.2A-1: Special fields for SL configured grant Type 2 scheduling activation PDCCH validation

|  | DCI format 3_0 |
| --- | --- |
| HARQ process number | set to all '0's |

Table 10.2A-2: Special fields for SL configured grant Type 2 scheduling release PDCCH validation

|  | DCI format 3_0 |
| --- | --- |
| HARQ process number | set to all '1's |
| Frequency resource assignment (if present) | set to all '1's |

## 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1

-	a payload size for DCI format 2_6 by size-DCI-2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by ps-PositionDCI-2-6

-	a '0' value for the Wake-up indication bit, when reported to higher layers, indicates to not start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a '1' value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a bitmap, when the UE is provided a number of groups of configured SCells by dormancyGroupOutsideActiveTime, where

-	the bitmap location is immediately after the Wake-up indication bit location

-	the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDurationTimer would start on the PCell or on the SpCell [11, TS 38.321]

-	for each search space set, the PDCCH monitoring occasions are the ones in the first Ts slots indicated by duration, or Ts=1 slot if duration is not provided, starting from the first slot of the first Ts slots and ending prior to the start of drx-onDurationTimer.

On PDCCH monitoring occasions associated with a same long DRX Cycle, a UE does not expect to detect more than one DCI format 2_6 with different values of the Wake-up indication bit for the UE or with different values of the bitmap for the UE.

The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321].

If a UE reports for an active DL BWP a MinTimeGap value that is X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the MinTimeGap value of the SCS of the active DL BWP in Table 10.3-1.

Table 10.3-1 Minimum time gap value X

| SCS (kHz) | Minimum Time Gap X (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [11, TS 38.321] for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report a value of the Wake-up indication bit to higher layers for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

-	is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or

-	does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle

the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field,

-	the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime,

-	each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells

-	if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if

-	the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if

-	a one-shot HARQ-ACK request field is not present or has a '0' value, and if

-	the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1

the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of

-	modulation and coding scheme

-	new data indicator

-	redundancy version

and of

-	HARQ process number

-	antenna port(s)

-	DMRS sequence initialization

as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

If an active DL BWP provided by dormantBWP-Id for a UE on an activated SCell is not a default DL BWP for the UE on the activated SCell, as described in Clause 12, the BWP inactivity timer is not used for transitioning from the active DL BWP provided by dormantBWP-Id to the default DL BWP on the activated SCell.

A UE is expected to provide HARQ-ACK information in response to a detection of a DCI format 1_1 indicating SCell dormancy after N symbols from the last symbol of a PDCCH providing the DCI format 1_1. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the DCI format 1_1, N=7 for μ=0, N=7.5 for μ=1, and N=15 for μ=2; otherwise, N=14 for μ=0, N=16 for μ=1, N=27 for μ=2, and N=31 for μ=3, where μ is the smallest SCS configuration between the SCS configuration of the PDCCH providing the DCI format 1_1 and the SCS configuration of a PUCCH with the HARQ-ACK information in response to the detection of the DCI format 1_1.

## 10.4	Search space set group switching

A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set.

If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList.

When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList.

A UE can be provided by searchSpaceSwitchDelay a number of symbols Pswitch where a minimum value of  Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ. UE processing capability 1 for SCS configuration μ applies unless the UE indicates support for UE processing capability 2.

Table 10.4-1: Minimum value of Pswitch [symbols]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |

A UE can be provided, by searchSpaceSwitchTimer, a timer value for a serving cell that the UE is provided searchSpaceGroupIdList or, if provided, for a set of serving cells provided by cellGroupsForSwitchList. The UE decrements the timer value by one after each slot based on a reference SCS configuration that is the smallest SCS configuration μ among all configured DL BWPs in the serving cell, or in the set of serving cells. The UE maintains the reference SCS configuration during the timer decrement procedure.

If a UE is provided by SearchSpaceSwitchTrigger a location of a search space set group switching flag field for a serving cell in a DCI format 2_0, as described in Clause 11.1.1;

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and the UE sets the timer value to the value provided by searchSpaceSwitchTimer

-	if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

If a UE is not provided SearchSpaceSwitchTrigger for a serving cell,

-	if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format, the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set

-	if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

A UE determines a slot and a symbol in the slot to start or stop PDCCH monitoring according to search space sets for a serving cell that the UE is provided searchSpaceGroupIdList or, if cellGroupsForSwitchList is provided, for a set of serving cells, based on the smallest SCS configuration μ among all configured DL BWPs in the serving cell or in the set of serving cells and, if any, in the serving cell where the UE receives a PDCCH and detects a corresponding DCI format 2_0 triggering the start or stop of PDCCH monitoring according to search space sets.

## 10.5	HARQ-ACK information for PUSCH transmissions

A UE can be configured a number of search space sets to monitor PDCCH for detecting a DCI format 0_1 with a DFI flag field and CRC scrambled with a CS-RNTI provided by cs-RNTI. The UE determines that the DCI format provides HARQ-ACK information for PUSCH transmissions based on when a DFI flag field value is set to '1', if a PUSCH transmission is configured by ConfiguredGrantConfig.

The HARQ-ACK information corresponds to transport blocks in PUSCH transmissions for all HARQ processes for a serving cell of a PDCCH reception that provides DCI format 0_1 or, if DCI format 0_1 includes a carrier indicator field, for a serving cell indicated by a value of the carrier indicator field.

For a PUSCH transmission configured by ConfiguredGrantConfig, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission, or of any repetition of the PUSCH transmission, by a number of symbols provided by cg-minDFIDelay.

For an initial transmission by a UE of a transport block in a PUSCH configured by ConfiguredGrantConfig, if the UE receives a CG-DFI that provides HARQ-ACK information for the transport block, the UE assumes that the transport block was correctly decoded if the HARQ-ACK information value is ACK; otherwise, the UE assumes that the transport block was not correctly decoded.

For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission by a number of symbols provided by cg-minDFIDelay or, if the PUSCH transmission is over multiple slots,

-	after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay, if a value of the HARQ-ACK information is ACK.

-	after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay, if a value of the HARQ-ACK information is NACK.

# 11	UE-group common signalling

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

## 11.1	Slot configuration

A slot format includes downlink symbols, uplink symbols, and flexible symbols.

The following are applicable for each serving cell.

If a UE is provided tdd-UL-DL-ConfigurationCommon, the UE sets the slot format per slot over a number of slots as indicated by tdd-UL-DL-ConfigurationCommon.

The tdd-UL-DL-ConfigurationCommon provides

-	a reference SCS configuration  by referenceSubcarrierSpacing

-	a pattern1.

The pattern1 provides

-	a slot configuration period of  msec by dl-UL-TransmissionPeriodicity

-	a number of slots  with only downlink symbols by nrofDownlinkSlots

-	a number of downlink symbols  by nrofDownlinkSymbols

-	a number of slots  with only uplink symbols by nrofUplinkSlots

-	a number of uplink symbols  by nrofUplinkSymbols

A value  msec is valid only for .  A value  msec is valid only for  or .  A value  msec is valid only for , or , or .

A slot configuration period of  msec includes  slots with SCS configuration . From the  slots, a first  slots include only downlink symbols and a last  slots include only uplink symbols. The  symbols after the first  slots are downlink symbols. The  symbols before the last  slots are uplink symbols. The remaining  are flexible symbols.

The first symbol every  periods is a first symbol in an even frame.

If tdd-UL-DL-ConfigurationCommon provides both pattern1 and pattern2, the UE sets the slot format per slot over a first number of slots as indicated by pattern1 and the UE sets the slot format per slot over a second number of slots as indicated by pattern2.

The pattern2 provides

-	a slot configuration period of  msec by dl-UL-TransmissionPeriodicity

-	a number of slots  with only downlink symbols by nrofDownlinkSlots

-	a number of downlink symbols  by nrofDownlinkSymbols

-	a number of slots  with only uplink symbols by nrofUplinkSlots

-	a number of uplink symbols  by nrofUplinkSymbols

The applicable values of  are same as the applicable values for .

A slot configuration period of  msec includes first  slots and second  slots.

From the  slots, a first  slots include only downlink symbols and a last  include only uplink symbols. The  symbols after the first  slots are downlink symbols. The  symbols before the last  slots are uplink symbols. The remaining  are flexible symbols.

A UE expects that  divides 20 msec.

The first symbol every  periods is a first symbol in an even frame.

A UE expects that the reference SCS configuration  is smaller than or equal to a SCS configuration  for any configured DL BWP or UL BWP. Each slot provided by pattern1 or pattern2 is applicable to  consecutive slots in the active DL BWP or the active UL BWP where the first slot starts at a same time as a first slot for the reference SCS configuration  and each downlink or flexible or uplink symbol for the reference SCS configuration  corresponds to  consecutive downlink or flexible or uplink symbols for the SCS configuration .

If the UE is additionally provided tdd-UL-DL-ConfigurationDedicated, the parameter tdd-UL-DL-ConfigurationDedicated overrides only flexible symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

The tdd-UL-DL-ConfigurationDedicated provides

-	a set of slot configurations by slotSpecificConfigurationsToAddModList

-	for each slot configuration from the set of slot configurations

-	a slot index for a slot provided by slotIndex

-	a set of symbols for a slot by symbols where

-	if symbols = allDownlink, all symbols in the slot are downlink

-	if symbols = allUplink, all symbols in the slot are uplink

-	if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible

For each slot having a corresponding index provided by slotIndex, the UE applies a format provided by a corresponding symbols. The UE does not expect tdd-UL-DL-ConfigurationDedicated to indicate as uplink or as downlink a symbol that tdd-UL-DL-ConfigurationCommon indicates as a downlink or as an uplink symbol, respectively.

For each slot configuration provided by tdd-UL-DL-ConfigurationDedicated, a reference SCS configuration is the reference SCS configuration  provided by tdd-UL-DL-ConfigurationCommon.

A slot configuration period and a number of downlink symbols, uplink symbols, and flexible symbols in each slot of the slot configuration period are determined from tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated and are common to each configured BWP.

A UE considers symbols in a slot indicated as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated to be available for receptions and considers symbols in a slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated to be available for transmissions.

If a UE is not configured to monitor PDCCH for DCI format 2_0, for a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE

-	the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format

-	the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.

For operation with shared spectrum channel access, if a UE is provided csi-RS-ValidationWith-DCI, is not provided CO-DurationsPerCell, and is not provided SlotFormatCombinationsPerCell, and if the UE is configured by higher layers to receive a CSI-RS in a set of symbols of a slot, the UE cancels the CSI-RS reception in the set of symbols of the slot if the UE does not detect a DCI format indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot.

If a UE is provided channelAccessMode ='dynamic' and is provided availableRB-SetsToAddModList and availableRB-SetsToRelease, the UE expects to be provided co-DurationsPerCellToAddModList and co-DurationsPerCellToReleaseList and/or slotFormatCombToAddModList and slotFormatCombToReleaseList.

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format, after a number of symbols that is smaller than the PUSCH preparation time Tproc,2 for the corresponding UE processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0

-	the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6. TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols

For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot.

For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive DL PRS in the set of symbols of the slot, if the UE is not provided with a measurement gap.

For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.

For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot.

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable', and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells.

For a set of symbols of a slot corresponding to a valid PRACH occasion and  symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

If a UE is scheduled by a DCI format to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot.

If a UE is scheduled by a DCI format to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot.

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable', and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

the UE determines a reference cell for a symbol as an active cell with the smallest cell index among serving cells where the symbol is configured as

-	downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated

-	uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

-	downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol

If a UE

-	is configured with multiple serving cells in a frequency band and is provided half-duplex-behavior = 'enable', and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE does not expect

-	a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated,

-	tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and

-	to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell.

If the reference cell and another cell for a UE operate in different frequency bands and if the UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable', and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

the UE

-	UE assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively,

-	transmits a signal/channel scheduled by a DCI format on a symbol of the other cell when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell,

-	is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on the other cell.

If a UE

-	is configured with multiple serving cells and is provided half-duplex-behavior = 'enable', and

-	is not capable of simultaneous transmission and reception on any cell from the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

the UE

-	does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell

-	does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell

-	does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell

-	does not transmit a SRS that is configured by higher layers on a set of symbols on another cell if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell

-	does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell

-	assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on another cell to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell

-	does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively

### 11.1.1	UE procedure for determining slot format

This Clause applies for a serving cell that is included in a set of serving cells configured to a UE by slotFormatCombToAddModList and slotFormatCombToReleaseList, availableRB-SetsToAddModList and availableRB-SetsToRelease, switchTriggerToAddModList and switchTriggerToReleaseList, or co-DurationsPerCellToAddModList and co-DurationsPerCellToReleaseList.

If a UE is configured by higher layers with parameter SlotFormatIndicator, the UE is provided an SFI-RNTI by sfi-RNTI and with a payload size of DCI format 2_0 by dci-PayloadSize.

The UE is also provided in one or more serving cells with a configuration for a search space set  and a corresponding CORESET p for monitoring Mp,sLSFI PDCCH candidates for DCI format 2_0 with a CCE aggregation level of LSFI CCEs as described in Clause 10.1. The Mp,sLSFI PDCCH candidates are the first Mp,sLSFI PDCCH candidates for CCE aggregation level LSFI for search space set s in CORESET p.

For each serving cell in the set of serving cells, the UE can be provided:

-	an identity of the serving cell by servingCellId

-	a location of a SFI-index field in DCI format 2_0 by positionInDCI

-	a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes

-	one or more slot formats indicated by a respective slotFormats for the slot format combination, and

-	a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId

-	for unpaired spectrum operation, a reference SCS configuration μSFI by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration μSFI,SUL by subcarrierSpacing2 for the supplementary UL carrier

-	for paired spectrum operation, a reference SCS configuration μSFI,DL for a DL BWP by subcarrierSpacing and a reference SCS configuration μSFI,UL for an UL BWP by subcarrierSpacing2

-	a location of an available RB set indicator field in DCI format 2_0 that is

-	one bit, if intraCellGuardBandsDL-List for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, by availableRB-SetsPerCell, and the serving cell remains available or unavailable for reception until the end of the indicated channel occupancy duration

-	a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandsDL-List for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetsPerCell and a RB set remains available or unavailable for receptions until the end of the indicated channel occupancy duration

-	a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationsPerCell, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from co-DurationList. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by co-DurationList. If CO-DurationsPerCell is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

-	a reference SCS configuration for co-DurationList, by subcarrierSpacing-r16

-	a location of a search space set group switching flag field in DCI format 2_0, by SearchSpaceSwitchTrigger, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling for the serving cell or the set of serving cells, provided by CellGroupsForSwitching, as described in Clause 10.4.

A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots for each DL BWP or each UL BWP starting from a slot where the UE detects the DCI format 2_0. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_0. The SFI-index field includes  bits where maxSFIindex is the maximum value of the values provided by corresponding slotFormatCombinationId. A slot format is identified by a corresponding format index as provided in Table 11.1.1-1 where 'D' denotes a downlink symbol, 'U' denotes an uplink symbol, and 'F' denotes a flexible symbol.

If a PDCCH monitoring periodicity for DCI format 2_0, provided to a UE for the search space set  by monitoringSlotPeriodicityAndOffset, is smaller than a duration of a slot format combination the UE obtains at a PDCCH monitoring occasion for DCI format 2_0 by a corresponding SFI-index field value, and the UE detects more than one DCI formats 2_0 indicating a slot format for a slot, the UE expects each of the more than one DCI formats 2_0 to indicate a same format for the slot.

A UE does not expect to be configured to monitor PDCCH for DCI format 2_0 on a second serving cell that uses larger SCS than the serving cell.

Table 11.1.1-1: Slot formats for normal cyclic prefix

| Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 0 | D | D | D | D | D | D | D | D | D | D | D | D | D | D |
| 1 | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 2 | F | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 3 | D | D | D | D | D | D | D | D | D | D | D | D | D | F |
| 4 | D | D | D | D | D | D | D | D | D | D | D | D | F | F |
| 5 | D | D | D | D | D | D | D | D | D | D | D | F | F | F |
| 6 | D | D | D | D | D | D | D | D | D | D | F | F | F | F |
| 7 | D | D | D | D | D | D | D | D | D | F | F | F | F | F |
| 8 | F | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 9 | F | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 10 | F | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 11 | F | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 12 | F | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 13 | F | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 14 | F | F | F | F | F | U | U | U | U | U | U | U | U | U |
| 15 | F | F | F | F | F | F | U | U | U | U | U | U | U | U |
| 16 | D | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 17 | D | D | F | F | F | F | F | F | F | F | F | F | F | F |
| 18 | D | D | D | F | F | F | F | F | F | F | F | F | F | F |
| 19 | D | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 20 | D | D | F | F | F | F | F | F | F | F | F | F | F | U |
| 21 | D | D | D | F | F | F | F | F | F | F | F | F | F | U |
| 22 | D | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 23 | D | D | F | F | F | F | F | F | F | F | F | F | U | U |
| 24 | D | D | D | F | F | F | F | F | F | F | F | F | U | U |
| 25 | D | F | F | F | F | F | F | F | F | F | F | U | U | U |
| 26 | D | D | F | F | F | F | F | F | F | F | F | U | U | U |
| 27 | D | D | D | F | F | F | F | F | F | F | F | U | U | U |
| 28 | D | D | D | D | D | D | D | D | D | D | D | D | F | U |
| 29 | D | D | D | D | D | D | D | D | D | D | D | F | F | U |
| 30 | D | D | D | D | D | D | D | D | D | D | F | F | F | U |
| 31 | D | D | D | D | D | D | D | D | D | D | D | F | U | U |
| 32 | D | D | D | D | D | D | D | D | D | D | F | F | U | U |
| 33 | D | D | D | D | D | D | D | D | D | F | F | F | U | U |
| 34 | D | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 35 | D | D | F | U | U | U | U | U | U | U | U | U | U | U |
| 36 | D | D | D | F | U | U | U | U | U | U | U | U | U | U |
| 37 | D | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 38 | D | D | F | F | U | U | U | U | U | U | U | U | U | U |
| 39 | D | D | D | F | F | U | U | U | U | U | U | U | U | U |
| 40 | D | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 41 | D | D | F | F | F | U | U | U | U | U | U | U | U | U |
| 42 | D | D | D | F | F | F | U | U | U | U | U | U | U | U |
| 43 | D | D | D | D | D | D | D | D | D | F | F | F | F | U |
| 44 | D | D | D | D | D | D | F | F | F | F | F | F | U | U |
| 45 | D | D | D | D | D | D | F | F | U | U | U | U | U | U |
| 46 | D | D | D | D | D | F | U | D | D | D | D | D | F | U |
| 47 | D | D | F | U | U | U | U | D | D | F | U | U | U | U |
| 48 | D | F | U | U | U | U | U | D | F | U | U | U | U | U |
| 49 | D | D | D | D | F | F | U | D | D | D | D | F | F | U |
| 50 | D | D | F | F | U | U | U | D | D | F | F | U | U | U |
| 51 | D | F | F | U | U | U | U | D | F | F | U | U | U | U |
| 52 | D | F | F | F | F | F | U | D | F | F | F | F | F | U |
| 53 | D | D | F | F | F | F | U | D | D | F | F | F | F | U |
| 54 | F | F | F | F | F | F | F | D | D | D | D | D | D | D |
| 55 | D | D | F | F | F | U | U | U | D | D | D | D | D | D |
| 56 – 254 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 | UE determines the slot format for the slot based on tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, if any, on detected DCI formats |  |  |  |  |  |  |  |  |  |  |  |  |  |

For unpaired spectrum operation for a UE on a serving cell, the UE is provided by subcarrierSpacing a reference SCS configuration μSFI for each slot format in a combination of slot formats indicated by an SFI-index field value in DCI format 2_0. The UE expects that for a reference SCS configuration μSFI and for an active DL BWP or an active UL BWP with SCS configuration μ, it is μ≥μSFI. Each slot format in the combination of slot formats indicated by the SFI-index field value in DCI format 2_0 is applicable to 2μ-μSFI consecutive slots in the active DL BWP or the active UL BWP where the first slot starts at a same time as a first slot for the reference SCS configuration μSFI and each downlink or flexible or uplink symbol for the reference SCS configuration μSFI corresponds to  consecutive downlink or flexible or uplink symbols for the SCS configuration μ.

For paired spectrum operation for a UE on a serving cell, the SFI-index field in DCI format 2_0 indicates a combination of slot formats that includes a combination of slot formats for a reference DL BWP and a combination of slot formats for a reference UL BWP of the serving cell. The UE is provided by subcarrierSpacing a reference SCS configuration μSFI,DL for the combination of slot formats indicated by the SFI-index field value in DCI format 2_0 for the reference DL BWP of the serving cell. The UE is provided by subcarrierSpacing2 a reference SCS configuration μSFI,UL for the combination of slot formats indicated by the SFI-index field value in DCI format 2_0 for the reference UL BWP of the serving cell. If μSFI,DL≥μSFI,UL and for each 2μSFI,DL-μSFI,UL+1 values provided by a value of slotFormats, where the value of slotFormats is determined by a value of slotFormatCombinationId in slotFormatCombination and the value of slotFormatCombinationId is set by the value of the SFI-index field value in DCI format 2_0, the first 2μSFI,DL-μSFI,UL values for the combination of slot formats are applicable to the reference DL BWP and the next value is applicable to the reference UL BWP. If μSFI,DL<μSFI,UL and for each 2μSFI,UL-μSFI,DL+1 values provided by slotFormats, the first value for the combination of slot formats is applicable to the reference DL BWP and the next 2μSFI,UL-μSFI,DL values are applicable to the reference UL BWP.

The UE is provided a reference SCS configuration μSFI,DL so that for an active DL BWP with SCS configuration μDL, it is μDL≥μSFI,DL. The UE is provided a reference SCS configuration μSFI,UL so that for an active UL BWP with SCS configuration μUL, it is μUL≥μSFI,UL. Each slot format for a combination of slot formats indicated by the SFI-index field value in DCI format 2_0 for the reference DL BWP, by indicating a value for slotFormatCombinationId that is mapped to a value of slotFormats in slotFormatCombination, is applicable to 2μDL-μSFI,DL consecutive slots for the active DL BWP where the first slot starts at a same time as a first slot in the reference DL BWP and each downlink or flexible symbol for the reference SCS configuration μSFI,DL corresponds to 2μDL-μSFI,DL consecutive downlink or flexible symbols for the SCS configuration μDL. Each slot format for the combination of slot formats for the reference UL BWP is applicable to 2μUL-μSFI,UL consecutive slots for the active UL BWP where the first slot starts at a same time as a first slot in the reference UL BWP and each uplink or flexible symbol for the reference SCS configuration μSFI,UL corresponds to 2μUL-μSFI,UL consecutive uplink or flexible symbols for the SCS configuration μUL.

For unpaired spectrum operation with a second UL carrier for a UE on a serving cell, the SFI-index field value in DCI format 2_0 indicates a combination of slot formats that includes a combination of slot formats for a reference first UL carrier of the serving cell and a combination of slot formats for a reference second UL carrier of the serving cell. The UE is provided by subcarrierSpacing a reference SCS configuration μSFI for the combination of slot formats indicated by the SFI-index field in DCI format 2_0 for the reference first UL carrier of the serving cell. The UE is provided by subcarrierSpacing2 a reference SCS configuration μSFI,SUL for the combination of slot formats indicated by the SFI-index field value in DCI format 2_0 for the reference second UL carrier of the serving cell. For each 2μSFI-μSFI,SUL+1 values of slotFormats, the first 2μSFI-μSFI,SUL values for the combination of slot formats are applicable to the reference first UL carrier and the next value is applicable to the reference second UL carrier.

The UE expects to be provided a reference SCS configuration μSFI,SUL so that for an active UL BWP in the second UL carrier with SCS configuration μSUL, it is μSUL≥μSFI,SUL. Each slot format for a combination of slot formats indicated by the SFI-index field in DCI format 2_0 for the reference first UL carrier is applicable to 2μ-μSFI consecutive slots for the active DL BWP and the active UL BWP in the first UL carrier where the first slot starts at a same time as a first slot in the reference first UL carrier. Each slot format for the combination of slot formats for the reference second UL carrier is applicable to 2μSUL-μSFI,SUL consecutive slots for the active UL BWP in the second UL carrier where the first slot starts at a same time as a first slot in the reference second UL carrier.

If a BWP in the serving cell is configured with μ=2 and with extended CP, the UE expects μSFI=0, μSFI=1, or μSFI=2. A format for a slot with extended CP is determined from a format for a slot with normal CP. A UE determines an extended CP symbol to be a downlink/uplink/flexible symbol if the overlapping normal CP symbols that are downlink/uplink/flexible symbols, respectively. A UE determines an extended CP symbol to be a flexible symbol if one of the overlapping normal CP symbols is flexible. A UE determines an extended CP symbol to be a flexible symbol if the pair of the overlapping normal CP symbols includes a downlink and an uplink symbol.

A reference SCS configuration μSFI, or μSFI,DL, or μSFI,UL, or μSFI,SUL is either 0, or 1, or 2 for FR1 and is either 2 or 3 for FR2.

For a set of symbols of a slot, a UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink and to detect a DCI format indicating to the UE to receive PDSCH or CSI-RS in the set of symbols of the slot.

For a set of symbols of a slot, a UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols in the slot as downlink and to detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot.

For a set of symbols of a slot that are indicated by a DCI format 2_0 as being within a remaining channel occupancy duration either by a channel occupancy duration field or by an SFI-index field, a UE does not expect to detect at a later time a DCI format 2_0 indicating, either by a channel occupancy duration field or by an SFI-index field, that any symbol from the set of symbols is not within a remaining channel occupancy duration.

For a set of symbols of a slot that are indicated as downlink/uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink/downlink, respectively, or as flexible.

For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in Clause 4.1, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

For a set of symbols of a slot corresponding to a valid PRACH occasion and Ngap symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as downlink.

For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255

-	if one or more symbols from the set of symbols are symbols in a CORESET configured to the UE for PDCCH monitoring, the UE receives PDCCH in the CORESET only if an SFI-index field value in DCI format 2_0 indicates that the one or more symbols are downlink symbols

-	if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format indicating to the UE to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives PDSCH or CSI-RS in the set of symbols of the slot

-	if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot the UE transmits the PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot

-	if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible, and the UE does not detect a DCI format indicating to the UE to receive PDSCH or CSI-RS, or the UE does not detect a DCI format, a RAR UL, fallbackRAR UL grant, or successRAR grant indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot, the UE does not transmit or receive in the set of symbols of the slot

-	if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink

-	if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink or flexible.

-	if the UE is configured by higher layers to transmit PUCCH, or PUSCH, or PRACH in the set of symbols of the slot, the UE transmits the PUCCH, or the PUSCH, or the PRACH in the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as uplink

-	if the UE is configured by higher layers to transmit SRS in the set of symbols of the slot, the UE transmits the SRS only in a subset of symbols from the set of symbols of the slot indicated as uplink symbols by an SFI-index field value in DCI format 2_0

-	a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot

-	a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink or flexible if the set of symbols of the slot includes symbols corresponding to any repetition of a PUSCH transmission activated by an UL Type 2 grant PDCCH as described in Clause 10.2

-	a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as uplink and also detect a DCI format indicating to the UE to receive PDSCH or CSI-RS in one or more symbols from the set of symbols of the slot

If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.

For operation with shared spectrum channel access, if a UE is configured by higher layers to receive a CSI-RS and the UE is provided CO-DurationsPerCell, for a set of symbols of a slot that are indicated as downlink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided, the UE cancels the CSI-RS reception in the set of symbols of the slot that are not within the indicated remaining channel occupancy duration.

If a UE is configured by higher layers to receive a DL PRS in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the DL PRS reception in the set of symbols of the slot.

If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format, after a number of symbols that is smaller than the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 or the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0

-	the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6. TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols.

If a UE is configured by higher layers to receive a CSI-RS or detects a DCI format 0_1 indicating to the UE to receive a CSI-RS in one or more RB sets and a set of symbols of a slot, and the UE detects a DCI format 2_0 with bitmap indicating that any RB set from the one or more RB sets is not available for reception, the UE cancels the CSI-RS reception in the set of symbols of the slot.

A UE assumes that flexible symbols in a CORESET configured to the UE for PDCCH monitoring are downlink symbols if the UE does not detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as flexible or uplink and the UE does not detect a DCI format indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH in the set of symbols.

For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot

-	the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format

-	the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR

-	the UE receives PDCCH as described in Clause 10.1

-	if the UE is configured by higher layers to receive PDSCH in the set of symbols of the slot, the UE does not receive the PDSCH in the set of symbols of the slot

-	if the UE is configured by higher layers to receive CSI-RS in the set of symbols of the slot, the UE does not receive the CSI-RS in the set of symbols of the slot, except when UE is provided CO-DurationsPerCell and the set of symbols of the slot are within the indicated remaining channel occupancy duration.

-	if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS

-	if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided enableConfiguredUL, the UE

-	does not transmit the PUCCH, or the PUSCH, or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is after PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0

-	does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is after the PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0

-	if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is provided EnableConfiguredUL, the UE can transmit the SRS, or PUCCH, or PUSCH, or PRACH, respectively.

For unpaired spectrum operation for a UE on a cell in a frequency band of FR1, and when the scheduling restrictions due to RRM measurements [10, TS 38.133] are not applicable, if the UE detects a DCI format indicating to the UE to transmit in a set of symbols, the UE is not required to perform RRM measurements [10, TS 38.133] based on a SS/PBCH block or CSI-RS reception on a different cell in the frequency band if the SS/PBCH block or CSI-RS reception includes at least one symbol from the set of symbols.

## 11.2	Interrupted transmission indication

If a UE is provided DownlinkPreemption, the UE is configured with an INT-RNTI provided by int-RNTI for monitoring PDCCH conveying DCI format 2_1 [5, TS 38.212]. The UE is additionally configured with

-	a set of serving cells by int-ConfigurationPerServingCell that includes a set of serving cell indexes provided by corresponding servingCellId and a corresponding set of locations for fields in DCI format 2_1 by positionInDCI

-	an information payload size for DCI format 2_1 by dci-PayloadSize

-	an indication granularity for time-frequency resources by timeFrequencySet

If a UE detects a DCI format 2_1 for a serving cell from the configured set of serving cells, the UE may assume that no transmission to the UE is present in PRBs and in symbols that are indicated by the DCI format 2_1, from a set of PRBs and a set of symbols of the last monitoring period. The indication by the DCI format 2_1 is not applicable to receptions of SS/PBCH blocks.

The set of PRBs is equal to the active DL BWP as defined in Clause 12 and includes  PRBs.

If a UE detects a DCI format 2_1 in a PDCCH transmitted in a CORESET in a slot, the set of symbols is the last  symbols prior to the first symbol of the CORESET in the slot where  is the PDCCH monitoring periodicity provided by the value of monitoringSlotPeriodicityAndOffset, as described in Clause 10.1,  is the number of symbols per slot,  is the SCS configuration for a serving cell with mapping to a respective field in the DCI format 2_1,  is the SCS configuration of the DL BWP where the UE receives the PDCCH with the DCI format 2_1. If the UE is provided tdd-UL-DL-ConfigurationCommon, symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon are excluded from the last  symbols prior to the first symbol of the CORESET in the slot. The resulting set of symbols includes a number of symbols that is denoted as .

The UE does not expect to be provided values of , , and  resulting to a value of  that is not an integer. The UE does not expect to be configured by monitoringSymbolsWithinSlot with more than one PDCCH monitoring occasion for DCI format 2_1 in a slot.

A UE is provided the indication granularity for the set of PRBs and for the set of symbols by timeFrequencySet.

If the value of timeFrequencySet is 'set0', 14 bits from MSB of a field in DCI format 2_1 have a one-to-one mapping with 14 groups of consecutive symbols from the set of symbols where each of the first  symbol groups includes  symbols, each of the last  symbol groups includes  symbols, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group.

If the value of timeFrequencySet is 'set1', 7 pairs of bits from MSB of a field in the DCI format 2_1 have a one-to-one mapping with 7 groups of consecutive symbols where each of the first  symbol groups includes  symbols, each of the last  symbol groups includes  symbols, a first bit in a pair of bits for a symbol group is applicable to the subset of first  PRBs from the set of  PRBs, a second bit in the pair of bits for the symbol group is applicable to the subset of last  PRBs from the set of  PRBs, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and subset of PRBs, and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group and subset of PRBs.

## 11.2A	Cancellation indication

If a UE is provided UplinkCancellation, the UE is provided, in one or more serving cells, a search space set for monitoring the first PDCCH candidate with a CCE aggregation level of LCI CCEs of the search space set for detection of a DCI format 2_4 [5, TS 38.212] with a CI-RNTI provided by ci-RNTI as described in Clause 10.1. UplinkCancellation additionally provides to the UE

-	a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI

-	a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier, if the serving cell is configured with a SUL carrier

-	an information payload size for DCI format 2_4 by dci-PayloadSizeForCI

-	an indication for time-frequency resources by timeFrequencyRegion

For a serving cell having an associated field in a DCI format 2_4, for the field denote by

-	NCI a number of bits provided by ci-PayloadSize

-	BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion

-	TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, from a number of symbols that

-	is provided by timeDurationforCI in timeFrequencyRegion, if the PDCCH monitoring periodicity for the search space set with the DCI format 2_4 is one slot and there are more than one PDCCH monitoring occasions in a slot, or

-	is equal to the PDCCH monitoring periodicity, otherwise.

-	GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion

GCI sets of bits from the MSB of the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

For a group of symbols, NBI=NCIGCI bits from MSB of each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as BCI=LRB  from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB or FrequencyInfoUL that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. If the PUSCH transmission or the SRS transmission is scheduled by a DCI format, the indication by the DCI format 2_4 is applicable to the PUSCH transmission or SRS transmission only if the last symbol of the PDCCH reception providing the DCI format is earlier than the first symbol of the PDCCH reception providing the DCI format 2_4. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming  d2,1=doffset⋅2-μUL2-μ where doffset is provided by deltaOffset, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming that d2,1=0 after a last symbol of a CORESET where the UE detects the DCI format 2_4.

A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetition Type B, as determined in Clauses 9 and 9.2.5 or in Clause 6.1 of [6, TS 38.214], or an SRS transmission on the serving cell if, respectively,

-	the transmission is PUSCH with priority 0, if the UE is provided uplinkCancellationPriority,

-	a group of symbols, from the TCI symbols, has at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and

-	a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission,

where

-	the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that is in a group of symbols having corresponding bit values of '1' in the DCI format 2_4;

-	the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4.

If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is later than the first symbol of the PDCCH reception providing the DCI format 2_4.

## 11.3	Group TPC commands for PUCCH/PUSCH

For PUCCH transmission on a serving cell, a UE can be provided

-	a TPC-PUCCH-RNTI for a DCI format 2_2 by tpc-PUCCH-RNTI

-	a field in DCI format 2_2 is a TPC command of 2 bits mapping to  values as described in Clause 7.2.1

-	an index for a location in DCI format 2_2 of a first bit for a TPC command field for the PCell, or the SpCell for EN-DC operation, or for a carrier of the PCell by tpc-IndexPCell

-	an index for a location in DCI format 2_2 of a first bit for a TPC command field for the PUCCH-SCell or for a carrier for the PUCCH-SCell by tpc-IndexPUCCH-Scell

-	a mapping for the PUCCH power control adjustment state , by a corresponding {0, 1} value of a closed loop index field that is appended to the TPC command field in DCI format 2_2 if the UE indicates a capability to support two PUCCH power control adjustment states by twoDifferentTPC-Loop-PUCCH, and if the UE is configured for two PUCCH power control adjustment states by twoPUCCH-PC-AdjustmentStates

The UE is also provided on a serving cell with a configuration for a search space set  and a corresponding CORESET  for monitoring PDCCH candidates for DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI as described in Clause 10.1.

For PUSCH transmission on a serving cell, a UE can be provided

-	a TPC-PUSCH-RNTI for a DCI format 2_2 by tpc-PUSCH-RNTI

-	a field in DCI format 2_2 is a TPC command of 2 bits mapping to  values as described in Clause 7.1.1

-	an index for a location in DCI format 2_2 of a first bit for a TPC command field for an uplink carrier of the serving cell by tpc-Index

-	an index for a location in DCI format 2_2 of a first bit for a TPC command field for a supplementary uplink carrier of the serving cell by tpc-IndexSUL

-	an index of the serving cell by targetCell. If targetCell is not provided, the serving cell is the cell of the PDCCH reception for DCI format 2_2

-	a mapping for the PUSCH power control adjustment state , by a corresponding {0, 1} value of a closed loop index field that is appended to the TPC command field for the uplink carrier or for the supplementary uplink carrier of the serving cell in DCI format 2_2 if the UE indicates a capability to support two PUSCH power control adjustment states, by twoDifferentTPC-Loop-PUSCH, and if the UE is configured for two PUSCH power control adjustment states by twoPUSCH-PC-AdjustmentStates

The UE is also provided for the serving cell of the PDCCH reception for DCI format 2_2 with a configuration for a search space set  and a corresponding CORESET  for monitoring PDCCH candidates for DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI as described in Clause 10.1.

## 11.4	SRS switching

DCI format 2_3 is applicable for uplink carrier(s) of serving cells where a UE is not configured for PUSCH/PUCCH transmission or for uplink carrier(s) of a serving cell where srs-PowerControlAdjustmentStates indicates a separate power control adjustment state between SRS transmissions and PUSCH transmissions.

A UE configured by higher layers with parameter carrierSwitching is provided

-	a TPC-SRS-RNTI for a DCI format 2_3 by tpc-SRS-RNTI

-	an index of a serving cell where the UE interrupts transmission in order to transmit SRS on one or more other serving cells by srs-SwitchFromServCellIndex

-	an indication of an uplink carrier where the UE interrupts transmission in order to transmit SRS on one or more other serving cells by srs-SwitchFromCarrier

-	a DCI format 2_3 field configuration type by typeA or typeB

-	for typeA, an index for a set of serving cells is provided by cc-SetIndex, indexes of serving cells in the set of serving cells are provided by cc-IndexInOneCC-Set, and a DCI format 2_3 field includes a TPC command for each serving cell from the set of serving cells and can also include a SRS request for SRS transmission on the set of serving cells

-	for typeB, DCI format 2_3 field includes a TPC command for a serving cell index and can also include a SRS request for SRS transmission on the serving cell

-	an indication for a serving cell for whether or not a field in DCI format 2_3 includes a SRS request by fieldTypeFormat2-3 where a value of 0/1 indicates absence/presence of the SRS request – a mapping for a 2 bit SRS request to SRS resource sets is as provided in [6, TS 38.214]

-	an index for a location in DCI format 2_3 of a first bit for a field for a non-supplementary uplink carrier of the serving cell by startingBitOfFormat2-3

-	an index for a location in DCI format 2_3 of a first bit for a field for a supplementary uplink carrier of the serving cell by startingBitOfFormat2-3SUL-v1530

# 12	Bandwidth part operation

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

A UE configured for operation in bandwidth parts (BWPs) of a serving cell, is configured by higher layers for the serving cell a set of at most four bandwidth parts (BWPs) for receptions by the UE (DL BWP set) in a DL bandwidth by parameter BWP-Downlink or by parameter initialDownlinkBWP with a set of parameters configured by BWP-DownlinkCommon and BWP-DownlinkDedicated, and a set of at most four BWPs for transmissions by the UE (UL BWP set) in an UL bandwidth by parameter BWP-Uplink or by parameter initialUplinkBWP with a set of parameters configured by BWP-UplinkCommon and BWP-UplinkDedicated.

If a UE is not provided initialDownlinkBWP, an initial DL BWP is defined by a location and number of contiguous PRBs, starting from a PRB with the lowest index and ending at a PRB with the highest index among PRBs of a CORESET for Type0-PDCCH CSS set, and a SCS and a cyclic prefix for PDCCH reception in the CORESET for Type0-PDCCH CSS set; otherwise, the initial DL BWP is provided by initialDownlinkBWP. For operation on the primary cell or on a secondary cell, a UE is provided an initial UL BWP by initialUplinkBWP. If the UE is configured with a supplementary UL carrier, the UE can be provided an initial UL BWP on the supplementary UL carrier by initialUplinkBWP.

If a UE has dedicated BWP configuration, the UE can be provided by firstActiveDownlinkBWP-Id a first active DL BWP for receptions and by firstActiveUplinkBWP-Id a first active UL BWP for transmissions on a carrier of the primary cell.

For each DL BWP or UL BWP in a set of DL BWPs or UL BWPs, respectively, the UE is provided the following parameters for the serving cell as defined in [4, TS 38.211] or [6, TS 38.214]:

-	a SCS by subcarrierSpacing

-	a cyclic prefix by cyclicPrefix

-	a common RB  and a number of contiguous RBs  provided by locationAndBandwidth that indicates an offset  and a length  as RIV according to [6, TS 38.214], setting , and a value  provided by offsetToCarrier for the subcarrierSpacing

-	an index in the set of DL BWPs or UL BWPs by respective BWP-Id

-	a set of BWP-common and a set of BWP-dedicated parameters by BWP-DownlinkCommon and BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon and BWP-UplinkDedicated for the UL BWP [12, TS 38.331]

For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP.

For each DL BWP in a set of DL BWPs of the PCell, or of the PUCCH-SCell, a UE can be configured CORESETs for every type of CSS sets and for USS as described in Clause 10.1. The UE does not expect to be configured without a CSS set on the PCell, or on the PUCCH-SCell, of the MCG in the active DL BWP.

If a UE is provided controlResourceSetZero and searchSpaceZero in PDCCH-ConfigSIB1 or PDCCH-ConfigCommon, the UE determines a CORESET for a search space set from controlResourcesetZero as described in Clause 13 and for Tables 13-1 through 13-10, and determines corresponding PDCCH monitoring occasions as described in Clause 13 and for Tables 13-11 through 13-15. If the active DL BWP is not the initial DL BWP, the UE determines PDCCH monitoring occasions for the search space set only if the CORESET bandwidth is within the active DL BWP and the active DL BWP has same SCS configuration and same cyclic prefix as the initial DL BWP.

For each UL BWP in a set of UL BWPs of the PCell or of the PUCCH-SCell, the UE is configured resource sets for PUCCH transmissions as described in Clause 9.2.1.

A UE receives PDCCH and PDSCH in a DL BWP according to a configured SCS and CP length for the DL BWP. A UE transmits PUCCH and PUSCH in an UL BWP according to a configured SCS and CP length for the UL BWP.

If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall

-	for each information field in the DCI format

-	if the size of the information field is smaller than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format information fields, respectively

-	if the size of the information field is larger than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE uses a number of least significant bits of the DCI format equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format information fields, respectively

-	set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format

If a bandwidth part indicator field is configured in a DCI format 0_1 and indicates an active UL BWP with different SCS configuration μ, or with different number  NRB-set,ULBWP of RB sets, than a current active UL BWP, the UE determines an uplink frequency domain resource allocation Type 2 based on X' bits and Y' bits that are generated by independently truncating or padding the X MSBs and the Y LSBs [6, TS 38.214] of the frequency domain resource assignment field of DCI format 0_1, where truncation starts from the MSBs of the X bits or the Y bits, zero-padding prepends zeros to the X bits or the Y bits, and

-	if the indicated active UL BWP has SCS configuration μ=1 and the current active BWP has SCS configuration μ=0, the X MSBs are truncated to X'=X-1 bits, or

-	if the indicated active UL BWP has SCS configuration μ=0 and the current active BWP has SCS configuration μ=1, the X MSBs are zero-padded to X'=X+1 bits

-	otherwise, the X MSBs are unchanged

and

-	the Y LSBs are truncated or zero-padded to  Y'=log2NRB-set,ULBWPNRB-set,ULBWP+12 bits where  NRB-set,ULBWP is a number of RB sets configured for the indicated active UL BWP

A UE does not expect to detect a DCI format with a BWP indicator field that indicates an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change, respectively [10, TS 38.133].

If a UE detects a DCI format with a BWP indicator field that indicates an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

If a UE detects a DCI format with SCell dormancy indication that indicates an active DL BWP change for an Scell in slot n of primary cell, the UE is not required to receive or transmit in the SCell during a time duration specified in [10, TS 38.133].

If a UE detects a DCI format indicating an active UL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in the scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

A UE does not expect to detect a DCI format indicating an active DL BWP change or an active UL BWP change for a scheduled cell within FR1 (or FR2) in a slot other than the first slot of a set of slots for the DL SCS of the scheduling cell that overlaps with a time duration where the UE is not required to receive or transmit, respectively, for an active BWP change in a different cell from the scheduled cell within FR1 (or FR2).

A UE expects to detect a DCI format with a BWP indicator field that indicates an active UL BWP change or an active DL BWP change only if a corresponding PDCCH is received within the first 3 symbols of a slot.

For a serving cell, a UE can be provided by defaultDownlinkBWP-Id a default DL BWP among the configured DL BWPs. If a UE is not provided a default DL BWP by defaultDownlinkBWP-Id, the default DL BWP is the initial DL BWP.

If a UE is provided by bwp-InactivityTimer a timer value for the serving cell [11, TS 38.321] and the timer is running, the UE decrements the timer at the end of a subframe for FR1 or at the end of a half subframe for FR2 if the restarting conditions in [11, TS 38.321] are not met during the interval of the subframe for FR1 or of the half subframe for FR2.

For a cell where a UE changes an active DL BWP due to a BWP inactivity timer expiration and for accommodating a delay in the active DL BWP change or the active UL BWP change required by the UE [10, TS 38.133], the UE is not required to receive or transmit in the cell during a time duration from the beginning of a subframe for FR1, or of half of a subframe for FR2, that is immediately after the BWP inactivity timer expires until the beginning of a slot where the UE can receive or transmit.

When a UE's BWP inactivity timer for a cell within FR1 (or FR2) expires within a time duration where the UE is not required to receive or transmit for an active UL/DL BWP change in the cell or in a different cell within FR1 (or FR2), the UE delays the active UL/DL BWP change triggered by the BWP inactivity timer expiration until a subframe for FR1 or half a subframe for FR2 that is immediately after the UE completes the active UL/DL BWP change in the cell or in the different cell within FR1 (or FR2).

If a UE is provided by firstActiveDownlinkBWP-Id a first active DL BWP and by firstActiveUplinkBWP-Id a first active UL BWP on a carrier of a secondary cell, the UE uses the indicated DL BWP and the indicated UL BWP as the respective first active DL BWP on the secondary cell and first active UL BWP on the carrier of the secondary cell.

A UE does not expect to monitor PDCCH when the UE performs RRM measurements [10, TS 38.133] over a bandwidth that is not within the active DL BWP for the UE.

# 13	UE procedure for monitoring Type0-PDCCH CSS sets

If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in Clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15.  and  are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and  and  are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index  overlaps in time with system frame  and slot . The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix.

For operation without shared spectrum channel access, a UE assumes that the offset in Tables 13-1 through 13-10 is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set, provided by subCarrierSpacingCommon, from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block. In Tables 13-7, 13-8, and 13-10  is defined in [4, TS 38.211].

For operation with shared spectrum channel access, a UE determines an offset from a smallest RB index of the CORESET for Type0-PDCCH CSS set to a smallest RB index of the common RB overlapping with a first RB of the corresponding SS/PBCH block

-	according to the offset in Table 13-1A or Table 13-4A, if the frequency position of the SS/PBCH block corresponds to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], and

-	according to a sum of a first offset and a second offset if the frequency position of the SS/PBCH block is provided by ssbFrequency in a measurement configuration associated with a reporting configuration providing reportCGI and does not correspond to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], where

-	the first offset is provided in Table 13-1A or Table 13-4A, and

-	the second offset is determined as the offset from a smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block indicated in the measurement configuration to a smallest RB index of the common RB overlapping with the first RB of a SS/PBCH block hypothetically located at the GSCN of a synchronization raster entry, where the single synchronization raster entry is located in the same channel as the SS/PBCH block used for the shared spectrum channel access procedure, as described in [15, TS 37.213]

where the offsets are defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set that is same as the SCS of the corresponding SS/PBCH block.

For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two consecutive slots starting from slot . For SS/PBCH block with index , the UE determines an index of slot  as  that is in a frame with system frame number (SFN)  satisfying  if , or in a frame with SFN satisfying  if .  and  are provided by Tables 13-11 and 13-12, and  based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots  and  is the first symbol index provided by Tables 13-11 and 13-12.

For operation with shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set with respect to average gain, quasi co-location  ‘typeA’ and  ‘typeD’ properties, when applicable [6, TS 38.214]. For a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two consecutive slots starting from slot n0 include the associated Type0-PDCCH monitoring occasions. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. M and O are provided by Table 13-11, and μ∈{0, 1} based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots n0 and n0+1 is the first symbol index provided by Table 13-11. The UE does not expect to be configured with M=1/2, or with M=2, when NSSBQCL=1.

For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, a UE monitors PDCCH in the Type0-PDCCH CSS set over one slot with Type0-PDCCH CSS set periodicity equal to the periodicity of SS/PBCH block. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, if the active DL BWP is the initial DL BWP, the UE is expected to be able to perform radio link monitoring, as described in Clause 5, and measurements for radio resource management [10, TS 38.133] using a SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set. For a SS/PBCH block with index , the UE determines the slot index  and  based on parameters provided by Tables 13-13 through 13-15.

Table 13-1: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 48 | 1 | 12 |
| 7 | 1 | 48 | 1 | 16 |
| 8 | 1 | 48 | 2 | 12 |
| 9 | 1 | 48 | 2 | 16 |
| 10 | 1 | 48 | 3 | 12 |
| 11 | 1 | 48 | 3 | 16 |
| 12 | 1 | 96 | 1 | 38 |
| 13 | 1 | 96 | 2 | 38 |
| 14 | 1 | 96 | 3 | 38 |
| 15 | Reserved |  |  |  |

Table 13-1A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands operated with shared spectrum channel access

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 96 | 1 | 10 |
| 1 | 1 | 96 | 1 | 12 |
| 2 | 1 | 96 | 1 | 14 |
| 3 | 1 | 96 | 1 | 16 |
| 4 | 1 | 96 | 2 | 10 |
| 5 | 1 | 96 | 2 | 12 |
| 6 | 1 | 96 | 2 | 14 |
| 7 | 1 | 96 | 2 | 16 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-2: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 5 |
| 1 | 1 | 24 | 2 | 6 |
| 2 | 1 | 24 | 2 | 7 |
| 3 | 1 | 24 | 2 | 8 |
| 4 | 1 | 24 | 3 | 5 |
| 5 | 1 | 24 | 3 | 6 |
| 6 | 1 | 24 | 3 | 7 |
| 7 | 1 | 24 | 3 | 8 |
| 8 | 1 | 48 | 1 | 18 |
| 9 | 1 | 48 | 1 | 20 |
| 10 | 1 | 48 | 2 | 18 |
| 11 | 1 | 48 | 2 | 20 |
| 12 | 1 | 48 | 3 | 18 |
| 13 | 1 | 48 | 3 | 20 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-3: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 2 |
| 1 | 1 | 48 | 1 | 6 |
| 2 | 1 | 48 | 2 | 2 |
| 3 | 1 | 48 | 2 | 6 |
| 4 | 1 | 48 | 3 | 2 |
| 5 | 1 | 48 | 3 | 6 |
| 6 | 1 | 96 | 1 | 28 |
| 7 | 1 | 96 | 2 | 28 |
| 8 | 1 | 96 | 3 | 28 |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-4: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 1 |
| 2 | 1 | 24 | 2 | 2 |
| 3 | 1 | 24 | 2 | 3 |
| 4 | 1 | 24 | 2 | 4 |
| 5 | 1 | 24 | 3 | 0 |
| 6 | 1 | 24 | 3 | 1 |
| 7 | 1 | 24 | 3 | 2 |
| 8 | 1 | 24 | 3 | 3 |
| 9 | 1 | 24 | 3 | 4 |
| 10 | 1 | 48 | 1 | 12 |
| 11 | 1 | 48 | 1 | 14 |
| 12 | 1 | 48 | 1 | 16 |
| 13 | 1 | 48 | 2 | 12 |
| 14 | 1 | 48 | 2 | 14 |
| 15 | 1 | 48 | 2 | 16 |

Table 13-4A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands operated with shared spectrum channel access

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 0 |
| 1 | 1 | 48 | 1 | 1 |
| 2 | 1 | 48 | 1 | 2 |
| 3 | 1 | 48 | 1 | 3 |
| 4 | 1 | 48 | 2 | 0 |
| 5 | 1 | 48 | 2 | 1 |
| 6 | 1 | 48 | 2 | 2 |
| 7 | 1 | 48 | 2 | 3 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-5: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 15} kHz for frequency bands with minimum channel bandwidth 40MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 4 |
| 1 | 1 | 48 | 2 | 4 |
| 2 | 1 | 48 | 3 | 4 |
| 3 | 1 | 96 | 1 | 0 |
| 4 | 1 | 96 | 1 | 56 |
| 5 | 1 | 96 | 2 | 0 |
| 6 | 1 | 96 | 2 | 56 |
| 7 | 1 | 96 | 3 | 0 |
| 8 | 1 | 96 | 3 | 56 |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-6: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 40MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 24 | 3 | 0 |
| 3 | 1 | 24 | 3 | 4 |
| 4 | 1 | 48 | 1 | 0 |
| 5 | 1 | 48 | 1 | 28 |
| 6 | 1 | 48 | 2 | 0 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 48 | 3 | 0 |
| 9 | 1 | 48 | 3 | 28 |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-7: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 60} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 0 |
| 1 | 1 | 48 | 1 | 8 |
| 2 | 1 | 48 | 2 | 0 |
| 3 | 1 | 48 | 2 | 8 |
| 4 | 1 | 48 | 3 | 0 |
| 5 | 1 | 48 | 3 | 8 |
| 6 | 1 | 96 | 1 | 28 |
| 7 | 1 | 96 | 2 | 28 |
| 8 | 2 | 48 | 1 | -41 if -42 if |
| 9 | 2 | 48 | 1 | 49 |
| 10 | 2 | 96 | 1 | -41 if -42 if |
| 11 | 2 | 96 | 1 | 97 |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-8: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 3 | 24 | 2 | -20 if -21 if |
| 5 | 3 | 24 | 2 | 24 |
| 6 | 3 | 48 | 2 | -20 if -21 if |
| 7 | 3 | 48 | 2 | 48 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-9: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {240, 60} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 96 | 1 | 0 |
| 1 | 1 | 96 | 1 | 16 |
| 2 | 1 | 96 | 2 | 0 |
| 3 | 1 | 96 | 2 | 16 |
| 4 | Reserved |  |  |  |
| 5 | Reserved |  |  |  |
| 6 | Reserved |  |  |  |
| 7 | Reserved |  |  |  |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-10: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {240, 120} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 0 |
| 1 | 1 | 48 | 1 | 8 |
| 2 | 1 | 48 | 2 | 0 |
| 3 | 1 | 48 | 2 | 8 |
| 4 | 2 | 24 | 1 | -41 if -42 if |
| 5 | 2 | 24 | 1 | 25 |
| 6 | 2 | 48 | 1 | -41 if -42 if |
| 7 | 2 | 48 | 1 | 49 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-11: Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR1

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 2 | 2 | 1 | 1 | 0 |
| 3 | 2 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 6 | 7 | 1 | 1 | 0 |
| 7 | 7 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 8 | 0 | 1 | 2 | 0 |
| 9 | 5 | 1 | 2 | 0 |
| 10 | 0 | 1 | 1 | 1 |
| 11 | 0 | 1 | 1 | 2 |
| 12 | 2 | 1 | 1 | 1 |
| 13 | 2 | 1 | 1 | 2 |
| 14 | 5 | 1 | 1 | 1 |
| 15 | 5 | 1 | 1 | 2 |

Table 13-12: Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 2 | 2.5 | 1 | 1 | 0 |
| 3 | 2.5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 6 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 7 | 2.5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 8 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 9 | 7.5 | 1 | 1 | 0 |
| 10 | 7.5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 11 | 7.5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 12 | 0 | 1 | 2 | 0 |
| 13 | 5 | 1 | 2 | 0 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Table 13-13: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 2 and {SS/PBCH block, PDCCH} SCS {120, 60} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 |  | 0, 1, 6, 7 for , , , |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

Table 13-14: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 2 and {SS/PBCH block, PDCCH} SCS {240, 120} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, …, 7) |
| --- | --- | --- |
| 0 | or | 0, 1, 2, 3, 0, 1 in , , , , , () 12, 13 in , () |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

Table 13-15: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {120, 120} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 |  | 4, 8, 2, 6 in , , , |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for  for FR1 or for  for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as .  is the GSCN of the first SS/PBCH block and  is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If the UE detects the second SS/PBCH block and the second SS/PBCH block does not provide a CORESET for Type0-PDCCH CSS set, as described in Clause 4.1, the UE may ignore the information related to GSCN of SS/PBCH block locations for performing cell search.

If a UE detects a SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for  for FR1 or for  for FR2, the UE determines that there is no SS/PBCH block having an associated Type0-PDCCH CSS set within a GSCN range . and  are respectively determined by controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1. If the GSCN range is , the UE determines that there is no information for a second SS/PBCH block with a CORESET for an associated Type0-PDCCH CSS set on the detected SS/PBCH block.

If a UE does not detect any SS/PBCH block providing a CORESET for Type0-PDCCH CSS set, as described in Clause 4.1, within a time period determined by the UE, the UE may ignore the information related to GSCN of SS/PBCH locations in performing cell search.

Table 13-16: Mapping between the combination of  and controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to  for FR1

|  | 16×controlResourceSetZero +searchSpaceZero |  |
| --- | --- | --- |
| 24 | 0, 1, …, 255 | 1, 2, …, 256 |
| 25 | 0, 1, …, 255 | 257, 258, …, 512 |
| 26 | 0, 1, …, 255 | 513, 514, …., 768 |
| 27 | 0, 1, …, 255 | -1, -2, …, -256 |
| 28 | 0, 1, …, 255 | -257, -258, …, -512 |
| 29 | 0, 1, …, 255 | -513, -514, …., -768 |
| 30 | 0, 1, …, 255 | Reserved, Reserved, …, Reserved |

Table 13-17: Mapping between the combination of  and controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to  for FR2

|  | 16×controlResourceSetZero +searchSpaceZero |  |
| --- | --- | --- |
| 12 | 0, 1, …, 255 | 1, 2, …, 256 |
| 13 | 0, 1, …, 255 | -1, -2, …, -256 |
| 14 | 0, 1, …, 255 | Reserved, Reserved, …, Reserved |

# 14	Integrated access-backhaul operation

Throughout this specification, unless otherwise noted, statements using the term "UE" in clauses 4 through 13 are equally applicable to the IAB-MT of an IAB node.

A procedure for an IAB-MT to perform cell search, system information acquisition, or random access procedure is same as a corresponding one for a UE except for the following.

For initial cell selection, an IAB-MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

For PRACH transmission, an IAB-MT determines frames and subframes/slots within the frames containing PRACH occasions as described in [4, TS 38.211].

The IAB-MT determines an association period for mapping SS/PBCH blocks to PRACH occasions based on a PRACH configuration period as described in Clause 8.1 and according to Table 14-1 instead of Table 8.1-1. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec. A PRACH occasion in a PRACH slot is valid according to the conditions in Clause 8.1.

Table 14-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period for an IAB-MT

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16, 32, 64} |
| 20 | {1, 2, 4, 8, 16, 32} |
| 40 | {1, 2, 4, 8, 16} |
| 80 | {1, 2, 4, 8} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

If an IAB-node is provided an index Tdelta in a Timing Delta MAC CE [11, TS 38.321] from a serving cell, the IAB-node may assume that NTA/2+Ndelta+Tdelta⋅Gstep⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-MT when NTA/2+Ndelta+Tdelta⋅Gstep>0, where NTA is obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell and Ndelta and Gstep are determined as

-	Ndelta=-70528 and Gstep=64, if the serving cell providing the Timing Delta MAC CE operates in FR1,

-	Ndelta=-17664 and Gstep=32, if the serving cell providing the Timing Delta MAC CE operates in FR2

The IAB-node may use the time difference to determine a DU transmission time.

A slot format for an IAB-DU or an IAB-MT includes downlink symbols, uplink symbols, and flexible symbols.

For each cell of an IAB-DU, the IAB-DU can be provided an indication for a slot format over a number of slots by gNB-DU Cell Resource Configuration [16, TS 38.473].

For each serving cell, an IAB-MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT. If the IAB-MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in Clause 11.1 that include "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-MT of an IAB node by replacing "tdd-UL-DL-ConfigurationDedicated" with "tdd-UL-DL-ConfigurationDedicated-IAB-MT" for the IAB-MT, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides

-	a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-	for each slot configuration from the set of slot configurations

-	a slot index for a slot provided by slotIndex

-	a set of symbols for a slot by symbols-IAB-MT where

-	if symbols-IAB-MT = allDownlink, all symbols in the slot are downlink

-	if symbols-IAB-MT = allUplink, all symbols in the slot are uplink

-	if symbols-IAB-MT = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible.

-	if symbols-IAB-MT = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

An IAB-MT can be provided, by SlotFormatCombinationsPerCell, a list of slot format combinations applicable for one serving cell and, by SlotFormatIndicator, a configuration for monitor a DCI format 2_0 indicating a slot format combination, from the list of slot format combinations, over a number of slots as described in Clause 11.1.1. In addition to the slot formats in Table 11.1.1-1, an SFI field for an IAB-MT in DCI format 2_0 can indicate to the IAB-MT a slot format from the slot formats in Table 14-2.

Table 14-2: Slot formats for normal cyclic prefix

| Slot Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 56 | U | U | U | U | U | U | U | U | U | U | U | U | U | F |
| 57 | U | U | U | U | U | U | U | U | U | U | U | U | F | F |
| 58 | U | U | U | U | U | U | U | U | U | U | U | F | F | F |
| 59 | U | U | U | U | U | U | U | U | U | U | F | F | F | F |
| 60 | U | U | U | U | U | U | U | U | U | F | F | F | F | F |
| 61 | U | U | U | U | U | U | U | U | F | F | F | F | F | F |
| 62 | U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| 63 | U | U | U | U | U | U | F | F | F | F | F | F | F | F |
| 64 | U | U | U | U | U | F | F | F | F | F | F | F | F | F |
| 65 | U | U | U | U | F | F | F | F | F | F | F | F | F | F |
| 66 | U | U | U | F | F | F | F | F | F | F | F | F | F | F |
| 67 | U | U | F | F | F | F | F | F | F | F | F | F | F | F |
| 68 | U | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 69 | U | F | F | F | F | F | F | F | F | F | F | F | F | D |
| 70 | U | U | F | F | F | F | F | F | F | F | F | F | F | D |
| 71 | U | U | U | F | F | F | F | F | F | F | F | F | F | D |
| 72 | U | F | F | F | F | F | F | F | F | F | F | F | D | D |
| 73 | U | U | F | F | F | F | F | F | F | F | F | F | D | D |
| 74 | U | U | U | F | F | F | F | F | F | F | F | F | D | D |
| 75 | U | F | F | F | F | F | F | F | F | F | F | D | D | D |
| 76 | U | U | F | F | F | F | F | F | F | F | F | D | D | D |
| 77 | U | U | U | F | F | F | F | F | F | F | F | D | D | D |
| 78 | U | U | U | U | U | U | U | U | U | U | U | U | F | D |
| 79 | U | U | U | U | U | U | U | U | U | U | U | F | F | D |
| 80 | U | U | U | U | U | U | U | U | U | U | F | F | F | D |
| 81 | U | U | U | U | U | U | U | U | U | U | U | F | D | D |
| 82 | U | U | U | U | U | U | U | U | U | U | F | F | D | D |
| 83 | U | U | U | U | U | U | U | U | U | F | F | F | D | D |
| 84 | U | F | D | D | D | D | D | D | D | D | D | D | D | D |
| 85 | U | U | F | D | D | D | D | D | D | D | D | D | D | D |
| 86 | U | U | U | F | D | D | D | D | D | D | D | D | D | D |
| 87 | U | F | F | D | D | D | D | D | D | D | D | D | D | D |
| 88 | U | U | F | F | D | D | D | D | D | D | D | D | D | D |
| 89 | U | U | U | F | F | D | D | D | D | D | D | D | D | D |
| 90 | U | F | F | F | D | D | D | D | D | D | D | D | D | D |
| 91 | U | U | F | F | F | D | D | D | D | D | D | D | D | D |
| 92 | U | U | U | F | F | F | D | D | D | D | D | D | D | D |
| 93 | U | U | U | U | U | U | U | U | U | F | F | F | F | D |
| 94 | U | U | U | U | U | U | F | F | F | F | F | F | D | D |
| 95 | U | U | U | U | U | U | F | F | D | D | D | D | D | D |
| 96 | U | U | U | U | U | U | U | D | D | D | D | D | D | D |

For a serving cell of an IAB-MT, the IAB-MT can be provided by Provided Guard Symbols MAC CE a number of symbols that will not be used for the IAB-MT in slots where the IAB-node transitions between IAB-MT and IAB-node DU and a SCS configuration for the number of symbols [11, TS 38.321].

With reference to slots of an IAB-DU cell, a symbol in a slot of an IAB-DU cell can be configured to be of hard, soft, or unavailable type. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive in the symbol.

When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU can respectively transmit, receive or either transmit or receive in the symbol only if

-	the IAB-MT does not transmit or receive in the symbol, or

-	the IAB-MT would transmit or receive in the symbol, and the transmission or reception in the symbol is not changed due to a use of the symbol by the IAB-DU, or

-	the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available

When a symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the symbol.

A symbol of a slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot.

If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSizeAI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH.

For each cell of an IAB-DU in a set of cells of the IAB-DU, the IAB-DU can be provided:

-	an identity of the IAB-DU cell by iab-DU-CellIdentity

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI

-	a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes

-	resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, and

-	a mapping for the soft symbol availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId

The IAB-DU can assume a same SCS configuration for availabilityCombinations for a cell as an SCS configuration provided by gNB-DU Cell Resource Configuration for the cell.

An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. The AI index field includes  bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol in a slot is identified by a corresponding value resourceAvailability as provided in Table 14-3.

Table 14-3: Mapping between values of resourceAvailability elements and types of soft symbol availability in a slot

| Value | Indication |
| --- | --- |
| 0 | No indication of availability for soft symbols |
| 1 | DL soft symbols are indicated available No indication of availability for UL and Flexible soft symbols |
| 2 | UL soft symbols are indicated available No indication of availability for DL and Flexible soft symbols |
| 3 | DL and UL soft symbols are indicated available No indication of availability for Flexible soft symbols |
| 4 | Flexible soft symbols are indicated available No indication of availability for DL and UL soft symbols |
| 5 | DL and Flexible soft symbols are indicated available No indication of availability for UL soft symbols |
| 6 | UL and Flexible soft symbols are indicated available No indication of availability for DL soft symbols |
| 7 | DL, UL, and Flexible soft symbols are indicated available |

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the IAB-MT obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the IAB-MT detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the IAB-MT expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot.

# 15	Dual active protocol stack based handover

If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG.

If a UE is configured with a target MCG using NR radio access in FR1 or in FR2 and with a source MCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

If a UE is configured with a target MCG using NR radio access in FR1 and a source MCG using NR radio access in FR1, the UE is configured a maximum power PMCG for transmissions on the target MCG by p-DAPS-Target and a maximum power PSCG for transmissions on the source MCG by p-DAPS-Source and with an inter-CG power sharing mode by uplinkPowerSharingDAPS-Mode. The UE determines a transmission power on the target MCG and a transmission power on the source MCG per frequency range.

If the UE indicates support for semi-static power sharing mode1 and is provided uplinkPowerSharingDAPS-Mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for nrdc-PCmode-FR1 = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG.

If the UE indicates support for semi-static power sharing mode2 and is provided uplinkPowerSharingDAPS-Mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for nrdc-PCmode-FR1 = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. The UE expects to be provided uplinkPowerSharingDAPS-Mode = Semi-static-mode2 only for synchronous DAPS HO operation [10, TS 38.133].

If the UE indicates support for dynamic power sharing and is provided uplinkPowerSharingDAPS-Mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for nrdc-PCmode-FR1 = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

If

-	the carrier frequencies of target MCG and source MCG are not intra-frequency, and

-	the UE does not indicate support of ul-TransCancellationDAPS, and

-	UE does not indicate a capability for power sharing between source and target MCG in DAPS handover or the UE is not provided with uplinkPowerSharingDAPS-Mode,

the UE does not expect transmissions on the target and source cell in overlapping time resources.

If

-	the UE indicates support of ul-TransCancellationDAPS, and

-	the carrier frequencies of target MCG and source MCG are not intra-frequency, and

-	UE does not indicate a capability for power sharing between source and target MCG in DAPS handover or the UE is not provided with uplinkPowerSharingDAPS-Mode, and

-	UE transmissions on the target cell and the source cell are in overlapping time resources,

the UE transmits only on the target cell, and cancels the transmission to source cell

If

-	the carrier frequencies of target MCG and source MCG are intra-frequency, and

-	UE transmissions on the target cell and the source cell overlapping time resources,

the UE transmits only on the target cell and cancels the transmission on the source cell

The UE does not expect to cancel a transmission on the source cell if a first symbol of the transmission on the source cell is less than Tproc,2+d after a last symbol of a CORESET where the UE receives a PDCCH providing a DCI format scheduling a transmission on the target cell. Tproc,2 is the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1=1,  d is a time duration corresponding to 2 symbols for SCS configuration μ, and μ is the smallest SCS configuration between the SCS configuration of the PDCCH providing the DCI format and the SCS configuration for the transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0.

A UE does not expect to cancel a transmission on the source cell in symbols from the set of symbols that occur, relative to a last symbol of a PDSCH reception conveying a RAR message with a RAR UL grant on the target cell, after a number of symbols that is smaller than NT,1+NT,2+0.5 msec, where NT,1  is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2  is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] and the UE considers that N1 and N2 correspond to the smaller of the SCS configurations for the PDSCH on the target cell and the transmission on the source cell. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

The UE determines intra-frequency as described in Clause 9.2.1 of [10, TS38.133].

For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively.

If a UE is provided search space sets on both the target MCG and the source MCG, in any slot the UE does not expect to have USS sets on both the target MCG and the source MCG that result in the number of monitored PDCCH candidates and the total number of non-overlapped CCEs in both cells that each exceed the corresponding maximum numbers per slot defined in Table 10.1-2 and Table 10.1-3.

For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot overlapping in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. For DAPS operation in a same frequency band, a UE does not transmit PRACH on the source MCG in a slot overlapping in time with a PUSCH/PUCCH/SRS transmission on the target MCG or when a gap between the first or last symbol of a PUSCH/PUCCH/SRS transmission on the target MCG is separated by less than N symbols from a last or a first symbol, respectively, of a PRACH transmission on the source MCG.  N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission.

# 16	UE procedures for sidelink

A UE is provided by SL-BWP-Config a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS 38.211]. For a resource pool within the SL BWP, the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by sl-SubchannelSize. The first RB of the first sub-channel in the SL BWP is indicated by sl-StartRB-Subchannel. Available slots for a resource pool are provided by timeresourcepool and occur with a periodicity provided by 'periodResourcePool'. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by sl-StartSymbol and be within a number of consecutive symbols indicated by sl-LengthSymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined.

The UE expects to use a same numerology in the SL BWP and in an active UL BWP in a same carrier of a same cell. If the active UL BWP numerology is different than the SL BWP numerology, the SL BWP is deactivated.

A UE transmitting using a Mode-1 grant uses the corresponding fields in SCI to reserve the next resource(s) allocated by the same grant.

A priority of a PSSCH according to NR radio access or according to E-UTRA radio access is indicated by a priority field in a respective scheduling SCI format. A priority of a PSSS/SSSS/PSBCH according to E-UTRA radio access is provided by sl-SSB-PriorityEUTRA [13, TS 36.213]. A priority of an S-SS/PSBCH block is provided by sl-SSB-PriorityNR. A priority of a PSFCH is same as the priority of a corresponding PSSCH.

A UE does not expect to be provided search space sets associated with CORESETs on more than one cell to monitor PDCCH for detection of DCI format 3_0 or DCI format 3_1.

## 16.1	Synchronization procedures

A UE receives the following SL synchronization signals in order to perform synchronization procedures based on S-SS/PSBCH blocks: SL primary synchronization signals (S-PSS) and SL secondary synchronization signals (S-SSS) [4, TS 38.211].

A UE assumes that reception occasions of a physical sidelink broadcast channel (PSBCH), S-PSS, and S-SSS are in consecutive symbols [4, TS 38.211] and form a S-SS/PSBCH block.

For reception of a S-SS/PSBCH block, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by sl-AbsoluteFrequencySSB. The UE assumes that a S-PSS symbol, a S-SSS symbol, and a PSBCH symbol have a same transmission power. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PSBCH block is aligned with a subcarrier with index 0 in an RB of the SL BWP.

A UE is provided, by sl-NumSSB-WithinPeriod, a number NperiodS-SSB of S-SS/PSBCH blocks in a period of 16 frames. The UE assumes that a transmission of the S-SS/PSBCH blocks in the period is with a periodicity of 16 frames. The UE determines indexes of slots that include S-SS/PSBCH block as NoffsetS-SSB+NintervalS-SSB⋅iS-SSB, where

-	index 0 corresponds to a first slot in a frame with SFN satisfying (SFN mod 16)=0

-	iS-SSB is a S-SS/PSBCH block index within the number of S-SS/PSBCH blocks in the period, with 0≤iS-SSB≤NperiodS-SSB-1

-	NoffsetS-SSB is a slot offset from a start of the period to the first slot including S-SS/PSBCH block, provided by sl-TimeOffsetSSB

-	NintervalS-SSB is a slot interval between S-SS/PSBCH blocks, provided by sl-timeInterval

For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots.

For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1],

-	a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1';

else

-	a0=0 if pattern1 is provided by tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by tdd-UL-DL-ConfigurationCommon as described in Clause 11.1

-	a1, a2, a3,a4 are determined based on

-	P in pattern1 as described in Table 16.1-1 for a0=0

-	P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1

where P and P2 are as described in Clause 11.1

-	a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively

-	for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1

-	for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w

where

-	L is the number of symbols in a slot: L=12 if cyclicPrefix = "ECP"; else, L=14

-	I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0

-	I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0

-	Y is the sidelink starting symbol index provided by sl-StartSymbol

-	w is the granularity of slots indication as described in Table 16.1-2

-	μref, uslots, usym, uslots,2, usym,2 are described in Clause 11.1

-	μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211]

Table 16.1-1: Slot configuration period when one pattern is indicated

| a1, a2, a3,a4 | Slot configuration period of pattern1 P (msec) |
| --- | --- |
| 0, 0, 0, 0 | 0.5 |
| 0, 0, 0, 1 | 0.625 |
| 0, 0, 1, 0 | 1 |
| 0, 0, 1, 1 | 1.25 |
| 0, 1, 0, 0 | 2 |
| 0, 1, 0, 1 | 2.5 |
| 0, 1, 1, 0 | 4 |
| 0, 1, 1, 1 | 5 |
| 1, 0, 0, 0 | 10 |
| Reserved | Reserved |

Table 16.1-2: Slot configuration period and granularity when two patterns are indicated

| a1, a2, a3,a4 | Slot configuration period of pattern1 P (msec) | Slot configuration period of pattern2 P2 (msec) | Granularity w in slots with different SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 15kHz | 30 kHz | 60 kHz | 120 kHz |
| 0, 0, 0, 0 | 0.5 | 0.5 | 1 |  |  |  |
| 0, 0, 0, 1 | 0.625 | 0.625 |  |  |  |  |
| 0, 0, 1, 0 | 1 | 1 |  |  |  |  |
| 0, 0, 1, 1 | 0.5 | 2 |  |  |  |  |
| 0, 1, 0, 0 | 1.25 | 1.25 |  |  |  |  |
| 0, 1, 0, 1 | 2 | 0.5 |  |  |  |  |
| 0, 1, 1, 0 | 1 | 3 | 1 |  |  | 2 |
| 0, 1, 1, 1 | 2 | 2 |  |  |  |  |
| 1, 0, 0, 0 | 3 | 1 |  |  |  |  |
| 1, 0, 0, 1 | 1 | 4 |  |  |  |  |
| 1, 0, 1, 0 | 2 | 3 |  |  |  |  |
| 1, 0, 1, 1 | 2.5 | 2.5 |  |  |  |  |
| 1, 1, 0, 0 | 3 | 2 |  |  |  |  |
| 1, 1, 0, 1 | 4 | 1 |  |  |  |  |
| 1, 1, 1, 0 | 5 | 5 | 1 |  | 2 | 4 |
| 1, 1, 1, 1 | 10 | 10 | 1 | 2 | 4 | 8 |

If a UE would transmit or receive an S-SS/PSBCH block, and the transmission or reception would overlap in time with transmissions or receptions on the sidelink using E-UTRA radio access, the UE transmits or receives the signal/channel with the higher priority.

If a UE would transmit or receive sidelink synchronization signals for E-UTRA radio access, and the transmission or reception would overlap in time with sidelink transmissions or receptions using NR radio access, the UE transmits or receives the signal/channel with the higher priority.

## 16.2	Power control

### 16.2.0	S-SS/PSBCH blocks

A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i as

PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PO,S-SSB is a value of dl-P0-PSBCH if provided; else, PS-SSB(i)=PCMAX

-	αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1

-	PL=PLb,f,c(qd) as described in Clause 7.1.1

-	MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ

### 16.2.1	PSSCH

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of sl-MaxTransPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTransPower-r16 is not provided, then PMAX,CBR=PCMAX;

-	if dl-P0-PSSCH-PSCCH is provided

-	PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-	elseif sl-P0-PSSCH-PSCCH is provided

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) [dBm]

-	else

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm]

where

-	PO,D is a value of dl-P0-PSSCH-PSCCH if provided

-	αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1

-	PLD=PLb,f,c(qd) as described in Clause 7.1.1 except that

-	the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0

-	the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0

-	MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration

-	if sl-P0-PSSCH-PSCCH is provided

-	PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm]

-	else

-	PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm]

where

-	PO,SL is a value of sl-P0-PSSCH-PSCCH, if provided

-	αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1

-	PLSL= referenceSignalPower – higher layer filtered RSRP, where

-	referenceSignalPower is obtained from a PSSCH transmit power per RE per antenna port of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-filterCoefficient, and

-	higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-filterCoefficient

-	MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration

The UE splits the power PPSSCH(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power.

A UE determines a power PPSSCH2,b,c(i) for a PSSCH transmission on a resource pool in the symbols where a corresponding PSCCH is transmitted in PSCCH-PSSCH transmission occasion i as

PPSSCH2(i)=10log10MRBPSSCHi-MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm]

where MRBPSCCH(i) is a number of resource blocks for the corresponding PSCCH transmission in PSCCH-PSSCH transmission occasion i.

The UE splits the power PPSSCH2(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power.

### 16.2.2	PSCCH

A UE determines a power PPSCCH,b,c(i) for a PSCCH transmission on a resource pool in PSCCH-PSSCH transmission occasion i as

PPSCCH(i)=10log10MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm]

where

-	PPSSCH(i) is specified in Clause 16.2.1

-	MRBPSCCH(i) is a number of resource blocks for the PSCCH transmission in PSCCH-PSSCH transmission occasion i

-	MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i

### 16.2.3	PSFCH

A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i as

-	if dl-P0-PSFCH is provided,

PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm]

where

-	PO,PSFCH is a value of dl-P0-PSFCH

-	αPFSCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1

-	PL=PLb,f,c(qd) as described in Clause 7.1.1 except that

-	the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0

-	the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0

-	if Nsch,Tx,PSFCH≤Nmax,PSFCH

-	if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	else

-	UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as

-	the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any

-	zero, otherwise

and

PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm]

where PCMAX	is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions

-	else

-	the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2

-	if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	else

-	the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in Clause 16.2.4.2 such that NTx,PSFCH≥max1,i=1KMi where Mi is a number of PSFCHs with priority value i and K is defined as

-	the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs assigned with priority values 1, 2, …, K, if any

-	zero, otherwise

and

PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm]

where PCMAX is determined for the  NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1]

-	else

PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm]

where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending priority order as described in Clause 16.2.4.2 such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

### 16.2.4	Prioritization of transmissions/receptions

#### 16.2.4.1	Simultaneous NR and E-UTRA transmission/reception

If a UE

-	would transmit a first channel/signal using E-UTRA radio access and second channels/signals using NR radio access, and

-	a transmission of the first channel/signal would overlap in time with a transmission of the second channels/signals, and

-	the priorities of the channels/signals are known to both E-UTRA radio access and NR radio access at the UE T msec prior to the start of the earliest of the two transmissions, where T≤4 and is based on UE implementation,

the UE transmits only the channels/signals of the radio access technology with the highest priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

If a UE

-	would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive a second channel/signal or transmit second channels/signals using NR radio access, and

-	a transmission or reception of the first channel/signal would respectively overlap in time with a reception of the second channel/signal or transmission of the second channels/signals, and

-	the priorities of the channels/signals are known to both E-UTRA radio access and NR radio access at the UE T msec prior to the start of the earliest transmission or reception, where T≤4 and is based on UE implementation,

the UE transmits or receives only the channels/signals of the radio access technology with the highest priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

#### 16.2.4.2	Simultaneous PSFCH transmission/reception

If a UE

-	would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and

-	transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs

the UE transmits or receives only a set of PSFCHs corresponding to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs.

If a UE would transmit Nsch,Tx,PSFCH PSFCHs in a PSFCH transmission occasion, the UE transmits NTx,PSFCH PSFCHs corresponding to the smallest NTx,PSFCH priority field values indicated in all SCI formats 1-A associated with the PSFCH transmission occasion.

#### 16.2.4.3	Simultaneous SL and UL transmissions

If a UE

-	would simultaneously transmit on the UL and on the SL of a serving cell, and

-	the UE is not capable of simultaneous transmissions on the UL and on the SL of the serving cell

the UE transmits only on the link, UL or SL, with the higher priority.

If a UE

-	is capable of simultaneous transmissions on the UL and on the SL of two respective carriers of a serving cell, or of two respective serving cells,

-	would transmit on the UL and on the SL of the two respective carriers of the serving cell, or of the two respective serving cells,

-	the transmission on the UL would overlap with the transmission on the SL over a time period, and

-	the total UE transmission power over the time period would exceed PCMAX

the UE

-	reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

-	reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

##### 16.2.4.3.1	Prioritizations for sidelink and uplink transmissions

A UE performs prioritization between SL transmissions and UL transmissions after performing the procedures described in Clause 9.2.5 and in Clause 6.1 of [6, TS 38.214].

PSFCH transmissions in a slot have a same priority value as the smallest priority value among PSSCH receptions with corresponding HARQ-ACK information provided by the PSFCH transmissions in the slot.

A priority of S-SS/PSBCH block transmission is provided by sl-SSB-PriorityNR.

For prioritization between PSFCH/S-SS/PSBCH block transmission and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR, or a PUCCH with sidelink HARQ-ACK information report

-	if the UL transmission is for a PUSCH or for a PUCCH with priority index 1,

-	if sl-PriorityThreshold-UL-URLLC is provided

-	the SL transmission has higher priority than the UL transmission if a smallest priority value of the SL transmission(s) is smaller than sl-PriorityThreshold-UL-URLLC; otherwise, the UL transmission has higher priority than the SL transmission

-	else

-	the UL transmission has higher priority than the SL transmission

-	else

-	the SL transmission has higher priority than the UL transmission if the smallest priority value of the SL transmission(s) is smaller than sl-PriorityThreshold; otherwise, the UL transmission has higher priority than the SL transmission

A PRACH transmission, or a PUSCH scheduled by an UL grant in a RAR, has higher priority than a PSFCH or a S-SS/PSBCH block transmission.

A PUCCH transmission with a sidelink HARQ-ACK information report has higher priority than a SL transmission if a priority value of the PUCCH is smaller than a priority value of the SL transmission. The priority value of the PUCCH transmission is as described in Clause 16.5. If the priority value of the PUCCH transmission is larger than the priority value of the SL transmission, the SL transmission has higher priority.

When one or more SL transmissions from a UE overlap in time with multiple non-overlapping UL transmissions from the UE, the UE performs the SL transmissions if at least one SL transmission is prioritized over all UL transmissions subject to the UE processing timeline with respect to the first SL transmission and the first UL transmission.

When one or more UL transmissions from a UE overlap in time with multiple non-overlapping SL transmissions, the UE performs the UL transmissions if at least one UL transmission is prioritized over all SL transmissions subject to the UE processing timeline with respect to the first SL transmission and the first UL transmission.

When one SL transmission overlaps in time with one or more overlapping UL transmissions, the UE performs the SL transmission if the SL transmission is prioritized over all UL transmissions subject to both the UE multiplexing and processing timelines with respect to the first SL transmission and the first UL transmission, where the UE processing timeline with respect to the first SL transmission and the first UL transmission is same as when one or more SL transmissions overlap in time with multiple non-overlapping UL transmissions.

When one SL transmission overlaps in time with one or more overlapping UL transmissions, the UE performs the UL transmission if at least one UL transmission is prioritized over the SL transmission subject to both the UE multiplexing and processing timelines with respect to the first SL transmission and the first UL transmission, where the UE processing timeline with respect to the first SL transmission and the first UL transmission is same as when one or more SL transmissions overlap in time with multiple non-overlapping UL transmissions.

## 16.3	UE procedure for reporting HARQ-ACK on sidelink

A UE can be indicated by an SCI format scheduling a PSSCH reception, in one or more sub-channels from a number of Nsubch PSSCH sub-channels, to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK.

A UE can be provided, by sl-PSFCH-Period, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled.

A UE expects that a slot t'kSL (0≤k<T'max) has a PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where t'kSL is defined in [6, TS 38.214], and T'max is a number of slots that belong to the resource pool within 10240 msec according to [6, TS 38.214], and NPSSCHPSFCH is provided by sl-PSFCH-Period.

A UE may be indicated by higher layers to not transmit a PSFCH in response to a PSSCH reception [11, TS 38.321].

If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A or a SCI format 2-B has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception.

A UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH.

A UE determines a number of PSFCH resources available for multiplexing HARQ-ACK information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH where NCSPSFCH is a number of cyclic shift pairs for the resource pool and, based on an indication by higher layers,

-	Ntype PSFCH=1 and the Msubch, slotPSFCH PRBs are associated with the starting sub-channel of the corresponding PSSCH

-	Ntype PSFCH=Nsubch PSSCH and the Nsubch PSSCH⋅Msubch, slotPSFCH PRBs are associated with one or more sub-channels from the Nsubch PSSCH sub-channels of the corresponding PSSCH

The PSFCH resources are first indexed according to an ascending order of the PRB index, from the Ntype PSFCH⋅Msubch, slotPSFCH PRBs, and then according to an ascending order of the cyclic shift pair index from the NCSPSFCH cyclic shift pairs.

A UE determines an index of a PSFCH resource for a PSFCH transmission in response to a PSSCH reception as PID+MIDmodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI format 2-A or 2-B [5, TS 38.212] scheduling the PSSCH reception, and  and MID is the identity of the UE receiving the PSSCH as indicated by higher layers if the UE detects a SCI format 2-A with Cast type indicator field value of "01"; otherwise, MID is zero.

A UE determines a m0 value, for computing a value of cyclic shift α [4, TS 38.211], from a cyclic shift pair index corresponding to a PSFCH resource index and from NCSPSFCH using Table 16.3-1.

Table 16.3-1: Set of cyclic shift pairs

| NCSPSFCH | m0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cyclic Shift Pair Index 0 | Cyclic Shift Pair Index 1 | Cyclic Shift Pair Index 2 | Cyclic Shift Pair Index 3 | Cyclic Shift Pair Index 4 | Cyclic Shift Pair Index 5 |
| 1 | 0 | - | - | - | - | - |
| 2 | 0 | 3 | - | - | - | - |
| 3 | 0 | 2 | 4 | - | - | - |
| 6 | 0 | 1 | 2 | 3 | 4 | 5 |

A UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], as in Table 16.3-2 if the UE detects a SCI format 2-A with Cast type indicator field value of "01" or "10", or as in Table 16.3-3 if the UE detects a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11". The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211].

Table 16.3-2: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes ACK or NACK

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | 0 | 6 |

Table 16.3-3: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes only NACK

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | 0 | N/A |

### 16.3.1	UE procedure for receiving HARQ-ACK on sidelink

A UE that transmitted a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, attempts to receive associated PSFCHs according to PSFCH resources determined as described in Clause 16.3. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [10, TS 38.133]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource.

For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10"

-	report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01"

-	report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers

-	if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11"

-	report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers

## 16.4	UE procedure for transmitting PSCCH

A UE can be provided a number of symbols in a resource pool, by sl-TimeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by sl-FreqResourcePSCCH, for a PSCCH transmission with a SCI format 1-A.

A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] sets

-	"Resource reservation period" as an index in sl-ResourceReservePeriod1 corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource

-	the values of the frequency resource assignment field and the time resource assignment field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices  yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where:

-	N=minNselected, Nmax_reserve, where Nselected is a number of resources in the set Ry with slot indices  yj, 0≤j≤Nselected-1, such that y0<y1<…<yNselected-1≤y0+31, and Nmax_reserve is provided by sl-MaxNumPerReserve

-	each resource, from the set of Ry resources, corresponds to LsubCH contiguous sub-channels and a slot in a set of slots {tySL}, where LsubCH is the number of sub-channels available for PSSCH/PSCCH transmission in a slot

-	t'0SL,t'1SL,t'2SL,... is a set of slots in a sidelink resource pool [6, TS 38.214]

-	y0 is an index of a slot where the PSCCH with SCI format 1-A is transmitted.

For decoding of a SCI format 1-A, a UE may assume that a number of bits provided by sl-NumReservedBits can have any value.

## 16.5	UE procedure for reporting HARQ-ACK on uplink

A UE can be provided PUCCH resources or PUSCH resources [12, TS 38.331] to report HARQ-ACK information that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions. The UE reports HARQ-ACK information on the primary cell of the PUCCH group, as described in Clause 9, of the cell where the UE monitors PDCCH for detection of DCI format 3_0.

For SL configured grant Type 1 or Type 2 PSSCH transmissions by a UE within a time period provided by sl-PeriodCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources.

For PSSCH transmissions scheduled by a DCI format 3_0, a UE generates HARQ-ACK information in response to PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource in a set of time resources provided by the DCI format 3_0.

For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission. The UE can be indicated by a SCI format to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10"

-	generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from a PSFCH reception in the PSFCH reception occasion and, if the UE determines that a PSFCH is not received at the PSFCH reception occasion, generate NACK

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01"

-	generate ACK if the UE determines ACK from at least one PSFCH reception occasion, from the number of PSFCH reception occasions, in PSFCH resources corresponding to every identity MID of the UEs that the UE expects to receive the PSSCH, as described in Clause 16.3; otherwise, generate NACK

-	if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11"

-	generate ACK when the UE determines absence of PSFCH reception for each PSFCH reception occasion from the number of PSFCH reception occasions; otherwise, generate NACK

After a UE transmits PSSCHs and receives PSFCHs in corresponding PSFCH resource occasions, the priority value of HARQ-ACK information is same as the priority value of the PSSCH transmissions that is associated with the PSFCH reception occasions providing the HARQ-ACK information.

The UE generates a NACK when, due to prioritization, as described in Clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 with CRC scrambled by a SL-RNTI or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH transmission.

The UE generates a NACK when, due to prioritization as described in Clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 with CRC scrambled by SL-RNTI or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization.

The UE generates an ACK if the UE does not transmit a PSCCH with a SCI format 1-A scheduling a PSSCH in any of the resources provided by a configured grant in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the ACK is same as the largest priority value among the possible priority values for the configured grant.

A UE does not expect to be provided PUCCH resources or PUSCH resources to report HARQ-ACK information that start earlier than N+1∙2048+144∙κ∙2-μ∙Tc after the end of a last symbol of a last PSFCH reception occasion, from a number of PSFCH reception occasions that the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission, where

-	κ and Tc are defined in [4, TS 38.211]

-	μ=min⁡(μSL,μUL), where μSL is the SCS configuration of the SL BWP and μUL is the SCS configuration of the active UL BWP on the primary cell

-	N is determined from μ according to Table 16.5-1

Table 16.5-1: Values of N

| μ | N |
| --- | --- |
| 0 | 14 |
| 1 | 18 |
| 2 | 28 |
| 3 | 32 |

With reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in Clause 9.2.5, where k is a number of slots indicated by a PSFCH-to-HARQ_feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-PSFCH-ToPUCCH-CG-Type1. k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion assuming that the start of the sidelink frame is same as the start of the downlink frame [4, TS 38.211].

For a PSSCH transmission by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSSCH transmission activated by a DCI format, the DCI format indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 1 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN and sl-PSFCH-ToPUCCH-CG-Type1. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions.

For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources for OUCI HARQ-ACK information bits, as described in Clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212] in a last DCI format 3_0, among the DCI formats 3_0 that have a value of a PSFCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are indexed in an ascending order across PDCCH monitoring occasion indexes.

A UE does not expect to multiplex HARQ-ACK information for more than one SL configured grants in a same PUCCH.

A priority value of a PUCCH transmission with one or more sidelink HARQ-ACK information bits is the smallest priority value for the one or more HARQ-ACK information bits.

In the following, the CRC for DCI format 3_0 is scrambled with a SL-RNTI or a SL-CS-RNTI.

### 16.5.1	Type-1 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

If a UE is configured a SL configured grant type 1, and the UE is configured a SL configured grant type 2 or to monitor PDCCH for detection of DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI, and the UE is provided a set of slot timing values K1 associated with a SL BWP by sl-PSFCH-ToPUCCH and sl-PSFCH-ToPUCCH-CG-Type1, the sl-PSFCH-ToPUCCH-CG-Type1 is one of sl-PSFCH-ToPUCCH.

A UE reports HARQ-ACK information for PSSCH transmissions with corresponding PSFCH reception occasions in slot n only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PSFCH-to-HARQ_feedback timing indicator field in a DCI format 3_0 scheduling the PSSCH transmissions, or by a value of PSFCH-to-HARQ feedback timing indicator field in a DCI format 3_0 activating a SL configured grant Type-2 transmission, or by a value of sl-PSFCH-ToPUCCH for a SL configured grant Type-1. If the UE reports HARQ-ACK information for the PSSCH transmissions with corresponding PSFCH reception occasions in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	PSFCH reception occasions associated with PSSCH transmissions scheduled by a DCI format 3_0 with counter SAI field value of 1, or

-	PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant

within the MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions as determined in Clause 16.5.1.1, the UE determines a HARQ-ACK codebook only for the PSFCH reception occasion associated with PSSCH transmission scheduled by DCI format 3_0 or only for the PSFCH reception occasion associated with PSSCH transmission corresponding to a SL configured grant according to corresponding MA occasions, where a value of a counter SAI in DCI format 3_0 is according to Table 16.5.2.1-1. Otherwise, the procedures in Clause 16.5.1.1 and in Clause 16.5.1.2 for a HARQ-ACK codebook determination apply.

#### 16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on:

a)	a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1

b)	the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively

c)	a set of sidelink resource pool bitmaps

d)	a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period

For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code.

Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions

Set MA=∅

Set CK1 to the cardinality of set K1

Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1

Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool

while k< CK1

if mod nU-K1,k+1,max2μUL-μSL,1=0

Set nS=0 – index of a SL slot within an UL slot

while nS<max2μSL-μUL,1

if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell

nS=nS+1;

else

if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool and includes PSFCH resources as indicated by a sidelink resource pool bitmap and sl-PSFCH-Period, where K1,k is the k-th slot timing value in set K1

Set nF=0 – index of a SL slot within an PSFCH period

while nF<NPSFCH

MA=MA∪j;

j=j+1;

nF=nF+1;

end while

end if

nS=nS+1;

end if

end while

end if

k=k+1;

end while

The cardinality of the set MA defines a total number M of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions corresponding to the HARQ-ACK information bits. A UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of  OACK HARQ-ACK information bits as ojACK = HARQ-ACK information bit for candidate PSSCH transmission with index j with corresponding PSFCH reception, for  0 ≤j<M, as described in Clause 16.5. If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions.

If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as nHARQ-ACK=m=0M-1Nmreceived where Nmreceived is a number of HARQ-ACK information bits determined for corresponding PSSCH transmissions with corresponding PSFCH reception occasions in PSFCH reception occasion m.

#### 16.5.1.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0, then

-	if the UE

-	has not received any PDCCH with a DCI format 3_0 scheduling PSSCH transmissions with corresponding PSFCH reception occasions that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PSFCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PSSCH transmissions or on the value of PSFCH-to-HARQ feedback timing indicator field in a DCI format 3_0 activating a SL configured grant Type-2 transmission, or

-	has not been provided PSSCH resources with corresponding PSFCH reception occasions that the UE transmits corresponding HARQ-ACK information based on the value of sl-PSFCH-ToPUCCH-CG-Type1 for a SL configured grant Type-1, in any of the MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions, as described in Clause 16.5.1.1,

the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else the UE generates the HARQ-ACK codebook as described in Clause 16.5.1.1, unless the UE generates HARQ-ACK information only for

-	PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant, or

-	PSFCH reception occasions associated with PSSCH transmissions that are scheduled by DCI format 3_0 with a counter SAI field value of 1 in the MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions,

in which case the UE generates HARQ-ACK information only for the PSFCH reception occasions as described in Clause 16.5.1.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PSFCH reception occasions associated with PSSCH transmissions scheduled by a DCI format 3_0 that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a SAI field, the UE generates the HARQ-ACK codebook as described in Clause 16.5.1.1 when a value of the SAI field in the DCI format is VT-SAIUL=1. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-SAIUL=0 unless the UE generates HARQ-ACK information only for

-	PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant, or

-	PSFCH reception occasions associated with PSSCH transmissions that are scheduled by a DCI format 3_0 with a counter SAI field value of 1 in the MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions as described in Clause 16.5.1.

VT-SAIUL=0 if the SAI field in the DCI format is set to '0'; otherwise, VT-SAIUL=1.

### 16.5.2	Type-2 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic.

#### 16.5.2.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH reception occasions on an active DL BWP of a serving cell c, as described in Clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PSFCH-to-HARQ_feedback timing indicator field values, or a value provided by sl-PSFCH-ToPUCCH-CG-Type1, for PUCCH transmission with HARQ-ACK information in slot n in response to PSFCH receptions

-	Time gap field in DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH receptions

-	Time resource assignment in DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH receptions

-	A set of configured sidelink resource pool bitmaps

-	A value of a period of PSFCH resources provided in sl-PSFCH-Period.

-	A value of a minimum time gap provided in sl-MinTimeGapPSFCH.

The set of PDCCH monitoring occasions for DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH reception occasions is defined as the PDCCH monitoring occasions across active DL BWPs of the configured serving cell, indexed in ascending order of start time of the associated search space sets. The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions.

A value of a counter sidelink assignment indicator (SAI) field in DCI format 3_0 denotes an accumulative number of PDCCH monitoring occasions where PSSCH transmissions with associated PSFCH receptions are scheduled, up to a current PDCCH monitoring occasion, in ascending order of PDCCH monitoring occasion index m, where 0≤m<M.

Denote by VC-SAI,mSL the value of the counter SAI in DCI format 3_0 in PDCCH monitoring occasion m according to Table 16.5.2.1-1.

If the UE transmits HARQ-ACK information in a PUCCH in slot n, the UE determines the o0ACK,o1ACK,…,oOACK-1ACK, for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code:

Set m=0 – PDCCH with DCI format 3_0 monitoring occasion index: lower index corresponds to earlier PDCCH with DCI format 3_0 monitoring occasion

Set j=0

Set Vtemp=0

Set Vs=∅

Set M to the number of PDCCH monitoring occasions

while m<M

if PDCCH monitoring occasion m is before an active UL BWP change on the PCell

m=M;

else

if there is a PSFCH reception occasion associated with a PSSCH transmission scheduled by a DCI format in PDCCH monitoring occasion m

if VC-SAI,mSL≤Vtemp

j=j+1;

end if

Vtemp=VC-SAI,mSL

o4j+VC-SAI,mSL-1ACK = HARQ-ACK information bit

Vs=Vs∪4j+VC-SAI,mSL-1

end if

end if

m=m+1;

end while

OACK=4⋅j+Vtemp

oiACK=NACK for any i ∈0,1,…,OACK-1\Vs

if a SL configured grant type 1 is configured for a UE, or a SL configured grant type 2 is configured and activated for a UE, and the SL configured grant provides a grant for PSSCH transmissions with PSFCH reception occasions in a slot n-K1, where K1 is the PSFCH-to-HARQ-feedback timing value for the SL configured grant

OACK=OACK+1;

oOACK-1ACK= HARQ-ACK information bit associated with the PSFCH reception occasions associated with the PSSCH transmissions scheduled by the SL configured grant

end if

If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as

nHARQ-ACK=VSAI,mlastSL-USAImod4+m=0M-1Nmreceived+NCG

where

-	VSAI,mlastSL is a value of a counter SAI field in a last DCI format 3_0 scheduling PSSCH transmissions associated with PSFCH reception occasions that the UE detects within the M PDCCH monitoring occasions

-	VSAI,mlastSL=0 if the UE does not detect any DCI format 3_0 scheduling PSSCH transmissions associated with PSFCH reception occasions in any of the M PDCCH monitoring occasions

-	USAI is a total number of DCI format 3_0, scheduling PSSCH transmissions associated with PSFCH reception occasions, that the UE detects within the M PDCCH monitoring occasions. USAI=0 if the UE does not detect any DCI format 3_0 scheduling PSSCH transmissions with associated PSFCH reception occasions in any of the M PDCCH monitoring occasions

-	Nmreceived is a number of DCI format 3_0 scheduling PSSCH transmissions with associated PSFCH reception occasions that the UE detects in PDCCH monitoring occasion m

-	NCG is a number of SL configured grants for which the UE transmits corresponding HARQ-ACK information in a same PUCCH as for HARQ-ACK information corresponding to PSFCH reception occasions within the M PDCCH monitoring occasions

Table 16.5.2.1-1: Value of counter SAI in DCI format 3_0

| SAI MSB, LSB | VC-SAISL | Number of PDCCH monitoring occasions in which DCI format 3_0 scheduling PSSCH transmissions with corresponding PSFCH reception occasions is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0,0 | 1 | Y-1mod4+1=1 |
| 0,1 | 2 | Y-1mod4+1=2 |
| 1,0 | 3 | Y-1mod4+1=3 |
| 1,1 | 4 | Y-1mod4+1=4 |

#### 16.5.2.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0, then

-	if the UE

-	has not received any PDCCH within the monitoring occasions for DCI format 3_0 for scheduling PSSCH with corresponding PSFCH reception occasions on any serving cell, and

-	does not have HARQ-ACK information in response to a PSSCH transmission with corresponding PSFCH reception occasions associated with a SL configured grant to multiplex in the PUSCH, as described in Clause 16.5.2.1,

the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates and multiplexes in the PUSCH transmission the HARQ-ACK codebook as described in Clause 16.5.2.1.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a SAI field, the UE generates the HARQ-ACK codebook as described in Clause 16.5.2.1, with the following modifications:

-	For the pseudo-code for the HARQ-ACK codebook generation in Clause 16.5.2.1, after the completion of the m loop, the UE sets Vtemp=VSAIUL where VSAIUL is the value of the SAI field in the DCI format according to Table 16.5.2.2-1.

If a UE

-	is scheduled for a PUSCH transmission by a DCI format that includes a SAI field with value VT-SAIUL=4, and

-	has not received any PDCCH within the monitoring occasions for PDCCH with DCI format 3_0 for scheduling PSSCH with corresponding PSFCH reception occasions on a serving cell, and

-	does not have HARQ-ACK information in response to PSFCH reception occasions associated with a SL configured grant to multiplex in the PUSCH, as described in Clause 16.5.2.1,

the UE does not multiplex HARQ-ACK information in the PUSCH transmission.

Table 16.5.2.2-1: Value of SAI

| SAI MSB, LSB | VT-SAIUL | Number of PDCCH monitoring occasions in which DCI format 3_0 scheduling PSSCH transmissions with corresponding PSFCH reception occasions is present, denoted as X and X≥1 |
| --- | --- | --- |
| 0,0 | 1 | X-1mod4+1=1 |
| 0,1 | 2 | X-1mod4+1=2 |
| 1,0 | 3 | X-1mod4+1=3 |
| 1,1 | 4 | X-1mod4+1=4 |

## 16.6	UE procedure for LTE sidelink transmission

If the UE detects a DCI format 3_1 with CRC scrambled by SL Semi-Persistent Scheduling V-RNTI in slot n, the DCI format 3_1 activates or releases an LTE sidelink SPS configuration that is indicated by a SL SPS configuration index field [5, TS 38.212]. If the DCI format 3_1 activates an SL SPS configuration, the UE procedure for transmitting a PSCCH and a PSSCH is as described in [13, TS 36.213] except that a transmission starts no earlier than TDCI-NTA2×TC×103+X+4+m  ms, where TDCI is a start time of slot n, NTA and TC are defined in [4, TS 38.211], X is a value indicated by a Timing offset field in DCI format 3_1, and m is a value indicated by SL index field in DCI format 3_1 if the SL index field is present; otherwise, m=0.

## 16.7	Operation for in-device coexistence

If a UE would transmit or receive a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, when

-	the first channel/signal and the second channel/signal are time-division multiplexed, and

-	the UE knows the frame indexes of the first channel/signal and the frame indexes of the second channel/signal,

the UE transmits or receives each channel/signal so that the subframe boundary of the second channel/signal is aligned with the subframe boundary of the first channel/signal where the subframe boundary alignment is achieved by UE implementation means.