## C.2	Performance of studied NR positioning enhancements

Including performance of positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency ((objective 1c).

### C.2.1	Positioning accuracy analysis for NR positioning enhancements

#### C.2.1.1	Results from source [4]

##### C.2.1.1.1	Description of evaluation scenarios

The following enhancements are evaluated for accuracy:

- LOS/NLOS identification

- PRS/SRS frequency aggregation

- 1-symbol PRS

- PRS punctured by SSB

- AoA with ULA

- E-CID enhancement

- Synchronization error compensation

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Tables C.2.1.1.1-1 to Table C.2.1.1.1-4.

Table C.2.1.1.1-1: NR positioning enhancements (LOS/NLOS identification 31x and PRS/SRS frequency aggregation 32x) - evaluation scenarios and parameters

| Parameter | Case 311 (InF-DH, FR1) | Case 312 (InF-DH, FR1) | Case 313 (InF-DH, FR1) | Case 314 (InF-DH, FR1) | Case 321 (InF-DH, FR1) | Case 322 (InF-DH, FR1) | Case 323 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 200MHz | 50MHz+100MHz (gap)+50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution RAIM | Super resolution LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any |  |  |  |  |  |  |  |

Table C.2.1.1.1-2: NR positioning enhancements (1-symbol PRS 33x and PRS punctured by SSB 34x) - evaluation scenarios and parameters

| Parameter | Case 331 (InF-SH, FR1) | Case 332 (InF-SH, FR1) | Case 333 (InF-SH, FR1) | Case 334 (InF-SH, FR1) | Case 341 (InF-SH, FR1) | Case 342 (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 1 symbol) | DL-PRS (Comb-12, 12 symbol) | DL-PRS (Comb-12, 1 symbol) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 1 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port | Gold, single port | Gold, single port | Gold, single port | Gold, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 10.8dB | 10.8dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any |  |  |  |  |  | PRS center 20RBs are punctured by SSB |

Table C.2.1.1.1-3: Rel.16 NR positioning (UL-AoA enhancement with ULA 36x and E-CID enhancement 37x) - evaluation scenarios and parameters

| Parameter | Case 361 (InF-DH, FR1) | Case 362 (InF-DH, FR1) | Case 363 (InF-DH, FR1) | Case 371 (InF-DH, FR1) | Case 372 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | CSI-RS+SRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 1 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA+UL-AoA PSO | UL-TDOA+UL-AoA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | E-CID (RTT+AoA only from the serving cell) |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | UPA 4x4x2 | ULA 1x4x1 assuming legacy AoA | ULA 1x4x1 with modified AoA |  |  |

Table C.2.1.1.1-4: NR positioning enhancements (Synchronization error compensation) - evaluation scenarios and parameters

| Parameter | Case 381 (InF-SH, FR1) | Case 382 (InF-SH, FR1) | Case 383 (InF-SH, FR1) | Case 384 (InF-SH, FR1) | Case 385 (InF-SH, FR1) | Case 386 (InF-SH, FR1) | Case 387 (InF-SH, FR1) | Case 388 (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Network synchronization assumptions | 0ns | 0.2ns | 0.5ns | 1ns | 0ns | 0.2ns | 0.5ns | 1ns |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | 100MHz contiguous | 100MHz contiguous | 100MHz contiguous | 100MHz contiguous | 200MHz contiguous | 200MHz contiguous | 200MHz contiguous | 200MHz contiguous |

##### C.2.1.1.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.1.2-1:

Table C.2.1.1.2-1: NR positioning enhancements - horizontal location error results from [4]

| Features | Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| LOS/NLOS identification | 311, InF-DH422, FR1, UL-TDOA | (Optional) All UEs | 0.4309 | 1.6317 | 4.9693 | 9.9453 |
|  |  | Convex UEs | 0.3354 | 1.2393 | 4.6237 | 9.6631 |
|  | 312, InF-DH422, FR1, UL-TDOA w/ RAIM | (Optional) All UEs | 0.0371 | 0.0748 | 0.1912 | 1.4517 |
|  |  | Convex UEs | 0.0301 | 0.0554 | 0.1217 | 0.6569 |
|  | 313, InF-DH422, FR1, UL-TDOA w/ LOS/NLOS identification | (Optional) All UEs | 0.0397 | 0.0701 | 0.1354 | 0.4836 |
|  |  | Convex UEs | 0.0311 | 0.0505 | 0.0862 | 0.2111 |
|  | 314, InF-DH422, FR1, UL-TDOA w/ ideal LOS selection | (Optional) All UEs | 0.0394 | 0.0608 | 0.1399 | 0.4452 |
|  |  | Convex UEs | 0.0310 | 0.0506 | 0.0855 | 0.2022 |
| PRS/SRS frequency aggregation | 321, InF-DH422, FR1, UL-TDOA, 100M contiguous | (Optional) All UEs | 0.0394 | 0.0608 | 0.1399 | 0.4452 |
|  |  | Convex UEs | 0.0310 | 0.0506 | 0.0855 | 0.2022 |
|  | 322, InF-DH422, FR1, UL-TDOA, 200M contiguous | (Optional) All UEs | 0.0106 | 0.0179 | 0.0332 | 0.0827 |
|  |  | Convex UEs | 0.0082 | 0.0122 | 0.0197 | 0.0384 |
|  | 323, InF-DH422, FR1, UL-TDOA, 50MHz+100MHz (Gap)+50MHz | (Optional) All UEs | 0.0148 | 0.0273 | 0.0570 | 0.2244 |
|  |  | Convex UEs | 0.0109 | 0.0189 | 0.0337 | 0.0912 |
| 1-symbol PRS | 331, InF-SH, FR1, DL-TDOA, Comb-4 and 4-symbol | (Optional) All UEs | 0.0252 | 0.0425 | 0.0718 | 0.1531 |
|  |  | Convex UEs | 0.0196 | 0.0314 | 0.0504 | 0.0939 |
|  | 332, InF-SH, FR1, DL-TDOA, Comb-4 and 1-symbol | (Optional) All UEs | 0.0268 | 0.0473 | 0.0836 | 0.1693 |
|  |  | Convex UEs | 0.0210 | 0.0347 | 0.0567 | 0.1123 |
|  | 333, InF-SH, FR1, DL-TDOA, Comb-12 and 12-symbol | (Optional) All UEs | 0.0279 | 0.0492 | 0.0835 | 0.1647 |
|  |  | Convex UEs | 0.0219 | 0.0347 | 0.0573 | 0.1091 |
|  | 334, InF-SH, FR1, DL-TDOA, Comb-12 and 1-symbol | (Optional) All UEs | 0.0284 | 0.0484 | 0.0866 | 0.1784 |
|  |  | Convex UEs | 0.0209 | 0.0359 | 0.0595 | 0.1199 |
| PRS punctured by SSB | 341, InF-SH, FR1, DL-TDOA | (Optional) All UEs | 0.0252 | 0.0425 | 0.0718 | 0.1531 |
|  |  | Convex UEs | 0.0196 | 0.0314 | 0.0504 | 0.0939 |
|  | 342, InF-SH, FR1, DL-TDOA, 20-RB of PRS punctured by SSB | (Optional) All UEs | 0.0293 | 0.0475 | 0.0846 | 0.1699 |
|  |  | Convex UEs | 0.0213 | 0.0346 | 0.0539 | 0.1090 |
| UL-AoA enhancement with ULA | 361, InF-DH422, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0216 | 0.0318 | 0.0460 | 0.0735 |
|  |  | Convex UEs | 0.0203 | 0.0276 | 0.0396 | 0.0665 |
|  | 362, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ legacy AoA) | (Optional) All UEs | 2.4977 | 3.3503 | 4.1807 | 4.9986 |
|  |  | Convex UEs | 2.2164 | 3.1038 | 3.9067 | 4.8161 |
|  | 363, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ modified AoA | (Optional) All UEs | 0.0406 | 0.0626 | 0.1027 | 0.2058 |
|  |  | Convex UEs | 0.0348 | 0.0525 | 0.0821 | 0.1694 |
| E-CID enhancement | 371, InF-DH422, FR1, Multi-RTT | (Optional) All UEs | 0.0423 | 0.0591 | 0.0901 | 0.1756 |
|  |  | Convex UEs | 0.0388 | 0.0555 | 0.0871 | 0.1694 |
|  | 372, InF-DH422, FR1, E-CID w/ single cell RTT/AoA | (Optional) All UEs | 0.0471 | 0.0651 | 0.1068 | 0.2497 |
|  |  | Convex UEs | 0.0473 | 0.0647 | 0.1040 | 0.2395 |
| Synchronization error compensation | 381, InF-SH, FR1, UL-TDOA, 100M，0ns gNB Sync error | (Optional) All UEs | 0.0369 | 0.0622 | 0.1026 | 0.1867 |
|  |  | Convex UEs | 0.0263 | 0.0433 | 0.0654 | 0.1136 |
|  | 382, InF-SH, FR1, UL-TDOA, 100M，0.2ns gNB Sync error | (Optional) All UEs | 0.0828 | 0.1153 | 0.1643 | 0.2672 |
|  |  | Convex UEs | 0.0662 | 0.0855 | 0.1096 | 0.1452 |
|  | 383, InF-SH, FR1, UL-TDOA, 100M，0.5ns gNB Sync error | (Optional) All UEs | 0.1811 | 0.2482 | 0.3330 | 0.5173 |
|  |  | Convex UEs | 0.1454 | 0.1877 | 0.2306 | 0.2828 |
|  | 384, InF-SH, FR1, UL-TDOA, 100M，1ns gNB Sync error | (Optional) All UEs | 0.3500 | 0.4709 | 0.6348 | 1.0315 |
|  |  | Convex UEs | 0.2731 | 0.3530 | 0.4353 | 0.5372 |
|  | 385, InF-SH, FR1, UL-TDOA, 200M, 0ns gNB Sync error | (Optional) All UEs | 0.0086 | 0.0144 | 0.0240 | 0.0456 |
|  |  | Convex UEs | 0.0065 | 0.0101 | 0.0149 | 0.0250 |
|  | 386, InF-SH, FR1, UL-TDOA, 200M, 0.2ns gNB Sync error | (Optional) All UEs | 0.0710 | 0.0971 | 0.1330 | 0.2158 |
|  |  | Convex UEs | 0.0558 | 0.0726 | 0.0901 | 0.1102 |
|  | 387 InF-SH, FR1, UL-TDOA, 200M, 0.5ns gNB Sync error | (Optional) All UEs | 0.1725 | 0.2328 | 0.3055 | 0.4895 |
|  |  | Convex UEs | 0.1406 | 0.1790 | 0.2178 | 0.2659 |
|  | 388 InF-SH, FR1, UL-TDOA, 200M, 1ns gNB Sync error | (Optional) All UEs | 0.3490 | 0.4658 | 0.6330 | 0.9466 |
|  |  | Convex UEs | 0.2735 | 0.3582 | 0.4407 | 0.5340 |

Figure C.2.1.1.2-1 provides the positioning evaluation results for LOS/NLOS identification.

Figure C.2.1.1.2-2 provides the positioning evaluation results for PRS/SRS frequency aggregation.

Figure C.2.1.1.2-3 provides the positioning evaluation results for 1-symbol PRS.

Figure C.2.1.1.2-4 provides the positioning evaluation results for PRS punctured by SSB.

Figure C.2.1.1.2-5 provides the positioning evaluation results for UL-AoA enhancement with ULA.

Figure C.2.1.1.2-6 provides the positioning evaluation results for E-CID enhancement.

Figure C.2.1.1.2-7 and Figure C.2.1.1.2-8 provide the positioning evaluation results for synchronization error compensation.

Figure C.2.1.1.2-1: Positioning error results for LOS/NLOS identification from [4]

Figure C.2.1.1.2-2: Positioning error results for PRS/SRS frequency aggregation from [4]

Figure C.2.1.1.2-3: Positioning error results for 1-symbol PRS from [4]

Figure C.2.1.1.2-4: Positioning error results for PRS punctured by SSB from [4]

Figure C.2.1.1.2-5: Positioning error results for UL-AoA enhancement with ULA from [4]

Figure C.2.1.1.2-6: Positioning error results for E-CID enhancement from [4]

Figure C.2.1.1.2-7: Positioning error results for synchronization error compensation from [4]

Figure C.2.1.1.2-8: Positioning error results for synchronization error compensation from [4]

#### C.2.1.2	Results from source [7]

##### C.2.1.2.1	Description of evaluation scenarios

The evaluation scenarios in this contribution for verifying achievable accuracy based on enhanced positioning methods  include,

- InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, ideal LOS identification is assumed.

- InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, Rician K-factor as assistance information to determine LOS and NLOS links.

- InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, coherence bandwidth as assistance information to determine LOS and NLOS links.

Some scenario common parameters can be found in Table C.1.2.2.1-1. In addition, Table C.2.1.2.1-1 reveals some controlled variables of all simulation cases.

Table C.2.1.2.1-1 All simulation cases for positioning accuracy evaluation based on enhanced positioning methods

| Simulation cases | Scenario | FR1/FR2 | Whether Rel-17 enhancement is implemented | UE horizontal drop procedure | gNB Tx calibration error | Clutter parameters |
| --- | --- | --- | --- | --- | --- | --- |
| Case 12 | InF-DH | FR1 | NO | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 25 | InF-DH | FR1 | Ideal LOS identification | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 26 | InF-DH | FR1 | Rician K-factor | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 27 | InF-DH | FR1 | Coherence bandwidth | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 17 | InF-DH | FR2 | NO | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 28 | InF-DH | FR2 | Ideal LOS identification | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 29 | InF-DH | FR2 | Rician K-factor | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 30 | InF-DH | FR2 | Coherence bandwidth | Inside convex hull | 0ns | {40%, 2m, 2m} |

##### C.2.1.2.2	Positioning accuracy evaluation results for NR positioning enhancements

Table C.1.1.2.2-1 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-DH scenario based on enhanced positioning method, and corresponding CDF curves can be found in Figure C.2.1.2.2-1.

Table C.2.1.2.2-1: NR positioning enhancements - horizontal location error results from [7]

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 12 | 0.414 | 1.060 | 7.337 | 12.345 |
| Case 25 | 0.261 | 0.371 | 0.493 | 0.648 |
| Case 26 | 0.294 | 0.4207 | 0.786 | 7.205 |
| Case 27 | 0.300 | 0.4232 | 0.659 | 2.003 |
| Case 17 | 0.062 | 0.097 | 0.174 | 12.174 |
| Case 28 | 0.047 | 0.062 | 0.080 | 0.107 |
| Case 29 | 0.053 | 0.074 | 0.101 | 9.248 |
| Case 30 | 0.051 | 0.069 | 0.091 | 0.228 |

Figure C.2.1.2.2-1-1 InF-DH, FR1                      Figure C.2.1.2.2-2 InF-DH, FR2

Figure C.2.1.2.2-1 Rel-17 enhancements with LOS identification

#### C.2.1.3	Results from source [8]

##### C.2.1.3.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.3.1-1.

- Case ID: 1 to 2.

- Scenario: InF-HH-2D.

- Frequency Band: FR1.

- Positioning Technique: DL-TDOA +DL-CPP, UL-TDOA + UL-CPP [R1-2007860].

Table C.2.1.3.1-1: NR positioning enhancements - evaluation scenarios and parameters [8]

| Parameter | [Case 1], [InF-HH-2D], [FR1], [DL-TDOA+ DL-CPP] | [Case 2], [InF-HH-2D], [FR1], [UL-TDOA+ UL-CPP] |
| --- | --- | --- |
| Channel model (baseline) | InF-HH-2D | InF-HH-2D |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 |
| Number of symbols used per occasion | 12 | 12 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC + PLL | MUSIC + PLL |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | EKF + Chan | EKF + Chan |
| Network synchronization assumptions | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No |
| Evaluated Enhancement for Rel.17 | DL-TDOA+ DL-CPP | UL-TDOA+ UL-CPP |
| Additional Notes, if any | Double differential with reference UE to overcome network synchronization error [R1-2007860] | Double differential with reference UE to overcome network synchronization error [R1-2007860] |

##### C.2.1.3.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.3.2-1:

Table C.2.1.3.2-1: NR positioning enhancements - horizontal location error results from [8]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-HH-2D], [FR1], [DL- TDOA+ DL-CPP] | Convex UEs | 0.0218 | 0.0321 | 0.0387 | 0.051 |
| [Case 2], [InF-HH-2D], [FR1], [UL- TDOA+ UL-CPP] | Convex UEs | 0.0196 | 0.0306 | 0.0368 | 0.049 |

#### C.2.1.4	Results from source [13]

##### C.2.1.4.1	Description of evaluation scenarios

| Parameter | Case 1 [InF-SH, UL-TDOA, FR1, 100 MHz] | Case 1 [InF-DH, UL-TDOA, FR1, 100 MHz] |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-2, 2 symbol SRS-Pos | Comb-2, 2 symbol SRS-Pos |
| Reference signal (type of sequence, number of ports, …) | NR SRS-Pos | NR SRS-Pos |
| Number of sites | 18 | 18 |
| Number of symbols used per slot per positioning estimate | 2 symbols per slot | 2 symbols per slot |
| Number of slots per positioning estimate | 1 slot | 1 slot |
| Power-boosting level | fixed power (max) | fixed power (max) |
| Uplink power control (applied/not applied) | Not applicable | Not applicable |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Thresholding, 0.5, (Oversampling x4) | Thresholding, 0.5, (Oversampling x4) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Taylor Series, Least Squares | Taylor Series, Least Squares |
| Network synchronization assumptions | Perfect Synchronization | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | N/A | N/A |
| Additional notes, if any | LoS detector and Outlier rejection algorithm based on relative path powers | LoS detector and Outlier rejection algorithm based on relative path powers |

##### C.2.1.4.2	Positioning accuracy evaluation results for NR positioning enhancements

| Case | UEs | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, [InF-SH, UL-TDOA, FR1,100 MHz] | Convex UEs | 0.38 | 0.55 | 1.06 | 1.94 |
| Case 2, [InF-DH, UL-TDOA, FR1, 100 MHz] | Convex UEs | 1.60 | 2.35 | 3.08 | 4.2 |

#### C.2.1.5	Results from source [5]

##### C.2.1.5.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.5.1-1- Table C.2.1.5.1-18.

Table C.2.1.5.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E1] (InF-SH, FR1) | [Case E2] (InF-SH, FR1) | [Case E3] (InF-SH, FR2) | [Case E4] (InF-SH, FR2) | [Case E5] (InF-DH, FR1) | [Case E6] (InF-DH, FR1) | [Case E7] (InF-DH, FR2) | [Case E8] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-2: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E9] (InF-SH, FR1) | [Case E10] (InF-SH, FR1) | [Case E11] (InF-SH, FR2) | [Case E12] (InF-SH, FR2) | [Case E13] (InF-DH, FR1) | [Case E14] (InF-DH, FR1) | [Case E15] (InF-DH, FR2) | [Case E16] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-3: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E17] (InF-SH, FR1) | [Case E18] (InF-SH, FR1) | [Case E19] (InF-SH, FR2) | [Case E20] (InF-SH, FR2) | [Case E21] (InF-DH, FR1) | [Case E22] (InF-DH, FR1) | [Case E23] (InF-DH, FR2) | [Case E24] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-4: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E25] (InF-SH, FR1) | [Case E26] (InF-SH, FR1) | [Case E27] (InF-SH, FR2) | [Case E28] (InF-SH, FR2) | [Case E29] (InF-DH, FR1) | [Case E30] (InF-DH, FR1) | [Case E31] (InF-DH, FR2) | [Case E32] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-5: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E33] (InF-SH, FR1) | [Case E34] (InF-SH, FR1) | [Case E35] (InF-SH, FR2) | [Case E36] (InF-SH, FR2) | [Case E37] (InF-DH, FR1) | [Case E38] (InF-DH, FR1) | [Case E39] (InF-DH, FR2) | [Case E40] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-6: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E41] (InF-SH, FR1) | [Case E42] (InF-SH, FR1) | [Case E43] (InF-SH, FR2) | [Case E44] (InF-SH, FR2) | [Case E45] (InF-DH, FR1) | [Case E46] (InF-DH, FR1) | [Case E47] (InF-DH, FR2) | [Case E48] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-7: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E49] (InF-SH, FR1) | [Case E50] (InF-SH, FR1) | [Case E51] (InF-SH, FR1) | [Case E52] (InF-SH, FR1) | [Case E53] (InF-SH, FR1) | [Case E54] (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | 100% LOS detection probability without RAIM | 100% LOS detection probability with RAIM | 95% LOS detection probability without RAIM | 90% LOS detection probability without RAIM | no LOS detection without RAIM |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-8: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E55] (InF-DH, FR1) | [Case E56] (InF-DH, FR1) | [Case E57] (InF-DH, FR1) | [Case E58] (InF-DH, FR1) | [Case E59] (InF-DH, FR1) | [Case E60] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | 100% LOS detection probability without RAIM | 100% LOS detection probability with RAIM | 95% LOS detection probability without RAIM | 90% LOS detection probability without RAIM | no LOS detection without RAIM |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-9: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E61] (InF-DH, FR1) | [Case E62] (InF-DH, FR1) | [Case E63] (InF-DH, FR1) | [Case E64] (InF-DH, FR1) | [Case E65] (InF-DH, FR1) | [Case E66] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM timing report granularity 0.5ns | RAIM timing report granularity 1ns | RAIM timing report granularity 2ns | RAIM timing report granularity 0.5ns | RAIM timing report granularity 1ns | RAIM timing report granularity 2ns |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-10: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E67] (InF-SH, FR1) | [Case E68] (InF-SH, FR1) | [Case E69] (InF-SH, FR1) | [Case E70] (InF-SH, FR1) | [Case E71] (InF-SH, FR1) | [Case E72] (InF-SH, FR1) | [Case E73] (InF-SH, FR1) | [Case E74] (InF-SH, FR1) | [Case E75] (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-11: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E76] (InF-DH, FR1) | [Case E77] (InF-DH, FR1) | [Case E78] (InF-DH, FR1) | [Case E79] (InF-DH, FR1) | [Case E80] (InF-DH, FR1) | [Case E81] (InF-DH, FR1) | [Case E82] (InF-DH, FR1) | [Case E83] (InF-DH, FR1) | [Case E84] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-12: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E85] (InF-SH, FR1) | [Case E86] (InF-SH, FR1) | [Case E87] (InF-SH, FR1) | [Case E88] (InF-SH, FR1) | [Case E89] (InF-SH, FR1) | [Case E90] (InF-SH, FR1) | [Case E91] (InF-SH, FR1) | [Case E92] (InF-SH, FR1) | [Case E93] (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-13: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E94] (InF-DH, FR1) | [Case E95] (InF-DH, FR1) | [Case E96] (InF-DH, FR1) | [Case E97] (InF-DH, FR1) | [Case E98] (InF-DH, FR1) | [Case E99] (InF-DH, FR1) | [Case E100] (InF-DH, FR1) | [Case E101] (InF-DH, FR1) | [Case E102] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-14: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E103] (InF-SH, FR1) | [Case E104] (InF-SH, FR1) | [Case E105] (InF-SH, FR1) | [Case E106] (InF-DH, FR1) | [Case E107] (InF-DH, FR1) | [Case E108] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 50MHz | 100MHz | 50MHz+50MHz | 50MHz | 100MHz | 50MHz+50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM Aggregation of DL positioning frequency layers | RAIM | RAIM | RAIM Aggregation of DL positioning frequency layers |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-15: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E109] (IOO, FR1) | [Case E110] (IOO, FR2) | [Case E111] (IOO, FR1) | [Case E112] (IOO, FR2) | [Case E113] (IOO, FR1) | [Case E114] (IOO, FR2) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 4GHz | 30GHz | 4GHz | 30GHz | 4GHz | 30GHz |
| Subcarrier spacing | 30kHz | 120kHz | 30kHz | 120kHz | 30kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-16: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E115] (InF-SH, FR1) | [Case E116] (InF-SH, FR1) | [Case E117] (InF-SH, FR1) | [Case E118] (InF-SH, FR1) | [Case E119] (InF-DH, FR1) | [Case E120] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (60%, 6, 2) | InF-DH (60%, 6, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN | 1 port, QPSK-PN | 1 port, ZC sequence | 1 port, ZC sequence | 1 port, QPSK-PN | 1 port, QPSK-PN |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | DL:7.78dB | DL:7.78dB | UL:6dB | UL:6dB | DL:7.78dB | DL:7.78dB |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution | super resolution | super resolution | Machine learning | Machine learning |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | UL-AOA+AOA taylor series first/median peak | UL-AOA+AOA taylor series first/median peak | Machine learning | Machine learning |
| Network synchronization assumptions | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 | BS timing error 5ns, UE timing error 0.5ns | 0 | BS timing error 5ns, UE timing error 0.5ns | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM Sync error reduced by differential positioning | RAIM Rx/Tx timing error reduced by differential positioning | RAIM Sync error reduced by UL-TDOA+AOA | RAIM Rx/Tx timing error reduced by UL-TDOA+AOA | Machine learning | Machine learning |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-17: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E121] (InF-SH, FR1) | [Case E122] (InF-SH, FR1) | [Case E123] (InF-SH, FR1) | [Case E124] (InF-SH, FR1) | [Case E125] (InF-DH, FR1) | [Case E126] (InF-DH, FR1) | [Case E127] (InF-DH, FR1) | [Case E128] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | nF-DH (40%, 2, 2) | nF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 50M+50M |  |  |  |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak |  |  |  |  |  |  |  |
| Network synchronization assumptions | Perfect sync |  |  |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM Aggregation of DL positioning frequency layers Timing offset 1ns | RAIM Aggregation of DL positioning frequency layers Timing offset 5ns | RAIM Aggregation of DL positioning frequency layers Timing offset 10ns | RAIM Aggregation of DL positioning frequency layers Timing offset 20ns | RAIM Aggregation of DL positioning frequency layers Timing offset 1ns | RAIM Aggregation of DL positioning frequency layers Timing offset 5ns | RAIM Aggregation of DL positioning frequency layers Timing offset 10ns | RAIM Aggregation of DL positioning frequency layers Timing offset 20ns |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-18: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E-V1] (InF-SH, FR1) | [Case E-V2] (InF-DH, FR1) | [Case E- V3] (InF-SH, FR1) | [Case E-V4] (InF-DH, FR1) | [Case E- V5] (InF-SH, FR1) | [Case E-V6] (InF-DH, FR1) | [Case E-V7] (InF-SH, FR1) | [Case E-V8] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH (BS height = 8m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = 8m UE height =1.5m) | InF-SH (BS height = {4,8}m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =1.5m) | InF-SH (BS height =8m UE height =[0.5,2]m) | InF-DH (40%, 2, 2), (BS height =8m UE height =[0.5,2]m) | InF-SH (BS height = {4,8}m UE height =[0.5,2]m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =[0.5,2]m) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

##### C.2.1.5.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.5.2-1.1- Table C.2.1.5.2-1.3:

Table C.2.1.5.2-1.1: NR positioning enhancements - horizontal location error results from [5]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.039 | 0.049 | 0.072 | 0.099 |
|  | (Optional) All UEs | 0.049 | 0.080 | 0.20 | 0.89 |
| [Case E2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.25 | 13.23 | 17.16 | 26.57 |
|  | (Optional) All UEs | 13.09 | 17.89 | 25.44 | 34.70 |
| [Case E3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.009 | 0.012 | 0.016 | 0.024 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.042 | 0.34 |
| [Case E4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.78 | 12.77 | 15.84 | 26.39 |
|  | (Optional) All UEs | 13.52 | 17.85 | 23.58 | 33.52 |
| [Case E5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.058 | 0.10 | 0.18 | 4.43 |
|  | (Optional) All UEs | 0.11 | 0.26 | 0.85 | 5.72 |
| [Case E6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.32 | 11.97 | 14.63 | 21.94 |
|  | (Optional) All UEs | 10.80 | 14.46 | 17.66 | 23.84 |
| [Case E7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.013 | 0.026 | 4.35 |
|  | (Optional) All UEs | 0.015 | 0.027 | 0.21 | 4.98 |
| [Case E8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.52 | 13.61 | 16.58 | 18.89 |
|  | (Optional) All UEs | 11.74 | 15.26 | 18.18 | 22.95 |
| [Case E9], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.30 |
| [Case E10], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.42 | 13.97 | 17.67 | 25.67 |
|  | (Optional) All UEs | 13.67 | 17.73 | 23.65 | 37.47 |
| [Case E11], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.009 | 0.012 | 0.017 | 0.024 |
|  | (Optional) All UEs | 0.012 | 0.019 | 0.033 | 0.10 |
| [Case E12], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 10.84 | 13.91 | 17.53 | 23.80 |
|  | (Optional) All UEs | 12.57 | 16.84 | 24.54 | 34.93 |
| [Case E13], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
|  | (Optional) All UEs | 0.087 | 0.14 | 0.25 | 0.62 |
| [Case E14], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 8.91 | 12.07 | 14.26 | 19.69 |
|  | (Optional) All UEs | 11.42 | 15.01 | 17.54 | 24.96 |
| [Case E15], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.011 | 0.015 | 0.018 | 0.034 |
|  | (Optional) All UEs | 0.015 | 0.022 | 0.040 | 0.32 |
| [Case E16], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 9.94 | 13.35 | 15.10 | 18.17 |
|  | (Optional) All UEs | 11.78 | 14.66 | 18.03 | 21.48 |
| [Case E17], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.037 | 0.049 | 0.061 | 0.10 |
|  | (Optional) All UEs | 0.051 | 0.086 | 0.17 | 0.98 |
| [Case E18], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.80 | 13.23 | 17.04 | 26.38 |
|  | (Optional) All UEs | 13.09 | 17.30 | 25.36 | 35.25 |
| [Case E19], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.009 | 0.012 | 0.016 | 0.034 |
|  | (Optional) All UEs | 0.012 | 0.019 | 0.036 | 0.31 |
| [Case E20], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.40 | 13.17 | 19.07 | 26.42 |
|  | (Optional) All UEs | 13.17 | 17.91 | 24.74 | 35.56 |
| [Case E21], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.058 | 0.098 | 0.18 | 5.48 |
|  | (Optional) All UEs | 0.10 | 0.21 | 1.14 | 5.98 |
| [Case E22], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.91 | 13.28 | 16.02 | 22.90 |
|  | (Optional) All UEs | 12.26 | 15.28 | 18.57 | 23.85 |
| [Case E23], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.020 | 0.089 | 4.55 |
|  | (Optional) All UEs | 0.016 | 0.029 | 0.50 | 5.96 |
| [Case E24], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.52 | 13.61 | 16.21 | 18.75 |
|  | (Optional) All UEs | 12.23 | 15.25 | 18.43 | 23.60 |
| [Case E25], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.037 | 0.048 | 0.060 | 0.083 |
|  | (Optional) All UEs | 0.050 | 0.072 | 0.13 | 0.25 |
| [Case E26], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.50 | 14.13 | 17.81 | 26.09 |
|  | (Optional) All UEs | 13.61 | 17.81 | 23.77 | 37.64 |
| [Case E27], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.009 | 0.010 | 0.016 | 0.032 |
|  | (Optional) All UEs | 0.011 | 0.018 | 0.033 | 0.11 |
| [Case E28], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.50 | 14.38 | 18.06 | 26.01 |
|  | (Optional) All UEs | 14.13 | 16.70 | 24.49 | 36.40 |
| [Case E29], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.053 | 0.073 | 0.11 | 0.19 |
|  | (Optional) All UEs | 0.086 | 0.16 | 0.27 | 0.65 |
| [Case E30], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 10.0 | 13.35 | 16.53 | 22.54 |
|  | (Optional) All UEs | 12.77 | 15.12 | 18.62 | 26.28 |
| [Case E31], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.011 | 0.014 | 0.019 | 0.043 |
|  | (Optional) All UEs | 0.015 | 0.022 | 0.051 | 0.34 |
| [Case E32], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 9.93 | 13.21 | 16.55 | 20.23 |
|  | (Optional) All UEs | 12.11 | 15.60 | 19.18 | 24.03 |
| [Case E33], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.040 | 0.052 | 0.071 | 0.11 |
|  | (Optional) All UEs | 0.054 | 0.076 | 0.12 | 0.51 |
| [Case E34], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.043 | 0.055 | 0.069 | 0.14 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.62 |
| [Case E35], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.014 | 0.019 | 0.049 |
|  | (Optional) All UEs | 0.013 | 0.021 | 0.036 | 0.14 |
| [Case E36], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.011 | 0.015 | 0.019 | 0.12 |
|  | (Optional) All UEs | 0.014 | 0.021 | 0.034 | 0.19 |
| [Case E37], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.068 | 0.12 | 0.23 | 4.89 |
|  | (Optional) All UEs | 0.091 | 0.14 | 0.78 | 6.39 |
| [Case E38], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.063 | 0.092 | 0.26 | 5.12 |
|  | (Optional) All UEs | 0.092 | 0.16 | 0.88 | 6.84 |
| [Case E39], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.013 | 0.019 | 0.11 | 4.12 |
|  | (Optional) All UEs | 0.018 | 0.065 | 0.59 | 4.82 |
| [Case E40], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.013 | 0.019 | 0.039 | 4.26 |
|  | (Optional) All UEs | 0.033 | 0.081 | 0.78 | 5.49 |
| [Case E41], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.040 | 0.050 | 0.068 | 0.092 |
|  | (Optional) All UEs | 0.044 | 0.066 | 0.097 | 0.24 |
| [Case E42], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.048 | 0.058 | 0.067 | 0.092 |
|  | (Optional) All UEs | 0.049 | 0.069 | 0.12 | 0.23 |
| [Case E43], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.010 | 0.014 | 0.018 | 0.030 |
|  | (Optional) All UEs | 0.013 | 0.019 | 0.030 | 0.055 |
| [Case E44], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.012 | 0.016 | 0.021 | 0.037 |
|  | (Optional) All UEs | 0.016 | 0.021 | 0.033 | 0.072 |
| [Case E45], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.059 | 0.083 | 0.11 | 0.19 |
|  | (Optional) All UEs | 0.078 | 0.12 | 0.18 | 0.31 |
| [Case E46], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.060 | 0.091 | 0.13 | 0.20 |
|  | (Optional) All UEs | 0.086 | 0.13 | 0.19 | 0.35 |
| [Case E47], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.012 | 0.016 | 0.020 | 0.048 |
|  | (Optional) All UEs | 0.013 | 0.018 | 0.029 | 0.11 |
| [Case E48], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.013 | 0.016 | 0.021 | 0.051 |
|  | (Optional) All UEs | 0.014 | 0.020 | 0.026 | 0.11 |
| [Case E49], [SH, perfect sync], [FR1], [DL-TDOA, RAIM] (Case E9) | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
| [Case E50], [SH, perfect sync], [FR1], [DL-TDOA, known LOS without RAIM] | Convex UEs | 0.037 | 0.049 | 0.073 | 0.096 |
| [Case E51], [SH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] | Convex UEs | 0.037 | 0.047 | 0.063 | 0.083 |
| [Case E52], [SH, perfect sync], [FR1], [DL-TDOA, 95% LOS without RAIM] | Convex UEs | 0.039 | 0.052 | 0.080 | 2.86 |
| [Case E53], [SH, perfect sync], [FR1], [DL-TDOA, 90% LOS without RAIM] | Convex UEs | 0.041 | 0.052 | 0.083 | 4.54 |
| [Case E54], [SH, perfect sync], [FR1], [DL-TDOA, unknown LOS without RAIM] | Convex UEs | 0.042 | 0.055 | 0.11 | 4.62 |
| [Case E55], [DH, perfect sync], [FR1], [DL-TDOA, RAIM] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
| [Case E56], [DH, perfect sync], [FR1], [DL-TDOA, known LOS without RAIM] | Convex UEs | 0.060 | 0.097 | 0.15 | 0.33 |
| [Case E57], [DH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] | Convex UEs | 0.055 | 0.071 | 0.11 | 0.17 |
| [Case E58], [DH, perfect sync], [FR1], [DL-TDOA, 95% LOS without RAIM] | Convex UEs | 0.042 | 0.064 | 0.30 | 3.40 |
| [Case E59], [DH, perfect sync], [FR1], [DL-TDOA, 90% LOS without RAIM] | Convex UEs | 0.043 | 0.065 | 1.82 | 3.43 |
| [Case E60], [DH, perfect sync], [FR1], [DL-TDOA, unknown LOS without RAIM] | Convex UEs | 0.048 | 1.05 | 3.49 | 8.64 |
| [Case E61], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] | Convex UEs | 0.070 | 0.092 | 0.12 | 0.16 |
| [Case E62], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] | Convex UEs | 0.10 | 0.13 | 0.16 | 0.21 |
| [Case E63], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] | Convex UEs | 0.21 | 0.28 | 0.32 | 0.47 |
| [Case E64], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] | Convex UEs | 0.063 | 0.086 | 0.11 | 0.17 |
| [Case E65], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] | Convex UEs | 0.12 | 0.15 | 0.21 | 0.35 |
| [Case E66], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] | Convex UEs | 0.22 | 0.27 | 0.33 | 0.59 |
| [Case E67], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.19 | 0.23 | 0.30 |
| [Case E68], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.34 |
| [Case E69], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.36 |
| [Case E70], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.14 | 0.18 | 0.22 | 0.35 |
| [Case E71], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.14 | 0.17 | 0.23 | 0.37 |
| [Case E72], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.18 | 0.23 | 0.29 | 0.42 |
| [Case E73], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.37 | 0.45 | 0.60 | 0.83 |
| [[Case E74], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.47 | 0.61 | 0.76 | 1.07 |
| [Case E75], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.80 | 1.00 | 1.24 | 1.87 |
| [Case E76], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.31 |
| [Case E77], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.13 | 0.17 | 0.21 | 0.32 |
| [Case E78], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.13 | 0.18 | 0.21 | 0.32 |
| [Case E79], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.20 | 0.28 |
| [Case E80], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.12 | 0.18 | 0.22 | 0.31 |
| [Case E81], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.18 | 0.24 | 0.33 | 0.40 |
| [Case E82 [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.40 | 0.48 | 0.62 | 0.76 |
| [[Case E83], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.54 | 0.62 | 0.75 | 0.88 |
| [Case E84], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.78 | 1.08 | 1.19 | 1.94 |
| [Case E85], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.089 | 0.13 | 0.18 | 0.24 |
| [Case E86], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.094 | 0.13 | 0.17 | 0.23 |
| [Case E87], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.12 | 0.15 | 0.19 | 0.27 |
| [Case E88], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.23 | 0.33 |
| [Case E89], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.16 | 0.25 | 0.34 | 0.44 |
| [Case E90], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.12 | 0.17 | 0.22 | 0.28 |
| [Case E91], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.17 | 0.21 | 0.28 | 0.34 |
| [[Case E92], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.39 | 0.49 | 0.61 | 0.76 |
| [Case E93], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.56 | 0.69 | 0.88 | 1.26 |
| [Case E94], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.10 | 0.12 | 0.17 | 0.24 |
| [Case E95], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.10 | 0.13 | 0.17 | 0.24 |
| [Case E96], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.11 | 0.15 | 0.20 | 0.28 |
| [Case E97], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.23 | 0.36 |
| [Case E98], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.18 | 0.25 | 0.33 | 0.46 |
| [Case E99], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.20 | 0.26 | 0.34 |
| [Case E100], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.21 | 0.26 | 0.35 | 0.48 |
| [[Case E101], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.37 | 0.51 | 0.62 | 0.87 |
| [Case E102], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.57 | 0.71 | 0.98 | 1.28 |
| [Case E103], [SH, perfect sync], [FR1], [50M] | Convex UEs | 0.12 | 0.16 | 0.20 | 0.31 |
|  | (Optional) All UEs | 0.16 | 0.25 | 0.38 | 0.95 |
| [Case E104], [SH, perfect sync], [FR1], [100M] (Case E9) | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.25 |
| [Case E105], [SH, perfect sync], [FR1], [50M+50M] | Convex UEs | 0.072 | 0.12 | 0.16 | 0.21 |
|  | (Optional) All UEs | 0.11 | 0.17 | 0.28 | 0.58 |
| [Case E106], [DH, perfect sync], [FR1], [50M] | Convex UEs | 0.11 | 0.16 | 0.21 | 0.44 |
|  | (Optional) All UEs | 0.17 | 0.26 | 0.50 | 0.98 |
| [Case E107], [DH, perfect sync], [FR1], [100M] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
|  | (Optional) All UEs | 0.087 | 0.14 | 0.25 | 0.62 |
| [Case E108], [DH, perfect sync], [FR1], [50M+50M] | Convex UEs | 0.069 | 0.095 | 0.15 | 0.23 |
|  | (Optional) All UEs | 0.11 | 0.20 | 0.39 | 0.91 |

Table C.2.1.5.2-1.2: NR positioning enhancements - horizontal location error results from [22]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E115], [SH, sync error 50ns], [FR1], [DL-TDOA] [sync error reduced by differential positioning] | Convex UEs | 0.046 | 0.061 | 0.077 | 0.11 |
| [Case E116], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by differential positioning] | Convex UEs | 0.060 | 0.074 | 0.087 | 0.13 |
| [Case E117], [SH, sync error 50ns], [FR1], [UL-TDOA] [sync error reduced by UL-TDOA+AOA] | Convex UEs | 1.24 | 1.39 | 1.69 | 3.16 |
| [Case E118], [SH, perfect sync], [FR1], [UL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by UL-TDOA+AOA] | Convex UEs | 0.64 | 0.85 | 1.08 | 1.50 |
| [Case E119], [DH, (60%,6,2), perfect sync], [FR1], [machine learning] | all UEs | 2.49 | 3.05 | 3.64 | 4.60 |
| [Case E120], [DH, (60%,6,2), sync error 50ns], [FR1], [machine learning] | all UEs | 2.24 | 2.99 | 3.68 | 5.12 |
| [Case E121], [SH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] | Convex UEs | 0.19 | 0.25 | 0.33 | 0.46 |
| [Case E122], [SH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] | Convex UEs | 0.93 | 1.17 | 1.58 | 2.03 |
| [Case E123], [SH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] | Convex UEs | 2.15 | 2.57 | 3.55 | 5.46 |
| [Case E124], [SH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] | Convex UEs | 4.60 | 5.21 | 6.81 | 10.05 |
| [Case E125], [DH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] | Convex UEs | 0.24 | 0.36 | 0.51 | 0.96 |
| [Case E126], [DH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] | Convex UEs | 1.03 | 1.40 | 2.15 | 3.90 |
| [Case E127], [DH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] | Convex UEs | 2.08 | 2.79 | 3.97 | 8.34 |
| [Case E128], [DH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] | Convex UEs | 3.70 | 5.94 | 7.48 | 10.69 |

Table C.2.1.5.2-1.3: NR positioning enhancements - IOO horizontal location error results from [23]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E109], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.19 | 0.35 | 0.80 |
| [Case E110], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.06 | 0.13 | 0.54 |
| [Case E111], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.19 | 0.38 | 0.84 |
| [Case E112], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.06 | 0.13 | 0.56 |
| [Case E113], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.17 | 0.37 | 0.68 |
| [Case E114], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR2], [Multi-RTT MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.05 | 0.11 | 0.50 |
| [Case E129], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.19 | 0.38 | 0.82 |
| [Case E130], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.06 | 0.22 | 0.56 |
| [Case E131], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.12 | 0.19 | 0.38 | 0.86 |
| [Case E132], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.07 | 0.19 | 0.62 |
| [Case E133], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.19 | 0.33 | 0.68 |
| [Case E134], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR2], [Multi-RTT MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.05 | 0.11 | 0.54 |

Evaluation results of vertical location error for NR positioning enhancements are provided in Table C.2.1.5.2-2:

Table C.2.1.5.2-2: NR positioning enhancements - vertical location error results from [23]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E-V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.19 | 0.30 | 0.46 | 0.58 |
|  | (Optional) All UEs | 0.19 | 0.33 | 0.54 | 0.84 |
| [Case E-V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.12 | 0.20 | 0.41 | 0.64 |
|  | (Optional) All UEs | 0.13 | 0.22 | 0.41 | 1.39 |
| [Case E-V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.24 | 0.42 | 0.62 | 1.25 |
|  | (Optional) All UEs | 0.24 | 0.39 | 0.70 | 1.38 |
| [Case E-V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 1.03 | 2.05 | 2.68 | 4.62 |
|  | (Optional) All UEs | 1.21 | 2.50 | 2.94 | 5.24 |
| [Case E-V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.16 | 0.25 | 0.45 | 0.66 |
|  | (Optional) All UEs | 0.19 | 0.30 | 0.46 | 0.76 |
| [Case E-V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.82 | 1.17 | 2.03 | 3.16 |
|  | (Optional) All UEs | 0.91 | 1.68 | 2.75 | 3.47 |
| [Case E-V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.28 | 0.45 | 0.72 | 1.27 |
|  | (Optional) All UEs | 0.28 | 0.45 | 0.76 | 1.48 |
| [Case E-V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.67 | 1.77 | 3.02 | 4.93 |
|  | (Optional) All UEs | 0.86 | 2.39 | 3.12 | 5.98 |

#### C.2.1.6	Results from source [12]

##### C.2.1.6.1	Description of evaluation scenarios

The following enhancements for NR positioning are evaluated:

- NLOS classification to mitigate NLOS

- Multipath mitigation method

- Combination of NLOS classification and multipath mitigation

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.6.1-1.

Table C.2.1.6.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | Case1 ( InF-SH, FR1) | Case2 ( InF-DH, FR1) |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-DH (40%,2,2) |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS | DL-PRS |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port |
| Number of sites | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | ML | ML |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA Chan | DL-TDOA Chan |
| Network synchronization assumptions | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based |
| Evaluated Enhancement for Rel.17 |  |  |
| Additional notes, if any |  |  |

The following cases are evalueated:

| Case number | Scenarios | Band | Positioning method | Enhancement |
| --- | --- | --- | --- | --- |
| Case 1 | InF-SH | FR1 | DL-TDOA | NLOS classification |
| Case 2 | InF-SH | FR1 | DL-TDOA | NLOS mitigation |
| Case 3 | InF-SH | FR1 | DL-TDOA | NLOS mitigation + NLOS classification |
| Case 4 | InF-DH | FR1 | DL-TDOA | NLOS classification |
| Case 5 | InF-DH | FR1 | DL-TDOA | NLOS mitigation |
| Case 6 | InF-DH | FR1 | DL-TDOA | NLOS mitigation + NLOS classification |
| Case 7 | InF-SH | FR1 | DL-TDOA | Ideal NLOS classification |
| Case 8 | InF-DH | FR1 | DL-TDOA | Ideal NLOS classification |

##### C.2.1.6.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.6.2-1:

Table C.2.1.6.2-1: NR positioning enhancements - horizontal location error results from [12]

| Cases1 | UEs | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case1, (InF-SH, NLOS classification FR1) | Convex UEs | 0.17 | 0.22 | 0.33 | 0.87 |
| Case2, (InF-SH, NLOS mitigation FR1) | Convex UEs | 0.20 | 0.32 | 1.07 | 3.12 |
| Case3, (InF-SH, NLOS classification + NLOS mitigation FR1) | Convex UEs | 0.16 | 0.20 | 0.24 | 0.31 |
| Case4, (InF-DH, NLOS classification FR1) | Convex UEs | 2.08 | 4.13 | 7.29 | 12.5 |
| Case5, (InF-DH, NLOS mitigation FR1) | Convex UEs | 0.73 | 1.73 | 3.0 | 5.49 |
| Case6, (InF-DH, NLOS classification+ NLOS mitigation FR1) | Convex UEs | 0.21 | 0.29 | 0.75 | 2.47 |
| Case7, (InF-SH, Ideal NLOS classification FR1) | Convex UEs | 0.14 | 0.17 | 0.21 | 0.23 |
| Case8, (InF-DH, Ideal NLOS classification FR1) | Convex UEs | 0.16 | 0.20 | 0.25 | 0.31 |

#### C.2.1.7	Results from source [10]

##### C.2.1.7.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.7.1-1.

Table C.2.1.7.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | Case 13, InF-SH, FR1, DL-TDOA | Case 14, InF-DH, FR1, DL-TDOA | Case 15, InF-SH, FR2, DL-TDOA | Case 16, InF-DH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port | DL PRS: single port | DL PRS: single port | DL PRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 17, InF-SH, FR1, UL-TDOA | Case 18, InF-DH, FR1, UL-TDOA | Case 19, InF-SH, FR2, UL-TDOA | Case 20, InF-DH, FR2, UL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | UL SRS: single port | UL SRS: single port | UL SRS: single port | UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Rx beam sweeping | Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 21, InF-SH, FR1, Multi-RTT | Case 22, InF-DH, FR1, Multi-RTT | Case 23, InF-SH, FR2, Multi-RTT | Case 24, InF-DH, FR2, Multi-RTT |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 25, InF-SH, FR1, Multi-RTT+ vertical AoA | Case 26, InF-DH, FR1, Multi-RTT+ vertical AoA | Case 27, InF-SH, FR2, Multi-RTT+ vertical AoA | Case 28, InF-DH, FR2, Multi-RTT+ vertical AoA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold Vertical AoA estimation based of detected FAP | FAP timing estimation based on adaptive threshold Vertical AoA estimation based of detected FAP | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 29, InF-SH, FR1, Multi-RTT with Tx/Rx timing error compensation | Case 30, InF-DH, FR1, Multi-RTT RTT with Tx/Rx timing error compensation | Case 31, InF-SH, FR2, Multi-RTT RTT with Tx/Rx timing error compensation | Case 32, InF-DH, FR2, Multi-RTT RTT with Tx/Rx timing error compensation |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 33, InF-SH, FR1, Multi-RTT | Case 34, InF-DH, FR1, Multi-RTT | Case 35, InF-SH, FR2, Multi-RTT | Case 36, InF-DH, FR2, Multi-RTT |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 400 MHz | 400 MHz | 800 MHz | 800 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | Ideal LOS classification Adjacent bandwidth | Ideal LOS classification Adjacent bandwidth | Ideal LOS classification Adjacent bandwidth | Ideal LOS classification Adjacent bandwidth |

##### C.2.1.7.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.7.2-1:

Table C.2.1.7.2-1: NR positioning enhancements - horizontal location error results from

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | Convex UEs | 0.11 | 0.15 | 0.23 | 0.37 |
| Case 14, InF-DH, FR1, DL-TDOA | Convex UEs | 0.22 | 0.56 | 1.2 | 1.94 |
| Case 15, InF-SH, FR2, DL-TDOA | Convex UEs | 0.06 | 0.08 | 0.11 | 0.21 |
| Case 16, InF-DH, FR2, DL-TDOA | Convex UEs | 0.08 | 0.14 | 0.22 | 0.89 |
| Case 17, InF-SH, FR1, UL-TDOA | Convex UEs | 0.12 | 0.17 | 0.25 | 0.4 |
| Case 18, InF-DH, FR1, UL-TDOA | Convex UEs | 0.39 | 0.81 | 1.45 | 4.4 |
| Case 19, InF-SH, FR2, UL-TDOA | Convex UEs | 0.08 | 0.12 | 0.15 | 0.24 |
| Case 20, InF-DH, FR2, UL-TDOA | Convex UEs | 0.12 | 0.17 | 0.28 | 0.68 |
| Case 21, InF-SH, FR1, Multi-RTT | Convex UEs | 0.07 | 0.1 | 0.13 | 0.17 |
| Case 22, InF-DH, FR1, Multi-RTT | Convex UEs | 0.1 | 0.15 | 0.2 | 0.3 |
| Case 23, InF-SH, FR2, Multi-RTT | Convex UEs | 0.05 | 0.07 | 0.08 | 0.1 |
| Case 24, InF-DH, FR2, Multi-RTT | Convex UEs | 0.07 | 0.09 | 0.12 | 0.17 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.05 | 0.07 | 0.09 | 0.1 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.07 | 0.1 | 0.13 | 0.19 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.04 | 0.05 | 0.06 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.06 | 0.07 | 0.09 | 0.2 |
| Case 29, InF-SH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.09 | 0.13 | 0.19 | 0.28 |
| Case 30, InF-DH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.15 | 0.24 | 0.36 | 0.55 |
| Case 31, InF-SH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.06 | 0.08 | 0.11 | 0.14 |
| Case 32, InF-DH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.09 | 0.14 | 0.5 | 8.7 |
| Case 33, InF-SH, FR1, Multi-RTT | Convex UEs | 0.03 | 0.04 | 0.05 | 0.06 |
| Case 34, InF-DH, FR1, Multi-RTT | Convex UEs | 0.05 | 0.07 | 0.09 | 0.11 |
| Case 35, InF-SH, FR2, Multi-RTT | Convex UEs | 0.02 | 0.03 | 0.04 | 0.05 |
| Case 36, InF-DH, FR2, Multi-RTT | Convex UEs | 0.04 | 0.05 | 0.06 | 0.07 |

Evaluation results of vertical location error for NR positioning enhancements are provided in Table C.2.1.7.2-2:

Table C.2.1.7.2-2: NR positioning enhancements - altitude location error results from

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | Convex UEs | 0.64 | 1.2 | 6.2 | 6.9 |
| Case 14, InF-DH, FR1, DL-TDOA | Convex UEs | 0.65 | 6.2 | 6.3 | 12.4 |
| Case 15, InF-SH, FR2, DL-TDOA | Convex UEs | 0.35 | 0.78 | 2.5 | 6.6 |
| Case 16, InF-DH, FR2, DL-TDOA | Convex UEs | 0.28 | 0.6 | 1.8 | 5.8 |
| Case 17, InF-SH, FR1, UL-TDOA | Convex UEs | 1.1 | 6.4 | 6.5 | 12.8 |
| Case 18, InF-DH, FR1, UL-TDOA | Convex UEs | 6.2 | 6.3 | 6.4 | 12.9 |
| Case 19, InF-SH, FR2, UL-TDOA | Convex UEs | 0.44 | 0.81 | 6.1 | 11.3 |
| Case 20, InF-DH, FR2, UL-TDOA | Convex UEs | 0.29 | 0.47 | 0.71 | 0.64 |
| Case 21, InF-SH, FR1, Multi-RTT | Convex UEs | 0.19 | 0.34 | 0.52 | 0.89 |
| Case 22, InF-DH, FR1, Multi-RTT | Convex UEs | 0.12 | 0.2 | 0.3 | 0.49 |
| Case 23, InF-SH, FR2, Multi-RTT | Convex UEs | 0.38 | 0.53 | 0.7 | 1 |
| Case 24, InF-DH, FR2, Multi-RTT | Convex UEs | 0.18 | 0.25 | 0.32 | 0.45 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.08 | 0.12 | 0.18 | 0.27 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.04 | 0.08 | 0.12 | 0.2 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.01 | 0.03 | 0.05 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.01 | 0.02 | 0.03 | 0.06 |
| Case 29, InF-SH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.38 | 0.64 | 1.1 | 1.86 |
| Case 30, InF-DH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.34 | 0.64 | 1.15 | 2.64 |
| Case 31, InF-SH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.33 | 0.58 | 1.37 | 12.7 |
| Case 32, InF-DH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.31 | 0.92 | 6.4 | 12.9 |
| Case 33, InF-SH, FR1, Multi-RTT | Convex UEs | 0.15 | 0.22 | 0.29 | 0.4 |
| Case 34, InF-DH, FR1, Multi-RTT | Convex UEs | 0.07 | 0.1 | 0.14 | 0.19 |
| Case 35, InF-SH, FR2, Multi-RTT | Convex UEs | 0.17 | 0.25 | 0.31 | 0.39 |
| Case 36, InF-DH, FR2, Multi-RTT | Convex UEs | 0.11 | 0.15 | 0.2 | 0.25 |

| Analysis for FR1 |  |
| --- | --- |
| Analysis for FR2 |  |

Figure C.2.1.7.2-1. Accuracy for Rel 17 positioning solutions with LOS/NLOS classification

| Analysis for FR1 and FR2 |  |
| --- | --- |

Figure C.2.1.7.2-2. Accuracy for Rel 17 positioning solutions based on Multi-RTT and vertical AoA measurements with LOS/NLOS classification

| Analysis for FR1 and FR2 |  |
| --- | --- |

Figure C.2.1.7.2-3. Accuracy for Rel 17 positioning solutions for Multi-RTT with Tx/Rx timing errors

| Analysis for FR1 |  |
| --- | --- |
| Analysis for FR2 |  |

Figure C.2.1.7.2-4. Accuracy for Rel 17 positioning solutions with aggregated bandwidth