C.2.2	Physical layer latency analysis for NR positioning enhancements

C.2.2.1	Results from source [4]

C.2.2.1.1	Description of evaluation scenarios

The physical layer latency for the following positioning methods are provided

- Enhanced UL E-CID

C.2.2.1.2	Latency analysis of NR positioning enhancements

Latency analysis for the Rel.16 NR positioning is provided ins Table C.2.2.1.2-1 and C.2.2.1.2-2.

Table C.2.2.1.2-1: NR positioning enhancements – latency analysis [4]

| Case L101, UL E-CID w/ measurements available Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A, Initial and Final RRC States CONNECTED Assuming UE Rx – Tx time difference is already available |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| gNB Rx – Tx time difference measurement and AoA measurements | 0-20ms | Assuming SRS periodicity is 20ms |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6-26ms |  |

Table C.2.2.1.2-2: NR positioning enhancements – latency analysis [4]

| Case L102, UL E-CID w/o measurements available Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A, Initial and Final RRC States CONNECTED Assuming UE Rx – Tx time difference is not available |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| gNB Tx higher layer processing | 3ms | Configuration of UE Rx – Tx time difference based on SRS and TRS/CSI-RS |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| UE Rx – Tx time difference measurement/gNB Rx – Tx time difference measurement and AoA measurements | 20ms | Assuming TRS/CSI-RS periodicity is 20ms Assuming SRS periodicity is 20ms |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for RRM measurement reporting, including UE Rx – Tx time difference measurement |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Processing UL-SCH containing the measurement and including gNB measurements |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 46-53.5ms |  |

C.2.2.2	Results from source [8]

C.2.2.2.1	Description of evaluation scenarios

For DL positioning enhancement, aperiodic PRS (A-PRS) could be introduced to reduce positioning latency. In this section, evaluation scenarios and parameters for positioning latency analysis of A-PRS based positioning solution are given as below:

- Case ID: 1.

- SCS: 15kHz.

- Frequency Band: FR1.

- Positioning Technique: DL-TDOA.

- Periodicity of PRS: Aperiodic PRS [R1-2007860]

- Duration time of PRS: 4ms

C.2.2.2.2	Latency analysis of NR positioning enhancements

For DL positioning enhancement, aperiodic PRS could be introduced. From our point of view, aperiodic measurement gap can be configured for aperiodic PRS [R1-2007860]. Therefore, whenever triggering, UE should measure those PRS resources. In addition, for measurement reporting, the SR request and UL grant procedures could be arranged before A-PRS triggering. Therefore, the reporting time could be reduced further. The enhanced procedure of DL-TDOA is shown in Figure C.2.2.2.2-1.

Figure C.2.2.2.2-1: Enhanced PHY procedure for DL-TDOA

The total delay could be calculated as

Ttotal=Ttrig+Tariv+Tprocess+Treport

where,

- Ttrig denotes the time to receive and decode the A-PRS triggering signaling.

- Tariv denotes the delay when DL PRS is received, which is related to the PRS offset value configured by the A-PRS triggering signaling

- Tprocess denotes the time from UE begins to measure PRS until the measurement result is ready, which is related to UE capability and the number of PRS resources needed to be measured.

- Treport denotes the time for measurement reporting, which includes SR request, PDCCH-based UL grant and PUSCH-based measurement result reporting..

Assume DCI signalling is used to trigger A-PRS, 1ms could be assumed for Ttrig. Current UE capability is defined as N ms of DL PRS symbols being processed in every T ms for a given maximum bandwidth (B) in MHz supported by UE. N = {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50}ms and T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}ms. Assume Y ms of PRS resources are configured, the processing time could be calculated as Tprocess=YN∙T.

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.2.2-1 with the assumption of SCS 15 kHz, and a moderate UE capability, i.e. N=4, T=8.

Table C.2.2.2.2-1: NR positioning enhancements – latency analysis [8]

| Case 1, 15kHz, FR1, DL-TDOA Source UE / Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | 1 | receive and decode the A-PRS triggering signaling |
| PRS arrival delay | 4 | DL PRS arrived for measurement, which is related to the PRS offset value configured by the A-PRS triggering signaling |
| Process time | ≤X, X=YN∙T (e.g. X=8 for Y=4, N=4, T=8 ) | UE begins to measure PRS until the measurement result is ready. This value depends on UE capability the number of PRS resources to be measured. |
| End trigger | 0.5 | PUSCH-based measurement result reporting are included |
| Total values | 13.5 |  |

C.2.2.3	Results from source [5]

C.2.2.3.1	Description of evaluation scenarios

Brief description of evaluation scenarios and key evaluation parameters.

C.2.2.3.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.3.2-1- Table C.2.2.3.2-6.

Table C.2.2.3.2-1: NR positioning enhancements – latency analysis [5]

| [Case 7], [IIoT/ Commercial], [Frequency Band], [Aperiodic PRS] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | 2.5-34.5 | Dynamic Scheduling latency for activating aperiodic PRS Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand PRS message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. Scheduling offset(TSlot offset) The slot offset can be 0-32 slots if the scheduling offset of SRS and PRS are consistent Process delay is assumed as 2ms |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 44.5~ | The minimum total value is 44.5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-2: NR positioning enhancements – latency analysis [5]

| [Case 8], [IIoT/ Commercial], [Frequency Band], [on-demand/aperiodic MG] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG | 0.5 | Dynamic Scheduling latency for activating aperiodic/on-demand MG Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. |
| 3: DL measurement &process delay. | [6~10240] Or [130~32770] | DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity. TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms Process delay is assumed as 2ms |
| 4: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 27.5~ | The minimum total value is 27.5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-3: NR positioning enhancements – latency analysis [5]

| [Case 9], [IIoT/ Commercial], [Frequency Band], [Positioning BWP] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PDCCH from the gNB carrying activate positioning BWP | 1.5 | Dynamic Scheduling latency for activating positioning BWP Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. BWP switching latency. The value of BWP switching time can refer to Table 8.6.2-1 in TS38.133. Here we take a typical value of 1ms as the BWP switching latency. |
| 3: DL measurement &process delay. | [6~10240] Or [130~32770] | DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity. TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms. Process delay is assumed as 2ms |
| 4: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 28.5~ | The minimum total value is 28.5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-4: NR positioning enhancements – latency analysis [5]

| [Case 10], [IIoT/ Commercial], [Frequency Band], [Physical layer triggered] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA, AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | 0.5 | Dynamic Scheduling latency for physical layer triggered positioning request Transmission of the PDCCH from the gNB carrying Positioning requesting message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. |
| 1: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 2: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 3: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 4: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 5: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| End trigger | 0.5 | PUCCH latency for physical layer triggered positioning reporting Transmission of the PUCCH from the UE carrying measurement/location message. Which value less than the DL/UL data transmission time, including Tprocess time_pucch, Talign time andTPUCCH and smaller than 0.5ms. |
| Total values | 44~ | The minimum total value is 44 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-5: NR positioning enhancements – latency analysis [5]

| [Case 11], [IIoT/ Commercial], [Frequency Band], [Combination of on-demand/aperiodic PRS, on-demand/aperiodic MG or Positioning BWP, and physical layer triggered request/report] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA, AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 0.5ms | Dynamic Scheduling latency for physical layer triggered positioning request Transmission of the PDCCH from the gNB carrying Positioning requesting message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms |
| 1: Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG/Positioning BWP switch message | 0.5 ms/1.5ms | Dynamic Scheduling latency for activating aperiodic/on-demand MG/Positioning BWP switch Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG/Positioning BWP switch message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. BWP switching latency: 1ms |
| 2.DL measurement &process delay. | 2.5-34.5 | Dynamic Scheduling latency for activating aperiodic/on-demand PRS Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand PRS message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. Scheduling offset(TSlot offset) The slot offset can be 0-32 slots if the scheduling offset of SRS and PRS are consistent Process delay is assumed as 2ms |
| End trigger | 0.5 ms | PUCCH latency for physical layer triggered positioning reporting Transmission of the PUCCH from the UE carrying measurement/location message. Which value less than the DL/UL data transmission time, including Tprocess time_pucch, Talign time andTPUCCH and smaller than 0.5ms. |
| Total values | 5 ms~ | The minimum total value is 5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-6: NR positioning enhancements – latency analysis [5]

| [Case 12], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [Idle/inactive] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| 1: Start trigger: DL measurement &process delay. | [6~10240] Or [130~32770] | DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity. TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms -Process delay is assumed as 2ms |
| 2. Positioning report by RACH procedures | [11.3~168.5] | Positioning report steps in idle/inactive state include below components: -Delay due to RACH scheduling period -Transmission of RACH Preamble -Preamble detection and processing in gNB -Transmission of RA response -UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Resume Request) -Transmission of RRC Resume Request Comparing to the steps of inacitve to connected state as in TR37910 table 5.7.2.1-1, steps 7~10 are ignored, which can save about 10ms. In addition, considering the delay due to RACH period, additional delay of 10~160ms should be added. Therefore, the latency of positioning report in idle/inactive state can be assumed as 11.3~168.5ms. (11.3~18.5ms-10ms+10~160ms) |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 27.3~ | The minimum total value is 27.3ms if only consider the minimum value for every step. |

C.2.2.4	Results from source [10]

C.2.2.4.1	Description of evaluation scenarios

In order to perform latency analysis for enhanced NR positioning solutions, we use the following assumptions:

- gNB location request: 2 symbols

  - Provides activation of DL PRS, UL SRS transmission, and scheduling information for UE measurement report (e.g. resources for UE measurement report transmission can be associated with resources of SRS for positioning or directly indicated)

  - DL PRS and UL SRS for positioning transmission are performed sequentially

- No MG configuration and BWP switching considerations

- DL PRS resource: 2 symbols, Comb-6, 4 symbols in total per DL PRS Resource Set

- SRS for positioning configuration – same as in reference system configuration

  - DL PRS processing delay – 2 ms (in scope of RAN WG4)

- UE measurement report

  - 7 symbols PUSCH duration (same as in reference system configuration)

Other assumptions are the same as in reference system configuration

C.2.2.4.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.4.2-1.

Table C.2.2.4.2-1: NR positioning enhancements – latency analysis

| Case 4, InF, FR1, R17 DL-TDOA/DL-AoD Case 5, InF, FR1, R17 UL-TDOA/UL-AOA Case 6, InF, FR1, R17 Multi-RTT Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: R17 NR Positioning techniques, type DL+UL, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB measurement request message |
| Step 1. gNB location request | 24.25 symbols 0.8861 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [7] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L2 UE MAC CE processing time] | [~28-84 symbols 1-3 ms] | [In this analysis MAC CE based on PDCCH+PDSCH is assumed for gNB location request. Other possible option is to use PDCCH only that can further reduce latency TBD by RAN WG2] |
| Step 2: DL PRS transmission | 64 symbols 2.2858 ms | Value is provided for L1 components only |
| 2A. DL PRS alignment time | 4 symbols 0.1429 ms | Worst case: 4ms for selected reference system configuration to reflect slot boundary |
| 2B. DL PRS transmission time | 4 symbols 0.1429 ms |  |
| 2C. DL PRS processing and report delay | [~56 symbols 2 ms] | [ Not defined. It is assumed that new UE capabilities for low latency NR positioning are to be introduced. Definition is in scope of RAN WG4] |
| Step 3. SRS for positioning transmission | 29 symbols 1.0357ms | Value is provided for L1 components only |
| 3A: SRS for positioning preparation time | 0 ms | Not defined. 0ms is assumed for analysis (can be prepared in advance, e.g. during DL PRS processing or earlier) |
| 3B: SRS for positioning alignment time | 14 symbols 0.5ms | Depends on SRS resource allocation. Worst case: 14 symbols for selected reference configuration |
| 3C: SRS for positioning transmission time | 1 symbol 0.0357 | 1 symbol for selected reference configuration |
| 3D: SRS for positioning processing time | [14 symbols 0.5 ms] | [Not defined: 0.5 ms is assumed. Definition is in scope of RAN WG4] |
| Step 4. UE measurement report | 20.5symbols 0.7321ms | Value is provided for L1 components only |
| 4A: [L2 MAC CE preparation] | [~28 symbols 1 ms] | [In this analysis MAC CE based on PUSCH is assumed. TBD by RAN WG2] |
| 4B: PUSCH alignment time | 7 symbols 0.2500 ms |  |
| 4C: PUSCH transmission time | 7 symbols 0.2500 ms | 7 symbols for selected reference system configuration |
| 4D: PUSCH processing time | 6.5 symbols 0.2321 ms | Not specified. For analysis, the assumption from the [7] is used. gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| End trigger |  | Transmission by the gNB measurement response message |

Based on analysis in table above, we have following observations

- DL-TDOA latency of L1 components = Step 1 + Step 2 + Step 4 = 3.8839 ms

  - Estimated higher layer signaling time (L2/L3 components) is up to 4ms

  - Total time is about 8 ms

- UL-TDOA latency of L1 components = Step 1 + Step 3 = 1.9018 ms

  - Estimated higher layer signaling time (L2/L3 components) is up to 3ms

  - Total time is about 5 ms

- Multi-RTT latency of L1 components = Step 1 + Step 2 + Step 3 = 4.1875 ms

  - It is assumed that Step 4 is done in parallel with Step 3B,3C,3D

  - Estimated higher layer signaling time (L2/L3 components) is up to 4ms

  - Total time is about 8.2 ms

C.2.2.5 Results from source [16]

C.2.2.5.1 Description of evaluation scenarios

The latency analysis of enhancements for the UE-assisted DL methods are presented here. All analysis are based on 30kHz SCS. The minimum latency value is presented here. Specifically, we perform the latency analysis of three following enhancements:

- No measurement gap for DL PRS reception

- Measurement gap activation/deactivation

- On-demand PRS

C.2.2.5.2 Latency analysis of NR positioning enhancements

Table C.2.2.5.2-1: Latency analysis for no measurement gap DL methods

| Source [NW]/Destination [NW] Positioning technique: DL methods, mode ; UE-A Initial and Final RRC States [CONNECTED] Positioning technique: No measurement gap for PRS reception methods |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message and PRS configuration activation. | 1 |  |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message |
| Step 3: The UE receives PRS without MG | 2 |  |
| Step 4: UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 16ms |  |

Table C.2.2.5.2-2: Latency analysis for measurement gap activation/deactivation DL methods

| Source [NW]/Destination [NW] Positioning technique: DL methods, mode ; UE-A Initial and Final RRC States [CONNECTED] Positioning technique: Measurement gap activation/deactivation DL methods |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message, MG and PRS configurations | 1 |  |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message |
| Step 3: The UE transmits MG activation request in an MAC CE or UCI | 1 |  |
| Step 4: The gNB successfully decodes the MG activation request. | 3 | MAC CE or UCI |
| Step 5: The gNB activates the MG for the UE using MAC CE or UCI | 1 |  |
| Step 6: UE receive MG activation and apply the MG configuration | 3 | MAC CE or UCI |
| Step 7: UE receives PRS in the MG | 2 |  |
| Step 8: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 24ms |  |

Table C.2.2.5.2-3: Latency analysis for on-demand DL PRS methods

| Source [NW]/Destination [NW] Positioning technique: DL methods, mode ; UE-A Initial and Final RRC States [CONNECTED] Positioning technique: On-demand PRS |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: The UE transmits measurement gap (MG) request in an RRC message. | 1 |  |
| Step 2: The gNB successfully decode the MG request message | 5 | RRC message |
| Step 3: The gNB configures MG in an RRC message | 1 |  |
| Step 4: The UE receive the MG configuration message and apply the MG configuration. | 10 | RRC message |
| Step 5: The UE receives PRS in the MG | 2 |  |
| Step 6: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 22ms |  |

C.2.2.6	Results from [17]

C.2.2.6.1	Description of evaluation scenarios

Brief description of evaluation scenarios and key evaluation parameters.

C.2.2.6.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.1.6-1.

Table C.2.2.1.6-1: NR positioning enhancements – latency analysis

| Proposal | L1 Latency | Explanation |
| --- | --- | --- |
| Support Low-layer (e.g., unicast/group-common DCI, MAC-CE) triggering of DL/UL PRS transmission/muting/Location-Request for DL-only and DL/UL methods. | < 1 slot (for DCI-based) 3 msec (for DL MAC-CE) | Instead of the UE receiving through a PDSCH carrying an RRC message the Location Information Request, a DCI-based triggering may be interpreted by the UE, depending on the scenario within the same slot that the PDCCH is received. A MAC-CE Request may be interested within 3 msec. |
| Support DCI/MAC-CE triggering of Measurement gaps (MG) for the purpose of positioning measurements | < 1 slot (for DCI-based) 3 msec (for DL MAC-CE) | Instead of the gNB transmitting an RRC configuration that includes the MG request, a DCI-based triggering may be interpreted by the UE, depending on the scenario within the same slot that the PDCCH is received. A MAC-CE Request may be interested within 3 msec. The smallest MG periodicity currently is 20 msec, so a UE may have to wait, after it has interpreted the RRC message, an additional 20 msec before receiving the MG in NR Rel-16. If the MG is triggered by DCI or MAC-CE, the UE would not have to wait for a long period of time for the first occasion of the MG. |
| Fast/real-time processing of short PRS instances: Support Enhanced PRS processing capabilities Support partially-staggered or no-staggered DL-PRS transmissions | 1-4 slots | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before it is ready to report. Furthermore, the maximum capability with respect to the number of PRS resources within a slot that is currently supported is 64 resources. If we want the UE to process, e.g. 18 TRPs, each one with at least 4 beams, with single-symbol comb-6 PRS, we would need UEs that can process 72 resources per slot. |
| Support Low-layer (e.g. UL MAC-CE or UCI) Measurement Reporting towards the serving gNB | 1 slot (for UCI) 3 msec (form UL MAC-CE) | Instead of the UE reporting the measurements as part of RRC in a NAS container (2-5 msec), a UCI-based would require < 1 slot, whereas a MAC-CE alternative would require 3 msec. |

C.2.2.7 Results from source [15]

C.2.2.7.1	Description of evaluation scenarios

To evaluate physical layer latency for the enhanced NR positioning solution(s), we use followings assumption as below:

- The gNB provides the UE with reporting resource of positioning measurements when the gNB configures MG.

  - Numerology: 15 kHz (for PDCCH, PDSCH, PUCCH, PUSCH, SRS)

- For PUSCH transmission:

  - Uplink switching gap is not configured.

  - No BWP switching

  - No overlapping symbols of the PUCCH and the scheduled PUSCH

  - The number of PUSCH symbols = from 4 to 14 for Type A

  - The number of PUSCH symbols = from 1 to 14 for Type B

  - According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For PDSCH transmission:

  - No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH

  - The number of PDSCH symbols = from 3 to 14 for Type A

  - The number of PDSCH symbols = from 2 to 14 for Type B

  - According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For SRS transmission:

  - One shot transmission (2 OS ~ 12 OS)

- According to resource type (periodic, Semi-persistent and aperiodic), the number of symbols can be changed.

C.2.2.7.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.7.2-1- Table C.2.2.7.2-5.

Table C.2.2.7.2-1: NR positioning enhancements latency analysis for DL method (Case 1) [15]

| Source/Destination: NW/UE Positioning technique DL-TDOA and DL-AOD, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Request Location Information message (RRC processing time, UE) | 5 ms ~ 5.5 ms | 38.331 section 12. RRC DL information transfer at UE : 5 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Constructing measurement gap request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 - 5ms |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3ms |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of measurement gap configuration (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5. The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 49.12 ms ~ 198.12 ms For UE capability-2: 47.68 ms ~ 196.26 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 23.12ms ~ 167.12ms For UE capability-2: 21.68ms ~ 165.26ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-2: NR positioning enhancements latency analysis for DL method (Case 2) [15]

| Source/Destination: UE/NW Positioning technique DL-TDOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3ms |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of measurement gap configuration (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 41.41ms ~ 185.84ms For UE capability-2: 40.33ms ~ 184.55ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 22.41ms ~ 165.34ms For UE capability-2: 21.33ms ~ 164.05ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-3: NR positioning enhancements latency analysis for UL method (Case 3) [15]

| Source/Destination: NW/UE Positioning technique UL-TDOA and UL-AOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| SRS configuration (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS configuration (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS configuration message (RRC processing time, UE) | 10 ms ~ 10.5 ms | 38.331 section 12. RRC Reconfiguration processing at UE : 10 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Reception of SRS activation message (RRC processing time, gNB UE) | 1 ms ~ 3.5 ms | 38.331 section 12. Processing delay of MAC-CE SRS Activation at UE: 1 ms ~ 3 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | NRPPa Processing delay at gNB Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| Total values | For UE capability-1: 14.78 ms ~ 20.14 ms For UE capability-2: 14.42 ms ~ 19.57 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 0.78 ms ~ 2.64 ms For UE capability-2: 0.42 ms ~ 2.07 ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-4: NR positioning enhancements latency analysis for DL+UL method (Case 4) [15]

| Source/Destination: NW/UE Positioning technique: Multi-RTT, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| *Constructing Provide assistance message (RRC processing time, gNB) | 3 ms | 38.331 section 12. |
| Transmission of LPP provide assistance data (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of LPP provide assistance data (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of LPP provide assistance data (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| *Constructing request location information (RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Transmission of Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| * Transportation of Request Location Information message (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| *Constructing measurement gap request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 - 5ms |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request (RRC processing time, gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3ms |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of measurement gap configuration (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | NRPPa Processing delay at gNB Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| measurement report transmission(including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 63.9 ms ~ 215.76 ms For UE capability-2: 62.1 ms ~ 213. 33 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 23.9 ms ~ 169.76 ms For UE capability-2: 22.1 ms ~167.33ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-5: : NR positioning enhancements latency analysis for E-CID (Case 5) [15]

| Source/Destination: NW/UE Positioning technique: E-CID, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| LPP Request Location Information message transmission(gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of LPP Request Location Information message (RRC processing time, UE) | 5 ms ~ 5.5 ms | 38.331 section 12. RRC DL information transfer at UE : 5 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| measurement report transmission(including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement report, SRS and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 14.56 ms ~ 17.56 ms For UE capability-2: 13.84 ms16.63 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 1.56ms ~ 3.56ms For UE capability-2: 0.84ms~2.63ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.