3GPP TSG RAN WG1 #104-e		   	R1-2100063

e-Meeting, January 25th – February 5th, 2021

Agenda Item:		8.1.1

Source:				InterDigital, Inc.

Title:					Discussions on Rel-17 beam management

Document for:	Discussion and Decision

# Introduction

In RAN1#103-e [1], agreements on L1-based beam indication by using existing DCI formats 1_1/1_2, how to define/apply beam application time, use cases and enhancement scopes for L1/L2-centric inter-cell mobility, UE-initiated panel selection/activation for MP-UE and UE reporting on MPE were made. In this contribution, we discuss further details on Rel-17 beam management.

# Unified TCI framework for DL and UL

In RAN1#103-e [1], RAN1 agreed to support L1-based joint TCI state and separate DL/UL TCI state based on the existing DCI formats 1_1 and 1_2 with a mechanism for UE to acknowledge successful decoding of beam indication. In this section, we provide discussions on the issues related to the unified TCI framework.

Group common DCI for L1-based beam indication

For L1-based beam indication, in addition to the existing DCI formats 1_1 and 1_2, utilization of group common DCI was proposed. The motivation is to achieve efficient beam indication based on reduced signaling overhead and latency by delivering beam information to multiple UEs simultaneously. However, performance benefits from group DCI are not clear as FR2 beam is narrow so that chances for simultaneous transmission can be rare events.

Observation 1: Performance benefits from group-common DCI for L1-based beam indication are not clear as FR2 beam is narrow so that chances for simultaneous transmission can be rare events.

Proposal 1: L1-based beam indication with group-common DCI is not supported.

DCI formats for separate UL beam indication

When gNB utilizes the existing DCI formats 1_1 and 1_2 for separate UL beam indication, the overall operation may not be efficient as the existing DCI formats 1_1 and 1_2 are DL DCI formats. Figures 1 – 2 show exemplary operations with the DCI formats 1_1/1_2 and the DCI formats 0_1/0_2, respectively, for separate UL beam indication.

Figure 1 Separate UL beam indication with DCI formats 1_1 and 1_2

Figure 2 Separate UL beam indication with DCI formats 0_1 and 0_2

As shown in Figure 1, when gNB utilizes the DCI formats 1_1 and 1_2, UE needs to decode both DL DCI for new beam indication and UL DCI for PUSCH scheduling. In addition, UL scheduling offset and processing time for new beam application should be independently applied as the new beam indication and the PUSCH utilize different DCI formats. On the other hand, when gNB utilizes the DCI formats 0_1 and 0_2 for both new beam indication and PUSCH scheduling as shown in Figure 2, UE needs to decode only UL DCI, so PDCCH decoding latency can be reduced. In addition, processing time for PUSCH transmission and new beam application can be combined if UE implementation allows simultaneous processing of both operations.

Observation 2: For separate UL beam indication, utilization of existing UL DCI formats 0_1 and 0_2 is more efficient as it doesn’t require multiple PDCCH decoding and combined UE processing time is possible if UE implementation allows simultaneous processing.

Proposal 2: For separate UL beam indication, utilization of the existing UL DCI formats 0_1 and 0_2 is preferred.

Beam application time for unified TCI framework

In RAN1#103-e, regarding application time of the beam indication, following alternatives are provided:

- Alt1: the first slot that is at least X ms or Y symbols after the DCI with the joint or separate DL/UL beam indication

- Alt2: the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication

When UE receives L1-based beam indication, corresponding PUCCH transmission for ACK/NACK should be based on previously indicated beam as the UE didn’t send the confirmation to gNB yet. In that regard, it is obvious that UE’s beam changing procedure initiates after sending the PUCCH not after decoding PDCCH which provides L1-based beam indication. Therefore, it is preferred to apply newly indicate beam X ms or Y symbols after the acknowledgement of the beam indication. However, the application time of the beam indication does not need to be applied for all PDCCH transmission with DCI formats 1_1 and 1_2. As DCI formats 1_1 and 1_2 are not dedicated DCI formats for beam indication, but for PDSCH scheduling, DCI formats 1_1 and 1_2 can schedule PDSCHs without any new beam indication (i.e., indicating an identical beam with previously indicated beam). Therefore, if DCI formats 1_1 and 1_2 indicate identical beam with previously indicated beam, the indicated beam can be applied to DL and UL without any application time.

Observation 3: As UE transmits PUCCH for ACK/NACK of beam indication with previously indicated beam, UE’s beam changing procedure initiates after sending the PUCCH not after decoding PDCCH which provides L1-based beam indication.

Proposal 3: For beam application time, the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication (Alt 2) should be supported.

Observation 4: As DCI formats 1_1 and 1_2 are not dedicated formats for beam indication, but for PDSCH scheduling, DCI formats 1_1 and 1_2 can schedule PDSCHs without any new beam indication (i.e., indicating an identical beam with previously indicated beam).

Proposal 4: If DCI formats 1_1 and 1_2 indicate identical beam with previously indicated beam, the indicated beam can be applied to DL and UL without any application time.

In RAN1#102-e, in addition to the definition of beam application time, how to configure beam application time and/or define beam application were discussed as follows:

- Alt1: The beam application time can be configured by the gNB based on UE capability

  - Support a UE capability for the minimum value of beam application time

  - FFS: the exact minimum values of beam application time supported by UE

  - FFS: whether existing UE capability can be reused as this UE capability.

  - FFS: whether different beam application time values are supported for uplink and downlink

  - FFS: whether UE capability needs to be introduced for the maximum value of beam application time

- Alt2: The beam application time is fixed and defined in specification

- Alt3: The beam application time can be configured by the gNB where the minimum value of beam application time is fixed and defined in specification

Among the above alternatives, Alt 1 considers UE implementation based on UE capability while other alternatives do not. Fixed beam application time or fixed minimum beam application time may ease gNB implementation, however, UE implementation may have more variation based on a budget. In that sense, in order to consider various UE implementations, it is preferred to consider UE implementation as well as gNB implementation.

Proposal 5: In order to consider various UE implementations, it is preferred to consider UE capability by supporting a UE capability for the minimum value of beam application time (Alt 1).

Configuration of SRS for BM as a source RS to represent a DL RX spatial filter

For the configuration of SRS as a source RS to represent a DL RS spatial filter, RAN1 should consider following aspects:

- Beam Correspondence

  - Since Rel-15, beam correspondence has been a mandatory UE capability for FR2 where its type of support, i.e., with or without network assistance is indicated through UE capability signaling. Meaning that, for FR2, beam correspondence is an essential feature that all UEs need to support for beam management operation. Based on this understanding, there’s no reason not to utilize existing channel information based on beam correspondence.

- Signaling overhead

  - For Rel-15 DL TCI state indication, a gNB needs to allocate CSI-RS resources for TCI state indication even ifthe gNB isalready aware of channel information based on SRS transmissions. Such CSI-RS transmission induces large signaling overhead considering maximum 64 DL beams and should be minimized by utilizing existing information

- More accurate beam information

  - A UE in FR2 may experience beam change due to UE mobility and dynamic blockage and latest information can help better utilization of transmission beam. For example, if SRS transmission is a more recent RS transmission than CSI-RS, it may provide better information and corresponding performance benefits.

Observation 5: Introduction of SRS in DL TCI state provides performance benefits based on beam correspondence, less signaling overhead and more accurate beam information.

Proposal 6: DL TCI state based on SRS for beam measurement should be supported.

Parameters to be included in the joint TCI state

Rel-15 UL beam indication mechanism allows beam specific UL transmit power control mechanism based on associations between an SRS resource and a set of UL power control parameters. The beam specific UL transmit power control allows beam-specific power allocation by measuring dedicated pathloss RS and applying dedicated parameters. The benefits of beam-specific power control should be maintained in joint TCI and at least following UL power control parameters should be considered:

- Pathloss RS

- P0 and alpha set

- Closed-loop index.

Observation 6: Rel-15 UL power control mechanism allows beam-specific power controls by having a set of power control parameters associated with an SRI.

Proposal 7: At least pathloss RS, P0/alpha set and closed-loop index should be supported as a component of joint TCI.

# L1/L2-centric inter-cell mobility

In RAN1#103-e [1], use cases and enhancement scopes are agreed for L1/L2-centric inter-cell mobility. Based on the agreed use cases and the scopes, we discuss further details on L1/L2-centric inter-cell mobility in this section.

Beam measurement

Beam measurement of reference signals from other cells is a basis to determine optimized beam for inter-cell operation. While other reference signals (e.g., CSI-RS or SRS) can be utilized based on gNB implementation for inter-cell operation (e.g., via scrambling identity), existing beam measurement framework does not allow to measure SSB transmitted by other cells than serving cell. Given that SSB based beam measurement provides efficient beam measurements without additional signaling overheads, it should be enhanced as a part of inter-cell beam measurement.

Observation 7: While SSB provides efficient beam measurement without additional signaling overheads, existing beam measurement framework does not allow SSB based beam measurement.

Proposal 8: SSB based beam measurement for inter-cell operation should be considered.

Beam indication

While existing TCI state allows beam indication based on a reference signal from a serving cell, a reference signal from other cells than serving cells is not allowed to be utilized. In order to achieve efficient operation for beam indication of inter-cell operation, enhancement on existing TCI state should be enhanced. In addition, enhanced beam indication method for inter-cell operation can be considered. One possible way to support enhanced beam indication would be an indication of beam group instead of indicating only one beam for the operation. Based on the indicated beam group, UE-centric determination can be considered based on the indicated beam group.

Observation 8: Existing TCI state does not allow beam indication based on a reference signal from other cells than serving cells.

Proposal 9: An indication of a reference signal from other cells than serving cells should be considered in TCI state for inter-cell operation.

Beam failure recovery

Given that beam failure recovery is an essential procedure which provides reliability and enables standalone operation of FR2, enhancement of beam failure recovery should be an important part of inter-cell mobility. For the enhancement, monitoring, candidate beam selection and reporting for other cells can be considered. For example, as number of cells to be monitored, selected and reported increases, UE implementation complexity and power consumption also increase. In that regard, enhanced operation of beam failure recovery should consider efficient operation with reasonable power consumption. For example, prioritization of beams from serving cells than other cells can be considered.

Observation 9: Beam failure recovery is an essential procedure which provides reliability and enables standalone operation of FR2.

Observation 10: As number of cells to be monitored, selected and reported increases, UE implementation complexity and power consumption also increase.

Proposal 10: Efficient beam failure recovery operation for inter-cell operation should be supported.

# MP-UE assumption to facilitate fast UL panel selection

CSI reporting for MP-UE assumption

In RAN1#103-e [1], UE-initiated UL panel selection/activation was agreed to facilitate fast uplink panel selection. For UE initiated UL panel selection/activation, CSI reporting can be a good method to indicate UE’s selection on UL panel to gNB. One possible enhancement with simple specification impact would be introducing a panel ID in a CSI-RS resource. For example, based on the panel ID, UE can report its panel selection/activation to gNB by reporting CRI and gNB can identify UE’s decision by determining associated panel ID with reported CRI.

Observation 11: CSI reporting can be a good method to indicate UE’s panel selection/activation with simple specification impact.

Proposal 11: Introduce panel ID in a CSI-RS resource for CSI reporting based on UE-initiated UL panel selection/activation.

NW-initiated panel selection/activation

In addition to UE-initiated UL panel selection/activation, NW-initiated panel selection/activation can be considered. While UE is able to identify UE-side information e.g., remaining battery power, MPE and interference from other UEs and gNB, gNB is able to identify network-side information e.g., scheduling information and traffic. In that regard, NW-initiated panel selection/activation can be beneficial as well as UE determination. For example, a panel ID can be introduced in a TCI state for joint beam indication and separate UL beam indication.

Observation 12: While UE is able to identify UE-side information for panel determination, gNB is able to identify gNB-side information for panel determination.

Proposal 12: Introduce NW-initiated panel selection/activation in addition to UE-initiated panel selection/activation.

# MPE mitigation

Support of UE reporting for MPE mitigation

In RAN1#103-e [1], reporting on P-MPR based on Rel.16 framework and SSBRI(s)/CRI(s) was agreed for further investigation. As the existing P-MPR reporting has a limitation that the reporting is not able to indicate a panel or a beam which requires the application of P-MPR while the application could be beam or panel specific. Thus, enhanced P-MPR reporting with associated panels or beams should be considered. For example, by reporting SSBRI(s)/CRI(s) associated with a panel ID, gNB can identify P-MPR for a desired panel.

Observation 13: P-MPR reporting based on Rel.16 framework has a limitation that the reporting is not able to indicate a panel or a beam which requires the application of P-MPR.

Proposal 13: Introduce SSBRI(s)/CRI(s) with an associated panel ID.

NW confirmation in response to the reported MPE event

When UE-centric determination methods are considered, major concern on the support is a mismatch of beam information between UE and gNB. For example, when the gNB couldn’t decode UE’s reporting on P-MPR, a different spatial filter may be applied and severely reduce uplink performance due to a loss of antenna gain.  In that regard, a gNB confirmation can be considered to align the understanding. For example, the gNB may transmit a PDCCH in a dedicated CORESET as a confirmation of the UE reporting on P-MPR.

Observation 14: Possible mismatch of beam information between UE and gNB may severely degrade uplink performance.

Proposal 14: Consider gNB confirmation mechanism to align beam information between UE and gNB.

# Conclusions

This contribution discussed potential enhancement for Rel-17 beam management. Based on the presented discussion, we make the following observations and proposals,

Observation 1: Performance benefits from group-common DCI for L1-based beam indication are not clear as FR2 beam is narrow so that chances for simultaneous transmission can be rare events.

Observation 2: For separate UL beam indication, utilization of existing UL DCI formats 0_1 and 0_2 is more efficient as it doesn’t require multiple PDCCH decoding and combined UE processing time is possible if UE implementation allows simultaneous processing.

Observation 3: As UE transmits PUCCH for ACK/NACK of beam indication with previously indicated beam, UE’s beam changing procedure initiates after sending the PUCCH not after decoding PDCCH which provides L1-based beam indication.

Observation 4: As DCI formats 1_1 and 1_2 are not dedicated formats for beam indication, but for PDSCH scheduling, DCI formats 1_1 and 1_2 can schedule PDSCHs without any new beam indication (i.e., indicating an identical beam with previously indicated beam).

Observation 5: Introduction of SRS in DL TCI state provides performance benefits based on beam correspondence, less signaling overhead and more accurate beam information.

Observation 6: Rel-15 UL power control mechanism allows beam-specific power controls by having a set of power control parameters associated with an SRI.

Observation 7: While SSB provides efficient beam measurement without additional signaling overheads, existing beam measurement framework does not allow SSB based beam measurement.

Observation 8: Existing TCI state does not allow beam indication based on a reference signal from other cells than serving cells.

Observation 9: Beam failure recovery is an essential procedure which provides reliability and enables standalone operation of FR2.

Observation 10: As number of cells to be monitored, selected and reported increases, UE implementation complexity and power consumption also increase.

Observation 11: CSI reporting can be a good method to indicate UE’s panel selection/activation with simple specification impact.

Observation 12: While UE is able to identify UE-side information for panel determination, gNB is able to identify gNB-side information for panel determination.

Observation 13: P-MPR reporting based on Rel.16 framework has a limitation that the reporting is not able to indicate a panel or a beam which requires the application of P-MPR.

Observation 14: Possible mismatch of beam information between UE and gNB may severely degrade uplink performance.

Proposal 1: L1-based beam indication with group-common DCI is not supported.

Proposal 2: For separate UL beam indication, utilization of the existing UL DCI formats 0_1 and 0_2 is preferred.

Proposal 3: For beam application time, the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication (Alt 2) should be supported.

Proposal 4: If DCI formats 1_1 and 1_2 indicate identical beam with previously indicated beam, the indicated beam can be applied to DL and UL without any application time.

Proposal 5: In order to consider various UE implementations, it is preferred to consider UE capability by supporting a UE capability for the minimum value of beam application time (Alt 1).

Proposal 6: DL TCI state based on SRS for beam measurement should be supported.

Proposal 7: At least pathloss RS, P0/alpha set and closed-loop index should be supported as a component of joint TCI.

Proposal 8: SSB based beam measurement for inter-cell operation should be considered.

Proposal 9: An indication of a reference signal from other cells than serving cells should be considered in TCI state for inter-cell operation.

Proposal 10: Efficient beam failure recovery operation for inter-cell operation should be supported.

Proposal 11: Introduce panel ID in a CSI-RS resource for CSI reporting based on UE-initiated UL panel selection/activation.

Proposal 12: Introduce NW-initiated panel selection/activation in addition to UE-initiated panel selection/activation.

Proposal 13: Introduce SSBRI(s)/CRI(s) with an associated panel ID.

Proposal 14: Consider gNB confirmation mechanism to align beam information between UE and gNB.

# References

- “Final Report of 3GPP TSG RAN WG1 #103-e”, 3GPP RAN1 Meeting #103-e, e-Meeting, October 26th – November 13th, 2020.