3GPP TSG RAN WG1 #104-e                                          R1-2100092

e-Meeting, January 25th – February 5th, 2021

Source:	ZTE

Title:          Remaining issue about Timing for Secondary cell activation and deactivation

Agenda item:	7.2.5

Document for: Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues on UL control enhancements about unexpected latency for SCell activation or deactivation.

Unexpected latency for SCell activation or deactivation

In the RAN1 #102-e meeting[1], some companies worried that there was an unexpected latency for SCell activation or deactivation when the sub-slot based PUCCH transmissions are configured as the SCell activation or deactivation timing is defined only considering the slot-based PUCCH transmissions as following in TS38.213[2].

- 4.3 Timing for secondary cell activation / deactivation With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following: - the actions related to CSI reporting on a serving cell that is active in slot - the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot - the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after in which the serving cell is active. The value of is where is a number of slots for a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and is a number of slots per subframe for the SCS configuration of the PUCCH transmission. <--------------------------------------------------Other parts are omitted -------------------------------------------------->

It can be seen that a UE applies the SCell activation or deactivation in slot n+k if the PDSCH carrying the MAC-CE command ends in slot n. It makes sense when the UE is configured with slot based PUCCH transmission. However, when the PUCCH with HARQ-ACK corresponding to the PDSCH is sub-slot based resource, the UE will apply the SCell activation or deactivation in slot n + k1 + 3+ 1 literally, which will cause the timing between PDSCH carrying the activation command and SCell activation or deactivation is unexpected large.

This issue was discussed in RAN1 #103-e meeting, and at least following three options are raised to solve it:

Option 1: Change the reference time to the slot of PUCCH. Option 2: Convert k1 from sub-slot to slot if sub-slot based PUCCH resource are configured. Option 3: No need change since the current spec is clear.

In our understanding, the slot with 2/7 OSs only exists in Clause 9 in TS 38.213, but in the other clauses, at least before Clause 9, a slot always contains 14 OSs. Therefore, even if a UE is provided subslotLengthForPUCCH-r16, the k1 in Clause 4 still should be a number of slots with 14 OSs for a PUCCH transmission with HARQ-ACK information for the PDSCH reception.

Proposal 1: The current spec about the timing for secondary cell activation or deactivation is clear, no changes are needed.

Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: The current spec about the timing for secondary cell activation or deactivation is clear, no changes are needed.

Reference

- 3GPP RAN1#102-e meeting, R1-2007054, Summary#1 of email thread [102-e-NR-L1enh_URLLC-UCI_Enh-02], Moderator (OPPO).

- 3GPP TS 38.213-g40, Physical layer procedures for control, Release 16.