3GPP TSG-RAN WG1 Meeting #104-e	R1-2100140

e-Meeting, January 25 – February 05, 2021

Agenda item:	7.2.11

Title:	Correction for V2X UE feature list

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

Due to the discussion outcome for FG15-2 in the last RAN1#103-e meeting [1], in this contribution, we identify a necessary correction in the UE feature list for 5G-V2X.

2. Discussion

In the last meeting, it was agreed and RAN1 modified FG15-2 (transmitting NR sidelink mode 1 scheduled by NR Uu) to cover only the case of same carrier scheduling. For the cross-carrier scheduling case, a new FG15-25 (Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier) was introduced. As such, a related note in FG15-2 (yellow highlight in the table below) should have been updated, but it was overlooked.

Proposal: Due to discussion outcomes that FG15-2 is only intended for same carrier scheduling and the new FG15-25 for cross-carrier scheduling is defined as an optional FG with capability signalling, a related sentence in the “note” column for FG15-2 should be corrected as:

“This is the basic FG for sidelink in licensed spectrum where gNB is operating on”

| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to 8 configured grants can be configured for a UE. Up to C sidelink HARQ processes are supported including those for configured grants 2) UE can transmit PSSCH according to the normal 64QAM MCS OFDM table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2 on the same carrier as sidelink. 6) UE can transmit using the subcarrier spacing and CP length it reports. 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 9) Support downlink pathloss based open loop power control 11) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1 |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Candidate values for C are {8,16} Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: For Component 6, if a band is not indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1, the reported numerology shall be the same for sidelink and uplink. Component (9) is only required to be supported in a band not indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E.1-1 In a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1, the UE supports at least 30 kHz with normal CP in FR1, and at least 120 kHz with normal CP in FR2 | Optional with capability signalling For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-25 | Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier | 1) UE can monitor DCI format 3_0 on a different carrier from sidelink for NR sidelink dynamic scheduling and configured grant type 2 | FG 15-2 | Yes | No |  | Per FS | N.A. | N.A. | N.A. | If the UE indicates support for FG 15-2 in a band indicated with only the PC5 interface in Table 5.2E.1-1 of 38.301-1, the UE must indicate that FG 15-25 is supported for a band combination with that band. | Optional with capability signalling |

References:

[1] R1-2009585 “Updated RAN1 UE features list for Rel-16 NR”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#103-e