3GPP TSG RAN WG1 meeting #104-e                                                                             R1-2100316

e-Meeting, January 25th – February 5th, 2021

Title:	Draft reply LS on SL switching priority

Response to:	R4-2017839

Release: 	Rel-16

Work Item: 	5G_V2X_NRSL-Core

Source:	CATT, GOHIGH [RAN1]

To: 	RAN4

Cc: 	RAN2

Contact person:	Rui Zhao

zhaorui@gohigh.com.cn

Attachments:	None

Overall Description

RAN1 would like to thank RAN4 for the LS R1-2100009[1] on SL switching priority, and questions about the priority between LTE SL and NR SL.

Question 1: Is there priority defined for LTE SL and NR SL?

Answer: Yes, the priority is defined for both LTE SL and NR SL. The priority information is carried in SCI format 1 and SCI format 1-A for LTE SL and NR SL respectively.

Question 2: How does RAN WG1 define the priority of LTE SL and NR SL? For example, which parameter is used and how to determine the priority?

Answer:

For LTE SL, the priority carried in SCI format 1 is determined by the PPPP of V2X traffic QoS.

For NR SL, the message to be transmitted in SL includes not only V2X traffic but also PC5 signaling(e.g. SL-CSI feedback, and PC5-RRC, and etc..). When V2X traffic is transmitted in SL, the priority carried in SCI format 1-A is determined by the priority level of V2X traffic QoS. When PC5-signaling is transmitted in SL, the priority carried in SCI format 1-A is fixed as “1”.

Additionally, NR V2X priority level and LTE V2X PPPP are directly comparable i.e. the same numerical value has the same meaning in both the RATs[2].

Question 3: Is it the case that there is no higher priority for LTE SL than NR SL?

Answer: No, the priority of V2X traffic is determined by application layer. From RAN1 understanding, LTE SL could have higher priority than NR SL.

Actions

To RAN2

ACTION: RAN1 respectfully asks RAN4 to take the above information into account.

Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #104b-e                                       12 Apr. – 20 Apr. 2021                       e-Meeting

TSG-RAN WG1 Meeting #105-e                                         19 May – 27 May 2021                       e-Meeting

Reference

  - R1-2100009, “LS on SL switching priority”, RAN4.

  - R1-1911675, “Reply LS on mapping between LTE V2X PPPP and NR V2X priority”, SA2