3GPP TSG RAN WG1 Meeting #104-e R1-2100341 e-Meeting, 25th January – 5th February, 2020 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; draft; rev; -; Current version:; 16.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:. Source to WG:; CATT. Source to TSG:; RAN WG1. Work item code:; NR_UE_pow_sav_CORE; Date:; 2021-01-18. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In Clause 10.3, the DCI format 2_6 is used to indicate to MAC layer to start or not to start the drx-onDurationTimer in the configured DRX operation. In Clause 5.7 of TS38.321, a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters in support of secondary DRX group introduced in Rel-16. However, the DCI format 2_6 for UE wakeup and/or SCell dormancy are not configured together with secondary DRX group to avoid the feature interaction, When two DRX groups are configured in support of secondary DRX group, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. In Clause 10.3, DCI format 2_6 is configured for the indication to the MAC layer for the start of drx-OnDurationTimer. The current specification needs to be corrected to state DCI format 2_6 is configured for indication of the start of drx-OnDurationTimer and/or SCell dormancy only when one DRX group is configured.. -. Summary of change:; Adding “with only one DRX group” to the sentence “A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]” to restrict the support for DCI format 2_6 when secondary DRX group is configured. Adding “when only one DRX group is configured” for DCI format 0_1/1_1 for case 1 and DCI format 1_1 for case 2 SCell dormancy indication.. Consequences if not approved:; This CR is to correct the UE behavior when UE is configured DCI format 2_6 for DRX adaptation and SCell dormancy indication only when one DRX group is configured without the support of additional. Clauses affected:; 10.3. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR is to correct the ambiguity of indication of DCI format 2_6 for the start of drx-OnDurationTimer and SCell dormancy only apply when UE is configured with one DRX group. If the CR is not included, the UE behavior of DRX operation with DCI format 2_6 for wakeup indication and SCell dormancy for configuring with one DRX group only is not clear and does not exclude the scenario of joint configuration with the secondary DRX group.. This CR's revision history:; This is the first version for this CR..

## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

A UE configured with DRX mode operation [11, TS 38.321] with only one DRX group can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1

-	a payload size for DCI format 2_6 by sizeDCI_2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by psPositionDCI-2-6

-	a '0' value for the Wake-up indication bit, when reported to higher layers, indicates to not start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a '1' value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a bitmap, when the UE is provided a number of groups of configured SCells by dormancyGroupOutsideActiveTime, where

-	the bitmap location is immediately after the Wake-up indication bit location

-	the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDuarationTimer would start on the PCell or on the SpCell [11, TS 38.321]

-	for each search space set, the PDCCH monitoring occasions are the ones in the first Ts slots indicated by duration, or Ts=1 slot if duration is not provided, starting from the first slot of the first Ts slots and ending prior to the start of drx-onDurationTimer.

On PDCCH monitoring occasions associated with a same long DRX Cycle, a UE does not expect to detect more than one DCI format 2_6 with different values of the Wake-up indication bit for the UE or with different values of the bitmap for the UE.

The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321].

If a UE reports for an active DL BWP a requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the requirement of the SCS of the active DL BWP in Table 10.3-1.

Table 10.3-1 Minimum time gap value X

| SCS (kHz) | Minimum Time Gap X (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [11, TS 38.321] for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report a value of the Wake-up indication bit to higher layers for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

-	is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or

-	does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle

the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field when only one DRX group is configured,

-	the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime,

-	each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells

-	if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1 when only one DRX group is configured, and if

-	the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if

-	a one-shot HARQ-ACK request field is not present or has a '0' value, and if

-	the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1

the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of

-	modulation and coding scheme

-	new data indicator

-	redundancy version

and of

-	HARQ process number

-	antenna port(s)

-	DMRS sequence initialization

as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

If an active DL BWP provided by dormantBWP-Id for a UE on an activated SCell is not a default DL BWP for the UE on the activated SCell, as described in Clause 12, the BWP inactivity timer is not used for transitioning from the active DL BWP provided by dormantBWP-Id to the default DL BWP on the activated SCell.

A UE is expected to provide HARQ-ACK information in response to a detection of a DCI format 1_1 indicating SCell dormancy after N symbols from the last symbol of a PDCCH providing the DCI format 1_1. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the DCI format 1_1, N=7 for μ=0, N=7.5 for μ=1, and N=15 for μ=2; otherwise, N=14 for μ=0, N=16 for μ=1, N=27 for μ=2, and N=31 for μ=3, where μ is the smallest SCS configuration between the SCS configuration of the PDCCH providing the DCI format 1_1 and the SCS configuration of a PUCCH with the HARQ-ACK information in response to the detection of the DCI format 1_1.