3GPP TSG RAN WG1 #104-e                                             R1-2100365

e-Meeting, January 25th –February 5th, 2021

Agenda Item:	8.15.1

Source:	CATT

Title:	         Applicable Scenarios to NB-IoT/eMTC

Document for:	Discussion and Decision

Introduction

In the RAN #103 e-meeting，the following agreements on scenarios and configuration have been reached[1]:

Agreement:

IoT NTN scenarios A, B, and C are included in the study as shown below:

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |

Agreement:

The following IoT NTN reference scenario parameters are agreed:

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C |
| --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Frequency Range (service link) | < 6 GHz (e.g. 2 GHz in S band) |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations , including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |
| Payload | Transparent type | Transparent Type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) |
| Min Elevation angle for both sat-gateway and C-IoT device | 10degree for service link and 10degree for feeder link | 10degree for service link and 10degree for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |

NOTE 1:    Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite.

NOTE 2:   This beam size refers to the Nadir pointing of the satellite.

NOTE 3: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam.

NOTE 4: The use of a Circular polarized antenna is optional.

NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888  for device can be assumed for link budget. The noise figure is device vendor implementation specific.

NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link.

NOTE 7: System bandwidth is FFS

In this contribution, we further anlayzed the related issues and prosented our views about potential application scenarios for eMTC and NB-IoT over NTN.

Discussion

  - Link budget

In this section, link budget under different scenarios are evaluated. According to agreed link budget parameters, we calculate the link budget in S band for related scenarios. Set-1 satellite set in TR38.321 are reused. Note that the max UL channel bandwidth can be 180 kHz for NB-IoT and 2*180 kHz for eMTC, so we use this worst assumption to check if SNR is reasonable for larger UL transmission bandwidth. Results are shown in the table 1 and table 2.

Table 1 Link budget result for eMTC NTN

|  | DL | UL | DL | UL | DL | UL |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | GEO | GEO | LEO1200 | LEO1200 | LEO600 | LEO600 |
| Frequency (GHz) | 2 | 2 | 2 | 2 | 2 | 2 |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 59 | -10 | 40 | -10 | 34 | -10 |
| RX: G/T [dB/K] | -31.62 | 19 | -31.62 | 1.1 | -31.62 | 1.1 |
| B(MHZ) | 1.08 | 0.36 | 1.08 | 0.36 | 1.08 | 0.36 |
| Free space path loss [dB] | 190.64 | 190.64 | 168.38 | 168.38 | 164.19 | 164.19 |
| Atmospheric loss [dB] | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Shadow fading margin [dB] | 3 | 3 | 3 | 3 | 3 | 3 |
| Scintillation Loss [dB] | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| Polarization loss [dB] | 0 | 0 | 0 | 0 | 0 | 0 |
| Additional losses [dB] | 0 | 0 | 0 | 0 | 0 | 0 |
| Central beam elevation | 10 degree | 10 degree | 10 degree | 10 degree | 10 degree | 10 degree |
| C-IoT device Noise Figure[dB] | -7 | -7 | -7 | -7 | -7 | -7 |
| CNR [dB] | 0.04 | -13.90 | 3.3 | -9.54 | 1.49 | -5.35 |

Table 2 Link budget result for NB-IoT NTN

|  | DL | UL | DL | UL | DL | UL |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | GEO | GEO | LEO1200 | LEO1200 | LEO600 | LEO600 |
| Frequency (GHz) | 2 | 2 | 2 | 2 | 2 | 2 |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 59 | -10 | 40 | -10 | 34 | -10 |
| RX: G/T [dB/K] | -31.62 | 19 | -31.62 | 1.1 | -31.62 | 1.1 |
| B(MHZ) | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 | 0.18 |
| Free space path loss [dB] | 190.64 | 190.64 | 168.38 | 168.38 | 164.19 | 164.19 |
| Atmospheric loss [dB] | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Shadow fading margin [dB] | 3 | 3 | 3 | 3 | 3 | 3 |
| Scintillation Loss [dB] | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| Polarization loss [dB] | 0 | 0 | 0 | 0 | 0 | 0 |
| Additional losses [dB] | 0 | 0 | 0 | 0 | 0 | 0 |
| Central beam elevation | 10 degree | 10 degree | 10 degree | 10 degree | 10 degree | 10 degree |
| C-IoT device Noise Figure[dB] | -7 | -7 | -7 | -7 | -7 | -7 |
| CNR [dB] | 0.04 | -10.89 | 3.3 | -8.53 | 1.49 | -4.34 |

Based on the above link budget results, it can be seen that in GEO system, the CNR of UL at the beam center will reach -14dB level for eMTC. Even for NB-IoT, this situation is also worse, about -11dB level. Considering that the maximum cell radius is 1750km, the worst CNR of cell edge users in eMTC will be far less than - 14dB.

Observation 1：In GEO system, the UL CNR at the beam center will reach about -14dB when large bandwidth is configured.

Proposal 1：In view of worse SNR range in large UL bandwidth, 360khz / 180khz UL bandwidth should not be set as typical configuation for calibration and performance evaluation.

  - Satellite related parameters configuration

In general, MTC deployment is targeted to provide wide coverage, especially for non-terrestrial network. Then the minimal beam footprint size should not be less than 100km. In order to ensure good coverage within the beam area, the 3dB beamwidth should be carefully configured. In table 3, 3dB beamwidth angles are listed according to TR38.821.

Table 3 The 3dB beamwidth for S band in TR 38.821

|  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| 3dB beamwidth—Set1 | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter—Set1 | 250 km | 90 km | 50 km |
| 3dB beamwidth—Set2 | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter—Set2 | 450 km | 190 km | 90 km |

From the table 3, it is observed that 3dB beamwidth is a bit narrow due to limited beam footprint size when use it into IoT NTN scenario. As agreed beam size, for GEO IoT case, the beam coverage is 3500km, and for LEO IoT case, the beam coverage is 1000km. Obviously, the 3dB beamwidth for IoT NTN needs to be updated.

Observation 2：The existing 3dB beamwidth for S band on satellite parameter configuration in TR 38.821 can’t guarantee beam coverage for IoT NTN.

Proposal 2: New 3dB beamwidth paremeter for IoT NTN needs to be defined.

According to the altitude of the satellite, the elevation angle and the radius of the earth, satellite coverage angles can be calculated. Related angle definitions are llustrated in figure 1, where one beam is applied for one satellite, α is elevation angle of satellite, β1 is the angle from left edge of the beam to beam center and β2 is the angle from right edge of the beam to beam center.

Figure 1 Beam coverage and angle mapping

Satellited coverage results are shown in the table 4, where the beam diameter corresponds to the steering beam case in which beam center is fixed for different elevation angle.

Table 4 Satellite coverage result for different cases

| Use cases | Elevation:α | the max angle of beam coverage:β1 | the max angle of beam coverage:β2 | the corresponding beam diameter assuming beamwidth angle equal to 2*β1 |
| --- | --- | --- | --- | --- |
| GEO | 90 degree | 8.6922 degree | 8.6922 degree | 5429*2 km |
| GEO | 45 degree | 2.5577 degree | 14.8266 degree | 1670*2 km |
| GEO | 30 degree | 1.1718 degree | 16.2125 degree | 789.62*2 km |
| GEO | 10 degree | 0.1331 degree | 17.2513 degree | 94.271*2 km |
| LEO-1200 | 90 degree | 57.2989 degree | 57.2989 degree | 1200*2 km |
| LEO-1200 | 45 degree | 20.7841 degree | 98.8137 degree | 573.1*2 km |
| LEO-1200 | 30 degree | 10.5163 degree | 104.0815 degree | 366.88*2 km |
| LEO-1200 | 10 degree | 1.3318 degree | 113.2660 degree | 313.09*2 km |
| LEO-600 | 90 degree | 66.0541 degree | 66.0541 degree | 691.72*2 km |
| LEO-600 | 45 degree | 25.7950 degree | 106.3132 degree | 366.83*2 km |
| LEO-600 | 30 degree | 13.7295 degree | 118.3787 degree | 257.62*2 km |
| LEO-600 | 10 degree | 1.8902 degree | 130.2180 degree | 63.725*2 km |

As agreed in agreement of RAN #103e, for GEO IoT case, the max beam coverage is 3500km, and for LEO IoT case, the max beam coverage is 1000km. Based on the above results, for GEO and LEO-600, the beam sizes in case of the elevation angle equal to 45 degree and less are less than 3500 km, and for LEO-1200 case the beam sizes in case of the elevation angle equal to 30 degree and less are less than 1000km. If taking agreed maximum beam size for GEO and LEO case as the evaluation assumption, some evaluated cases same as that of TR38.821 may not be suitable. Hence we suggest to change the maximum beam coverage or select other elevation angles for evaluation.

Based on above results, we can get the following observations:

Observation 3: For IoT NTN evaulation, due to larger beam size, mutiple tiers of beam layout may not be suitable.

Observation 4: For steering beam case, smaller beam size is to be defined to fit different elevation angles.

Proposal 3: For evaluation purpose, it is suggested to limited beam tier and adjust the maximum beam size to get reasonble results.

Conclusion

In this contribution, we analzyed typical scenarions and related parameters for NB-IoT and eMTC over satellite. Observations and proposals are summarized below:

Observation 1：In GEO system, the UL CNR at the beam center will reach about -14dB when large bandwidth is configured.

Observation 2：The existing 3dB beamwidth for S band on satellite parameter configuration in TR 38.821 cannot guarantee beam coverage for IoT NTN.

Observation 3: For IoT NTN evaulation, due to larger beam size, mutiple tiers of beam layout may not be suitable.

Observation 4: For steering beam case, smaller beam size is to be defined to fit different elevation angles.

Proposal 1：In view of worse SNR range in large UL bandwidth, 360khz / 180khz UL bandwidth should not be set as typical configuation for calibration and performance evaluation.

Proposal 2: New 3dB beamwidth paremeter for IoT NTN needs to be defined.

Proposal 3: For evaluation purpose, it is suggested to limited beam tier and adjust the maximum beam size to get reasonble results.

References

- RAN1 #103e Chairman notes

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN).