3GPP TSG RAN WG1 #104-e			R1-2100403

e-Meeting, January 25th – February 5th, 2021

Title:	Draft reply LS on the use of simultaneous CSI-RS and ports

Source:	CATT

Agenda 	5

Document for:	   Discussion and Decision

- Background

An LS was received from RAN2 (c.f. [1]) on the use of simultaneous CSI-RS resources and ports. Specifically, In RAN2 understanding, the field maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC is supposed to indicate a restriction on the number of active CSI-RS resources. Furthermore, the field totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC is supposed to indicate a restriction on the total number of ports to be used in active CSI-RS resources.  RAN2 would like to ask whether the fields maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC and totalNumberPortsSimultaneousNZP-CSI-RS are supposed to indicate: the number of active CSI-RS resources and ports, respectively; or the number of configured CSI-RS resources and ports (including active CSI-RS resources and ports), respectively?

The reason for the question is that the current field description for maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC in TS 38.306 includes “This parameter limits the total number of NZP-CSI-RS resources that the NW may configure across all CCs, and across MCG and SCG in case of NR-DC (irrespective of the associated codebook type).”; and the current field description for totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC  in TS 38.306 includes “This parameter limits the total number of ports that the NW may configure across all NZP-CSI-RS resources across all CCs”.

- Discussion

Our understanding is that RAN2’s understanding is correctly aligned with RAN’1 intention when defining the UE capability as to CSI-RS resources/ports reflected in UE capability maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC and totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC.

References:

### R2-2011274, “LS on the use of simultaneous CSI-RS resources and ports,” RAN2