3GPP TSG RAN WG1 #104-e			R1-2100532

e-Meeting, January 25th – February 5th, 2021

Source: 	ZTE, Sanechips

Title:	Discussion on DL 1024QAM for NR FR1

Agenda Item:	8.16

Document for:	Discussion/Decision

- Introduction

In RAN1-103e meeting, the agreements about DL 1024QAM for NR FR1 are as follows [1]:

Agreements: - Introduce new RRC signaling to indicate use of 1024-QAM CQI table. - For supporting 1024-QAM in NR downlink, adopt the LTE 1024-QAM constellation. - 1024-QAM MCS table can be used only with DCI format with CRC scrambled by C-RNTI or CS-RNTI Agreements: - Introduce new RRC signaling to indicate use of 1024-QAM MCS table for at least DCI format 1_1 - FFS : support of 1024-QAM MCS table for DCI format 1_2 - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, separate RRC signaling is used for each of the two DCI formats 1_1 and 1_2, respectively - FFS : whether the RRC signaling is only introduced in PDSCH-Config or it can also be separately configured in SPS-Config Agreements: - RRC signaling (mcs-Table-r17) to indicate use of 1024-QAM MCS table for DCI format 1_1 is present only in PDSCH-config - When UE is configured with mcs-Table-r17 set to 1024-QAM MCS table for DCI - UE uses 1024-QAM MCS table for PDSCH scheduled with a DCI format 1_1 with CRC scrambled by C-RNTI, - UE uses 1024-QAM MCS table for PDSCH scheduled with the DCI format 1_1 with CRC scrambled by CS-RNTI if the UE is not configured with mcs-Table in SPS-Config - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, similar approach is used for 1024-QAM MCS table usage with DCI format 1_2

However, the design of 1024QAM CQI table and 1024QAM MCS table has not been determined in RAN1-103e meeting, due to different design principles among companies. More specifically, the divergence largely comes from the granularity of 256QAM and 1024QAM. Therefore, it is necessary to analyze the transition point between 256QAM and 1024QAM. Moreover, some TPs are proposed.

- Discussion on design of 1024QAM CQI table and MCS table

Transition point of 256QAM and 1024QAM

In RAN1-103e meeting, link level simulation assumption is given as follows [1]:

Agreements: - Companies are encouraged to use below link-level simulation assumptions for assessing at least transition point between 256-QAM and 1024-QAM. Parameter; Value. Carrier frequency, SCS, System BW; 3.5GHz, 30kHz, 100 MHz. Channel model; AWGN, CDL-B or CDL-C in TR 38.901 with up to 30ns delay spread. UE speed; 3km/h, 0km/h. Number of UE antennas; 1T4R, 2T4R or 4T4R. Number of gNB antennas; 32T32R or 64T64R or 2T or 8T. Tx EVM; 0, 2%. Rx EVM; 0, 3%. MCS; 256 QAM, 1024 QAM Coding Rate*: 0.70, 0.75, 0.80, 0.85, 0.90, 0.925 Other coding rates are not precluded and, if simulated, to be reported by each company. DMRS type; DM-RS type 1. Number of DMRS symbols; 1. Number of scheduled RBs; 273. PDSCH mapping; Type A, Start symbol 2, Duration 12. Rank; Rank1, Rank 2,. Channel estimation; Realistic channel estimation. Metric; Crossover SNR at transition points between 256-QAM and 1024-QAM. Note*: Coding rates are used for 1024QAM, while coding rates for 256QAM are selected from TS38.214 MCS table 2.

As the transition point of 256QAM and 1024QAM is SE =7.4063, the corresponding CQI alternative entries for 256QAM and 1024QAM are shown in the Table 1.

Table 1 Alternative modulation order and code rate combination in transition point

|  | Modulation Order | Target code rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| Alt1 | 8 | 948 | 7.4063 |
| Alt2 | 10 | 758.5 | 7.4063 |

The SE vs. SNR performance at BLER of 10% of the above two alternative entries for AWGN, CDL-B and CDL-C channel models with and without EVM are shown in Figure 1, Figure 2 and Figure 3 respectively based on the simulation assumptions in Table A.1. The SNR values of Alt1 and Alt 2 at target BLER=10% in AWGN, CDL-B and CDL-C channel models with and without EVM are provided in Table 2.

| (a) ) EVM: (Tx, Rx)= (0, 0) | (b) ) EVM: (Tx, Rx)= (2%, 3%) |
| --- | --- |

Figure 1 SE vs. SNR performance at target BLER = 10% in AWGN channel

| (a) EVM: (Tx, Rx)= (0, 0) | (b) EVM: (Tx, Rx)= (2%, 3%) |
| --- | --- |

Figure 2 SE vs. SNR performance at target BLER = 10% in CDL-B channel

| (a) EVM: (Tx, Rx)= (0, 0) | (b) EVM: (Tx, Rx)= (2%, 3%) |
| --- | --- |

Figure 3 SE vs. SNR performance at target BLER = 10% in CDL-C channel

Table 2 SNR@BLER=10% of 256QAM and 1024QAM at SE = 7.4063 with and without EVM

| Channel model |  | AWGN |  | CDL-B |  | CDL-C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EVM: (Tx, Rx) |  | (0, 0) | (2%, 3%) | (0, 0) | (2%, 3%) | (0, 0) | (2%, 3%) |
| SNR(dB)@BLER=10% | Alt1: (256QAM, 948) | 24.94 | 26.95 | 15.81 | 16.67 | 15.05 | 15.88 |
|  | Alt2: (1024QM, 758.5) | 24.73 | 26.76 | 16.40 | 16.98 | 15.85 | 16.49 |
| ∆SNR(dB)=Alt2-Alt1 |  | -0.21 | -0.19 | 0.59 | 0.31 | 0.80 | 0.61 |

According the simulation results in Figure 1- Figure 3 and Table 2, it can be seen that the performance of Alt2 (1024QAM, 758.5) is almost the same with that of Alt1 (256QAM, 948) in AWGN channel, while the performance of Alt1 (256QAM, 948) is better than that of Alt2 (1024QAM, 758.5) in fading channel, i.e, CDL-B, CDL-C.

Proposal 1: The same CQI entry with LTE, i.e., Alt1 (256QAM, 948), is adopted as the CQI entry.

1024QAM CQI Table

In LTE, the CQI table with 1024QAM is shown as Table 3 [2]. The SE vs SNR performance of CQI entries with index of 1-15 in Table 3 at BLER of 10% is evaluated and shown in Figure 4. The simulation parameters are based on Table A.2.

Table 3 4-bit CQI Table 4 in LTE

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.330121 |
| 15 | 1024QAM | 948 | 9.2578 |

Figure 4 SE vs. SNR performance at target BLER = 10% of the LTE 1024QAM CQI table

Based on the simulation results in Figure 4, it can be seen that the SNR spacing between CQI entries with the same modulation order in SE vs. SNR curves is approximately the same.

Observation 1: The SNR spacing between CQI entries with the same modulation order is approximately the same.

Therefore, to simplify standardization workload, we prefer to reuse LTE CQI table with 1024QAM entries for NR PDSCH.

Proposal 2: To simplify the standardization workload, LTE CQI table with 1024QAM entries, that is, Table 3, should be reused for NR PDSCH.

1024QAM MCS Table

Based on the revised WID as follows, it is clear that only 5-bit MCS table should be specified [4].

The objective is to specify downlink 1024QAM for NR PDSCH operation in FR1, together with related procedures, signalling and necessary RF requirements. The main objectives are: - Specify high order modulation for PDSCH [RAN1] - Specify 1024QAM constellation as specified in E-UTRA for DL PDSCH - Specify corresponding 5-bit MCS table with 1024QAM entries as defined in E-UTRA, with 5 bit DCI overhead for MCS indication - Specify corresponding CQI feedback with 1024QAM entries as defined in E-UTRA, with no changes to the CQI field and table sizes

Observation 2: Only 5-bit MCS table with 1024QAM entries is specified according to the WID scope.

For 5-bit 1024QAM MCS table, the MCS design principle is to start with NR 256QAM MCS table and remove M=5 entries to accommodate 1024QAM MCS entries according the discussion in the last meeting. However, the removed MCS entries from NR 256QAM MCS table have the following alternatives [5].

Regarding M=5 entries to be removed, different alternatives were mentioned: - Alt1: remove {5,7,9,12,14} from the 256-QAM table - Alt2: remove {6, 8, 10, 12, 14} from the 256QAM table - Alt3: remove {2,4,6,8,10} from the 256-QAM table - Alt4: remove {1,3,5,7,9} from the 256-QAM table

According to agreements in RAN1#90bis meeting [6] and in RAN1#92 meeting [7], the design principles of 1024QAM MCS table are shown as follows.

Agreements in RAN1#90bis: - For introduction of 1024QAM MCS table: - Remove M entries from the 256QAM table while maintaining (close to) uniformly spaced SE, while keeping the lowest MCS - Add M new entries for 1024QAM, with (close to) uniformly spaced SE - Including 1 entry to support re-transmission with 1024 QAM Agreements in RAN1#92: The removed entries from 256QAM table are {5, 7, 9, 12, 14}.

Hence, Alt 1, i.e., remove {5, 7, 9, 12, 14} from the 256-QAM table is preferable based on the agreements for LTE 1024QAM MCS table in RAN1-92 meeting [7].

Proposal 3: The removed entries from 256QAM MCS table are Alt 1: {5, 7, 9, 12, 14}

Herein, the proposed 5-bit MCS table with 1024QAM entries for NR PDSCH is shown in Table 4.

Table 4 Proposed MCS table with 1024QAM entries for NR PDSCH

| MCS Index | Modulation Order | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 434 | 1.6953 |
| 6 | 4 | 553 | 2.1602 |
| 7 | 4 | 658 | 2.5703 |
| 8 | 6 | 466 | 2.7305 |
| 9 | 6 | 567 | 3.3223 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.3320 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 806 | 7.8711 |
| 24 | 10 | 853 | 8.330121 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 |  | reserved |
| 28 | 4 |  |  |
| 29 | 6 |  |  |
| 30 | 8 |  |  |
| 31 | 10 |  |  |

More specifically, Table 4 is derived according to the following steps:

- The modulation order corresponding to each MCS index in Table 4 is the same with that in the LTE 1024QAM MCS table. Add 5 new entries for 1024QAM including 1 entry to support re-transmission with 1024QAM.

- The entries with MCS index of 24 and 26 in Table 4 are extracted from the 1024QAM entries in 1024QAM CQI table as shown in Table 3.

- To ensure uniformly spaced SE, the SE of MCS index of 23 and 25 with 1024QAM are obtained by averaging the SE of adjacent MCS entries, i.e., = (+)/27.8711 and = (+)/28.7939. Herein, the target code rate in Table 4 is approximately equal to (SE/Qm).

- Remove 5 entries with MCS index of {5, 7, 9, 12, 14} from NR 256QAM MCS table (i.e., Table A.3) to obtain the MCS entries with index of 0-22 and 27-30 in Table 4.

Observation 3: The 1024QAM MCS table in Table 4 follows the design rule of LTE 1024QAM MCS table agreed in RAN1#90bis and RAN1#92 meeting.

The SE vs SNR performance of MCS entries with index of 0-27 in Table 4 at BLER of 10% is evaluated and shown in Figure 5. The simulation parameters are based on Table A.2.

Figure 5 SE vs SNR performance at BLER=10% of the 1024QAM MCS table in Table 4

Observation 4: The SNR spacing between the adjacent MCS entries in Table 4 is around 2dB for MCS index of 0-10, around 1dB for MCS index of 11-22, and around 1.5dB for MCS index of 23-26 with 1024QAM.

According to the analysis above, the SNR spacing of SE vs. SNR curves is uniformly spaced. Therefore, Table 4 should be used as the MCS table with 1024QAM MCS entries for NR PDSCH.

Proposal 4: Table 4 should be used as the MCS table with 1024QAM entries for NR PDSCH.

- Remaining TPs for DL 1024QAM

In RAN1-103e meeting, some TPs were agreed in corresponding specs. Besides, the remaining TPs for support of DL 1024QAM are listed as follow.

TS38.214 V16.4.0 5.1.3.1 Modulation order and target code rate determination Unchanged parts are omitted Table 5.1.3.1-4: MCS index table 4 for PDSCH MCS entries are omitted Unchanged parts are omitted 5.1.6.3 PT-RS reception procedure Unchanged parts are omitted If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig, - the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, respectively. - if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna port' presence and pattern is a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, - if the higher layer parameter timeDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume LPT-RS = 1. - if the higher layer parameter frequencyDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume KPT-RS = 2. - otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when - the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or - the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or - the scheduled MCS from Table 5.1.3.1-3 is smaller than 15, or - the scheduled MCS from Table 5.1.3.1-4 is smaller than 5, or - the number of scheduled RBs is smaller than 3, or - otherwise, if the RNTI equals RA-RNTI, [MsgB-RNTI], SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present Unchanged parts are omitted

- Conclusion

In this contribution, the observations and proposals for DL 1024QAM for NR are as follows:

Observation 1: The SNR spacing between CQI entries with the same modulation order is approximately the same.

Observation 2: Only 5-bit MCS table with 1024QAM entries is specified according to the WID scope.

Observation 3: The 1024QAM MCS table in Table 4 follows the design rule of LTE 1024QAM MCS table agreed in RAN1#90bis and RAN1#92 meeting.

Observation 4: The SNR spacing between the adjacent MCS entries in Table 4 is around 2dB for MCS index of 0-10, around 1dB for MCS index of 11-22, and around 1.5dB for MCS index of 23-26 with 1024QAM.

Proposal 1: The same CQI entry with LTE, i.e., Alt1 (256QAM, 948), is adopted as the CQI entry.

Proposal 2: To simplify the standardization workload, LTE CQI table with 1024QAM entries, that is, Table 3, should be reused for NR PDSCH.

Proposal 3: The removed entries from 256QAM MCS table are Alt 1: {5, 7, 9, 12, 14}

Proposal 4: Table 4 should be used as the MCS table with 1024QAM entries for NR PDSCH.

- References

- Draft Report of 3GPP TSG RAN WG1 #103-e v0.2.0 (Online meeting, 26th October - 13th November 2020).

- 3GPP TS 36.213 V16.4.0, 2020-12.

- 3GPP TS 38.214 V16.4.0, 2020-12.

- RP-202886, “Revised WID on Introduction of DL 1024QAM for NR FR1”, Nokia, RAN#90e.

- R1-2009799, Summary of Email discussion [103-e-NR-1024QAM-01].

- Chairman’s Notes in RAN1#90 bis.

- Chairman’s Notes in RAN1#92.

- Appendix

- A.1 Simulation parameters for transition point of 256QAM and 1024QAM

### Table A.1	Simulation assumptions for transition point

| Parameter | Value |
| --- | --- |
| Carrier frequency, SCS, System BW | 3.5GHz, 30kHz, 100 MHz |
| Channel model | AWGN, CDL-B, CDL-C |
| Channel coding | LDPC |
| UE speed | 0km/h, 3km/h |
| Number of UE antennas | 1R,4R |
| Number of gNB antennas | 1T,2T |
| Tx EVM | 0, 2% |
| Rx EVM | 0, 3% |
| MCS | {256QAM, 682.5}, {256QAM, 711}, {256QAM, 754}, {256QAM, 797}, {256QAM, 841}, {256QAM, 885}, {256QAM, 916}, {256QAM, 948}; {1024QAM, 716.8}, {1024QAM, 758.5}, {1024QAM, 806}, {1024QAM, 853}, {1024QAM, 900.5}. |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 1 |
| Number of scheduled RBs | 273(200 for data) |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Rank | Rank1, Rank 2, |
| Channel estimation | Realistic channel estimation |

  - A.2 Simulation parameters for 1024QAM CQI table and MCS table

### Table A.2	Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency, SCS, System BW | 3.5GHz, 30kHz, 100 MHz |
| Channel model | AWGN |
| Channel coding | LDPC |
| UE speed | 0km/h |
| Number of UE antennas | 1R |
| Number of gNB antennas | 1T |
| Tx EVM | 0% |
| Rx EVM | 0% |
| MCS | 1024QAM CQI table in Table 3 1024QAM MCS table in Table 4 |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 1 |
| Number of scheduled RBs | 273 (200 for data) |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Rank | Rank1, Rank 2, |
| Channel estimation | Realistic channel estimation |

  - A.3 256QAM MCS Table in NR

### Table A.3	NR 256QAM MCS table [3]

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |