3GPP TSG RAN WG1 #104-e                                                            R1-2100567

e-Meeting, January 25th – February 5th, 2021

Source: 	ZTE

Title:	Discussion on UL and DL 16QAM for NB-IoT

Agenda Item:	8.9.1

Document for:	Discussion

- Introduction

In RAN #103 e-meeting, DL and UL 16QAM for NB-IoT was further discussed. The relevant working assumptions and agreements are given as below [1]:

- Working assumptions

- The following TBS indices are introduced for downlink

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS: Support of legacy TBS indices with 16-QAM at least for some deployment modes.

- FFS: Mapping of (a subset of) TBS entries to modulation schemes for different deployment modes.

- FFS for I_SF > 7

- For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16QAM.

- For inband deployments, the downlink TBS entries between 11 (TBS of 2024 for I_SF=7) and [17] are used for 16QAM.

- The following TBS indices are introduced for uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | [2472, 2536] |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | [2536] |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | [2536] |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

- Agreements:

- At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits with ISF=7.

- For in-band deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits (ISF=7).

- Repetitions larger than 2 are not supported in case of 16QAM for downlink

- FFS: Whether repetition of 2 is supported or not

- Different breaking points (QPSK16QAM) are used for standalone/guardband and inband deployments.

- FFS the details of the breaking point.

- Repetitions larger than 2 are not supported in case of 16QAM for downlink

- FFS: Whether repetition of 2 is supported or not

- Explicit or implicit signaling of ratios of NPDSCH EPRE to NRS EPRE for the following cases is supported.

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

In this contribution, we discuss remaining issues for DL and UL 16QAM for NB-IoT.

- DL 16QAM

TBS table

According to the working assumption in last meeting, the new TBS entries are introduced in NB-IoT to support DL 16QAM, as shown in Table 1. Specifically, TBS 14~21 in LTE TBS table are reused for Rel-17 NB-IoT. We propose to confirm this working assumption for DL TBS table. Further, the range of TBS indexes is 0≤ITBS≤21 for guard-band and standalone deployment and 0≤ITBS≤17 for in-band deployment.

Proposal 1: Confirm the working assumption for DL TBS table for NB-IoT 16QAM.

- 0≤ITBS≤21 for guard-band and standalone deployment

- 0≤ITBS≤17 for in-band deployment

Table 1: New TBS entries for DL 16QAM

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

If the number of subframes for NPDSCH is extended to larger than 10, more resource elements can be configured to load a transport block. Thus, the soft buffer size will be increased since it is dependent on the number of resource elements in NB-IoT. Considering that the TBS table in the working assumption can meet DL peak data rate requirement, I_SF > 7 should not be supported for DL TBS Table.

Proposal 2: I_SF > 7 is not supported for DL TBS table for NB-IoT 16QAM.

MCS table

Switching point between QPSK and 16QAM

Guard-band/Standalone

In LTE, the modulation order is switch to 16QAM at TBS 9 (1544 bits for 10PRBs) which has the code rate of round 0.6 for QPSK modulation. And TBS 9 is overlapped in two adjacent MCS entries with QPSK and 16QAM. This is because both AWGN and fading channel are considered for the design of switching point. In AWGN channel, for a TBS, low-order modulation has better performance than high-order modulation. However, in a fading channel, the performance of a MCS with lower modulation order and higher code rate may be worse than that of a MCS with higher modulation order and lower code rate because some deep fading data is more difficult to decode at very high code rate.

In the working assumption, the switching point is set to TBS 14 (2856 bits for I_SF=7) only based on AWGN simulation results for NB-IoT. The code rate of TBS 14 is round 0.9 for QPSK modulation, which is significantly larger than that of LTE switching point. Since the real propagation condition is mainly fading channel, this switching point may not be conducive to NB-IoT transmission performance. In our opinion, the modulation switching point for NB-IoT should follow LTE design principle. That is, the modulation switching point also should be based on fading channel. Figure 1 and 2 show BLER performance for NPDSCH in standalone deployment. It can be observed that:

Observation 1: For NPDSCH with 1T1R antenna, 16QAM performance is about 1.8 dB better than QPSK performance at BLER =0.1 for standalone deployment when TBS 12 (2280 bits with ISF=7) is used. And the performance gap further expands when TBS 13 is used.

Observation 2: For NPDSCH with 2T1R antenna, 16QAM performance is close to 1 dB better than QPSK performance at BLER =0.1 for standalone deployment when TBS 13 (2536 bits with ISF=7) is used.

For these cases, if 16QAM is only applied for the transport blocks larger than or equal to TBS 14, a performance loss will be expected for NPDSCH. Therefore, we propose that 16QAM can be applied for the transport blocks larger than or equal to TBS 12. And overlapped TBS 12 and 13 could be set for MCS entries with QPSK and 16QAM to be used for different configurations and scenarios.

Proposal 3: For standalone and guard-band deployment, 16QAM can be used for TBS indexes from 12 to 21.

Figure 1 BLER performance for NPDSCH with 1T1R in standalone deployment

Figure 2 BLER performance for NPDSCH with 2T1R in standalone deployment

In-band

Since the number of NPDSCH REs on a PRB in in-band deployment is less than that in guard-band/standalone deployment, the modulation switching point for in-band deployment has smaller TB size. Figure 3 shows that DL 16QAM performance is better than QPSK performance at BLER=0.1 when TBS 10 (1736 bits with ISF=7) is used for 1 Tx antenna. Hence, it can be considered that 16QAM can be applied for the transport blocks larger than or equal to TBS 10. And overlapped TBS 10 could be set for MCS entries with QPSK and 16QAM to be used for different configurations and scenarios.

Observation 3: For NPDSCH with 1T1R antenna, 16QAM performance is better than QPSK performance at BLER=0.1 for in-band deployment when TBS 10 (1736 bits with ISF=7) is used.

Proposal 4: For in-band deployment, 16QAM can be used for TBS indexes from 10 to 17.

Figure 3 BLER performance of NPDSCH in in-band deployment

MCS entries

Due to introduction of new TBS indexes, both 4-bit and 5-bit MCS table are feasible and need to be down-selected for DL 16QAM. In last meeting, it is proposed that the MCS field for 16QAM is supported without increasing the DCI size [1].

4-bit MCS

In guard-band/standalone deployment, the 4-bit MCS table cannot contain all DL TBSs since number of MCS entries is limited. So it is required to select 16 TBSs to map into the MCS table. Considering that 16QAM is mainly used to increase the data rate, high SNR region and larger TBS assignment should be the main objective of the 16QAM MCS table. Thus, for higher MCS entries, the continuous and larger TBS indexes can be set in order to ensure the data transmission efficiency under high SNR. And for lower MCS entries, the interval and smaller TBS indexes can be set to guarantee transmission reliability when the channel or SNR deteriorates.

Specifically, based on the current MCS entries of Rel-16 NB-IoT, the 6 MCS entries corresponding to TBS 1, 3, 5, 7, 9 and 11 are removed, and the 8 MCS entries corresponding to TBS 0, 2, 4, 6, 8, 10, 12 and 13 are retained. Meanwhile, new 8 MCS entries corresponding to TBS 14~21 are added in MCS table. Thus, a possible 4-bit MCS table is provided as shown in Table 2.

Table 2: Possible 4-bit MCS table for DL 16QAM

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 2 |
| 2 | 2 | 4 |
| 3 | 2 | 6 |
| 4 | 2 | 8 |
| 5 | 2 | 10 |
| 6 | 4 | 12 |
| 7 | 4 | 13 |
| 8 | 4 | 14 |
| 9 | 4 | 15 |
| 10 | 4 | 16 |
| 11 | 4 | 17 |
| 12 | 4 | 18 |
| 13 | 4 | 19 |
| 14 | 4 | 20 |
| 15 | 4 | 21 |

Proposal 5: If 4-bit MCS table is adopted for DL 16QAM for guard-band and standalone deployment, it should be considered to remove the existing 6 MCS entries and add new 8 MCS entries.

For in-band deployment, the range of available TBS is smaller than that for guard-band/standalone deployment for DL 16QAM. A dedicated 4-bit MCS table can be introduced to support DL 16QAM for in-band deployment. Specifically, it can be considered to select 16 TBSs from TBS 0~17 to map to 4-bit MCS table.

5-bit MCS

The 5-bit MCS table has sufficient MCS entries to cover all TBSs and match detailed channel state. For guard-band/standalone deployment, TBS 0~13 with QPSK can be mapped to MCS 0~13, which is the same as Rel-16 NB-IoT MCS. And TBS 12~21 with 16QAM can be mapped to MCS 14~23. The advantage of this design is that the new MCS table is compatible with all legacy MCS. For Rel-17 NB-IoT UE with 16QAM capability, this MCS table can be used regardless of whether 16QAM is enabled. When 16QAM is not enabled, the default MCS range used for NPDSCH is 0~13 for guard-band/standalone deployment. Moreover, overlapped TBS 12 and 13 for MCS entries with QPSK and 16QAM are beneficial for MCS assignment in different scenarios. The detail of MCS are given in Table 3.

Table 3: Possible 5-bit MCS table for DL 16QAM for guard-band/standalone deployment

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12 | 2 | 12 |
| 13 | 2 | 13 |
| 14 | 4 | 12 |
| 15 | 4 | 13 |
| 16 | 4 | 14 |
| 17 | 4 | 15 |
| 18 | 4 | 16 |
| 19 | 4 | 17 |
| 20 | 4 | 18 |
| 21 | 4 | 19 |
| 22 | 4 | 20 |
| 23 | 4 | 21 |
| 24~31 | reserved | reserved |

Furthermore, if 5-bit MCS table is adopted for DL 16QAM, the definition of MCS field in DCI needs to be studied to avoid increasing the DCI size. It can be considered to utilize ‘repetition number’ field to assist in MCS indication for DL 16QAM. In the existing DCI format N1, the size of ‘repetition number’ field is 4 bits, as shown in Table 4. When DL 16QAM is enabled, the most significant bit of ‘repetition number’ field can be repurposed to indicate MCS since large number of repetitions is usually not configured for NPDSCH. Thus, the bitwidth of MCS indication is extended from 4 bits to 5 bits. Correspondingly, the number of repetitions configured for NPDSCH should be less than or equal to 128 due to the reduced ‘repetition number’ field size when DL 16QAM is enabled.

Proposal 6: If 5-bit MCS table is adopted for DL 16QAM, for guard-band and standalone deployments,

- MCS 0~13 correspond to TBS 0~13 with QPSK modulation

- MCS 14~23 correspond to TBS 12~21 with 16QAM modulation

- The most significant bit of ‘repetition number’ field can be repurposed to indicate MCS

Table 4: Number of repetitions () for NPDSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 8 | 192 |
| 9 | 256 |
| 10 | 384 |
| 11 | 512 |
| 12 | 768 |
| 13 | 1024 |
| 14 | 1536 |
| 15 | 2048 |

For in-band deployment, the same design principle is used for 5-bit MCS table and MCS indication. In this table, TBS 0~10 with QPSK can be mapped to MCS 0~10, which is the same as Rel-16 NB-IoT MCS. And TBS 10~17 with 16QAM can be mapped to MCS 11~18.

Proposal 7: If 5-bit MCS table is adopted for DL 16QAM, for in-band deployment,

- MCS 0~10 correspond to TBS 0~10 with QPSK modulation

- MCS 11~18 correspond to TBS 10~17 with 16QAM modulation

Repetitions for DL 16QAM

According to the agreements, repetitions larger than 2 are not supported in case of 16QAM for downlink. In our view, whether repetition of 2 is supported or not could investigate DL data rate under different number of repetitions and modulation schemes. From Table 5, it can be observed that 16QAM with two repetitions can still provide higher DL peak data rate compared to Rel-16. For example, for a channel state which matches the configuration with TBS=4968 bits, 16QAM and two repetitions, if two repetitions are not supported for 16QAM, QPSK with TBS= 2536 bits may be used for NPDSCH. This implies a performance loss on data rate. Therefore, repetition of 2 could be supported for DL 16QAM.

Proposal 8: Repetition of 2 could be supported for DL 16QAM.

Table 5: DL peak data rate for 2 progresses

| Deployment scenario | Modulation order and Transport block size | Number of repetitions | Period of 2-progress transmission (ms) | Data rate (kbps) |
| --- | --- | --- | --- | --- |
| Guard-band/ Standalone | Rel-16, QPSK, 2536 | 1 | 40 | 126.8 |
|  | 16QAM, 4968 | 2 | 60 | 165.6 |
| In-band | Rel-16, QPSK, 1736 | 1 | 40 | 86.8 |
|  | 16QAM, 3624 | 2 | 60 | 120.8 |

UL 16QAM

Subcarrier allocation for UL 16QAM

In aspect of subcarrier allocation, it is agreed that at least multi-tone transmission with 12 subcarriers can use 16QAM and that with 3 and 6 subcarriers are FFS. In our opinion, since 3 and 6 subcarriers allocation have the same number of NPUSCH REs as 12 subcarriers allocation for a transport block, 3 and 6 subcarriers allocation can also pursue high data rate. Hence, 16QAM should be supported for multi-subcarrier with 3 and 6 subcarriers.

Proposal 9: UL 16QAM should be supported for 3/6 subcarriers allocation.

TBS table

Based on working assumption, the extended TBS indices for uplink 16QAM are introduced in Table 6. We can accept this TBS table to support UL 16QAM.

Table 6: Extended TBS for UL 16QAM

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

MCS table

Switching point between QPSK and 16QAM

To determine UL modulation switching point, NPUSCH performance for different modulation orders and TBS are evaluated under fading channel and the results are given in Figure 4. The results show 16QAM performance is significantly better than QPSK performance at TBS 13 (2536 bits with IRU=7). Hence, 16QAM can be applied for the transport blocks larger than or equal to TBS 13 for NPUSCH. And overlapped TBS 13 could be set for MCS entries with QPSK and 16QAM.

Observation 4: For NPUSCH with 12 subcarriers, 16QAM performance is significantly better than QPSK performance at TBS 13 (2536 bits with IRU=7).

Proposal 10: For NPUSCH, 16QAM can be used for TBS indexes from 13 to 21.

Figure 4 BLER performance for NPUSCH with 1T2R

MCS entries

Because UL TBS range is extended to 0~21, a 5-bit MCS table is proposed for UL 16QAM. It can be considered to use the same design principle as DL 5-bit 16QAM MCS table. That is, UL 16QAM MCS table is compatible for all the legacy MCS. Specifically, MCS 0~13 correspond to TBS 0~13 with QPSK modulation. And MCS 14~22 correspond to TBS 13~21 with 16QAM modulation. The detail of MCS are given in Table 7.

Proposal 11: 5-bit MCS table can be defined for UL 16QAM for NB-IoT.

- MCS 0~13 correspond to TBS 0~13 with QPSK

- MCS 14~22 correspond to TBS 13~21 with 16QAM

Table 7: Possible 5-bit MCS table for UL 16QAM

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12 | 2 | 12 |
| 13 | 2 | 13 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 4 | 16 |
| 18 | 4 | 17 |
| 19 | 4 | 18 |
| 20 | 4 | 19 |
| 21 | 4 | 20 |
| 22 | 4 | 21 |
| 23~31 | reserved | reserved |

DL Power allocation

In LTE/eMTC, downlink power allocation is performed depending on the ρA and ρB, where ρA represents the ratio of PDSCH EPRE and CRS EPRE on CRS OFDM symbols and ρB represents the ratio of PDSCH EPRE and CRS EPRE on non-CRS OFDM symbols. Through ρA and ρB, eNB can adjust transmit power between different OFDM symbols. However, in practice, the same total transmit power is usually configured for different OFDM symbols since different transmit power between OFDM symbols may cause phase diversion.

Observation 5: In practice, different transmit power configured between OFDM symbols may cause phase diversion.

Also, Rel-16 NB-IoT uses the same transmit power between different OFDM symbols when a given NRS power is configured, as shown in following specifications.

A UE may assume the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports.

Observation 6: The transmit power between different OFDM symbols is constant in R16 NB-IoT when a given NRS power is configured.

Therefore, for Rel-17 NB-IoT DL power allocation, the same total transmit power could be defined for each OFDM symbol to reduce complexity.

That is, the following three symbols have the same total transmit power.

- Symbol A: NPDSCH in symbols without NRS and CRS

- Symbol B: NPDSCH in symbols with CRS ( only for “In-band” deployment)

- Symbol C: NPDSCH in symbols with NRS

Proposal 12: The same total transmit power is set for each OFDM symbol for Rel-17 NB-IoT DL power allocation.

For Symbol C, a new UE-specific higher layer parameter Pn can be introduced to indicate the ratio of NPDSCH EPRE and NRS EPRE for symbol C to achieve UE-specific DL power allocation. Since NRS EPRE is configured by the existing cell-specific higher layer parameter, NPDSCH EPRE in Symbol C can be determined. Furthermore, the transmit power of entire Symbol C can be determined.

Proposal 13: A new UE-specific higher layer parameter is introduced to indicate the ratio of NPDSCH EPRE and NRS EPRE in symbols with NRS.

For Symbol A, because Symbol A, B and C have the same total transmit power, NPDSCH EPRE in Symbol A can be determined.

For Symbol B, the ratio of NRS EPRE and CRS EPRE is configured by the existing higher layer parameter nrs-CRS-PowerOffset in SIB1. According to CRS EPRE, NPDSCH EPRE in Symbol B can be determined since Symbol A, B and C have the same total transmit power. Wherein, the ratio of NRS EPRE to CRS EPRE is indicated by nrs-CRS-PowerOffset only when same PCI is configured in the existing specifications. For Rel-17 NB-IoT DL power allocation, it can be supported to indicate the ratio of NRS EPRE to CRS EPRE in in-band deployment for both same PCI and different PCI.

Proposal 14: For in-band deployment, the ratio of NRS EPRE to CRS EPRE is indicated for both same PCI and different PCI.

Others

DL 16QAM introduces larger TBS, so the new UE buffer size needs to be defined. In Rel-16 NB-IoT, the downlink physical layer parameter values of UE category are shown in Table 4.1C-1 of TS 36.306.

Table 4.1C-1: Downlink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits |
| --- | --- | --- | --- |
| Category NB1（R13） | 680 | 680 | 2112 |
| Category NB2 （R14） | 2536 | 2536 | 6400 |

In NB-IoT, DL soft buffer size is calculated based on the number of NPDSCH REs, the maximum modulation order and the number of HARQ processes. Since the maximum modulation order is increased from 2 to 4, new DL soft buffer size is doubled compared to Rel-16, i.e. 12800 bits

Proposal 15: DL soft buffer size of 12800 bits is specified for NB-IoT 16QAM.

- Conclusion

In this contribution, we have discussed the issues related to DL and UL 16QAM for NB-IoT. And the following observations and proposals are given:

Observation 1: For NPDSCH with 1T1R antenna, 16QAM performance is about 1.8 dB better than QPSK performance at BLER =0.1 for standalone deployment when TBS 12 (2280 bits with ISF=7) is used. And the performance gap further expands when TBS 13 is used.

Observation 2: For NPDSCH with 2T1R antenna, 16QAM performance is close to 1 dB better than QPSK performance at BLER =0.1 for standalone deployment when TBS 13 (2536 bits with ISF=7) is used.

Observation 3: For NPDSCH with 1T1R antenna, 16QAM performance is better than QPSK performance at BLER=0.1 for in-band deployment when TBS 10 (1736 bits with ISF=7) is used.

Observation 4: For NPUSCH with 12 subcarriers, 16QAM performance is significantly better than QPSK performance at TBS 13 (2536 bits with IRU=7).

Observation 5: In practice, different transmit power configured between OFDM symbols may cause phase diversion.

Observation 6: The transmit power between different OFDM symbols is constant in R16 NB-IoT when a given NRS power is configured.

Proposal 1: Confirm the working assumption for DL TBS table for NB-IoT 16QAM.

- 0≤ITBS≤21 for guard-band and standalone deployment

- 0≤ITBS≤17 for in-band deployment

Proposal 2: I_SF > 7 is not supported for DL TBS table for NB-IoT 16QAM.

Proposal 3: For standalone and guard-band deployment, 16QAM can be used for TBS indexes from 12 to 21.

Proposal 4: For in-band deployment, 16QAM can be used for TBS indexes from 10 to 17.

Proposal 5: If 4-bit MCS table is adopted for DL 16QAM for guard-band and standalone deployment, it should be considered to remove the existing 6 MCS entries and add new 8 MCS entries.

Proposal 6: If 5-bit MCS table is adopted for DL 16QAM, for guard-band and standalone deployments,

- MCS 0~13 correspond to TBS 0~13 with QPSK modulation

- MCS 14~23 correspond to TBS 12~21 with 16QAM modulation

- The most significant bit of ‘repetition number’ field can be repurposed to indicate MCS

Proposal 7: If 5-bit MCS table is adopted for DL 16QAM, for in-band deployment,

- MCS 0~10 correspond to TBS 0~10 with QPSK modulation

- MCS 11~18 correspond to TBS 10~17 with 16QAM modulation

Proposal 8: Repetition of 2 could be supported for DL 16QAM.

Proposal 9: UL 16QAM should be supported for 3/6 subcarriers allocation.

Proposal 10: For NPUSCH, 16QAM can be used for TBS indexes from 13 to 21.

Proposal 11: 5-bit MCS table can be defined for UL 16QAM for NB-IoT.

- MCS 0~13 correspond to TBS 0~13 with QPSK

- MCS 14~22 correspond to TBS 13~21 with 16QAM

Proposal 12: The same total transmit power is set for each OFDM symbol for Rel-17 NB-IoT DL power allocation.

Proposal 13: A new UE-specific higher layer parameter is introduced to indicate the ratio of NPDSCH EPRE and NRS EPRE in symbols with NRS.

Proposal 14: For in-band deployment, the ratio of NRS EPRE to CRS EPRE is indicated for both same PCI and different PCI.

Proposal 15: DL soft buffer size of 12800 bits is specified for NB-IoT 16QAM.

- Reference

- R1-2009730, Feature lead summary #3 on [103-e-LTE-Rel17_NB_IoT_eMTC-01], Moderator (Huawei) , RAN1 #103-e