3GPP TSG RAN WG1 #104-e                                                            R1-2100569

e-Meeting, January 25th – February 5th, 2021

Source:              ZTE

Title:                   DL TBS increase for eMTC

Agenda Item:     8.9.3

Document for:   Discussion

- Introduction

In RAN #88 e-meeting, the revised WID on additional enhancements for NB-IoT and LTE-MTC was agreed [1]. One of the objectives is to add a Rel-17 optional UE capability to support a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A. In this contribution, we provide our considerations on DL TBS of 1736 bits.

- Discussion

Rel-16 DL MCS and TBS

If the UE is not configured with higher layer parameter ce-PDSCH-64QAM-Config-r15, DL modulation scheme supports up to 16QAM in CE mode A. A 4-bit "modulation and coding scheme" field is used in DCI for PDSCH scheduling and the MCS index indicated by this field is less than or equal to 15 based on Table 7.1.7.1-1 in TS36.213 [2].

Table 7.1.7.1-1: Modulation and TBS index table for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 1 |
| 2 | 2 | 2 | 2 |
| 3 | 2 | 2 | 3 |
| 4 | 2 | 2 | 4 |
| 5 | 2 | 4 | 5 |
| 6 | 2 | 4 | 6 |
| 7 | 2 | 4 | 7 |
| 8 | 2 | 4 | 8 |
| 9 | 2 | 4 | 9 |
| 10 | 4 | 6 | 9 |
| 11 | 4 | 6 | 10 |
| 12 | 4 | 6 | 11 |
| 13 | 4 | 6 | 12 |
| 14 | 4 | 6 | 13 |
| 15 | 4 | 6 | 14 |
| 16 | 4 | 6 | 15 |
| 17 | 6 | 6 | 15 |
| 18 | 6 | 6 | 16 |
| 19 | 6 | 6 | 17 |
| 20 | 6 | 6 | 18 |
| 21 | 6 | 6 | 19 |
| 22 | 6 | 6 | 20 |
| 23 | 6 | 6 | 21 |
| 24 | 6 | 6 | 22 |
| 25 | 6 | 6 | 23 |
| 26 | 6 | 6 | 24 |
| 27 | 6 | 6 | 25 |
| 28 | 6 | 6 | 26/26A |
| 29 | 2 | 2 | reserved |
| 30 | 4 | 4 |  |
| 31 | 6 | 6 |  |

Thus, it can be observed that the TBS range for the BL/CE UE configured with up to 16QAM is TBS 0 to 14 which are shown in Table 7.1.7.2.1-1 in TS36.213 [2].

Table 7.1.7.2.1-1: Transport block size table for up to 16QAM (6 PRBs)

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 |
| 13 | 224 | 488 | 744 | 1000 | 1256 | 1544 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 |

However, the transport block size lager than 1000 bits in Table 7.1.7.2.1-1 cannot be used for UE category M1 since Table 4.1A-1 restricts the DL maximum TBS of UE category M1 to 1000 bits in existing TS36.306 [3].

Table 4.1A-1: Downlink physical layer parameter values set by the field ue-CategoryDL

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category M1 | 1000 | 1000 | 25344 | 1 |

If the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15, DL modulation scheme supports up to 64QAM in CE mode A. A 5-bit "modulation and coding scheme" field is used in DCI for PDSCH scheduling. The MCS for PDSCH is indicated based on Table 7.1.7.1-1 in TS36.213 [2]. But the maximum DL TBS is also restricted to 1000 bits.

Clarification of TBS range

For the range of available TBS, it needs to be clarified which one of the following options is applicable to support a maximum DL TBS of 1736 bits.

- Option 1: DL TBS less than or equal to 1736 bits can be used for each NPRB

- Option 2: DL TBS less than or equal to TBS14 can be used for each NPRB

If DL modulation scheme supports up to 16QAM in CE mode A, for Option 1, TBS table, MCS table and CQI table need to be redesigned since additional TBS and MCS entries are introduced for 4-bit modulation and coding schemes. For Option 2, the current TBS table, MCS table, CQI table and DCI indication are not affected. According to the objective of WI, Option 2 should be adopted.

Proposal 1: For DL TBS increase for 16QAM, TBS less than or equal to TBS14 can be used for each NPRB.

If DL modulation scheme supports up to 64QAM in CE mode A, it needs to be confirmed whether 64QAM and maximum DL TBS of 1736 bits can be configured together. In our opinion, it is beneficial that 64QAM is configured with 1736 bits to achieve higher DL data rate. Moreover, the current TBS table, MCS table, CQI table and DCI indication are not affected since the existing 64QAM MCS table support 5-bit modulation and coding schemes. So Option 1 should be adopted.

Proposal 2: For DL TBS increase for 64QAM, TBS less than or equal to 1736 bits can be used for each NPRB.

TBS restriction in 36.213

Maximum DL TBS of 1736 bits can be supported for HD-FDD Cat. M1 UEs in CE mode A. Then, some descriptions in TS36.213 need to be modified. In particular, for the UE configured with 6PRB PDSCH bandwidth, TS36.213 has following specifications:

-	if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the MPDCCH DCI format 6-1A is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1,

-	set  to the TBS determined by the procedure in Subclause 7.1.7.2.1,

-	TBS=minTBS',1000

-	otherwise

-	TBS is determined by the procedure in Subclause 7.1.7.2.1

Wherein, the description of “TBS=minTBS',1000” can be change to “TBS=minTBS',1736 when maximum DL TBS of 1736 bits is configured and TBS=minTBS',1000 otherwise.”

Proposal 3: In TS36.213, TBS restriction needs to be modified to 1736 if maximum DL TBS of 1736 bits is configured.

Soft buffer size

The motivation of DL TBS increase is mainly to improve data rate and spectral efficiency for CE mode A. In the current specifications, the features that affect DL spectral efficiency for CE mode A include:

- Number of HARQ processes: 8/10/14 HARQ processes

- Modulation scheme: 16QAM/64QAM

- Multi-TB scheduling

- The maximum TBS of 1736 bits

Wherein, the number of HARQ processes and the maximum transport block size impact DL soft buffer size for category M1 UEs. The existing soft buffer size is calculated based on the maximum TBS of 1000 bits and 8 HARQ processes. Though 10 HARQ processes are introduced in Rel-14, the soft buffer size is not increased to avoid hardware updates. Thus, considering UE cost and complexity, new soft buffer size should be determined according to low UE capability requirement for category M1 UEs. So it may be defined based on the maximum TBS of 1736 bits and 8 HARQ processes in CE mode A. And the detail is shown as below.

8∙32∙3∙1736+24+432=43008 soft channel bits

Proposal 4: New soft buffer size may be defined based on the maximum TBS of 1736 bits and 8 HARQ processes.

- Conclusion

In this contribution, we discuss DL TBS increase for Cat. M1 UEs in CE mode A. And the relevant proposals are given as below:

Proposal 1: For DL TBS increase for 16QAM, TBS less than or equal to TBS14 can be used for each NPRB.

Proposal 2: For DL TBS increase for 64QAM, TBS less than or equal to 1736 bits can be used for each NPRB.

Proposal 3: In TS36.213, TBS restriction needs to be modified to 1736 if maximum DL TBS of 1736 bits is configured.

Proposal 4: New soft buffer size may be defined based on the maximum TBS of 1736 bits and 8 HARQ processes.

- Reference

[1] 3GPP, RP-201306, WID revision: Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN #88e

[2] TS36.213 V16.1.0

[3] TS36.306 V16.1.0