3GPP TSG RAN WG1 Meeting #104-E															R1-2100629

e-Meeting, January 25th – February 5th, 2020

Source:	Intel Corporation

Title:	Correction to FD-OCC for PSCCH

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This contribution presents a correction for PHY structure of Release 16 NR V2X. Corrections to other selected aspects of Release 16 NR V2X are presented in [2] and [3].

PSCCH DMRS

During 3GPP RAN1 meeting #101-E the working assumption was made:

Decision: As per decisions from GTW on June 4th, Working assumption: - The frequency-domain OCC length for PSCCH is {2, 3, 4} - The same LTE requirement and procedure for UE blind decoding (w.r.t. to OCC vs. LTE’s cyclic shifts) for PSCCH applies

However, the current definition in clause 8.4.1.3.2 of [1] in our opinion does not result in a FD-OCC but rather a cyclic shift. For FD-OCC with OCC length 3 one element in the sequence rlm does need to be spread to 3 consecutive PSCCH DMRS REs in frequency direction. As there are 3 PSCCH DMRS REs per PRB this means each PSCCH DMRS symbol in one PRB needs to be generated from one element in rlm. In our opinion this is currently not the case as the element taken from the sequence rlm is different for each PSCCH DMRS RE. As a coincidence the combination of the phase shifts in different PRBs would be a cyclic shift as applied to the LTE DMRS. However, we think as the given working assumption was never revoked it should be reflected in the specification. This can be achieved by changing the index of the reference symbol sequence in the PSCCH DMRS mapping as in the following TP.

  - Adopt the following TP to align PSCCH DMRS definition with RAN1 agreement

| Title: | Correction to PSCCH DMRS Mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification for PSCCH DMRS does not follow FD-OCC agreement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct indexing of reference symbol sequence resulting in FD-OCC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different PSCCH antenna ports are orthogonalized via CS not FD-OCC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------begin text proposal for 38.211, section 8.4.1.3.2 -------------------------------------------------

##### 8.4.1.3.2	Mapping to physical resources

The sequence rlm shall be multiplied with the amplitude scaling factor βDMRSPSCCH in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with rl0 to resource elements k,lp,μ in a slot on antenna port p=2000 according to

ak,l(p,μ)=βDMRSPSCCHwf,i(k')rln3n+k'
k=nNscRB+4k'+1
k'=0,1,2
n=0,1,…

<<<unchanged text omitted>>>

----------------------------------------------------end text proposal for 38.211 -----------------------------------------------------------

Conclusions

In this contribution, we provided views on the issue of FD-OCC for PSCCH. Based on discussions, we propose a TP to correct current specification.

References

- 3GPP TS 38.211, “Physical channels and modulations”, version 16.4.0, December 2020.

- R1-2100630, “Corrections to Mode-2 resource allocation”, Intel Corporation, e-Meeting, January 2021.

- R1-2100631, “Corrections to sidelink procedures”, Intel Corporation, e-Meeting, January 2021.