3GPP TSG RAN WG1 #104-E			     R1-2100630

e-Meeting, January 25th – February 5th, 2021

Source:	Intel Corporation

Title:	Corrections to Mode-2 resource allocation

Agenda item:	7.2.4

Document for:	Discussion and Decision

- Introduction

This contribution is a part of Rel.16 maintenance work and it presents a text proposal to implement one RAN1#103-e agreement on Mode-2. Other corrections to R16 NR V2X are presented in [1]-[2].

- Discussion

At the last meeting, the following late agreement was made which needs to be implemented in specifications:

Agreements: - When a UE checks pre-emption for a resource, the UE identifies a candidate resource set based on steps 1-7 in clause 8.1.4 TS 38.214 - After the candidate set is identified, the UE checks SL-RSRP measurement and priority condition as per agreements, for resource(s) {r’} subject to pre-emption overlapping with received SCI 1-A and not included in the candidate set, where the RSRP threshold is the final threshold after executing steps 1-7 i.e. includes all necessary increments for reaching X%.

It was already discussed in RAN1#103-e that the agreement may be implemented by letting UE to check the RSRP threshold for a resource which is even not in the identified resource set. The text proposal for this agreement is presented in section 3.

- Text Proposal

| Title: | Correction to RSRP threshold checking procedure with pre-emption enabled |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Implementation of RAN1#103-e agreement to correct PHY procedure of candidate resource set identification for the purpose of pre-emption checking |  |  |  |  |  |  |  |  |
| Summary of change: |  | The condition for pre-emption triggering has been revised so that RSRP can be checed even for a resource not in the identified resource set |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Pre-emption could not be triggered even in cases it should be triggered, if periodic reservatino is in use |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <<< Unchanged parts omitted >>> If a resource ri' from the set (r0',r1',r2',…) - is not a member of SA, and - if the RSRP measurement performed according to clause 8.4.2.1 for a received SCI format 1-A overlapped with the resource ri' is higher than ThprioRX,prioTX including all increments after execution of steps 1-7 above, and - if an associated priority prioRX, satisfy one of the following conditions, then the UE shall report pre-emption of the resource ri' to higher layers. - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX <<< Unchanged parts omitted >>>

- Conclusions

In this contribution, we provided a text proposal implementing a RAN1#103-e agreement and have the following proposal:

- Adopt the text proposal in section 3 of this document implementing RAN1#103-e agreement which fixes pre-emption checking for periodic reservations enabled

- References

- R1-2100629 “Correction to FD-OCC for PSCCH”, Intel Corporation, e-Meeting, January 2021.

- R1-2100631, “Corrections to sidelink procedures”, Intel Corporation, e-Meeting, January 2021.