3GPP TSG RAN WG1 #104-e                                                              	         R1-2100681

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.14.2

Source: 	Intel Corporation

Title:	On evaluation methodology for XR

Document for: 	Discussion/Decision

# Introduction

In this contribution we provide our views on the evaluation methodology and KPIs for capacity, power and mobility aspects for XR SID.

# Capacity

## Evaluation methodology

In terms of capacity evaluation most of the agreeable parameters have been raised in RAN1#103-e and the agreements are summarized here:

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor hotspot FR1/FR2 | Dense urban FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound ISD: 200m |
| UE distribution |  | FR1 : 80% indoor, 20% outdoor FR2 : 100% outdoor |
| Carrier frequency | FR1: 4 GHz FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT = 1.5m |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic FFS: Ideal (optional) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| BS Antenna parameters | FR1 : 32TxRU,(M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ FR2 : 2TxRU,(M, N, P, Mg, Ng; Mp, Np) = (16,8,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ | FFS FR1 : • Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) • Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) • Option 3: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4) (dH, dV) = (0.5λ, 0.5λ) FR2 : 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx Power | FR1 : 24dBm per 20 MHz FR2 : 23dBm per 80MHz | FR1: 44dBm per 20 MHz FR2 : 40dBm per 80MHz |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |
| UE Antenna parameters | FR1 : Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ FR1 : Optional : 4T/4R, 1T/2R, 2T2R FR2 : FFS (downselect) FR2: Opt1 : (M,N,P)=(1,4,2), 3 panels (left, right,top), (Mp,Np) upto company FR2 : Opt2 : 4Tx/4Rx : (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ |  |
| UE max Tx power | FR1 : 23dBM FR2 : 23dBm (max EIRP 43dBm) |  |
| Downtilt | 90 degrees (pointing to ground) | FFS:6 or 12 degree |
| TDD configuration | FR1: Opt1: DDDSU Opt2: DDDUU FR2 : Opt1 : DDDSU |  |
| System BW | FR1 : Baseline-100MHz, Optional -20/40MHZ(FFS:200MHZ) FR2: FFS |  |
| Power Control Param | Companies to report |  |
| Transmission Scheme | Companies to report |  |
| Scheduler | SU/MU-Mimo PF Scheduler (Company to report SU or MU) Other scheduler up to company |  |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, Assumptions on SRS: periodicity, processing gain, processing delay, etc |  |
| PHY Processing delay | Baseline: UE PDSCH processing capability #1 Optional : UE PDSCH processing capability #2 Companies to report processing delay |  |
| PDCCH overhead, DMRS overhead, Target BLER, Max HARQ transmission | Companies to report |  |

It Dense Urban scenario, indoor UEs are generally considered to be evenly distributed in different floors of a building (buildings ranging in height from 4 floors to 8 floors) while the RAN1#103-e agreement seems to suggest that UE height is fixed at 1.5m.

Proposal-1: Clarify that the UE height for indoor UEs is according to typical RAN1 assumption where UEs are uniformly distributed in different floors of buildings in Dense Urban scenario.

## Key performance indicators

KPI for capacity evaluation comprises of the following:

Average data-rate requirement

Media traffic in XR has wide variations depending on the particular use-case. In the uplink pose and controller information can be implemented in aperiodic manner as well. Media traffic can also be encoded with adaptive bit-rate encoders. For simplicity of modeling in RAN1, particularly for short time durations, average bit-rate (averaged over simulation time) can be a requirement and a key performance indicator. If we consider simple (no memory) statistical traffic models (similar to FTP Models 1-3) it is analogous to average file-size times arrival rate.

Packet delay statistics and Packet delay budget (PDB)

Packet delay can be defined roughly as the time interval between packet arrival at L1 at the transmitter to successful reception at the receiver including HARQ re-transmissions (if any). Various components of packet delay includes processing time at the transmitter, PDCCH alignment delay, processing time at the receiver, alignment delay for ACK/NACK etc. as studied in [1].  Packet delay budget or air-interface latency can be defined as the maximum packet delay (for each and every packet) that can be tolerated for the given service. Consequently packet delay statistics for a given UE may not be averaged over simulation time.

Figure 1: definition of packet delay

Average packet error rate (PER) statistics and reliability requirement

Average packet error rate (PER) can be defined as the ratio of packets that are not received successfully at the receiver within the PDB – such a ratio can be determined by considering the entire simulation time for each UE. Reliability requirement translates to a threshold for maximum PER.

User satisfaction ratio

The fraction of active UEs in a cell that can satisfy an average bit-rate requirement, an average PER requirement and a PDB - all at the same time can be considered as the user satisfaction ratio for capacity evaluations.

Proposal-2: Consider defining the following KPIs for capacity evaluations:

  - Average data-rate requirement

  - Packet delay statistics and Packet delay budget (PDB)

  - Average packet error rate (PER) statistics and reliability requirement

  - User satisfaction ratio

# Power

## Evaluation methodology

UE power savings mechanisms were studied for connected mode UEs in Release 16 NR [6]. As a part of the study a detailed system level evaluation methodology was adopted where a UE power consumption model was agreed for smartphone devices operating in FR1 and FR2. Based on the UE power consumption model, RAN1 evaluated UE power saving gains using a baseline assumption for C-DRX for different traffic models namely FTP Model 3, Web-browsing, video streaming, instant messaging, VoIP, gaming and background app sync. Different eMBB deployment scenarios were also considered e.g., Dense Urban, Indoor Hotspot and Rural Macro. Given the diverse traffic types and deployments considered in [6], a similar EVM may be adopted for NR XR study.

An example of the UE power consumption model is shown in the following table.

UE power consumption model for FR1

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

As can be observed, the power consumption model is defined in terms of relative power and based on the state of UE in every slot. For the SLS evaluation a value from the table above can be chosen to represent the power consumption in a given slot. At the end of the evaluation, the total consumed power by each UE can be evaluated across the simulated subframes. However, the values in [6] are relevant to smartphone class devices and would need to be updated to match the assumptions for a wearable device using NR XR.

Proposal-3: Update UE power consumption model from Rel-16 UE power saving study [6] to reflect NR XR device cases.

Additionally, most of the traffic models used in [6] are derived from [7] which are possibly outdated with respect to traffic types relevant to XR. Therefore, updated traffic models under discussion in NR XR should be adopted. To this end, it may be necessary to re-define the baseline C-DRX assumptions to evaluate power savings.

Proposal-4: Use new XR traffic models and define related baseline C-DRX parameters for UE power saving evaluations.

## Key performance indicators

The KPIs for evaluation comprises of the following:

UE Power Saving Gain

The percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme

System Performance

The system performance parameters should be similar to the KPIs introduced for Capacity evaluation. For a given power saving scheme, the KPIs for capacity evaluation should be considered as a trade-off for power saving gain.

Proposal-5: Consider defining the following KPIs for power consumption evaluations:

  - Power saving gain compared to a baseline scheme

  - KPIs for capacity evaluation for a power saving scheme

To enable consistent evaluations, initial calibration may be performed among interested companies based on a set of agreed evaluation assumptions including power consumption model and C-DRX settings.

# Mobility

## Evaluation methodology

In Release-17 NR feMIMO WI, system level mobility evaluation assumptions were agreed in [8]. Two types of mobility evaluations were considered i.e., intra-cell mobility where the UE moves within a given cell; and inter-cell mobility, where the UE can cross the cell boundary triggering a handover. Assumptions for modelling inter-cell handover at a system level were also agreed in [8]. The assumptions are summarized in the following tables

Baseline assumptions for SLS: Intra-cell mobility scenarios

| Parameters | Values |
| --- | --- |
| Scenarios | High speed @FR2: - Dense Urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 100% outdoor - One UE is dropped for each of the 21 sectors/cells (see mobility description below) |
| UE Speed | For Dense Urban: 60 km/hr and 120 km/hr |
| UE Mobility and trajectory handling | Linear trajectory, intra-cell mobility (constrained within one cell) - Trajectory sampling at most spaced by decorrelation distance Dense Urban: For each of the 21 cells: One UE is dropped as follows: For the upper right sector/cell (can be extended analogously to the upper left and lower sectors/cells, see Appendix B) with d=30m, the UE starts at P and moves along the 120-deg line downward to Q Each sector is a cell and that the cell association for intra-cell mobility is geographic and not RSRP based. Note: Optionally, if for some reason a company would like to simulate only one cell/sector, the company should clearly state this in the contribution, including the assumed interference model. |
| UE and panel orientation | Companies will describe the assumed UE and panel orientation relative to the direction of the trajectory Note: When the UE orientation is deemed fixed - For dense urban, the three panels located facing the right, left and to the front of the direction of motion tend to result in maximum signal reception. |

Baseline assumptions for SLS: Inter-cell mobility scenarios

| Parameters | Values |
| --- | --- |
| Scenarios | High speed @FR2: - Dense Urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 100% outdoor - One UE is dropped on the entire network of 21-cell/sector topography (see mobility description below) |
| UE Mobility and trajectory handling | Linear trajectory, inter-cell mobility - Trajectory sampling at most spaced by decorrelation distance Dense Urban: Here X (in meter) is a uniformly distributed random variable U[26,34]. One UE is dropped and starts at P and moves along the 120-deg line downward to Q. Each sector is a cell and that the cell association for inter-cell mobility is L1-RSRP based. Inter-cell interference is modeled by assuming the transmission of random beams with random precoders (assuming Type I codebook) from the non-associated cells. Note: Optionally, if for some reason a company would like to simulate only a portion of the trajectory with at least one cell boundary crossing, the company should clearly state this in the contribution. |
| UE and panel orientation | Just as for intra-cell mobility, companies will describe the assumed UE and panel orientation relative to the direction of the trajectory. Note: When the UE orientation is deemed fixed - For dense urban, the three panels located facing the right, left and to the front of the direction of motion tend to result in maximum signal reception. |
| Modeling for Rel.15 L3-based handover (HO) | Handover (HO) timeline can be summarized as follows: - The UE detects that a target becomes better than source, with an additional offset from the HO margin (e.g. 3dB) - After U[1.0s,1.5s], the UE stops communicating with source - After another 80ms interruption, the UE starts communicating with target. It is assumed that the best TX/RX beam is used at this point. The HO latency (the time measured from the initiation and completion of the UE association with the new destination cell) is modeled as a uniformly distributed random variable of U[1.0 sec,1.5 sec]. That is, when the L1-RSRP for the target cell is larger than the L1-RSRP for the source cell by the HO margin (e.g. 3 dB), handover is initiated. Handover to the target cell is completed after HO latency. - To aid comparison, companies should state the assumed HO margin. Additional elements such as potential RLF due to hysteresis, additional latency due to potential ping-pong effect, additional latency due to DRX assumption (as opposed to non-DRX assumed above) –if added by companies– need to be described and justified in the contribution. |

The mobility evaluation assumptions can be adopted from the above tables. Mobility events and their impact on XR performance can be further evaluated. For intra-cell mobility, the MIMO model can be updated to include multiple UEs moving in straight line trajectories and MU-MIMO performance under such mobility can be evaluated. For inter-cell case, the handover modelling can be further refined and the impact of such handovers to XR KPIs can be considered.

Proposal-6: For impact of mobility events on XR performance, the L1-mobility EVM from Release-17 MIMO can be adopted as a baseline.

## Key performance indicators

The KPIs specified in NR MIMO [6] can be considered in addition to the capacity evaluation KPIs

| Performance metrics | - CDF of UE throughput, avg. and 5% UE throughput - TCI state update (beam indication) signaling overhead (separate analysis from SLS) - Beam switching latency (only for FR2) - RSRP distribution |
| --- | --- |

Proposal-7: Consider defining the following KPIs for mobility evaluations:

  - KPIs for capacity evaluation

  - RSRP Distribution

  - Beam Switching latency (FR2)

# Conclusions

In this paper, EVM for NR XR study was discussed and the following proposals were made:

- Proposal-1: Clarify that the UE height for indoor UEs is according to typical RAN1 assumption where UEs are uniformly distributed in different floors of buildings in Dense Urban scenario.

- Proposal-2: Consider defining the following KPIs for capacity evaluations:

  - Average data-rate requirement

  - Packet delay statistics and Packet delay budget (PDB)

  - Average packet error rate (PER) statistics and reliability requirement

  - User satisfaction ratio

- Proposal-3: Update UE power consumption model from Rel-16 UE power saving study [6] to reflect NR XR device cases.

- Proposal-4: Use new XR traffic models and define related baseline C-DRX parameters for UE power saving evaluations.

- Proposal-5: Consider defining the following KPIs for power consumption evaluations:

  - Power saving gain compared to a baseline scheme

  - KPIs for capacity evaluation for a power saving scheme

- Proposal-6: For impact of mobility events on XR performance, the L1-mobility EVM from Release-17 MIMO can be adopted as a baseline.

- Proposal-7: Consider defining the following KPIs for mobility evaluations:

  - KPIs for capacity evaluation

  - RSRP Distribution

  - Beam Switching latency (FR2)

# References

| [1] | 3GPP RAN1, "TR 38.824, Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)," 27 03 2019. [Online]. |
| --- | --- |
| [2] | Qualcomm, "S4-201399, FS_XR Traffic: Permanent document v0.3.1," [Online]. |
| [3] | 3GPP SA4, "TR 22.842, Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS)," 20 12 2019. [Online]. |
| [4] | 3GPP SA4, "TR 22.925, Quality of Service (QoS) and network performance 3.1.1," 23 04 1999. [Online]. |
| [5] [6] [7] [8] | 3GPP SA4, "TR26.928, Extended Reality (XR) in 5G 16.1.0," 23 12 2020. [Online]. 3GPP RAN1 TR 38.840, " Study on User Equipment (UE) power saving in NR (Release 16)" R1-070674, "LTE Physical Layer framework for performance verification", RAN WG1 #48, February 2007, St. Louis, MI, USA. R1-2007151, "Moderator Summary for multi-beam enhancement: EVM", Moderator (Samsung), RAN1 WG1 #102-e, August 2020. |