3GPP TSG RAN WG1 104-e                                                                 R1-2100684

e-Meeting, January 25th – February 5th, 2021

Source: 	Intel Corporation

Title:	On timing relationship for NB-IoT and eMTC NTN

Agenda item:	8.15.3

Document for:	Discussion and Decision

# Introduction

At the 3GPP TSG RAN meeting #86 new study item (SI) on NB-IoT/eMTC support for Non-Terrestrial Network (NTN) was approved in [1]. This SI aims to evaluate and confirm solutions to address the minimum necessary specifications for NB-IoT and eMTC NTN. One of the objectives which is related to RAN1 is to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821 [2] including the following items.

- Aspects related to random access procedure/signals

- Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication

- Timing offset related to scheduling and HARQ-ACK feedback

- Aspects related to HARQ operation

In this contribution enhancements to timing relationship for NB-IoT/eMTC to support NTN are discussed.

# Discussion

One important topic which was considered for NR NTN SI [2] and should be considered for NB-IoT/eMTC NTN is timing relationship enhancements. Due to large delay observed in NTN large timing advance (TA) is applied to UL transmission. In order to support large TA additional slot offset should be added for some physical channels and procedures on top of the already supported slot offset value.

For eMTC additional slot offset K_offset is needed at least for PUSCH transmission, HARQ-ACK feedback transmission on PUCCH, aperiodic SRS transmission and CSI reference resource definition in time domain. For NB-IoT additional slot offset K_offset is needed at least for NPUSCH format 1 and HARQ-ACK feedback on NPUSCH format 2.

Proposal 1:

- For eMTC additional slot offset K_offset is needed at least for PUSCH, HARQ-ACK feedback on PUCCH, aperiodic SRS and CSI reference resource definition

- For NB-IoT additional common slot offset K_offset is needed at least for NPUSCH format 1 and HARQ-ACK feedback on NPUSCH format 2

In order to reduce UE complexity and UE cost NB-IoT and eMTC devices support half-duplex FDD mode. In such mode simultaneous transmission and reception is not possible as it is done in normal FDD operation. It is assumed that in order to achieve time synchronisation for UL transmission UE applies additional TA to compensate delay introduced in the service link. The value of additional TA is calculated at the UE side autonomously based on GNSS information and satellite ephemeris. If the value of the additional delay is not known by the gNB, UL transmission can collide with DL reception at the UE side. Hence, reporting of additional TA applied by the UE to compensate service link delay should be supported.

Proposal 2:

- Reporting of additional TA applied by the UE to compensate service link delay calculated based on GNSS information and satellite ephemeris should be supported

# Conclusion

In this contribution aspects related to timing relationship enhancements for NB-IoT/eMTC NTN were discussed. The following proposals are made.

Proposal 1:

- For eMTC additional slot offset K_offset is needed at least for PUSCH, HARQ-ACK feedback on PUCCH, aperiodic SRS and CSI reference resource definition

- For NB-IoT additional common slot offset K_offset is needed at least for NPUSCH format 1 and HARQ-ACK feedback on NPUSCH format 2

Proposal 2:

- Reporting of additional TA applied by the UE to compensate service link delay calculated based on GNSS information and satellite ephemeris should be supported

# References

- RP-193235 Study on NB-Io/eMTC support for Non-Terrestrial Network

- 3GPP TR 38.821 V16.0.0 (2019-12); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)