3GPP TSG RAN WG1 #104-e																			R1-2100686

e-Meeting, January 25th – February 5th, 2021

Source:	Intel Corporation

Title:	Support of 1024QAM

Agenda item:	8.16

Document for:	Discussion and Decision

# Introduction

In RAN#89e meeting the work item on downlink 1024QAM support in FR1 was agreed for Rel-17 [1] and revised in [2]. The objectives of WID include specification of MCS and CQI tables supporting 1024QAM.

The objective is to specify downlink 1024QAM for NR PDSCH operation in FR1, together with related procedures, signalling and necessary RF requirements. The main objectives are: - Specify high order modulation for PDSCH [RAN1] - Specify 1024QAM constellation as specified in E-UTRA for DL PDSCH - Specify corresponding 5-bit MCS table with 1024QAM entries as defined in E-UTRA, with 5 bit DCI overhead for MCS indication - Specify corresponding CQI feedback with 1024QAM entries as defined in E-UTRA, with no changes to the CQI field and table sizes

In this contribution, we propose details of the corresponding enhancements for Rel-17.

# CQI table design for 1024QAM

According to 1024QAM WID objective 1024QAM CQI table for NR should support the same 1024QAM entries as in LTE. It is, therefore, proposed to adopt the corresponding table without modifications as shown in Table 1.

Table 1 CQI table for 1024QAM

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

To verify validity for the switching point between 256QAM and 1024QAM from LTE, link-level evaluations were carried out in 1x1 AWGN channel for the different spectral efficiencies. The detailed simulation assumptions can be found in the Appendix. It can be seen Figure 1 that MCS supporting 1024QAM modulation start outperforming 256QAM from spectral efficiency of ~7.1 bps/Hz, which roughly requires replacing 256QAM entry with SE ~ 7.4 with 1024QAM in Table 1. The above observation is taken into account for MCS table design.

Figure 1 Spectral efficiency vs SNR for target BLER = 10%

# MCS table design for 1024QAM

For 5-bit MCS table design with 1024QAM, we propose to use the following principles:

- 256QAM MCS table is used as a starting point for introduction of 1024QAM

- Remove six MCS entries from 256QAM MCS table

  - Remove every other MCS entry with lowest spectral efficiency

  - Consider MCS entries which were not sub-sampled for 256QAM

  - MCS #5, #7, #9, #12, #14 can be considered as candidate for removal from 256QAM table

  - Remove MCS#22 with SE of 7.406

- Introduce 6 additional MCS entries supporting 1024 QAM

  - 5 MCS with TBS indication

  - 1 for adaptive HARQ without TBS indication

- Define spectral efficiency for 1024QAM entries as follows

  - Two MCS entries have the same spectral efficiency as in CQI table

  - Two additional MCS entries have spectral efficiency linearly interpolated from spectral efficiencies of the nearest MCS entries

  - Change modulation for MCS#22 from 256QAM to 1024QAM

  - Quantize the spectral efficiency and coding rates

  - Order according to modulation order and spectra efficiency

Using the above principles, the following MCS table supporting 1024QAM is proposed as show in Table 2, where 1024QAM entries are highlighted in yellow:

Table 2 5-bit MCS table with 1024QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 434 | 1.6953 |
| 6 | 4 | 553 | 2.1602 |
| 7 | 4 | 658 | 2.5703 |
| 8 | 6 | 466 | 2.7305 |
| 9 | 6 | 567 | 3.3223 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.3320 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 10 | 948 | 7.4063 |
| 23 | 10 | 806 | 7.8711 |
| 24 | 10 | 853 | 8.3321 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |

# Support of 1024QAM for DCI Format 1_2

Similar to 256QAM, support of 1024QAM should be enabled for PDSCH scheduled by DCI format 1_2. For this purpose, a new RRC parameter should be supported to enable 1024QAM MCS table for PDSCH scheduled by DCI format 1-2.

# Conclusion

In this contribution, we have provided our views on the details of 1024QAM support in NR. The following proposal were made:

- For CQI design with 1024QAM, adopt the Table 1 (same to LTE CQI table)

- For MCS design with 1024QAM, adopt Table 2 (similar to LTE MCS table)

- Support 1024QAM for PDSCH scheduled by DCI format 1_2

# References

[1] RP-202044, New WID on Introduction of DL 1024QAM for NR FR1, Ericsson, 3GPP TSG RAN Meeting #89e, Electronic Meeting, September 14 - 18, 2020.

[2] RP-202886, Revised WID on Introduction of DL 1024QAM for NR FR1, Nokia 3GPP TSG RAN Meeting #90e, Electronic Meeting, December 7 – 11, 2020.

# Appendix

| Parameter | Value |
| --- | --- |
| Carrier frequency, SCS, System BW | 3.5GHz, 30kHz, 100 MHz |
| Channel model | AWGN |
| UE speed | 0km/h |
| Number of UE antennas | 1T1R |
| Number of gNB antennas | 1T1R |
| Tx/Rx EVM | 0/0%, 2/3% |
| MCS | 256 QAM, 1024 QAM |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 2 |
| Number of scheduled RBs | 36 |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Rank | Rank 1 |
| Channel estimation | Realistic channel estimation |
| Metric | Crossover SNR at transition points between 256-QAM and 1024-QAM |