3GPP TSG RAN WG1 #104-e		R1-2100758

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.4.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on NTN uplink time synchronization

Document for:	Discussion

- Introduction

In RAN1#103e, the agreements related to time and frequency synchronization is as following:

In this contribution, we discuss the issues related to time synchronization.

- Discussion

It is already agreed to support common timing offset indication before PRACH transmission, and whether the indication is a TA or a time offset value is FFS. If it is a TA, then the unit is same as legacy TA command, e.g. 16*64/2u*Tc, and if it is a time offset, the unit can be separately configured explicitly or implicitly. The indicated common timing offset is corresponding to the RTT between the satellite and the reference point. The reference point is where the DL and UL frame are aligned. The selection of TA command unit in legacy release is to address UE moving within the cell coverage area. However, the common timing offset indication is due to moving of satellite, so the unit of common timing offset can be a larger value to save signaling overhead. In this case, the formula in last meeting should be updated as:

TA=(NTA+NTA,offset)*Tc+X

Where NTA is derived from the UE specific TA self-estimation.

X is derived from the common timing offset if broadcasted by the network.

NTA,offset depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2.

Tc is specified in TS 38.211 section 4.1.

Proposal 1: The unit of common timing offset indicated can be different from that of TA command and separately configured.

Regarding whether TA margin indication is necessary or not, as common TA indication is already agreed, and the common TA can absorb the maximum TA uncertainty, so TA margin indication is not need.

Proposal 2: TA margin indication is not supported.

There is also discussion on the TA update in RRC connected state. There are two kinds of TA update methods: open-loop and close-loop. Open-loop TA update can be based on GNSS to calculate the RTT between UE and satellite (e.g. satellite moving) or based on DL RS to track the small-scale change of RTT, e.g. UE moving. Close-loop TA update is based on TA command indication from gNB. The TA indicated from gNB may also include a timing drift rate as satellite moving. There are concerns on the redundancy when UE estimates the RTT change between UE and satellite based on GNSS and gNB indicates the RTT change between UE and gNB simultaneously. From our perspective, the open-loop TA update can be for the RTT between UE and satellite, and the close-loop TA update can be for the RTT between the satellite and the reference point. The reference point is where DL and UL frames are aligned. The close-loop TA indication can be based on a value and a drift rate. The value can be the common timing offset value indicated in system information, and the drift rate is corresponding to the RTT change rate of the link between satellite and the reference point.

Proposal 3: Support indication of timing offset drift rate.

Furthermore, for NTN based on regenerative payload, the TA change rate is proportional to the satellite velocity’s component along the UE-satellite line, which is also proportional to the Doppler shift observed by the UE. Therefore, the UE can use Doppler measurements to estimate a TA change rate and update the TA value with lower-overhead signaling.

- Conclusion

In this contribution, we discussed the issues related to TA in NTN network, and our proposals are as following:

Proposal 1: The unit of common timing offset indicated can be different from that of TA command and separately configured.

Proposal 2: TA margin indication is not supported.

Proposal 3: Support indication of timing offset drift rate.

- References

RP-201256, RANP#88e, “Solutions for NR to support non-terrestrial networks (NTN)”

TR 38.821 v16.0.0

RAN1#103e chairman notes

R1-2009697, Thales, “FL Summary on enhancements on UL time and frequency synchronization for NR NTN”