3GPP TSG RAN WG1 #104e		 	R1-2100800

e-Meeting, January 25th – February 05th, 2021

Agenda Item:	7.2.4

Source:		Spreadtrum Communications

Title:		Remaining issues on sidelink physical layer procedure

Document for:		Discussion and decision

Introduction

This contribution provides remaining issues on sidelink physical layer procedure, with the corresponding TP.

Discussion

Power control

For the SL open-loop power control, a UE can be configured to use DL pathloss only, SL pathloss only, or both DL pathloss and SL pathloss. When p0-DL-PSCCHPSSCH is not provided but p0-SL-PSCCHPSSCH is provided, SL open-loop power control is based on SL pathloss only. When neither p0-DL-PSCCHPSSCH nor p0-SL-PSCCHPSSCH is provided, SL open-loop power control is not used. In both cases, to remove the DL pathloss effect, a proper value should be assigned for PPSSCH,D(i) to ensure that the minimum of valid power values can be achieved. Considering the CBR control has been taken in to account in determining PPSSCH(i) when maximumtransmitPower-SL is provided, including PMAX,CBR in PPSSCH,D(i) is redundant and ambiguous especially when maximumtransmitPower-SL is not provided.

Proposal 1 Remove the CBR part in determining open power value for PSSCH based on DL pathloss and adopt TP #1 in subsection 2.1.

TP #1

------------------------------------------------------ Start of Draft TP of 213------------------------------------------------

16.2.1	PSSCH

<Unchanged parts omitted>

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as:

-	if maximumtransmitPower-SL is provided then

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

-	else

PPSSCH(i)=minPCMAX,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of maximumtransmitPower-SL based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]

-	if p0-DL-PSCCHPSSCH is provided

-	PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-	elseif p0-SL-PSCCHPSSCH is provided

PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i)

-	PPSSCH,D(i)=minPCMAX,PPSSCH,SL(i) [dBm]

-	else

PPSSCH,D(i)=minPCMAX,PMAX,CBR

-	PPSSCH,D(i)=PCMAX [dBm]

<Unchanged parts omitted>

---------------------------------------------------------- End of Draft TP ----------------------------------------------------

UE procedure for reporting HARQ-ACK on sidelink

For PSFCH resource implicit mapping, a set of PRBs is determined for slot i and sub-channel j. Considering both i and j start from 0, it’s better to indicate such set of PRBs by PRBs [1, 2, 3, …] rather than [0th, 1th, 2nd , …] PRBs, thus we propose to change i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs into PRBs i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1.

Proposal 2 Adopt TP #2 in subsection 2.2.

TP #2

------------------------------------------------------ Start of Draft TP of 213------------------------------------------------

16.3  UE procedure for reporting HARQ-ACK on sidelink

<Unchanged parts omitted>

A UE is provided by rbSetPSFCH a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by numSubchannel, and a number of NPSSCHPSFCH PSSCH slots associated with a PSFCH slot, provided by periodPSFCHresource, the UE allocates the PRBs i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 from the MPRB, setPSFCH PRBs to slot i  and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH.

<Unchanged parts omitted>

---------------------------------------------------------- End of Draft TP ----------------------------------------------------

Conclusions

The following conclusions are proposed:

Proposal 1 Remove the CBR part in determining open power value for PSSCH based on DL pathloss and adopt TP #1 in subsection 2.1.

Proposal 2 Adopt TP #2 in subsection 2.2.

References

“Chairman notes ”, RAN 1 meeting #98bis.

3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for control(Release 16), 3GPP TS 38.213.