3GPP TSG RAN WG1 #104e		 	R1-2100801

e-Meeting, January 25th – February 05th, 2021

Agenda Item:	8.11.2.1

Source:		Spreadtrum Communications

Title:		Discussion on sidelink resource allocation for power saving

Document for:		Discussion and decision

Introduction

According to NR sidelink enhancement WID [1], for vulnerable road users (VRUs) in V2X use cases and UEs in public safety and commercial use cases, solutions for power saving in Rel-17 are required to minimize  power consumption. Furthermore, the principle of Rel-14 LTE sidelink random resource selection and partial sensing can be introduced to Rel-16 NR sidelink resource allocation mode 2 as baseline. In the following, we provide our considerations on power saving solutions based on Rel-14 LTE sidelink random resource selection and partial sensing mechanism, respectively.

Discussion

Rel-14 power reduction is mainly targeted for pedestrian UEs and it has been concluded that P-UE sends V2X messages but not receives V2X messages is substantially more power efficient than V2P services. Therefore, random selection is supported at least for the P-UEs which do not have sidelink Rx capability. Furthermore, for the other P-UEs having sidelink Rx capability, partial sensing is supported and (pre)-configuration instructs whether a P-UE uses partial sensing only, random selection only, or either of the two. In Rel-17, besides vulnerable road users (VRUs) in V2X use cases, UEs in public safety and commercial use cases also desire power efficient solutions, which may be more likely to have sidelink Rx capability. To provide the flexibility of such UEs, similar principle can be adopted.

Proposal 1: A Rel-17 sidelink UE can be NW-configured or pre-configured to use partial sensing only, random selection only, or either of the two.

Resource allocation mode 2: random resource selection

For UEs having sidelink Rx capability, using random selection may help to further reduce power reduction compared with partial sensing. In order to benefit from random selection and further reduce resource collision, additional information can be provided for UEs performing random selection, i.e., among multiple resource pools pre-configured to power saving UEs with random selection permitted, RSU can instruct such UEs at least one resource pool to be used via sidelink signaling, and the selection of resource pool can be based on RSU’s CBR measurements.

Proposal 2: Assistant information can be provided via sidelink signaling to the UEs performing random selection.

Resource allocation mode 2: Partial sensing

For UEs without sidelink Rx capability, it cannot receive PSCCH/PSSCH, which is a necessity of sensing, and thus, partial sensing is up to UE capability.

Proposal 3: Support of partial sensing is up to UE capability.

In Rel-14, resource reservation interval shorter than 100 ms is not supported. While in Rel-17, a larger set of period values {0, 1:99, 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000 ms} are supported, among which 16 values will be configured on a per resource pool basis. Therefore, resource reservation interval for Rel-17 power saving UEs should be discussed.

Proposal 4: RAN1 discusses whether additional value of interval is needed for Rel-17 power saving UEs.

Partial sensing mechanism in Rel-14 can be interpreted as in Figure 1, when P-UE makes resource selection/reselection decision at TTI m, the possible candidates resources, i.e., Y subframes, are selected in [m+T1, m+T2], the minimum allowed value of Y is (pre)configured and how to select Y subframes is up to P-UE implementation. If a subframe  is included in the set of Y subframes, the UE shall monitor any subframe  if k-th bit of the high layer parameter gapCandidateSensing is set to 1[2]. For example, TTI y1, y2, y3 are selected in figure 1, denoted by red, purple, and green, and thus the TTI moments marked with the same color should be monitored for resource exclusion purpose.

Figure 1 Partial sensing mechanism in Rel-14

In Rel-16, besides periodic resource reservation for different TB, resource reservation is also supported for the retransmissions of the same TB. And the maximum number of SL resources reserved by one transmission including current transmission is 3 within a window of 32 logical slots. Furthermore, periodic resource reservation can be enabled and disabled on a per resource pool basis and thus, T0 is (pre)-configured between: 1000+100ms and 100ms, for periodic resource reservation enabled and disabled, respectively.

By considering the resource reservation for the retransmissions of the same TB within 32 logical slots, for any of Y slots selected in [m+T1, m+T2], up to 3 resources reserved by one transmission including current transmission should also be considered for resource exclusion, which has been shown in Figure 2. For each of the selected slots n1, n2, n3, denoted by red, purple, and green, besides the possible slots of periodic resource reservation, the possible slots of resource reservation for the retransmissions of the same TB should be also monitored.

Figure 2 Partial sensing consideration in Rel-17

Proposal 5: For partial sensing, UE shall monitor any possible slots of resource reservation for the retransmissions of the same TB.

Conclusions

The following conclusions are proposed:

Proposal 1: A Rel-17 sidelink UE can be NW-configured or pre-configured to use partial sensing only, random selection only, or either of the two.

Proposal 2: Assistant information can be provided via sidelink signaling to the UEs performing random selection.

Proposal 3: Support of partial sensing is up to UE capability.

Proposal 4: RAN1 discusses whether additional value of interval is needed for Rel-17 power saving UEs.

Proposal 5: For partial sensing, UE shall monitor any possible slots of resource reservation for the retransmissions of the same TB.

References

“RP-201385 WID revision: NR sidelink enhancement ”, RAN meeting #88e.

3rd Generation Partnership Project;Technical Specification Group Radio Access Network;Evolved Universal Terrestrial Radio Access (E-UTRA)Physical layer procedures, 3GPP TS 36.213.