3GPP TSG RAN WG1 #104-e                               R1-2100882

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.3.2

Source: 	LG Electronics

Title: 	Discussion on unlicensed band URLLC/IIOT

Document for:	Discussion and decision

- Introduction

In RAN1#103-e, following agreements were made for support of URLLC in unlicensed band, in terms of UE-initiated COT based FBE operation and URLLC/NR-U CG harmonization [1].

Agreements: - In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms Conclusion: - For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB. Agreements: - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported Conclusion: If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X. If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y. Note 1: One of the devices X and Y is a UE and the other is its serving gNB. Note 2: Whether or not there is additional restriction on idle period is still FFS. Agreements: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions. Agreements: - The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s). - Note: UE initiated FBE configuration is configured per serving cell Agreements: In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT. o Note: Any value for the period, shall be at least 1ms and at most 10ms. o Note: Aim for low complexity operation to handle gNB and UE COT interactions Agreements: In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. - UE determines the initiator of a COT based on at least one of the following alternatives: - Alt 1: Introduce additional bit field in the scheduling DCI - Alt 2: Based on ChannelAccess-CPext field in DCI - Alt. 3: Based on a predetermined rule(s) - Alt. 4: Based on RRC signalling - Alt. 5: Based on MAC CE - FFS other alternatives - FFS on overriding possibility and/or the assumption - Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

In this contribution, based on the above agreements, we discuss and provide our views on UL enhancements for URLLC support in controlled unlicensed band environments.

- Support of UE-initiated COT for FBE based URLLC

Regarding UE-initiated COT for the purpose of supporting URLLC in controlled U-band environments operating based on FBE structure, basically, it is desirable that the UE-initiated COT is able to be controlled in gNB side, in order to avoid potential collision/blocking between UE’s UL transmission and gNB’s essential DL transmission (such as SSB transmission, system information, paging, and RACH messages), which would induce significant/critical impacts to the system/network, by allowing the UE-initiated COT based on the contention even with the gNB. With this consideration, we discuss on potential gNB-controlled UE-initiated COT mechanisms to support URLLC in FBE operation based U-band environments.

- Configuration of unaligned FFP between gNB and UE

One possible mechanism to support UE-initiated COT in gNB-controlled manner is to configure the FFP starting with gNB-initiated COT (i.e., FFP-g) and the FFP starting with UE-initiated COT (i.e., FFP-u) separately, where the starting timing of FFP-g is configured to be earlier than that of FFP-u.

Given this structure, from UE perspective, the UE could try to do LBT before the starting timing of its own FFP-u, then it could make an UE-initiated COT if the LBT is successful due to idle channel. Otherwise (it means, if there is LBT failure due to busy channel), the UE could follow Rel-16 FBE behaviour assuming FFP-g starting with gNB-initiated COT. Observing from gNB perspective, it would try to make gNB-initiated COT for its own FFP-g transmission when it has something to transmit in DL. Otherwise (it means, in case when nothing needs to be transmitted in DL), the gNB could hand over the chance of COT initiation to the UE, then it would be beneficial for the UEs having something to transmit in UL with low latency.

Figure 1 shows an example of the unaligned FFP structure between gNB and UE. Firstly, considering the FFP-g transmission based on gNB’s COT initiation, the gNB could transmit and share its COT for the duration in blue. Secondly, considering the FFP-u transmission based on UE’s COT initiation, the UE could transmit and share its COT for the duration in orange. In this case, it would be required to align the starting timing of the idle period between FFP-g and FFP-u, to avoid LBT blocking to other gNBs since the idle period of FFP-g may need to be aligned among multiple gNBs to well coexist each other.

Figure 1. An example of unaligned FFP structure between gNB and UE.

Proposal #1: Support unaligned FFP timing between the FFP stating with gNB-initiated COT and the FFP starting with UE-initiated COT.

- Indication to allow UE-initiated COT for the next FFP

Another possibility to support gNB-controlled UE-initiated COT could be to indicate dynamically (in the current FFP) whether to allow making UE-initiated COT for the next FFP, based on the transmission of UE (group)-common DCI like SFI signalling.

In this case, if gNB indicates that making UE-initiated COT for the next FFP is allowed for UEs, the UEs could try to do LBT and make the UE-initiated COT (then start the FFP with UL transmission) if the LBT is successful. For the above, the UE (group)-common DCI used for the indication of UE-initiated COT could either explicitly indicate whether or not to allow UE-initiated COT for the next FFP, or implicitly indicate by allocating UL resource at the beginning of FFP or by not cancelling pre-configured UL resource at the beginning of FFP. With this, the gNB could control potential congestion/collision among multiple UEs in the next FFP, by indicating allowance of UE-initiated COT differently per UE group.

For the above DCI signalling to indicate allowance of UE-initiated COT, basically, the structure of the common DCI signalling designed in Rel-16 NR-U can be reused with some modification or reinterpretation. For example, based on the combination of COT duration and SFI length indicated via the DCI (and the boundary of FFP), usage of the next FFP in terms of the initiated COT type (e.g. either FFP-g or FFP-u planned in the gNB) could be determined by the UE.

Proposal #2: Consider to support dynamic indication of whether to allow UE-initiated COT for the next FFP based on the transmission of UE (group)-common DCI, at least for the control of potential congestion among multiple UEs in a same FFP.

- Structure of the common DCI signaling (with indication of COT duration and SFI information) designed in Rel-16 NR-U can be reused.

- Configuration of LBT type to allow UE-initiated COT

For the configured UL transmission such as RACH occasion and CG PUSCH, it might be necessary to pre-configure the associated LBT type for the configured UL resource in terms of whether the configured UL is allowed to initiate COT, in other words, whether the detection of DL is required to transmit the configured UL (before LBT). This would be reasonable for gNB to well manage DL/UL configuration and FFP-g/FFP-u combination in time domain.

In addition, considering the fixed timing to start transmission of the UE-initiated COT per FFP, it may need to be considered/discussed whether some handling is required to apply the CP extension for transmission of the configured UL at the beginning of FFP.

Proposal #3: Consider LBT type configuration for the configured UL resource in terms of whether the configured UL is allowed to initiate COT by UE.

- It is FFS whether some handling is needed to apply the CP extension for transmission of the configured UL at the beginning of FFP.

- Consideration of default FFP-g without UE-initiated COT

Considering the case where some essential DL transmission occasions such as SSB or CORESET#0 are at the beginning of FFP or included within FFP duration, the FFP may need to be assumed by UE as a default FFP-g based on gNB-initiated COT. With the assumption, the UE is not allowed to initiate COT for the FFP, and thus the UE would not try to initiate COT for the FFP. By defining the default FFP-g, potential UL-to-DL interference due to COT initiation by UE could be avoided.

Similarly for the case where essential UL transmission occasion such as RACH occasion is configured at the beginning of FFP, the FFP may need to be considered (or reserved) by gNB as a default FFP-u based on UE-initiated COT.

Proposal #4: Consider to define the FFP including or starting with essential DL/UL transmission occasions (such as SSB or CORESET#0) as default FFP-g.

- Consideration of FFP alignment for multiple RB sets

Considering the case where multiple RB sets (requiring individual LBT) are within a carrier or BWP under the above unaligned FFP timing structure between gNB and UE, it may be required for UE to assume a same type of FFP (i.e., FFP-g or FFP-u) for the multiple RB sets to avoid potential UL-to-DL interference, even if the result of LBT for the multiple RB sets is different.

For example, in case when the LBT is successful for RB set #1 while the LBT is failed (and DL signal is detected) for RB set #2, it would be reasonable for the UE not to try initiate COT for RB set #1 by assuming the RB set #1 as FFP-g based on gNB-initiated COT, in order for avoiding UL-to-DL interference to RB set #2 operating as FFP-g.

Proposal #5: Consider to align the assumption of FFP type for multiple RB sets in a carrier/BWP under the unaligned FFP structure between UE and gNB.

- Configuration of UE FFP with UE-initiated COT

On the detailed configuration of FFP-u in terms of the period and starting offset, it is necessary to consider following aspects.

- Granularity of the starting offset (O_u) for FFP-u

- Relationship between FFP-u period (P_u) and FFP-u starting offset

- Relationship between FFP-u period and FFP-g period (P_g)

- Relationship between FFP-u period and the period (P_c) of configured UL

- Relationship between FFP-u period and the duration of FFP-u

Regarding the granularity of O_u, considering possible starting timing configured for the configured UL (e.g. CG PUSCH) resource and for aligning the starting timing of FFP-u with the configured UL to make UE-initiated COT possible by transmitting the configured UL, the value of O_u would need to be configured in symbol level. In addition, considering that the value of P_u is configured with absolute time as, for example, X msec, candidate values of O_u (in symbol level) might need to be scaled according to SCS value.

Besides, in case where the P_u is configured as X msec, candidate O_u values can be provided within the range from 0 to X msec. Similarly, in case where the P_g is configured as Y msec, in order for ensuring low complexity operation to handle interactions between gNB COT and UE COT as agreed, possible P_u values configurable for the UE need to be equal to or smaller than Y msec (e.g. as divider of Y msec). Moreover, in case where the P_u is configured as X msec, possible P_c values configurable for the UE can be either divider of Y msec or multiple of Y msec.

Regarding the FFP period (as the interval of adjacent FFP starting) and the FFP duration (as the time duration for maintaining the COT), currently for gNB’s FFP-g, those are the same. In case of FBE operation based on both gNB’s FFP-g and UE’s FFP-u, on the other hand, it could be beneficial to configure period and duration differently for FFP-u (e.g. period > duration) to provide the FFP dedicated to gNB without overlapping with FFP-u for guaranteeing reliable gNB transmission/controllability.

Proposal #6: Consider the following aspects for the configuration of the starting offset and the period/duration for UE FFP-u.

- Granularity of the starting offset (O_u) for FFP-u

- Relationship between FFP-u period (P_u) and FFP-u starting offset

- Relationship between FFP-u period and FFP-g period (P_g)

- Relationship between FFP-u period and the period (P_c) of configured UL

- Relationship between FFP-u period and the duration of FFP-u

- Scheduled UL transmission based on UE/gNB-initiated COT

It was agreed in RAN1#103-e that a UE should be able to determine whether a scheduled UL transmission is to be done according to shared-gNB COT or UE-initiated COT, and the method to support the UE’s determination of the scheduled UL is remained as FFS. If only the alternatives themselves (Alt 1/2/3/4/5) are focused without considering other aspect, Alt 3 (based on a predetermined rule) is preferred, for example, as UE’s determination according to whether the scheduled UL is aligned with the starting of FFP-u).

On the other hand, it is required to discuss/clarify first on the energy detection (ED) threshold used to perform LBT for UE-initiated COT before deciding the method for UE’s determination of the scheduled UL. Currently in LBE mode, on top of the default ED threshold (i.e. EDT_no_sh) given by calculation based on the maximum UE TX power, UE can be configured with another ED threshold (i.e., EDT_sh) from gNB for the purpose of shared-UE COT transmission by gNB. With those two different ED thresholds, for a CG PUSCH resource, the UE can select one of them and perform LBT based on the selected ED threshold, and the UE reports the selected ED threshold to the gNB via the CG PUSCH.

From the perspective of coexistence based on the ED threshold used for LBT, there might be no significant difference between LBE mode and FBE mode. Therefore, in this context, it would need to consider the dependency between UE’s determination of the scheduled UL (e.g. whether it is based on shared-gNB COT or UE-initiated COT) and the ED threshold used to perform LBT for the scheduled UL transmission, especially for the alternatives based on DCI indication.

Proposal #7: Clarify first on the energy detection (ED) threshold used to perform LBT for UE-initiated COT before deciding the method for UE’s determination of the scheduled UL transmission (e.g. whether it is done based on shared-gNB COT or UE-initiated COT).

- Configured UL transmission based on UE/gNB-initiated COT

On the UE behaviour/assumption for a configured UL transmission aligned with a UE FFP boundary, following three alternatives are on the table.

- Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

- Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

- Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

Regarding the above alternatives, first of all, it should be considered whether the reliable transmission and interaction between UE and gNB would be guaranteed without undesirable misalignment between them.

Figure 2. An example of FFP overlapping between gNB and UE.

Figure 3. Another example of FFP overlapping between gNB and UE.

Figures 2 and 3 show some examples of FFP overlapping between gNB and UE where some configured UL (i.e. C-UL) resource are configured at the UE FFP boundary or within UE FFP. Firstly in Figure 2, if the transmission of C-UL #1 is assumed as UE-initiated COT, then gNB is not allowed to transmit DL signal/channel in the idle period of FFP-u #1, but if the C-UL #1 is assumed as shared-gNB COT, then the gNB can transmit DL in the idle period of FFP-u #1. Secondly in Figure 3, if the transmission of C-UL #1 is assumed as UE-initiated COT, then UE can also transmit C-UL #3 within FFP-u #2, but if the C-UL #1 is assumed as shared-gNB COT, then the UE is not allowed to transmit C-UL #3 in outside of FFP-g #1. With the above observation, considering the case where gNB initiates COT but UE fails to detect the DL signal corresponding to the gNB-initiated COT, Alt-a might be able to cause unexpected and unreliable transmission from/to UE/gNB.

In this context, Alt-b might be able to guarantee the reliable transmission and interaction between UE and gNB, with slight modification as: The UE assumes that the configured UL transmission corresponds to UE-initiated COT if the LBT gap is provided before the configured UL (e.g. between the DL detection (before the configured UL) and the configured UL). On the other hand, considering coarse UL resource granularity due to the use of PRB interlace under U-band regulatory requirement (in terms of OCB and PSD) and short PUSCH duration to reduce transmission latency, it may be hard to ensure that all of potential URLLC UEs are well configured with the FDMed CG PUSCH resources to be aligned with a same UE FFP boundary. For this reason and to handle/control potential UE congestion by distributing the UEs over multiple UE FFPs, it is preferred for gNB to configure whether each configured UL is allowed for UE-initiated COT as in Alt-c. It is noted that with Alt-c, some of FFPs could be dedicated to gNB without contention/interference.

Proposal #8: Prefer Alt-c (The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration) for the configured UL transmission aligned with UE FFP boundary.

Moreover, the following UE behaviour/assumption was agreed for a configured UL transmission not aligned with a UE FFP boundary.

- If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT.

- Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

However, the above behaviour/assumption would cause potential problem in UE/gNB interaction aspect and FBE regulation perspective. For example, considering the case in Figure 3, in case when the UE has initiated COT by transmitting C-UL #1 and also it has detected gNB-initiated COT in FFP-g #2 (before C-UL #3), it is reasonable that the UE assumes the C-UL #3 (not aligned with a UE FFP boundary) corresponds to gNB-initiated COT. Therefore, the above agreement needs to be slightly updated accordingly (e.g. change of the sentence starting with “If” and the sentence starting with “Otherwise”).

Proposal #9: Consider the following UE behaviour/assumption for the configured UL transmission not aligned with UE FFP boundary (by slightly updating the previous agreement).

- If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- Otherwise, If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT.

- Harmonization of CG features in Rel-16 NR-U and URLLC

Regarding the CG configuration in terms of enabling/disabling CG-UCI based procedure and CG-DFI based procedure and CG retransmission timer, following four options are currently on the table.

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

Considering UE complexity and specification impacts to implement various combinations of CG-UCI, CG-DFI and CG timer, and focusing main use case rather than just introducing any workable variants, Option 1 is preferable and Option 2-b is undesirable. In case of Option 2-a and Option 3, it might be beneficial in terms of reducing DCI overhead (by enabling CG-UCI and disabling CG-DFI) or UL-SCH loss (by disabling CG-UCI and enabling CG-DFI). In this sense, those options can be considered further if there is consensus to pursue the above benefits.

Proposal #10: Option 1 is preferable for the CG configuration in terms of enabling/disabling CG-UCI based procedure and CG-DFI based procedure and CG retransmission timer.

- Option 2-a and Option 3 can be considered further if there is consensus to pursue the reduction of DCI overhead or UL-SCH loss.

Moreover, we also consider to design harmonized resource allocation scheme with taking existing RRC parameter and principle. In Rel-16, configured grant for NR URLLC and NR-U has been developed in different ways. Configured grant in NR-U has differences from configured grant in URLLC in following two aspects: (1) resource allocation method and (2) How to determine HARQ process ID.

- Resource allocation method for CG PUSCH

In Rel-16 NR URLLC discussion, PUSCH repetition type B has been introduced for support flexible resource allocation. As a result, NR URLLC has two parameters meaning the number of repetitions and repetition type. In NR-U, configured grant configuration also has the number of repetitions parameter for determine how many transmission occasions are used for a TB and separately has two parameters for resource allocation. Up to gNB configuration, configured grant configurations in NR URLLC and NR-U can make equivalent resource allocation

As mentioned above, the easiest way to support both resource allocation is to make a new parameter to select either resource allocation method. However, it is hard to say that it is harmonization. To make unified method, it can be considered to adopt repetition type B to NR-U CG resource allocation. If there is no segmentation of PUSCH, NR-U CG resource allocation is more generalized method to allocate resource.

Proposal #11: Consider to adopt PUSCH repetition type B for NR-U CG resource allocation.

If PUSCH repetition type B is adopted, to make NR-U CG RA and repetition type B work together, it could be necessary to combine existing RRC parameter to remove ambiguities and duplicated information.

Considering definitions of periodicity and repK of NR CG and definitions of cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16 of NR-U CG configuration, following three RRC parameter can be introduced in order to support harmonized CG operation:

- A RRC parameter for the number of consecutive PUSCH occasions (tentatively, cg-nrofconsecutivePUSCH)

  - This parameter determines the number of PUSCH repeated in back-to-back manner. This parameter follows the principle of repK with PUSCH repetition type B and cg-nrofPUSCH-InSlot-r16

- A RRC parameter for the number of repetition of consecutive PUSCH occasions in slot-level (tentatively, cg-nrofSlotRepetitons)

  - This parameter determines the number of a group of slot where consecutive PUSCH occasion is. This parameter follow the principle of cg-nrofSlots-r16 with extension to multiple slots.

- A RRC parameter for the number of PUSCH occasion used for a TB (tentatively, cg-nrofPUSCH-perTB)

  - This parameter determine how many PUSCH occasion is used for single transport block. This parameter follow the principle

By adopting above three parameters instead of existing parameters, it can be facilitated to truly support all of PUSCH repetition type A/B and NR-U CG RA simultaneously.

For an example, UE can be configured with {cg-nrofconsecutivePUSCH = 1, cg-nrofSlotRepetitons = cg-nrofPUSCH-perTB = 4} in order to achieve Rel-15 CG resource allocation with K=4, as shown in Figure 2.

Figure 2. An example of Rel-15 PUSCH allocation

For another example, UE can be configured with {cg-nrofconsecutivePUSCH = 4, cg-nrofSlotRepetitons = 1, cg-nrofPUSCH-perTB = 4} in order to achieve Rel-16 PUSCH repetition type B with K=4, as shown in Figure 3.

Figure 3. An example of Rel-16 PUSCH repetition type B

For another example, UE can be configured with {cg-nrofconsecutivePUSCH = 2, cg-nrofSlotRepetitons = 2, cg-nrofPUSCH-perTB = 2} in order to achieve NR-U CG operation with CG-UCI with cg-nrofPUSCH-InSlot-r16 = 2, cg-nrofSlots-r16 =2 and repK=2, as shown in Figure 4.

Figure 4. An example of NR-U CG operation

For another example, UE can be configured with {cg-nrofconsecutivePUSCH = 4, cg-nrofSlotRepetitons = 2, cg-nrofPUSCH-perTB = 2} in order to achieve a kind of harmonized operation between PUSCH repetition type B and NR-U CG operation as shown in Figure 5.

Figure 5. An example of harmonized CG resource allocation

As exemplify above, it is possible to support resource allocation and repetition scheme of both NR-U and NR URLLC CG by adopting three RRC parameters. Furthermore, adjusting values of parameters, it is also possible to facilitate harmonized operation naturally. Given point of view, we would like to propose the following.

Proposal #12: Introduce following three resource allocation parameters replacing existing parameters to support harmonized CG operation.

- A RRC parameter for the number of consecutive PUSCH occasions

- A RRC parameter for the number of repetition of consecutive PUSCH occasions in slot-level

- A RRC parameter for the number of PUSCH occasion used for a TB

If PUSCH repetition type B is used for unlicensed band, the segmentation would trigger LBT/CAP procedure. In most cases of the segmentation, it is reasonable to attempt LBT/CAP procedure since the segmentation occurs to avoid DL symbols. One controversial case is orphan symbol case. In NR URLLC, UE skipped 1 symbol length PUSCH after segmentation for efficiency. However, it could make UE attempt LBT/CAP unnecessarily. Therefore, in unlicensed band, it could be beneficial to utilize orphan symbol under some conditions rather than drop them.

Proposal #13: Consider to keep 1 symbol length PUSCH after segmentation under some condition in unlicensed band

- HARQ process ID determination without CG-UCI

NR-U specification uses CG-UCI for configured grant PUSCH. UE can selects HARQ process ID, RV and NDI itself for its configured grant PUSCH transmission. On the other hands, NR URLLC configured grant has an equation for determine a HARQ process ID for a period. Thus, differences in this aspect is that NR URLLC configured grant only support single TB per a period and doesn’t support autonomous re-transmission.

Those differences come from CG-UCI. However, CG-UCI has some drawbacks in terms of reliability. For URLLC services, all signals involving communication must meet at least reliability requirement to achieve overall reliability requirement. Since CG-UCI should be encoded separately from UL-SCH, it could be reliability bottleneck. In addition, CG-UCI may consume resource element for UL-SCH, so that degrades UL-SCH reliability. Form those point of view, it can be considered to support multiple TB transmission per periods and autonomous re-transmission, without CG-UCI. To support multiple TB transmission without CG-UCI, it would be necessary to specify new HARQ process ID equation suitable for NR-U configured grant.

In Rel-15/16, HARQ process ID in SPS/CG is determined by symbol index of initial repetition per period. The equation for HARQ process ID was design with basic assumptions that only one TB is transmitted per period and every initial transmission occasions for a TB are equally spaced. Both assumption are not aligned with resource allocation in NR-U CG operation.

One big difference is that two or more HARQ process can be used in a period of CG. To remove this problem and re-use current equation, we can consider one additional term “HPN interval” to make a space between HARQ process IDs in two adjacent period, as following equation:

(((symbol index /periodicty)*HPN interval) mod nrofHARQProcesses) + harqProcIDOffset

“HPN interval” can be determined by “how many TB can be transmitted in a period”. If two TB can be transmitted per period, HPN interval can be set 2.

For example, let assume 1 slot periodicity and two 7symbol PUSCH occasion per period in order to transit two individual TBs per period (Figure 6).

Figure 6. Two PUSCH per 1 period.

With HPN interval =2, nrofHARQProcesses = 8 and harqProcIDOffset =0, HARQ process ID of first PUSCH in each period can be determined as following figure 7.

Figure 7. an example of HPN interval

For remaining PUSCH occasion, we can allocate subsequent HARQ process ID of first PUSCH. Thus, for a period, HARQ process ID for nth initial PUSCH repetition can be X+modn-1,N, where X is HARQ process ID of first PUSCH in the period.

Even if HARQ process ID is successfully determined with above equation, it can be still required to indicate NDI from UE side at least to facilitate autonomous re-transmission with CG-UCI. For this cases, it is also can be considered to support CG-UCI having reduced information field.

Proposal #14: Consider new equation for determining HARQ process ID in order to support multiple TB transmission per periods.

Proposal #15: Consider NDI indication with less overhead other than CG-UCI.

- Conclusions

In this contribution, UL enhancements for URLLC in unlicensed controlled environments were discussed, and the followings are proposed.

Proposal #1: Support unaligned FFP timing between the FFP stating with gNB-initiated COT and the FFP starting with UE-initiated COT.

Proposal #2: Consider to support dynamic indication of whether to allow UE-initiated COT for the next FFP based on the transmission of UE (group)-common DCI, at least for the control of potential congestion among multiple UEs in a same FFP.

- Structure of the common DCI signaling (with indication of COT duration and SFI information) designed in Rel-16 NR-U can be reused.

Proposal #3: Consider LBT type configuration for the configured UL resource in terms of whether the configured UL is allowed to initiate COT by UE.

- It is FFS whether some handling is needed to apply the CP extension for transmission of the configured UL at the beginning of FFP.

Proposal #4: Consider to define the FFP including or starting with essential DL/UL transmission occasions (such as SSB or CORESET#0) as default FFP-g.

Proposal #5: Consider to align the assumption of FFP type for multiple RB sets in a carrier/BWP under the unaligned FFP structure between UE and gNB.

Proposal #6: Consider the following aspects for the configuration of the starting offset and the period/duration for UE FFP-u.

- Granularity of the starting offset (O_u) for FFP-u

- Relationship between FFP-u period (P_u) and FFP-u starting offset

- Relationship between FFP-u period and FFP-g period (P_g)

- Relationship between FFP-u period and the period (P_c) of configured UL

- Relationship between FFP-u period and the duration of FFP-u

Proposal #7: Clarify first on the energy detection (ED) threshold used to perform LBT for UE-initiated COT before deciding the method for UE’s determination of the scheduled UL transmission (e.g. whether it is done based on shared-gNB COT or UE-initiated COT).

Proposal #8: Prefer Alt-c (The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration) for the configured UL transmission aligned with UE FFP boundary.

Proposal #9: Consider the following UE behaviour/assumption for the configured UL transmission not aligned with UE FFP boundary (by slightly updating the previous agreement).

- If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- Otherwise, If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal #10: Option 1 is preferable for the CG configuration in terms of enabling/disabling CG-UCI based procedure and CG-DFI based procedure and CG retransmission timer.

- Option 2-a and Option 3 can be considered further if there is consensus to pursue the reduction of DCI overhead or UL-SCH loss.

Proposal #11: Consider to adopt PUSCH repetition type B for NR-U CG resource allocation.

Proposal #12: Introduce following three resource allocation parameters replacing existing parameters to support harmonized CG operation.

- A RRC parameter for the number of consecutive PUSCH occasions

- A RRC parameter for the number of repetition of consecutive PUSCH occasions in slot-level

- A RRC parameter for the number of PUSCH occasion used for a TB

Proposal #13: Consider to keep 1 symbol length PUSCH after segmentation under some condition in unlicensed band

Proposal #14: Consider new equation for determining HARQ process ID in order to support multiple TB transmission per periods.

Proposal #15: Consider NDI indication with less overhead other than CG-UCI.

- References

RAN1 chairman’s notes, RAN1#103-e