3GPP TSG RAN WG1 #104-e		                         R1-2100885

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.13.1

Source: 	LG Electronics

Title: 	Discussion on cross-carrier scheduling from SCell to PCell

Document for:	Discussion and decision

- Introduction

In RAN1#103-e meeting [1], the following agreements and conclusions were made for cross-carrier scheduling from sSCell to PCell/PSCell.

Agreements: - When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell Conclusion - When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16 - FFS: DCI format 2_5 and DCI Format 2_6 handling - Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’ Conclusion - When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5). Agreements: - Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured. Below alternatives can be considered in the discussion (other alternatives are not precluded) - Alt 1: UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s) - Alt 2: UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded) - Alt 2-1: - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-2: - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported - FFS: Details of switching mechanism - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-3: - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously - Alt 2-4: - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously - FFS following aspects - Impact of sSCell activation/deactivation and sSCell dormancy - Impact on BD/CCE limit handling - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a USS set configured for scheduling of PCell/PSCell)

In this contribution, we discuss and provide our views on the support of cross-carrier scheduling from sSCell to P(S)Cell.

- Discussion

DSS WI is mainly motivated from the scenario where gNB operates LTE and NR on the same carrier and that carrier is configured as NR P(S)Cell to a UE. In this case, if always-on signals such as LTE CRS and reserved region for LTE PDCCH are taken into account, a resource to carry NR PDCCH on that carrier may not be sufficient. To overcome this problem, it would be beneficial to allow cross-carrier scheduling from SCell to P(S)Cell. However, since NR system hasn’t supported yet cross-carrier scheduling where P(S)Cell is configured as scheduled cell, several issues need to be resolved to support P(S)Cell as scheduled cell.

The first aspect to be considered to support cross-carrier scheduling where P(S)Cell is configured as scheduled cell is which type of search space sets configured on P(S)Cell can be offloaded to sSCell. Our view is summarized as follows:

- Type0/0A/2-PDCCH CSS set: As in Rel-15, that is, can be configured only in PCell, as agreed in RAN1#103-e

- Type1-PDCCH CSS set: As in Rel-15/16, that is, can be configured only in P(S)Cell, as agreed in RAN1#103-e

- Type3-PDCCH CSS set: As in Rel-15/16, that is, can be configured in either of P(S)Cell and SCell, for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 as agreed in RAN1#103-e, also for DCI formats 2_5 and 2_6, applicability of the information in DCI formats 2_5 and 2_6 are the same as in Rel-15/Rel-16

- USS set associated with fallback DCI formats (i.e., DCI format 0_0/1_0): FFS whether it remains in P(S)Cell regardless of cross-carrier scheduling, or it can be configured in SCell scheduling P(S)Cell (e.g., by replacing one of the USS sets configured on SCell)

- USS set associated with non-fallback DCI formats (e.g., DCI format 0_1/0_2/1_1/1_2): Alt 2-1, i.e., UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously based on SS set configuration for which cross-carrier scheduling or self-carrier scheduling can be configured per each USS set

Furthermore, if USS sets are configured on the active DL BWP of the scheduling SCell and their indices exist in the USS sets configured on scheduled P(S)Cell, the USS sets can be monitored for cross-carrier scheduling from SCell to P(S)Cell. However, it is necessary to discuss further whether to also support PDCCH monitoring for USS sets, if the USS sets are configured on the active DL BWP of the scheduling SCell but their indices exist in the CSS sets configured on scheduled P(S)Cell. In addition, it can be considered how to handle the case where USS set index for the active DL BWP of scheduling SCell corresponding to a search space set configured for P(S)Cell does not exist.

Proposal 1: When cross-carrier scheduling from sSCell to P(S)Cell is configured,

- For Type3-PDCCH CSS set, the configuration for DCI formats 2_5 and 2_6 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

- For USS set associated with fallback DCI formats (i.e., DCI format 0_0/1_0) scheduling P(S)Cell: FFS whether it remains in P(S)Cell regardless of cross-carrier scheduling configuration, or it can be monitored in the sSCell

- For USS set associated with non-fallback DCI formats (e.g., DCI format 0_1/0_2/1_1/1_2) scheduling P(S)Cell,  UE can monitor DCI formats 0_1, 1_1, 0_2, 1_2 on both PCell USS set(s) and sSCell USS sets simultaneously (i.e., Alt 2-1 as per previous agreement) based on SS set configuration for which cross-carrier scheduling or self-carrier scheduling can be configured per each USS set.

Next, in case all or part of PDCCH candidates configured for P(S)Cell can be monitored on sSCell in addition to P(S)Cell, USS set dropping rule due to PDCCH overbooking may need to be clarified. To be specific, since there can be two scheduling cells (i.e., P(S)Cell and sSCell) corresponding to a single scheduled cell (i.e., P(S)Cell), the number of PDCCH candidates or non-overlapping CCEs needs to be counted for both two scheduling cells. In order to determine whether the number of PDCCH candidates or non-overlapping CCEs exceeds UE’s corresponding capability or not, a reference cell (for checking PDCCH overbooking) needs be defined especially considering the case where two scheduling cells are configured with different numerologies. In our view, choosing sSCell as the reference cell would be reasonable (regardless of configured SCS for P(S)Cell and sSCell) since the sSCell is configured as the scheduling cell for P(S)Cell from the perspective of cross-carrier scheduling configuration.

Proposal 2: When cross-carrier scheduling from sSCell to P(S)Cell is configured, UE counts the number of PDCCH candidates or non-overlapping CCEs from both P(S)Cell and sSCell and then, checks the PDCCH overbooking for the scheduled P(S)Cell based on the maximum number of PDCCH candidates or non-overlapping CCEs corresponding to (the SCS of) sSCell.

Furthermore, we may need to consider the case when the sSCell configured as scheduling cell for P(S)Cell can be deactivated (or dormant). UE is supposed to stop any PDCCH monitoring on a deactivated SCell. However, if the deactivated SCell was configured for cross-carrier scheduling from sSCell to P(S)Cell and PDCCH monitoring on the sSCell was turned off, gNB may not have sufficient resource for PDCCH scheduling PDSCH/PUSCH on P(S)Cell. One solution could be to allow PDCCH monitoring for P(S)Cell on sSCell, even though the corresponding sSCell is in deactivated or dormant state.

Proposal 3: Consider whether/how to handle the case when sSCell is deactivated or in dormant state.

- Conclusion

In this contribution, we provided our views on the support of cross-carrier scheduling from sSCell to P(S)Cell and proposals are as follows.

Proposal 1: When cross-carrier scheduling from sSCell to P(S)Cell is configured,

- For Type3-PDCCH CSS set, the configuration for DCI formats 2_5 and 2_6 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

- For USS set associated with fallback DCI formats (i.e., DCI format 0_0/1_0) scheduling P(S)Cell: FFS whether it remains in P(S)Cell regardless of cross-carrier scheduling configuration, or it can be monitored in the sSCell

- For USS set associated with non-fallback DCI formats (e.g., DCI format 0_1/0_2/1_1/1_2) scheduling P(S)Cell,  UE can monitor DCI formats 0_1, 1_1, 0_2, 1_2 on both PCell USS set(s) and sSCell USS sets simultaneously (i.e., Alt 2-1 as per previous agreement) based on SS set configuration for which cross-carrier scheduling or self-carrier scheduling can be configured per each USS set.

Proposal 2: When cross-carrier scheduling from sSCell to P(S)Cell is configured, UE counts the number of PDCCH candidates or non-overlapping CCEs from both P(S)Cell and sSCell and then, checks the PDCCH overbooking for the scheduled P(S)Cell based on the maximum number of PDCCH candidates or non-overlapping CCEs corresponding to (the SCS of) sSCell.

Proposal 3: Consider whether/how to handle the case when sSCell is deactivated or in dormant state.

- References

RAN1#103-e chairman’s note