3GPP TSG RAN WG1 #104-e			R1-2100945

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	7.2.4

Source: 	NEC

Title:	Remaining issues on resource allocation mode 2

Document for:	Discussion/Decision

- Introduction

This contribution is a revision of R1-2008081 of RAN1#103-e.

This contribution shared our TPs on the current sensing procedure in Rel.16 sidelink.

- Discussion

## Issue 1: n'-m≤Prsvp_RX'

In current sidelink sensing procedure specified in TS 38.214, when UE received higher layer trigger for resource (re-)selection at slot n, the corresponding sensing window is defined as the slots range of [n –T0,n–Tproc,0SL) where T0 is the start slot and Tproc,0SL is specified in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. As shown in table 8.1.4-1 below, Tproc,0SL may take value of 1 or 2 or 4 slots, which means there will exist 1 or 2 or 4 slots between the end slot of sensing window and the sensing trigger slot n (e.g., in Figure 1 below, trigger at slot n while sensing window ends before slot n-4).

Table 8.1.4-1: Tproc,0SL depending on sub-carrier spacing

| μSL | Tproc,0SL [slots] |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |

In current step 6), in order to exclude candidate resources that are occupied by other UEs, the procedures are:

6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

Figure 1. NR sensing window and resource selection window

Take figure 1 for example and assume Nmax = 1 for simplicity, if sensing UE detects SCI in slot m, there will be four cases to be considered because of different reservation periods from slot m: the reservations with black, cyan, red and green colors. According to step c):

- For the black one, because n'-m>Prsvp_RX', it falls into otherwise case, i.e., Q = 1, no resources within the selection window will be excluded. This is aligned between LTE and NR.

- For the green one, because n'-m≤Prsvp_RX' while Prsvp_RX>Tscal, it falls into otherwise case, i.e., Q = 1 and resource (n + b) within selection window will be excluded. This is aligned between LTE and NR.

- For the red one, because Prsvp_RX< Tscal and n'-m≤Prsvp_RX' , it falls into Q=TscalPrsvp_RX , so resources (n + 1) and (n + c) will be excluded, this is also aligned between LTE and NR.

- But for the cyan one, we think current spec by reusing LTE procedure will have bugs because there was no such gap slots (n-1, n-2, n-3, and n-4) in LTE, which exist in NR however. To be more specific,

The reservation case marked in cyan falls into the "otherwise" category and Q will only takes value "1" because of n'-m>Prsvp_RX'.

Then q=1 and only slot tm+1×Prsvp_RX'SL (i.e., slot (n -2) in the figure) is identified occupied while the other periodic reserved slots within resource selection window (i.e., slot (n + a) and slot (n + T2 - 2) in the example) cannot be identified occupied and will not be excluded from the selection window.

This will cause resource collisions if the sensing UE selects resources in slot (n + a) and slot (n + T2 - 2) for transmission. The right way is to treat the cyan one similar as red one and green one, i.e., let the cyan case fall into Q=TscalPrsvp_RX based on actual Prsvp_RX but not arbitrarily Q = 1 in this case.

Observation 1:

- There are Tproc,0SL gap slots between sensing window and trigger slot, which may have reservations within selection window and will not be excluded by current specification.

Based on the observation and discussion above, we propose following text proposals to solve the problems which are caused by reusing LTE excluding procedure:

Proposal 1:

- Adopt following text to TS 38.214 8.1.4:

c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …,Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-Tproc,0SL-m≤Prsvp_RX', where tn'-Tproc,0SLSL = n-Tproc,0SL if slot n-Tproc,0SL belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'- Tproc,0SLSL is the first slot after slot n-Tproc,0SL belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

## Issue 2: Tscal

Currently, Tscal is set to selection window size T2 converted to units of ms. However, as discussed in issue 1 section, Tproc,0SL gap slots should also be taken into account to calculate the number of possible reservations after sensing window, i.e., by Q=TscalPrsvp_RX. Otherwise, some reservations near the end of resource selection window will not be excluded. So, we propose that:

Proposal 2:

- Tscal is set to selection window size T2 converted to units of ms plus Tproc,0SL converted to units of ms if proposal 1 is adopted.

- Conclusion

In this contribution, we shared our concerns on current resource excluding procedure for NR sidelink and proposed that:

Observation 1:

- There are Tproc,0SL gap slots between sensing window and trigger slot, which may have reservations within selection window and will not be excluded by current specification.

Proposal 1:

- Adopt following text to TS 38.214 8.1.4:

c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …,Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-Tproc,0SL-m≤Prsvp_RX', where tn'-Tproc,0SLSL = n-Tproc,0SL if slot n-Tproc,0SL belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'- Tproc,0SLSL is the first slot after slot n-Tproc,0SL belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

Proposal 2:

- Tscal is set to selection window size T2 converted to units of ms plus Tproc,0SL converted to units of ms if proposal 1 is adopted.

- Reference

[1] 3GPP TS 38.214 V16.4.0 (2020-12).