3GPP TSG RAN WG1 #104-e			R1-2100986

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.4.3

Source:	InterDigital, Inc.

Title:	On HARQ enhancement for NTN

Document for:	Discussion and Decision

# Introduction

In RAN1 #103-e, the following agreement has been made as a progress for the HARQ enhancement for NTN [1]:

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until [X] after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- FFS: value of X and units in which it is defined.

- FFS: Whether TB of the two PDSCHs needs to be different

Agreement:

- Enhanced HARQ process ID indication is supported for DCI 0-2/1-2 and DCI 0-1/1-1 by at least one of following:

  - Option 1: Slot index as the MSB

  - Option 1-a: Slot index as the LSB

  - Option 2: Reusing one bit from other bit field

  - Option 3: Extending the HARQ process ID field up to 5 bits

- FFS: DCI 0-0/1-0

- Note: 32 is taken as maximal supported HARQ processes number for both UL and DL

Agreement:

HARQ codebook enhancement is supported as:

- For Type-2 HARQ codebook:

  - Option-1: Reduce codebook size with:

    - HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes

      - FFS: the details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

    - FFS: at least DCI for SPS release/SPS PDSCH

  - Option-2: No enhancement

  - Other options are not precluded.

- For Type-1 HARQ codebook, further discuss is needed with down selection among following options:

  - Option-1: No enhancement

  - Option-2: Report NACK on disabled process

  - Option-3: Reduce codebook size with criteria

- FFS: Enhancements for Type-3 HARQ codebook

In this contribution, we discuss on the remaining issues on HARQ enhancement for NTN.

# Discussion

## HARQ feedback enabling/disabling

During SI, RAN2 has agreed that HARQ feedback can be enabled/disabled per UE per HARQ process number. In the previous RAN1 meeting, RAN1 also made an agreement that the HARQ feedback enabling/disabling per UE per HARQ process number is configured via UE-specific RRC signaling.

The HARQ feedback disabling for a HARQ process could address the issue of the UE throughput performance loss due to long round trip delay with a limited HARQ process numbers, and the rely on the higher layer ARQ.

The L1 retransmission with HARQ feedback allows the scheduler to use an aggressive MCS selection for initial transmission as the retransmission could handle the failed TB reception, which in general improves the UE throughput. Therefore, in general, the PDSCH target BLER is assumed 10% (e.g., for eMBB) and its associated CQI is also derived based on the PDSCH target BLER.

When the HARQ feedback is disabled, it would be beneficial to lower the target BLER for PDSCH as no retransmission is available so that the number of retransmissions in upper layer can be reduced. Note that the retransmission in higher layer would increase the latency and resource waste significantly especially when frequent error occurs for TB transmission.

Observation-1: lowering target BLER for PDSCH when HARQ feedback is disabled is beneficial in terms of resource utilization and latency as it can reduce the number of retransmissions in higher layer

The HARQ feedback is also used for outer-loop link adaptation as the CQI feedback is typically not accurate since interference is changed dynamically, therefore the outer-loop link adaptation based on ACK/NACK has been used. However, if HARQ feedback is disabled and CQI feedback is based on the target BLER 10%, it is hard to determine MCS to meet a lower target BLER (e.g., 1%). Therefore, it would be beneficial to use a CQI table with a lower BLER target (but not as low as URLLC CQI table) when HARQ feedback is disabled.

Observation-2: use of a CQI table with a lower BLER target (e.g., 1%) could provide a better link adaptation with lower PDSCH BLER target when HARQ feedback is disabled

Therefore, a same CQI table with a different target BLER may be considered based on the HARQ feedback enabling/disabling status.

Proposal-1: a CQI table with a new target BLER (e.g., 1%) is considered when HARQ feedback is disabled

In addition, a blind retransmission could be also used to lower the target BLER of the initial transmission when HARQ is disabled for a HARQ process. A couple of options for the blind retransmission could be considered such as Rel-15 slot aggregation, multiple DCI with same TB scheduling, and single DCI scheduling multi-slot PDSCH with frequency hopping, and so forth. Since the Rel-15 slot aggregation is limited to the consecutive slots and the same frequency allocation, an enhancement of blind retransmission may be considered.

Proposal-2: blind retransmission is considered when HARQ feedback is disabled

## HARQ process number indication

It has been agreed in RAN1 #103-e that the maximum HARQ process number can be increased up to 32 for both UL and DL to address the throughput loss due to the long RTD in NTN. Since the maximum HARQ process number supported in current specification is 16, the options to indicate larger HARQ process number were discussed as following:

- Option 1: Slot index as the MSB

  - Option 1-a: Slot index as the LSB

- Option 2: Reusing one bit from other bit field

- Option 3: Extending the HARQ process ID field up to 5 bits

Among the Options, all Options except for the Option 3 tries to keep the same number of bits for the HARQ process number indication field while requiring additional standards efforts to identify additional details and/or potential issues. On the other hand, the Option 3 seems to be straightforward and not require any additional discussion to finalize the issue.

In addition, the gain from saving 1bit is not clear as the coverage of PDCCH may not be much different with saving 1bit considering the payload size of DCI formats. Also, the HARQ process ID bit field could be scalable based on the number of HARQ process IDs configured which is already supported in compact DCI. Therefore, there is no overhead issues unless the configured HARQ process number is greater than 16.

Proposal-3: support extending the HARQ process ID field up to 5 bits (i.e., Option 3)

## HARQ codebook enhancement

Following options were listed for further discussion as a HARQ codebook enhancement in RAN1 #103-e:

- For Type-1 HARQ codebook, further discuss is needed with down selection among following options:

  - Option-1: No enhancement

  - Option-2: Report NACK on disabled process

  - Option-3: Reduce codebook size with criteria

- For Type-2 HARQ codebook:

  - Option-1: Reduce codebook size with:

    - HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes

      - FFS: the details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

    - FFS: at least DCI for SPS release/SPS PDSCH

  - Option-2: No enhancement

  - Other options are not precluded.

Disabling of HARQ feedback for a HARQ process ID has been agreed as well as the increased maximum number of HARQ processes. Due to a larger HARQ process numbers, the HARQ codebook size optimization seems to be required to minimize the uplink coverage loss for the HARQ feedback. It also makes sense that a UE is not reporting HARQ information for the HARQ process ID disabled. Among the options, the Option-3 for Type-1 HARQ codebook and the Option-1 for Type-2 HARQ codebook seem to be appropriate in that regards.

Proposal-4: support Option-3 for Type-1 HARQ CB and Option-1 for Type-2 HARQ CB

# Summary

In this contribution, we discussed on remaining issues on the HARQ feedback enhancement for NTN. Based on the discussion, we have following observations and proposals:

Observation-1: lowering target BLER for PDSCH when HARQ feedback is disabled is beneficial in terms of resource utilization and latency as it can reduce the number of retransmissions in higher layer

Observation-2: use of a CQI table with a lower BLER target (e.g., 1%) could provide a better link adaptation with lower PDSCH BLER target when HARQ feedback is disabled

Proposal-1: a CQI table with a new target BLER (e.g., 1%) is considered when HARQ feedback is disabled

Proposal-2: blind retransmission is considered when HARQ feedback is disabled

Proposal-3: support extending the HARQ process ID field up to 5 bits (i.e., Option 3)

Proposal-4: support Option-3 for Type-1 HARQ CB and Option-1 for Type-2 HARQ CB

# References

- RAN1 #103e Chairman’s note