3GPP TSG-RAN WG1 Meeting #104-e	R1-2101091

e-Meeting, January 25th – February 05th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.213 CR on UE procedure for reporting HARQ-ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Physical Layer Enhancements for NR URLLC |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For simplicity, only one PUCCH with harq-ACK feedback is supported within a slot in release 15. In R16, in order to timely feedback harq-ACK and enable to support PDSCH retransmission with low latency, one of the primary task of the R16 URLLC UCI enhancement is to support sub-slot based PUCCH and multiple harq-ack feedbacks within a single uplinking slot. And the sub-slot length is indicated by RRC signaling subslotLengthForPUCCH in PUCCH-config. However, the current TS38.213 still shows a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot, which is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that a UE could transmit sub-slot based and multiple PUCCH with HARQ-ACK information in a slot. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification on UE behavior to feedback HARQ-ACK information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 38.213 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on RAN1’s common understanding, which has an impact on UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | None |  |  |  |  |  |  |  |  |

9.2.3  UE procedure for reporting HARQ-ACK

If UE is provided subslotLengthForPUCCH in PUCCH-Config, a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a subslot, otherwise, a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot

.

And there are some agreements here to support it.