3GPP TSG RAN WG1 #104-e                                          	         R1-2101094

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:                     Enhancement on Inter-cell Multi-TRP operations

Document for:	Discussion and Decision

Introduction

In RAN-86 meeting, the work item of Further enhancements on MIMO for NR [1] was approved. It aims to specify the further enhancements identified for NR MIMO. The detailed objectives are as follows:

- Extend specification support in the following areas [RAN1]

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

  - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception

In RAN1-102e meeting, the following agreements were reached for inter cell Multi-TRP operations [2]:

Agreement

Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation.

- Details on configuration of non-serving cell RS;

- Allowed source and target RS types for RS transmitted from the non-serving cell TRP;

- Allowed QCL types for RS transmitted from the non-serving cell TRP ;

- Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any;

- Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels;

- Other details not precluded.

In RAN1-103e meeting, the following agreements were reached for inter cell Multi-TRP operations [3]:

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

In this contribution, we discuss about the enhancement on inter-cell Multi-TRP operation.

Inter-cell Multi-TRP operation

Non-serving cell information

It was agreed in RAN-103e meeting that the information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information. Thus the non-serving cell information can include physical cell ID, SSB index and SSB configurations of neighboring cell. As for the definition of TCI state, it indicate the TCI state ID and the QCL-info. The information provided by QCL-info includes cell, bwp-Id and referenceSignal. Here cell refers to the serving cell index, referenceSignal refers to the NZP-CSI-RS-ResourceId or SSB-index. Since “Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”” has been agreed, it is straightforward to associate the “Non-serving cell information” with QCL-info. And the “Non-serving cell information” in QCI-info can be a neighborcellindex similar to existed ServCellIndex in QCI-info. It means that a new IE neighborcellindex should be added into QCI-info. And the mapping between physical cell ID of neighboring cell and the neighborcellindex should be indicated to UE.

Proposal 1: Add neighborcellindex into the definition of QCL-info. And the mapping relation between physical cell ID of neighboring cell and the neighborcellindex should be indicated to UE too.

Inter-cell beam management

In order to support the inter-cell multi-TRP operation, inter-cell beam management is necessary, which also introduces much more complexity. For UE with beam correspondence capability, it is enough to perform DL beam management or UL beam management only. But for UE without beam correspondence capability, both DL beam management and UL beam management are necessary.

Inter-cell beam management by UE

With inter-cell beam management by UE, UE need to measure the beam management reference signals (SSB or CSI-RS) of neighboring cell and send the measurement report to the serving gNB periodically. While for CSI-RS configuration from neighboring cell for beam management, CSI-RS configuration for L1 beam management from serving cell can be reused. The difference is that this CSI-RS is transmitted from non-serving cell TRP. Whether a CSI-RS is from serving cell or neighboring cell TRP is transparent to UE with no spec impact. Thus only enhancement on SSB from non-serving cell is needed for non-serving cell RS.

Proposal 2: SSB from non-serving cell can be supported to be configured as non-serving cell RS.

In addition, considering the UL signaling overhead, beam measurement report need to be further studied because of inter-cell beam management. Both non-group based and group based beam measurement report should be considered as a baseline. And group based beam measurement report should be considered with higher priority. With group based beam measurement report, it is possible that in the group there are two RSs associated with serving cell and neighboring cell respectively.

Proposal 3: Group based beam reporting is slightly preferred for inter-cell beam pairing.

Inter-cell beam management by gNB

With inter-cell beam management by gNB, UE need not to measure the beam management reference signal. But UE need to transmit the beam measurement reference signal, such as SRS. It is preferred to reuse the SRS for other use cases if possible, for example, reuse beam management SRS for serving cell. In this case, there will be not more impact on UE side. It is the neighboring gNB that measures the beam management reference signal and send the measurement report to the UE’s serving gNB.

Thus inter-cell beam management by both UE and gNB should be supported. And inter-cell beam management by gNB is much more preferred.

Proposal 4: Inter-cell beam management by gNB can be supported.

Multi-DCI for Inter-cell Multi-TRP operation

The assumption for inter-cell Multi-TRP operation is Multi-DCI based inter-cell PDSCH reception. Thus the CORESETPoolIndex for CORESETs of neighboring cell should be different from that of CORESETs from serving cell. But as noted in TS 38.213, the maximum number of monitored PDCCH candidate per slot is defined per serving cell as well as the maximum number of non-overlapped CCEs. If inter-cell Multi-TRP operation is supported, whether the PDCCH candidate associated with neighboring cell CORESETs should be considered as same as that of serving cell or not? If it is considered as same as that of serving cell,  the sum of the number of monitored PDCCH candidates from serving cell and neighboring should be no more than the maximum number in TS 38.213. While for the number of non-overlapped CCE, it is also need to limit the sum of CCEs from serving cell and neighboring cell.

Proposal 5: Whether the PDCCH candidate or CCE from CORESETs associated with neighboring cell should be considered as same as that of serving cell or not when calculating the maximum number of monitored PDCCH candidates and the maximum number of non-overlapped CCE.

Conclusion

In this contribution, we discuss about the inter-cell Multi-TRP operation. Based on above discusses, we provide the following proposals.

Proposal 1: Add neighborcellindex into the definition of QCL-info. And the mapping relation between physical cell ID of neighboring cell and the neighborcellindex should be indicated to UE too.

Proposal 2: SSB from non-serving cell can be supported to be configured as non-serving cell RS.

Proposal 3: Group based beam reporting is slightly preferred for inter-cell beam pairing.

Proposal 4: Inter-cell beam management by gNB can be supported.

Proposal 5: Whether the PDCCH candidate or CCE from CORESETs associated with neighboring cell should be considered as same as that of serving cell or not when calculating the maximum number of monitored PDCCH candidates and the maximum number of non-overlapped CCE.

References

- RP-193133, “New WID: Further enhancements on MIMO for NR”

- RAN1-102e meeting chairman notes.

- RAN1-103e meeting chairman notes.