3GPP TSG RAN WG1 #104-e						          R1-2101100

e-Meeting, January 25th – February 5th, 2021

Agenda item:	8.13.1

Source:	Xiaomi

Title:	Discussion on Cross-carrier scheduling from SCell to PCell

Document for:	Discussion

Introduction

A new work item on “New WID on NR Dynamic spectrum sharing (DSS)” was approved in RAN#86 and the objectives are as follows [1].

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS):

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In RAN1#103-e meeting, the following agreements were achieved for cross-carrier scheduling from SCell to PCell/PSCell) [2].

Agreement:

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell

Conclusion:

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Conclusion:

- When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5).

Agreements:

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured. Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a USS set configured for scheduling of PCell/PSCell)

Search space sets

Table 1 summarizes Rel-15/16 search space, corresponding RNTI/DCI format, and search space location [3].

Table 1 Search space, corresponding RNTI/DCI format, and location

| Search space | RNTI | DCI format | Rel-15/16 search space location (PCell/SCell) |
| --- | --- | --- | --- |
| Type0 CSS | SI-RNTI | 1_0 | PCell of the MCG |
| Type0A CSS | SI-RNTI | 1_0 | PCell of the MCG |
| Type1 CSS | RA-RNTI, MsgB-RNTI | 1_0 | PCell /PSCell |
|  | TC-RNTI | 1_0, 0_0 | PCell /PSCell |
| Type2 CSS | P-RNTI | 1_0 | PCell of the MCG |
| Type3 CSS | INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, AI-RNTI | 2_0, 2_1, 2_2, 2_3, 2_4, 2_5 | PCell/SCell or SCell configured by high layer |
|  | PS-RNTI | 2_6 | PCell /PSCell |
|  | C-RNTI, MCS-C-RNTI, CS-RNTI(s) | 0_0, 0_1, 1_0, 1_1 | PCell /PSCell |
| USS | C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, SL-L-CS-RNTI | 0_0, 0_1, 0_2, 1_0, 1_1, 1_2, 3_1 | PCell/SCell |

Group common DCI format 2_5 in Type 3 CSS can be configured on PCell/PSCell or SCell. So there are no additional issues to decide the location of Type 3 CSS with DCI format 2_5.

Proposal 1: When sSCell scheduling PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI format 2_5 and applicability of the information in the DCI format are the same as in Rel-16.

Group common DCI format 2_6 in Type 3 CSS can be only configured on PCell/PSCell because this format is used for SCell dormancy indication. If DCI format 2_6 in Type 3 CSS can be configured on the SCell used for scheduling PCell/PSCell, whether this SCell should be always active like PCell/PSCell and whether this SCell can be indicated as dormancy should be discussed. From this perspective, it is prefered to keep the DCI format 2_6 on Pcell/PSCell only.

Proposal 2: When sSCell scheduling PCell/PSCell is configured, UE monitors DCI format 2_6 in Type 3 CSS set  only on the PCell/PSCell and not on the sSCell.

When sSCell scheduling PCell/PSCell is configured, how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s) is discussed in the last RAN1 meeting. Below alternatives can be considered in the discussion.

- Alt 1: UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

- Alt 2: UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

- Alt 2-1:

  - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

- Alt 2-2:

  - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

    - FFS: Details of switching mechanism

  - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

- Alt 2-3:

  - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

- Alt 2-4:

  - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

From the perspective of UE processing complexity, UE monitoring both on PCell/PSCell and sSCell is not preferred. It should be a feature that can be activated/deactivated by network under the configuration of the feature, instead of always activating the feature when it is configured. By this way, network can have more scheduling flexibility on UE-specific search space, such as switch the scheduling cell back to the ‘PCell/PSCell’ upon the case of beam failure on the special SCell, and switch the scheduling cell to only the special SCell upon the case for UE to reduce power consumption.

Proposal 3: When sSCell scheduling PCell/PSCell is configured, UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously.

Proposal 4: When sSCell scheduling PCell/PSCell is configured, Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS is supported.

PDCCH monitoring

In this WI, there is a restriction that the total PDCCH blind decoding budget should not be changed as a result of this work. There are two PDCCH blind decoding budget, one is  or , the other is  or . For each scheduled cell, the UE is not required to monitor on the active DL BWP with SCS configuration  of the scheduling cell more than  PDCCH candidates or more than  non-overlapped CCEs per slot.

If the fallback DCI is always on the P(S)Cell, PDCCH blind decoding for this P(S)Cell will be performed on the P(S)Cell. Meanwhile, in case that PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell is supported, PDCCH blind decoding for this P(S)Cell will be additional performed on the SCell. That means for this P(S)Cell, PDCCH blind decoding will be performed on two cells and the total blind decoding for this P(S)Cell may be increased.

For another parameter of the budget in the current spec, if a UE is configured with  downlink cells with DL BWPs having SCS configuration , where , a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells. In case that PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell is supported, PDCCH blind decoding will be performed on two cells for this P(S)Cell. In this case, parameter  will be calculated twice.

Observation 1: The total blind decoding for P(S)Cell may be increased if the PDCCH blind decoding is performed on the two cells.

Proposal 5: For SCell scheduling PCell, further consider how to calculate/configure PDCCH candidates of USS to meet the BD/CCE budget.

Conclusions

In this contribution, we discuss Cross-carrier scheduling from SCell to P(S)Cell, the following observation and proposals have been made:

Observation 1: The total blind decoding for P(S)Cell may be increased if the PDCCH blind decoding is performed on the two cells.

Proposal 1: When sSCell scheduling PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI format 2_5 and applicability of the information in the DCI format are the same as in Rel-16.

Proposal 2: When sSCell scheduling PCell/PSCell is configured, UE monitors DCI format 2_6 in Type 3 CSS set  only on the PCell/PSCell and not on the sSCell.

Proposal 3: When sSCell scheduling PCell/PSCell is configured, UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously.

Proposal 4: When sSCell scheduling PCell/PSCell is configured, Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS is supported.

Proposal 5: For SCell scheduling PCell, further consider how to calculate/configure PDCCH candidates of USS to meet the BD/CCE budget.

References

- 3GPP RP-193260 “New WID on NR Dynamic spectrum sharing (DSS)”

- Chairman’s Notes RAN1#103-e

- 3GPP TS 38.213 v16.2.0 "NR; Physical layer procedures for control "