3GPP TSG-RAN WG1 Meeting #104-e		R1-2101148

e-Meeting, January 25th – February 5th, 2021

Title:	[DRAFT] Reply LS on SL switching priority

Response to:	R1-2100009 (R4-2017839)

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	vivo [RAN1]

To:	RAN4

Cc:

Attachments:

# Contact Person:

# Name:	               Zichao Ji

E-mail Address:	zichao.ji@vivo.com

# 1. Overall Description:

RAN1 thanks RAN4 for the LS regarding TX switching between LTE Sidelink and NR Sidelink. RAN1 would like to provide answers to the following questions:

  - Question 1: Is there priority defined for LTE SL and NR SL?

    - Answer: From RAN1 perspective, there is no priority defined per interface (i.e., a specific priority value associated with either LTE SL or NR SL). However, there is priority defined per channel/signal for transmission/reception in either LTE SL or NR SL. The details of priority handling are defined in section 16.2.4, TS38.213.

  - Question 2: How does RAN WG1 define the priority of LTE SL and NR SL? For example, which parameter is used and how to determine the priority?

    - Answer: For transmission/reception of each channel/signal associated with a TB, the priority is defined according to the logical channel priority defined in TS38.321. Especially, the priority of MAC CE is fixed to ‘1’ as defined in TS38.321. For NR S-SSB or LTE PSBCH/SSSS/PSSS, the priority is configured by higher layer parameter sl-SSB-PriorityNR-r16 and sl-SSB-PriorityEUTRA-r16, respectively.

  - Question 3: Is it the case that there is no higher priority for LTE SL than NR SL?

    - Answer: No, it is possible that transmission/reception of a channel/signal in LTE SL has higher priority than that in NR SL.

# 2. Actions:

To RAN4

RAN1 respectfully asks RAN4 to take the above information into account in related work.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #104-bis-e			April 12th – April 20th, 2021   	 					e-meeting

TSG RAN WG1 Meeting #105-e			May 19th – May 27th, 2021 	    		     			e-meeting