3GPP TSG RAN WG1#104-e		R1-2101152

e-Meeting, January 25th – February 5th, 2021

Source:	vivo

Title:	Discussion on Paging enhancement

Agenda Item:	5

Document for:	Discussion and Decision

- Background

In RAN2#112-e, RAN2 discussed UE grouping for paging enhancement in Rel-17 UE power saving WI. RAN2 confirmed that UE grouping is considered as a candidate of paging enhancements for UE power saving. Regarding paging for UE subgroups, RAN2 has discussed and considered the following methods:

- Paging indication for UE subgroups using paging DCI, with either same-slot or cross-slot scheduling;

- Paging early indication (PEI) / wake-up signal (WUS) for UE subgroups;

- UE subgroup indication by using multiple P-RNTIs;

- Paging for UE subgroups using different time/frequency resources.

From RAN2 perspective, the last two methods are de-prioritized. Notice that these methods are not mutually exclusive.

- Discussion on subgrouping on PEI and Paging DCI

In RAN 1#103-E meeting, multiple companies had provided simulation results on this issue and the following observation is agreed,

Option 1: For Paging indication for UE subgroups using paging DCI,

Agreements:

Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication within a PO can provide the following power saving gains w.r.t. Rel-16:

- If the original group paging rate is 10%:

  - [0.3%] - [1.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.4%] - [0.8%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.3%] - [1.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 80% and showed following results:

  - [0.7%] - [7.6%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.8%] - [3.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.5%] - [4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The number of UE sub-groups evaluated ranges from 2 to 16.

Some companies show concern on assuming group paging rate larger than 60%.

Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement.

Option 2: for Paging early indication (PEI) / wake-up signal (WUS) for UE subgroups,

Agreements:

Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication carried in paging early indication can provide the following power saving gains w.r.t Rel-16:

- If the original group paging rate is 10%:

  - [10.6%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [16.0%] –[36.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [14.3%] –[46.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results:

  - [8.0%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [18.1%] –[34.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [20.6%] –[42.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The additional power saving gains w.r.t. paging early indication without UE sub-grouping are given as follows:

- If the original group paging rate is 10%:

  - [0.6%] –[2.7%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.6%] –[4.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.6%] –[4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results:

  - [1.3%] –[8.0%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [2.1%] –[13.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [3.3%] –[16.1%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The number of UE sub-groups evaluated ranges from 2 to 16.

The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB.

Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement.

Note: Not all sources providing results for paging early indication without UE sub-grouping also provide results for paging early indication with UE sub-grouping.

It is clearly option 1(a.k.a. subgrouping on Paging DCI) can provide marginal power saving gain about 0.3 - 1.1%, while option 2(a.k.a. subgrouping on PEI) can provide significant power saving gain about 10.6 - 46% assuming PO rate is 10%. The big power saving gain difference between these two options exists even with higher PO rate.

To this extend, it is proposed that the sub-grouping indication by using paging PDCCH is not supported. And a corresponding reply LS (R2-2010884) to RAN2 is needed as attched.

Proposal: the sub-grouping indication by using paging PDCCH is not supported. A reply LS of R2-2010884 to RAN2 is proposed.

- Conclusion

Proposal: the sub-grouping indication by using paging PDCCH is not supported. A reply LS of R1-2100020(R2-2010884) to RAN2 is proposed.

References

- R2-2010884, LS on Paging Enhancement, RAN2

- R1-2100452, Paging enhancements for idle/inactive mode UE power saving, vivo