3GPP TSG-RAN WG1 Meeting #104-e		R1-210xxxx

e-Meeting, January 25th – February 5th, 2021

Title:	[DRAFT] Reply LS on Paging Enhancement

Response to:	R1-2100020 (R2-2010884)

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	vivo [RAN1]

To:	RAN2

Cc:

Attachments:

# Contact Person:

# Name:

E-mail Address:

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS on paging enhancement. Regarding the RAN2 prioritized two options for UE grouping for paging enhancement in Rel-17 UE power saving WI, RAN1 has the following observations,

For Paging indication for UE subgroups using paging DCI,

Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication within a PO can provide the following power saving gains w.r.t. Rel-16:

- If the original group paging rate is 10%:

  - [0.3%] - [1.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.4%] - [0.8%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.3%] - [1.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 80% and showed following results:

  - [0.7%] - [7.6%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.8%] - [3.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.5%] - [4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The number of UE sub-groups evaluated ranges from 2 to 16.

Some companies show concern on assuming group paging rate larger than 60%.

Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement.

For Paging early indication (PEI) / wake-up signal (WUS) for UE subgroups,

Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication carried in paging early indication can provide the following power saving gains w.r.t Rel-16:

- If the original group paging rate is 10%:

  - [10.6%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [16.0%] –[36.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [14.3%] –[46.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results:

  - [8.0%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [18.1%] –[34.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [20.6%] –[42.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The additional power saving gains w.r.t. paging early indication without UE sub-grouping are given as follows:

- If the original group paging rate is 10%:

  - [0.6%] –[2.7%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.6%] –[4.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.6%] –[4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results:

  - [1.3%] –[8.0%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [2.1%] –[13.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [3.3%] –[16.1%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The number of UE sub-groups evaluated ranges from 2 to 16.

The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB.

Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement.

Note: Not all sources providing results for paging early indication without UE sub-grouping also provide results for paging early indication with UE sub-grouping.

Hence, RAN1 recommends that the sub-grouping indication by using paging PDCCH is not supported.

# 2. Actions:

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in related work.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #104-bis-e			April 12th – April 20th, 2021   	 					e-meeting

TSG RAN WG1 Meeting #105-e		May 19th – May 27th, 2021 	    		     	e-meeting