3GPP TSG RAN WG1 #104e 		R1-2101156

# e-Meeting, January 25th – February 5th, 2021

Title:	[Draft] Reply LS to RAN4 on measuring CSI-RS during SCell activation

Response to:	R4-2017381

Release:	Rel-16

Work Item:	NR_unlic

Source:	vivo [RAN1]

To:	RAN4

Cc:

# Contact Person:

# Name:	                 	Gen Li

# E-mail Address:	reagan.li@vivo.com

# 1. Overall Description:

RAN1 would like to thank RAN4 for the LS R4-2017381 on the information on measuring CSI-RS during SCell. Regarding the questions included in the RAN4 LS, .

(1)   When none of the RRC parameters CO-DurationPerCell-r16, SlotFormatIndicator, and CSI-RS-ValidationWith-DCI-r16 is configured for a UE on the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell?

Answer: UE will proceed with the P/SP CSI-RS measurement and report on the being-activated SCell.

(2)   When RRC parameters CSI-RS-ValidationWith-DCI-r16 is configured, but SlotFormatIndicator and CO-DurationPerCell-r16 are not configured for the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell? Does UE need to decode a DCI format from other active serving cell (indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot) for this being-activated SCell to validate this P/SP CSI-RS?

Answer: Yes, UE proceeds with the P/SP CSI-RS measurement in the set of symbols of the slot if the UE detects a DCI format indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot; otherwise, UE cancels the CSI-RS reception.

(3)   When RRC parameters CO-DurationPerCell-r16 is configured but SlotFormatIndicator is not configured for the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell? Does UE need to decode a DCI format 2_0 (indicating remaining channel occupancy duration) from other active serving cell for this being-activated SCell to validate the CSI-RS?

Answer: Yes, UE proceeds with the P/SP CSI-RS measurement in the set of symbols of the slot if the UE decodes a DCI format 2_0 indicating the set of symbols are within remaining channel occupancy duration; otherwise, UE cancels the CSI-RS reception.

(4)   When RRC parameters CO-DurationPerCell-r16 is not configured but SlotFormatIndicator is configured for the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell? Does UE need to detect a DCI format 2_0 (indicating the starting point of CO duration and the slot format) from other active serving cell for this being-activated SCell to validate the CSI-RS?

Answer: Yes, UE proceeds with the P/SP CSI-RS measurement in the set of symbols of the slot if the UE decodes a DCI format 2_0 indicating the set of symbols are downlink symbols; otherwise, UE cancels the CSI-RS reception.

# 2. Actions:

# To RAN4

# ACTION: RAN1 respectfully ask RAN4 to take the above information into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104-bis-e,	April. 12, 2020– April. 20, 2021