3GPP TSG-RAN WG1 Meeting #104-e	R1-2101162

# e-Meeting, January 25th – February 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft]CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction of RRC parameter name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 38.331, following RRC parameter enableBeamSwitchTiming-r16 is defined, however in 38.214 there are multiple occasions where this parameter is captured as “enableBeamSwitchTiming-r16” or “enableBeamSwitchTiming”. For better readability and alignment with 38.331, mulitple occasions of “enableBeamSwitchTiming” are changed to “enableBeamSwitchTiming-r16” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multiple occasions of “enableBeamSwitchTiming” are changed to “enableBeamSwitchTiming-r16” in sections 5.2.1.5.1, 5.2.1.5.1a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC parameter name is not aligned with 38.331 and readability is poor. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.5.1, 5.2.1.5.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is editorial CR, no backward compability issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', 'periodic', or 'semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP. A UE is not expected to receive more than one DCI with non-zero CSI request per slot. A UE is not expected to be configured with different TCI-StateId's for the same aperiodic CSI-RS resource ID configured in multiple aperiodic CSI-RS resource sets with the same triggering offset in the same aperiodic trigger state. A UE is not expected to receive more than one aperiodic CSI report request for transmission in a given slot. If a UE does not indicate its capability of CSItriggerStateContainingNonactiveBWP the UE is not expected to be triggered with a CSI report for a non-active DL BWP. Otherwise, when a UE is triggered with a CSI report for a DL BWP that is non-active when expecting to receive the most recent occasion, no later than the CSI reference resource, of the associated NZP CSI-RS, the UE is not expected to report the CSI for the non-active DL BWP and the CSI report associated with that BWP is omitted. When a UE is triggered with aperiodic NZP CSI-RS in a DL BWP that is non-active when expecting to receive the NZP CSI-RS, the UE is not expected to measure the aperiodic CSI-RS. In the carrier of the serving cell expecting to receive that associated NZP CSI-RS, if the active DL BWP when receiving the NZP CSI-RS is different from the active DL BWP when receiving the triggering DCI,

-	the last symbol of the PDCCH span of the DCI carrying the BWP switching shall be no later than the last symbol of the PDCCH span of the DCI carrying the CSI trigger, irrespective of whether they are in the same carrier of a serving cell or not and irrespective of whether they are in the same SCS or not;

-	the UE is not expected to have any other BWP switching in that carrier after the last symbol of the PDCCH span covering the DCI carrying the CSI trigger and before the first symbol of the triggered NZP CSI-RS or CSI-IM.

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the UE provides beamSwitchTiming-r16, enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameter repetition, or is smaller than the UE reported threshold beamSwitchTiming-r16, when enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'on'.

-	If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS triggered by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS triggered by a PDCCH associated with the same coresetPoolIndex as the PDCCH triggering the aperiodic CSI-RS and scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS;

-	else, the UE applies the QCL parameter(s) of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH triggering that aperiodic CSI-RS, in the latest slot in which one or more CORESETs are associated with the same value of coresetPoolIndex as the PDCCH triggering that aperiodic CSI-RS

-	else if a UE is configured with enableTwoDefaultTCIStates and at least one TCI codepoint is mapped to two TCI states

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS. If there is a PDSCH indicated with two TCI states in the same symbols as the CSI-RS, the UE applies the first TCI state of the two TCI states when receiving the aperiodic CSI-RS.

-	else, the UE applies the first one of two TCI states corresponding to the lowest TCI codepoint among those mapped to two TCI states and applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received when receiving the aperiodic CSI-RS.

-	else if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and when enableBeamSwitchTiming-r16 is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info , aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48 when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming-r16 is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming-r16 is provided;

-	else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored;

-	else if the UE is configured with enableDefaultBeamForCCS and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided and the NZP-CSI-RS-ResourceSet is not configured with higher layer parameter trs-Info, or is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and the NZP-CSI-RS-ResourceSet is configured with higher layer parameter trs-Info, or is equal to or greater than 48 when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info, or is equal to or greater than the UE reported threshold beamSwitchTiming-r16, when enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'on', the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

-	The UE is not expected to receive aperiodic CSI-RS and PDSCH/aperiodic CSI-RS associated with different values of coresetPoolIndex in overlapped symbol(s). The UE is not expected to receive aperiodic CSI-RS and semi-persistent/periodic CSI-RS with different 'QCL-type D' in overlapped symbol(s).

-	A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

For a UE configured with the higher layer parameter CSI-AperiodicTriggerStateList, if a Resource Setting linked to a CSI-ReportConfig has multiple aperiodic resource sets, only one of the aperiodic CSI-RS resource sets from the Resource Setting is associated with the trigger state, and the UE is higher layer configured per trigger state per Resource Setting to select the one CSI-IM/NZP CSI-RS resource set from the Resource Setting.

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. The aperiodic CSI-RS is transmitted in a slot , , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and in slot , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16,

-	Nslot, offset, PDCCHCA is the Nslot, offsetCA which is determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, CSIRSCA is the Nslot, offsetCA which is determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5.

The UE does not expect that aperiodic CSI-RS is transmitted before the OFDM symbol(s) carrying its triggering DCI. When the minimum scheduling offset restriction is applied, UE is not expected to be triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min.

If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

If the UE is configured with a single carrier for uplink, the UE is not expected to transmit more than one aperiodic CSI report triggered by different DCIs on overlapping OFDM symbols.

5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameter repetition, or is smaller than the UE reported threshold beamSwitchTiming-r16 + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, when enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'on', where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

-	if one of the associated trigger states has the higher layer parameter qcl-Type set to 'typeD',

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48} and when enableBeamSwitchTiming-r16 is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the UE provides beamSwitchTiming-r16  and enableBeamSwitchTiming-r16 is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' and scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols when enableBeamSwitchTiming-r16 is provided;

-	else,

-	if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else if the UE is configured with enableDefaultBeamForCCS, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet is equal to or greater than the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided and the NZP-CSI-RS-ResourceSet is not configured with higher layer parameter trs-Info, or is equal to or greater than the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values of {14,28,48} and the NZP-CSI-RS-ResourceSet is configured with higher layer parameter trs-Info, or is equal to or greater than 48+d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info, or is equal to or greater than the UE reported threshold beamSwitchTiming-r16 + d 2μCSIRS/2μPDCCH in CSI-RS symbols when enableBeamSwitchTiming-r16 is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'on', where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

Table 5.2.1.5.1a-1: Additional beam switching timing delay d

| µPDCCH | d [PDCCH symbols] |
| --- | --- |
| 0 | 8 |
| 1 | 8 |
| 2 | 14 |

Aperiodic CSI-RS timing:

-	When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16, including the case that the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or minimumSchedulingOffsetK2-r16 for any UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16,

-	μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and  μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5

-	If the µPDCCH < µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than the first symbol of the CSI-RS carrier's slot that starts at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

-	If the µPDCCH > µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

Table 5.2.1.5.1a: Ncsirs as a function of the subcarrier spacing of the triggering PDCCH

| µPDCCH | Ncsirs [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | [14] |