3GPP TSG RAN WG1#104-e			R1-2101183

e-Meeting, 25th January – 5th February, 2021

Agenda item:	7.2.6

Source: 	Samsung

Title: 	On UL full power transmission

Document for:	Discussion and Decision

# Introduction

The following are captured in the NR specification regarding TPMI group signaling for mode 2.

TS 38.331 [1]

-- R1 16-5c-3: TPMI group for Mode 2 ul-FullPwrMode2-TPMIGroup-r16 SEQUENCE { twoPorts-r16 BIT STRING(SIZE(2)) OPTIONAL, fourPortsNonCoherent-r16 ENUMERATED{g0, g1, g2, g3} OPTIONAL, fourPortsPartialCoherent-r16 ENUMERATED{g0, g1, g2, g3, g4, g5, g6} OPTIONAL

TS 38.306 [2]

| ul-FullPwrMode2-TPMIGroup-r16 Indicates the UE supported TPMI group(s) which delivers full power. The capability signalling comprises the following values: - twoPorts-r16 indicates a 2-bit bitmap - fourPortsNonCoherent-r16 indicates the TPMI groups {G0-3} - fourPortsPartialCoherent-r16 indicates the TPMI groups (G0-6) UE indicates support of this feature shall also indicate support of ul-FullPwrMode2-MaxSRS-ResInSet. NOTE 1: When a full coherent UE operates in mode 2, it reports TPMIs the same as a partial-coherent UE. NOTE 2: For 4 port partial-coherent or full-coherent UE, UE can report: 2-port {2-bit bitmap} and one of 4-port non-coherent {G0~G3} and one of 4-port partial-coherent {G0~G6} For 4 port non-coherent UE, UE can report: 2-port {2-bit bitmap} and one of 4-port non-coherent {G0~G3} For 2 port UE, UE can report: 2-port {2-bit bitmap} NOTE 3: A UE that supports this feature must report at least one of the values. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

We propose the following clarifications in TS 38.306.

- The NOTE 2 in 38.306 needs some clarification. For 4 port non-coherent UE, the note says that “UE can report: 2-port {2-bit bitmap} and one of 4-port non-coherent {G0~G3}.” Due to the text “the UE can report…” in the beginning, there could be the following misunderstanding.

  - First, it is not clear from the text that the a non-coherent UE is not allowed to report 4-port partial-coherent {G0~G6}.

  - Second, whether the UE can report 2-port {2-bit bitmap} only or one of 4-port non-coherent {G0~G3} only needs to be clarified.

The second misunderstanding exists also for the 4 port partial-coherent UE. A simple fix for this could be by revising NOTE 2 as follows.

NOTE 2: For 4 port partial-coherent or full-coherent UE, UE can only report: 2-port {2-bit bitmap} or/and one of 4-port non-coherent {G0~G3} or/and one of 4-port partial-coherent {G0~G6} For 4 port non-coherent UE, UE can only report: 2-port {2-bit bitmap} or/and one of 4-port non-coherent {G0~G3} For 2 port UE, UE can report: 2-port {2-bit bitmap}

- Based on 38.331, fourPortsNonCoherent-r16 and fourPortspartialCoherent-r16 can indicate only one TPMI group from g0 through g6. However, their description in 38.306 implies that the UE indicates a set of multiple groups {g0-g3} for non-coherent and {g0-g6} for partial-coherent. It should be clarified by adding a text “one of” in their description in 38.306.

- The 38.331 uses lowercase g0-g6 for the TPMI groups, but 38.306 uses uppercase G0-G6. For consistency, it is better to align the capitalization. For instance, the capitalization in 38.306 can be replaced with lowercase.

A TP based on the above clarification is presented in Section 2.

# TP on TPMI group signaling

The TP based on the clarification in Section 1 is as follows.

Proposal 1: adopt the following TP for TS 38.306 v16.3.0

| ul-FullPwrMode2-TPMIGroup-r16 Indicates the UE supported TPMI group(s) which delivers full power. The capability signalling comprises the following values: - twoPorts-r16 indicates a 2-bit bitmap - fourPortsNonCoherent-r16 indicates one of the TPMI groups {Gg0-3} - fourPortsPartialCoherent-r16 indicates one of the TPMI groups (Gg0-6) UE indicates support of this feature shall also indicate support of ul-FullPwrMode2-MaxSRS-ResInSet. NOTE 1: When a full coherent UE operates in mode 2, it reports TPMIs the same as a partial-coherent UE. NOTE 2: For 4 port partial-coherent or full-coherent UE, UE can only report: 2-port {2-bit bitmap} or/and one of 4-port non-coherent {G0~G3} or/and one of 4-port partial-coherent {G0~G6} For 4 port non-coherent UE, UE can only report: 2-port {2-bit bitmap} or/and one of 4-port non-coherent {G0~G3} For 2 port UE, UE can report: 2-port {2-bit bitmap}} NOTE 3: A UE that supports this feature must report at least one of the values. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Conclusion

In this contribution, the following TP is proposed.

Proposal 1: adopt the following TP for TS 38.306 v16.3.0

| ul-FullPwrMode2-TPMIGroup-r16 Indicates the UE supported TPMI group(s) which delivers full power. The capability signalling comprises the following values: - twoPorts-r16 indicates a 2-bit bitmap - fourPortsNonCoherent-r16 indicates one of the TPMI groups {Gg0-3} - fourPortsPartialCoherent-r16 indicates one of the TPMI groups (Gg0-6) UE indicates support of this feature shall also indicate support of ul-FullPwrMode2-MaxSRS-ResInSet. NOTE 1: When a full coherent UE operates in mode 2, it reports TPMIs the same as a partial-coherent UE. NOTE 2: For 4 port partial-coherent or full-coherent UE, UE can only report: 2-port {2-bit bitmap} or/and one of 4-port non-coherent {G0~G3} or/and one of 4-port partial-coherent {G0~G6} For 4 port non-coherent UE, UE can only report: 2-port {2-bit bitmap} or/and one of 4-port non-coherent {G0~G3} For 2 port UE, UE can report: 2-port {2-bit bitmap}} NOTE 3: A UE that supports this feature must report at least one of the values. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

# References

- TS 38.331, v16.3.1

- TS 38.306, v16.3.0