3GPP TSG RAN WG1#104-e			R1-2101192

e-Meeting, 25th January – 5th February, 2021

Agenda item:	8.1.4

Source: 	Samsung

Title: 	Views on Rel. 17 CSI enhancements

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s view on the two Rel. 17 CSI enhancement items included in the WID [1]: CSI enhancements for NC-JT transmission, and CSI enhancements for FDD reciprocity.

# NC-JT CSI enhancements

In RAN1#102-e [2], it was agreed to study NC-JT CSI enhancements under the following two categories:

Agreement For CSI enhancement for multi-TRP, study following aspects taking into account trade-off among UE complexity, performance and reporting/RS overhead - Category 1 - For a reporting setting CSI-ReportConfig, more than one CSI-RS port groups in a resource or resources or resource sets are associated to different TRPs/TCI states, - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/UE-selected channel and interference hypotheses across TRPs /TCI states - and then report one or more CSIs within a single CSI report. - Category 2 – Within an implicit/explicit set of reporting settings CSI-ReportConfigs, which are associated to different TRPs/TCI states, - the UE will determine CSI reporting quantities based on pre-defined/indicated/configured/ UE-selected channel and interference hypotheses - and then report multiple CSIs with multiple CSI reports (including one or more CSIs per report or selected CSI with single CSI report) - Other enhancement are not excluded, e.g. CQI enhancements for multi-TRP transmission including CQI format, CQI reporting mechanism Note that companies are encouraged to clarify applicable transmission schemes/scenarios and strive to unify Rel-17 MTRP CSI framework enhancements

Each category fits for different deployment scenario for multi-TRP. Category 1 is suitable for the scenario where multiple TRPs are connected via an ideal or low-latency backhaul. In this case, CSIs of multiple TRPs can be transmitted via a single PUCCH/PUSCH of which UL beam targets for one of the TRP. This is exactly same with the case where the HARQ-ACK is transmitted in single-DCI based framework, or joint HARQ-ACK is transmitted in multi-DCI based framework. Meanwhile, Category 2 is suitable for the case where multiple TRPs are connected via a non-ideal backhaul. In this case, CSIs of multiple TRPs would be transmitted with separate PUCCH/PUSCH targeting for the respective TRPs. This is exactly same with the case where separate HARQ-ACK is transmitted in multi-DCI based framework. Hence, our understanding is that Category 1 is applicable for both single- and multi-DCI based framework while Category 2 is applicable for multi-DCI based framework where UCI needs to be transmitted separately per TRP. NW configured with multi-DCI based framework would know which is suitable between Category 1 or 2, and it can inform UE which category to be used by a higher layer signaling.

In Rel-16 CSI framework, a CSI-RS resource can be used as a CMR and/or NZP-IMR. To accurately measure CSI for NC-JT transmission, CMR and NZP-IMR should be measured together as a pair depicted in Figure 1. To reduce the RS overhead, it is desirable to reuse CMR as NZP-IMR as well. Rel-16 specification already allows such reuse only when CSI-RS is precoded, since NZP-IMR needs to be precoded such that each port corresponds to the interference transmission layer. To provide more flexibility on CSI resource configuration, it is beneficial to allow non-precoded CSI-RS for CMR to be reused as IMR.

Figure 1. CMR and IMR configuration for NC-JT CSI measurement.

Proposal 1: On CSI enhancements for multi-TRP,

- Support Category 1 for single-DCI based multi-TRP

- Allow UE to be configured between Category 1 and 2 for multi-DCI based multi-TRP

- Support CMR to be re-used as IMR for both non pre-coded and pre-coded CSI-RS

In RAN1#103-e [3], it was agreed to study CSI report configured by a single CSI reporting settings as follows:

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report - two RIs, two PMIs, two LIs and one CQI per codeword, for single-DCI based NCJT when the maximal transmission layers is less than or equal to 4 - FFS: Maximal transmission layers larger than 4 - FFS: Whether/how a subset of above reporting quantities are allowed to be configured to the UE - FFS: whether/how to support two RIs, two PMIs, two LIs and two CQIs, for multi-DCI based NCJT - FFS: whether/how to support CRI(s) to be reported in a CSI - FFS: restrictions among reported CSI quantities, e.g. among reported RIs and PMIs - FFS: whether/how to support non-PMI based port-selection - FFS: whether/how to support single value of reported LI Note that other NCJT CSI measurement/reporting enhancement for other scenarios is not precluded, e.g. for HST-SFN

When coordinated scheduler is available for multi-TRP, NW would dynamically switch between NC-JT and non-NC-JT transmission according to the traffic condition and channel quality. For such operation, non-NC-JT CSI report can be configured as well as NC-JT CSI report but it is redundant since only one of those reports are utilized for data scheduling. Another approach is to allow UE to choose only one of those reports according to channel condition and omit the others from reporting. If {CMR, IMR} pairs for non-NC-JT CSI and those for NC-JT CSI are configured together within a same resource setting, such omission can be done implicitly by CRI. One example of such resource setting is depicted in Figure 2. On each {CMR, IMR} pair in Figure 2, CMR indicates the CSI-RS from corresponding TRP, and IMR would indicate CSI-RS/CSI-IM according to the non-NC-JT or NC-JT interference hypothesis. If UE would like to report NC-JT CSI, two corresponding {CMR, IMR} pairs would be chosen, else a single {CMR, IMR} pair will be selected to report non-NC-JT CSI. The chosen {CMR, IMR} pairs can be indicated to gNB by the CRI value. Further, since NC-JT CSI would be burdensome for UE to calculate appropriate precoder, non-PMI based port-selection and some restrictions among reported RIs or PMIs can be beneficial in order to reduce the burden of UE side and CSI report quantity.

Figure 2. Example of CSI resource setting for dynamic NC-JT CSI reporting

Also, optimized UCI structure for dynamics NC-JT CSI report is also beneficial and can be extended by using Rel-16 two parts UCI structure as a baseline. For example, for CRI based solution, the amount of UCI for the proposed CSI report can vary much according to the selected CRI value. If the selected CRI value is for NC-JT CSI, the UCI would contain two sets of {RI, PMI, CQI} for cooperating TRPs. Otherwise, the UCI would contain one set of {RI, PMI, CQI} for a single TRP. To handle the varying amount of UCI, we can extend two-part UCI structure in Rel-16 for the NC-JT CSI report. For example, UCI comprises a two parts (UCI#1, UCI#2), where

- UCI#1 is always reported, has fixed payload, and comprises (1) partial CSI for two TRPs and (2) an indication about remaining CSI for two TRPs included in UCI2. Note that (2) determines the payload of UCI2; and

- UCI#2 has a variable payload, and comprises remaining CSI for two TRPs.

Proposal 2: For NC-JT CSI reporting enhancement, support and study followings:

- Support CRI-based dynamic reporting between NC-JT and non-NC-JT CSI

- Support non-PMI based port-selection

- Support restrictions among reported RIs or PMIs

- Study UCI structure optimized for dynamic NC-JT CSI report

The proposed CSI report requires additional CSI computational complexity to take into account the mutual interference in NC-JT. As CPU occupation rule in the current spec is designed for legacy CSI, the proposed CSI report needs new CPU occupation rule considering the additional CSI computational complexity. Practical implementation aspects should be taken into account in designing the new CPU occupation rule.

Proposal 3: Design new CPU occupation rule for dynamic NC-JT CSI report

We provide a preliminary SLS result to evaluate the gain by the proposed NC-JT CSI reporting. In the proposed CSI reporting, UE reports both or one of NC-JT and non-NC-JT CSIs from the two best TRPs having the highest RSRP. From those CSIs, NW schedules NC-JT or DPS dynamically according to the reported CSI and NW traffic conditions. For a fair comparison, we set the two baseline schemes:

- Scheme 1 (DPS): UE always reports non-NC-JT CSI from the two best TRPs. NW schedules DPS according to the reported CSI.

- Scheme 2 (non-NC-JT CSI only): UE reports non-NC-JT CSI only from the two best TRPs. NW schedules NC-JT according to the non-NC-JT CSI so that CSI mismatch would occur. When one of the two best TRPs is not available for scheduling, NW schedules a UE via DPS.

Table 1 shows the UPT gain by NC-JT with proposed CSI reporting compared to the baseline schemes. We can observe that proposed CSI reporting achieves substantial 50% and cell-edge UPT gain compared to scheme 2 (case with CSI mismatch), 36.42% and 16.41% respectively, which implies that proposed CSI reporting is necessary to enhance NW throughput when NC-JT is used.

Table 1. UPT gains of NC-JT by proposed CSI reporting.

|  | 5% UPT |  | 50% UPT |  | Mean UPT |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Value | Gain over DPS | Value | Gain over DPS | Value | Gain over DPS |
| Scheme 1 (DPS) | 87.0 Mbps | - | 162.9 Mbps | - | 169.0 Mbps | - |
| Scheme 2 (NC-JT w. CSI mismatch) | 89.8 Mbps | 3.25% | 185.0 Mbps | 13.59% | 206.9 Mbps | 22.38% |
| Proposed (NC-JT w. dynamic CSI report) | 104.5 Mbps | 20.19% (+16.41% from scheme 2) | 252.4 Mbps | 54.95% (+36.42% from scheme 2) | 252.1 Mbps | 49.15% (+21.87% from scheme 2) |

Observation 1: NC-JT scheduling according to dynamic NC-JT CSI report provides substantial UPT gain versus that according to non-NC-JT CSI (with the CSI mismatch).

CSI enhancements for FDD

## Baseline and reference performance

For study and design of the Rel. 17 codebook alternatives, the best possible (codebook) scheme from Rel.15/16 should be considered as a reference performance. In this subsection, we present simulation results to compare different schemes based on Rel. 15/16 NR codebooks (CBs) in order to determine the best possible scheme. The following Rel. 15/16 codebooks are compared.

- Rel. 15 Type I (T1), Config1

- Rel. 15 regular Type II (reg. T2)

- Rel. 16 reg. T2

- Rel. 16 port selection (PS) T2

  - DFT-based CSI-RS beamforming

  - Ideal (Eigenvector-based) CSI-RS beamforming

The UPT vs overhead tradeoff results for these codebooks are shown in Figure 1, where the simulation assumptions are according to the agreed EVM in RAN1#102-e, which are also copied in Table 2 in the appendix. We can make the following observation from these results.

Observation 2: based on UPT vs overhead tradeoff, Rel. 16 reg. T2 CB is better than Rel. 16 PS T2 CB for both DFT-based and ideal CSI-RS beamforming

- Note: ideal beamforming is a performance outer bound

Based on the above observation, we propose the following.

Proposal 4: for the study of Rel. 17 codebook alternatives, use Rel. 16 reg. T2 CB as a reference performance, in addition to the Rel. 16 PS T2 CB “baseline”

Figure 1

## Discussion on Rel. 17 codebook alternatives

The following agreement was made in RAN1#103-e [3] about the Rel. 17 codebook alternatives at least for rank 1.

Agreement Study following alternatives, and select one or a combination of multiple alternatives for Rel-17 in RAN1#104-e:

A summary of our view on these alternatives is provided in Table 1, wherein for the purpose of discussion, the alternatives in the agreement can be categorized based on the following discussion points.

- P1: CSI-RS port to SD-FD bases mapping (Of=1 vs Of>1)

  - Of=1 (i.e. 1-to-1 mapping)

  - Of>1 (i.e. many-to-one mapping, multiple SD-FD bases per CSI-RS port)

- P2: Codebook structure

  - W2 or W2Wf^H

  - W1W2 (i.e. Rel. 15 type)

  - W1W2Wf^H (Rel. 16 type)

- P3: Wf

  - DFT (i.e. Rel. 16 type Wf)

  - PS (i.e. new port selection Wf)

Table 1

| CB structure |  | W1 |  | Wf | View |
| --- | --- | --- | --- | --- | --- |
|  |  | Of | Selection |  |  |
| W2 | Alt0 | 1 or >1 | Identity (all ports or bases are selected) |  | For small number of CSI-RS ports (e.g. 4,8), W1 can be identity |
| W2Wf^H | Alt0 | 1 or >1 | No SD selection | FD PS or DFT |  |
| R15 type: W1W2 | Alt1 | 1 | CSI-RS ports |  | Simplest in terms of CB design |
|  | Alt2 | >1 | SD-FD bases |  | Concern |
| R16 type: W1W2Wf^H | Alt3-0 | 1 | CSI-RS ports | DFT | Very similar to R16 CB |
|  | Alt3-1 | >1 |  | DFT | Concern |
|  | Alt3-2 | >1 |  | FD PS | Concern |
|  | Alt4 | 1 | CSI-RS port groups | FD PS | R16 FD DFT bases are replaced with FD PS |
|  | Alt5 | >1 | SD-FD bases | DFT | Concern |

The first discussion point (P1) is about the mapping between CSI-RS ports and SD-FD bases. Let Of be the number of SD-FD bases mapped to each CSI-RS port. Then, in legacy (e.g. in Rel. 15/16 PS T2 codebooks) mapping, Of=1, i.e., each CSI-RS port maps to one beamforming vector (1-to-1 mapping). Based on the discussion in RAN1#103-e, some companies proposed to enhance this mapping by considering Of>1, i.e., multiple beamforming vectors are mapped to each CSI-RS port (many-to-one mapping), resulting in Alt2, 3-1, 3-2, and 5. In our view, there are several issues that need to be studied/addressed carefully in order to justify this enhancement. Some of issues are discussed as follows.

- WID scope: the first issue is about the scope of this enhancement. If Of>1 is indeed supported, then the scope of this work is not limited to codebook design only. It has impact on other aspects also (cf. the second issue). It is unclear whether this additional work can be accommodated based on the current WID.

- Specification impact: the second issue is about potential large specification impact due to a more involved CSI reporting mechanism (when compared with Rel. 15/16 based CSI reporting). In particular, the specification impact at least includes the Rel. 17 codebook design, UE behavior for PMI and CQI calculation, CSI-RS port processing, and R>1 (if supported).

- Performance: the third issue is related to performance loss when compared with Of=1 (cf. Section 3.3). Since multiple SD-FD bases are conveyed via one CSI-RS port, the resolution of conveying each SD-FD basis is reduced (e.g. if FDM based scheme is used, then the available set of REs needs to be subsampled/distributed across multiple SD-FD bases), and this can lead to performance loss, which can be large.

- UE complexity: The fourth issue is related to the additional UE complexity for this feature since it requires new UE implementation for CSI-RS port processing (the legacy CSI-RS port processing won’t work), and more involved calculations for PMI, CQI and R>1 (if supported).

- Unclear need and benefit: the fifth issue is unclear need and benefits that can be achieved with this feature. Based on the discussion in RAN1#103-e, the proponents argued about the following.

  - Supporting large number SD-FD bases (e.g. > 32): if reciprocity indeed exists, then there is no need for supporting large #beam-formed ports since #dominant angle-delay profiles is likely to be small. In particular, a maximum of 32 SD-FD bases (that can already be supported) is sufficient.

  - CSI-RS overhead reduction: the CSI-RS overhead issue can be handled by gNB implementation of beam-formed CSI-RS. For example, gNB can form groups of multiple UEs (based on their SRS based channel estimation), and obtain a common set of SD-FD bases for each group of UEs. The common set of SD-FD bases can then be used to beam-form CSI-RS ports that are common across the corresponding set of UEs, just like non-precoded CSI-RS case. It is clear that CSI-RS overhead can be reduced due the grouping of UEs that share the common set of beam-formed CSI-RS ports.

In summary, the codebook alternatives that are based on Of>1 feature have unclear benefits. Even so, when considered together with all the metrics (e.g. performance, UE complexity, overhead, specification work etc.), this feature lacks justification.

Observation 3: conveying multiple SD-FD bases per CSI-RS port (Of>1) have at least the following issues.

- WID scope: the current WID may not accommodate this feature

- Specification impact: at least on Rel. 17 codebook, PMI, CQI, and R>1 (if supported)

- Performance: potentially large performance loss when compared with legacy scheme (Of=1)

- UE complexity: requires new CSI-RS port processing, and more involved UE behaviour (implementation) for PMI, CQI, and R>1 (if supported)

- Unclear need and benefits: the need for supporting > 32 SD-FD bases is unclear, and CSI-RS overhead saving can be handled by gNB implementation

Based on the above observation, we propose the following.

Proposal 5: codebook alternatives (Alt2, 3-1, 3-2, and 5) that are based on conveying multiple SD-FD bases per CSI-RS port (Of>1) require further study and justification, hence should be deprioritized.

For the discussion points (P2 and P3) below, we focus on the remaining codebook alternatives: Alt0, Alt1, Alt3-0, and Alt4.

The second discussion point (P2) is about the codebook structure. The codebook alternatives from the agreement can be categorized into the following codebook structures:

- W2 or W2Wf^H: this correspond to Alt0 wherein the W1 component (for the SD port selection) is removed, which essentially means that all CSI-RS ports are selected for PMI calculation, hence W1 can be fixed to be an identity matrix, and can be removed from the codebook. In our view, for small number of CSI-RS ports (e.g. 4, 8), W1 can be identity since the likelihood that the CSI-RS port selection will bring gains is small.

- W1W2 (i.e. Rel. 15 type): this corresponds to Alt1. In our view, this is the simplest codebook design alternative since it includes the least number of components, namely, port selection via W1 and WB coefficients for the selected ports via W2. More importantly, it can perform better or similar (if not better) to the other category (i.e. W1W2Wf^H), as shown in Section 3.3.

- W1W2Wf^H (Rel. 16 type): this corresponds to Alt3-0 and Alt4. Alt3-0 is very similar to the Rel. 16 PS T2 codebook. The differences perhaps include free port selection (as opposed to restricted selection), and configurable Wf. Regarding free vs restricted port selection, the difference can be marginal if the gNB can perform beam-forming of CSI-RS ports in an order (e.g. based on power levels or eigenvalues associated with the eigenvectors). In addition, the CSI-RS beam-forming in Rel. 16 PS T2 codebook can be based on SD-FD bases (since it is up to gNB, hence there is no restrictions such as the beam-forming is only in SD). So, the difference between Alt3-0 and the Rel. 16 PS T2 codebook is marginal, i.e., only in terms of Wf, whether it is reported as in Rel. 16, and is configured (or/and reported) in Alt3-0. This difference is marginal, and may not have any impact on performance. Now, the only alternative left in this category is Alt4. When compared with Alt1, Alt4 is either worse (e.g. when DFT based CSI-RS beamforming is used) or similar (e.g. when Eigen beam-forming is used) in performance, as shown in Section 3.3.

The third discussion point (P3) is about Wf, whether Wf is DFT (i.e. Rel. 16 type Wf) or PS (i.e. new port selection Wf). If Wf is DFT, then we have Alt3-0, which is very similar to the Rel. 16 PS T2 codebook, hence as explained above, need not be specified. When Wf is PS, then we have Alt3-2, but its performance is worse than Alt1 due to Of>1, as shown in Section 3.3.

Based on the above discussion on P2 and P3, we make the following observation and proposal.

Observation 4: regarding codebook structure,

- W1 can be identity when number of CSI-RS ports is small

- W1W2 is the simplest in terms codebook design

- W1W2Wf^H is not preferred since it is either very similar to Rel. 16 PS T2 codebook (cf. Alt3-0 where Wf is DFT) or can incur performance loss (cf. Alt3-2 where Wf is PS and Of>1)

Proposal 6: for Rel. 17 codebook design, support Alt1 (W=W1W2)

- Alt0 (W=W2) can be supported when number of CSI-RS ports is small

## Simulation results

For performance evaluation, we provide simulation results for the following schemes.

- Reference codebook: R15 T1, Config1

- Baseline codebooks: R16 reg. T2 and Rel. 16 PS T2

- R17 enhancements

  - Codebook alternatives according to agreement in RAN1#103-e

  - paramCombination = 1-8 (same as Rel. 16).

The simulation assumptions are according to the agreed EVM in RAN1#102-e, which are also copied in Table 2 in the appendix. As reference, Rel. 15 Type I, Config 1 is considered.

In the first evaluation, we provide simulation results comparing Of=1 and Of=4, wherein the FDM scheme is considered for conveying multiple SD-FD bases per CSI-RS port when Of=4. In particular, the following codebook alternatives from the agreement in RAN1#103-e are evaluated.

- W1W2 CB structure: Alt1, Of=1 vs. Alt2, Of=4

- W1W2Wf^H CB structure: Alt4, Of=1 vs. Alt3-2, Of=4

The UPT vs overhead tradeoff results are shown in Figure 2 through Figure 5. We can observe the following.

Observation 5: Conveying multiple SD-FD bases per CSI-RS port (Of=4) incurs large loss (up to 8%) when compared with Of=1 for both DFT and ideal CSI-RS beamforming.

Figure 2

Figure 3

Figure 4

Figure 5

In the second evaluation, we provide simulation results comparing the two codebook structures: W1W2 vs W1W2Wf^H. In particular, the following codebooks are evaluated.

- W1W1 codebook structure: Alt1 (RAN1#103-e)

- W1W2Wf^H codebook structure: Alt4 (RAN1#103-e)

The UPT vs overhead tradeoff results for these codebooks are shown in Figure 6 and Figure 7. We can observe the following.

Observation 6: based on UPT vs overhead tradeoff,

- For DFT beamforming,

  - Alt4 shows marginal gain over R16 PS T2, but is worse than Alt1

  - Alt1 achieves similar performance-overhead tradeoff as R16 regular T2

- For ideal beamforming, Alt1 and Alt4 achieve similar performance

Figure 6

Figure 7

Conclusion

In this contribution, the following observations and proposals are made:

NC-JT CSI enhancements

Proposal 1: On CSI enhancements for multi-TRP,

- Support Category 1 for single-DCI based multi-TRP

- Allow UE to be configured between Category 1 and 2 for multi-DCI based multi-TRP

- Support CMR to be re-used as IMR for both non pre-coded and pre-coded CSI-RS

Proposal 2: For NC-JT CSI reporting enhancement, support and study followings:

- Support CRI-based dynamic reporting between NC-JT and non-NC-JT CSI

- Support non-PMI based port-selection

- Support restrictions among reported RIs or PMIs

- Study UCI structure optimized for dynamic NC-JT CSI report

Proposal 3: Design new CPU occupation rule for dynamic NC-JT CSI report

Observation 1: NC-JT scheduling according to dynamic NC-JT CSI report provides substantial UPT gain versus that according to non-NC-JT CSI (with the CSI mismatch).

FDD CSI enhancements

Observation 2: based on UPT vs overhead tradeoff, Rel. 16 reg. T2 CB is better than Rel. 16 PS T2 CB for both DFT-based and ideal CSI-RS beamforming

- Note: ideal beamforming is a performance outer bound

Observation 3: conveying multiple SD-FD bases per CSI-RS port (Of>1) have at least the following issues.

- WID scope: the current WID may not accommodate this feature

- Specification impact: at least on Rel. 17 codebook, PMI, CQI, and R>1 (if supported)

- Performance: potentially large performance loss when compared with legacy scheme (Of=1)

- UE complexity: requires new CSI-RS port processing, and more involved UE behaviour (implementation) for PMI, CQI, and R>1 (if supported)

- Unclear need and benefits: the need for supporting > 32 SD-FD bases is unclear, and CSI-RS overhead saving can be handled by gNB implementation

Observation 4: regarding codebook structure,

- W1 can be identity when number of CSI-RS ports is small

- W1W2 is the simplest in terms codebook design

- W1W2Wf^H is not preferred since it is either very similar to Rel. 16 PS T2 codebook (cf. Alt3-0 where Wf is DFT) or can incur performance loss (cf. Alt3-2 where Wf is PS and Of>1)

Observation 5: Conveying multiple SD-FD bases per CSI-RS port (Of=4) incurs large loss (up to 8%) when compared with Of=1 for both DFT and ideal CSI-RS beamforming.

Observation 6: based on UPT vs overhead tradeoff,

- For DFT beamforming,

  - Alt4 shows marginal gain over R16 PS T2, but is worse than Alt1

  - Alt1 achieves similar performance-overhead tradeoff as R16 regular T2

- For ideal beamforming, Alt1 and Alt4 achieve similar performance

Proposal 4: for the study of Rel. 17 codebook alternatives, use Rel. 16 reg. T2 CB as a reference performance, in addition to the Rel. 16 PS T2 CB “baseline

Proposal 5: codebook alternatives (Alt2, 3-1, 3-2, and 5) that are based on conveying multiple SD-FD bases per CSI-RS port (Of>1) require further study and justification, hence should be deprioritized.

Proposal 6: for Rel. 17 codebook design, support Alt1 (W=W1W2)

- Alt0 (W=W2) can be supported when number of CSI-RS ports is small

# References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- Chairman’s notes, RAN1#102-e

- Chairman’s notes, RAN1#103-e

Appendix

Table 2: Simulation assumptions for FDD reciprocity

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL |
| Inter-BS distance |  | 200m |
| Reciprocity model |  | Based on Section 5.3 of TR 36.897, to generate FDD DL and UL channels |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 44 dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz with 15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO, rank 1 only |
| MIMO layers |  | Up to 4 layers |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | CSI-RS, DMRS |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% for SU/MU-MIMO, rank 1 only |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | User throughput vs CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 regular and PS eTypeII codebooks |
| SRS modeling for UL channel estimation |  | SRS periodicity with 5ms SRS error modeling in Table A.1-2 in 36.897 with Δ=9 BW: same as CSI-RS Number of ports = 2 Tx power = 23 dBm |
| FDD DL/UL calibration error model at gNB |  | According to R1-144943, with amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop at time, and constant per 4 PRB at frequency. |