3GPP TSG RAN WG1 #104             	                                            	R1-2101195

e-Meeting, January 25th – February 5th, 2020

Agenda Item:		8.2.2

Source:			Samsung

Title:			PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz

Document for:		Discussion and decision

# 1 Introduction

Revised WID approved in RAN#90-e [1] supports the following features of PDCCH monitoring enhancements

- Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

This contribution discusses issues and techniques to support PDCCH monitoring with high SCS for NR from 52.6GHz to 71GHz, and corresponding limitations to PDCCH configurations.

# 2 PDCCH Monitoring Enhancements

NR Rel-15 supports slot-based PDCCH monitoring with blind decodes (BD) and CCE limits defined per slot for SCS of 15KHz, 30KHz, 60KHz, and 120KHz. The corresponding BD and CCE limits are defined in Table 10.1-2 and Table 10.1-3 from TS 38.213 [2].

Table 10.1-2: Maximum number  of monitored PDCCH candidates per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of monitored PDCCH candidates per slot and per serving cell |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-3: Maximum number  of non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

In NR Rel-16, span-based PDCCH monitoring are supported for Ultra-Reliable Low-Latency Communication (URLLC) traffic, where the BD and CCE limits are defined in Table 10.1-2A and Table 10.1-3A from TS 38.213 [2] for different UE capabilities. Three types of PDCCH monitoring capabilities with combination (X, Y) are supported. The span gap, X, can be 2 or 4 or 7 symbols, while the span duration, Y, is 2 or 3 symbols for SCS configuration, μ=0,or 1.

Table 10.1-2A: Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 14 | 28 | 44 |
| 1 | 12 | 24 | 36 |

Table 10.1-3A: Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 18 | 36 | 56 |
| 1 | 18 | 36 | 56 |

However, NR from 52.6GHz to 71GHz would require higher SCS to overcome phase noise and use the same FFT size as FR1/2 at high carrier frequency. Therefore, new BD and CCE limits for the higher SCS, such as 480KHz and 960KHz should be defined in NR Rel-17.

Observation 1: New BD and CCE limits with high SCS (480KHz and 960KHz) for NR from 52.6 GHz to 71 GHz is needed.

The TTI in terms of a span duration or a slot duration decreases linearly with respect to the increase of SCS. Given the same UE PDCCH monitoring capability as NR Rel-15/16, the PDCCH monitoring overhead within a TTI would increase dramatically due to the short TTI for high SCS. The large PDCCH monitoring burden within a short TTI can be an issue for UE implementation in practice, such that UE is not able to process the same number of BDs/CCEs per slot due to the shorter slot duration.

Observation 2: PDCCH monitoring burden is high due to short TTI at high SCS (480KHz and 960KHz).

## Multi-slot span based PDCCH monitoring

Multi-slot span based PDCCH monitoring can be considered to address the large PDCCH monitoring burden within a short TTI at high SCS.  The time separation between two consecutive PDCCH monitoring slots or spans can be extended to a gap over multiple slots. So UE is able to relax PDCCH processing in a PDCCH monitoring slot or span over the extended gap.

Follow the same principle from Rel-16, a combination (X, Y) can be reused, such that X limits the PDCCH monitoring gap, while Y limits a PDCCH monitoring duration.  Both X and Y can be extended from a number of symbols to a number of slots. The minimum PDCCH monitoring gap X should be larger than one slot, so that UE can distribute PDCCH processing/monitoring burden over multiple slots. For maximum PDCCH monitoring span, Y, the applicable value for Y can be same as Rel-15, i.e. one slot, slot-based PDCCH monitoring. Alternatively, Y can also be multiple slots to provide more PDCCH monitoring occasions and higher scheduling flexibility to NW. As UE expects much narrower beam direction from 52.6GHz to 71GHz compared with FR1 or FR2, the additional occasions when Y is larger than one slot can be used to for PDCCH receptions associated with different beam directions.

In practice, UE can support multiple applicable values for combination (X, Y). The larger X value is, the more PDCCH monitoring burden reduction UE achieves. To provide more configuration or scheduling flexibility to NW, multiple combinations of (X, Y) can be supported for multi-slot span based PDCCH monitoring at high SCS, such as 480KHz and 960KHz. The combination (X, Y) can either be determined based on UE capability or predetermined for applicable SCS configurations, such as , μ=5,or 6.

Proposal 1: Support multi-slot span based PDCCH monitoring based on combination (X, Y), where the minimum PDCCH monitoring gap X is larger than one slot, and the maximum PDCCH monitoring span Y is one or more slots, for SCS of 480KHz and 960KHz.

However, there are some potential issues with multi-slot span based PDCCH monitoring. Firstly, the extended span gap will increase latency. A scheduling delay of (X - Y) slots can be large for some cases, such that X is much larger than Y, or Y is small, e.g. 1. In addition, there will be some loss of data rate if only single PDSCH/PUSCH scheduling per slot is supported. To overcome those issues, adaptation on combination (X, Y) can be considered when a UE is capable of supporting multiple combinations (X, Y). For example, when a UE reports a capability of multiple combinations (X, Y), the UE can be indicated with a selected combination (X, Y) from the multiple combinations based on L1 signaling. The UE can deactivate or activated some PDCCH monitoring occasions according to the PDCCH configuration limitations based on the selected combination (X, Y). Meanwhile, UE can perform joint adaptation on maximum number of BDs based on the indicated (X, Y), if the BD/CCE budget is defined per combination (X, Y),

In addition, a UE can report its preferred minimum multi-slot span gap, X, and/or maximum multi-slot span Y according to UE requirements on power savings, latency, and data rate.

Proposal 2: Support adaptation and UE assistance information report for X and/or Y when UE supports multiple combinations (X, Y).

## BD/CCE budgets for high SCS

The BD/CCE limits at high SCS can be defined either per slot or per multi-slot span depending on PDCCH monitoring capabilities. For legacy slot based PDCCH monitoring, the value of maximum number MPDCCHmax,slot, μ of monitored PDCCH candidates per slot and maximum number CPDCCHmax,slot, μ of non-overlapped CCEs per slot for high SCS should be smaller than the values for lower SCS in Table 10.1-2 and Table 10.1-3, respectively.

For multi-slot span based PDCCH monitoring, the BD/CCE limit can be defined per combinations of (X, Y). Similar as multi-symbol span based PDCCH monitoring in NR Rel-16, the maximum number MPDCCHmax,(X,Y), μ of monitored PDCCH candidates and maximum number CPDCCHmax,(X,Y), μ of non-overlapped CCEs are determined according to the selection of multi-slot span gap, X, and multi-slot span duration, Y. The larger X or Y a UE supports, the larger values of BD/CCE limits the UE may expect.

As MPDCCHmax,slot, μ and CPDCCHmax,slot, μ are quite small for SCS of 120KHz, there is no much room to reduce the BD/CCE limit for higher SCS. PDCCH blocking may become an issue when the BD/CCE limit is too small. Therefore, it’s better to consider BD/CCE limits per multi-slot span for high SCS, such as μ=5, or 6.

Proposal 3: Support maximum number MPDCCHmax,(X,Y), μ of PDCCH candidates per multi-slot span for combination (X, Y), where X >1 slots, Y>=1 slots, and μ=5, or 6.

Proposal 4: Support maximum number CPDCCHmax,(X,Y),μ of non-overlapped CCEs per multi-slot span for combination (X, Y), where X >1 slots, Y>=1 slots, and μ=5, or 6.

# Limitation of PDCCH configurations

In this section, some limitations to PDCCH configuration are discussed in order to support multi-slot span based PDCCH monitoring and new BD/CCE limits as proposed in Section 2.

## Search space set configuration

To support multi-slot span based PDCCH monitoring as proposed in Section 2.1, the UE would expect that time gap between any two consecutive PDCCH monitoring spans should not be smaller than the multi-slot span gap, X, while any PDCCH monitoring span duration is up to Y slots. In NR, PDCCH monitoring occasions are determined according to configured search space sets, where PDCCH monitoring periodicity, ks, and duration, Ts, are configured in terms of a number of slots for a search space set s. The two consecutive PDCCH monitoring spans can either be configured PDCCH monitoring occasions from the same search space set or be configured PDCCH monitoring occasions from different search space sets. Therefore, both intra search space set span gap and inter search space set span gap should be considered.

Figure 1 illustrates an example of multi-slot span based PDCCH monitoring with combination of (X = 4, Y =2), the configuration of search space set 3 is invalid because the two consecutive PDCCH monitoring occasions from search space set 1 and search space set 3 is smaller than X, but is not smaller than Y.

Figure 1: Illustration of search space set configurations limited by combination of (X = 4, Y =2).

For intra search space set span gap, it’s obvious that the PDCCH monitoring periodicity of ks slots should be limited by the multi-slot span gap X, while PDCCH monitoring duration of Ts slots should be limited by multi-slot span, Y. Therefore, a limitation on search space set configuration should be supported, such that PDCCH monitoring periodicity is not smaller than multi-slot span gap, i.e. ks≥X, while PDCCH monitoring duration is not larger than multi-slot span, i.e. Ts≤Y.

For inter search space set span gap, two consecutive PDCCH monitoring occasions from different search space sets may belong to different PDCCH monitoring spans. In this case, the gap between the two consecutive PDCCH monitoring occasions from search space set i  and search space set j is limited by multi-slot span gap, X, such that |(ki-Oi)-(kj-Oj)|≥X, where Oi and Oj are offsets for search space set i  and search space set j. In another case, the two consecutive PDCCH monitoring occasions from different search space sets can belong to the same PDCCH monitoring span. In the latter case, the gap between the two consecutive PDCCH monitoring occasions from search space set i and search space set j should be limited by multi-slot span duration, Y, such that |(ki-Oi)-(kj-Oj)|<Y.

In practice, it’s much more challenging to restrict the inter search space set span gap than that for intra search space set span gap. To avoid misunderstanding between NW and UE on valid PDCCH monitoring occasions, it’s necessary to specify that UE is not required to monitor configured PDCCH occasion which breaks the PDCCH configuration limitations indicated by combination (X, Y).

Proposal 5: For multi-slot span based PDCCH monitoring based on combination (X, Y), support limitations on search space set configurations, including

- PDCCH monitoring periodicity, ks,

- PDCCH monitoring duration, Ts.

## PDCCH candidates allocation/dropping per multi-slot span

Currently, NR supports PDCCH candidates allocation/dropping per slot or span in a slot as specified in TS38.213 [2]. Given the PDCCH allocation/dropping rule, a UE determines PDCCH candidates to monitor based on the configuration of search space set for each PDCCH monitoring slot or span within a slot. When configured PDCCH candidates or non-overlapped CCEs exceeds BD or CCE limit, UE will drop remaining search space set with higher index.

Figure 2: Illustrating of PDCCH candidates allocation with combination (X = 4, Y =2).

For multi-slot span based PDCCH monitoring as illustrated in Figure 2, a span for PDCCH monitoring can be over multiple slots based on combination (X, Y).  The PDCCH candidates allocation/dropping rule from Rel-16 should be extended to support PDCCH monitoring in a multi-slot span.

Proposal 6: Support PDCCH candidates allocation/dropping per a span over multiple slots.

# 4. Conclusion

This contribution considered PDCCH monitoring enhancements and limitations to PDCCH configurations for NR from 52.6GHz to 71GHz.  Following proposals and observation were made:

Observation 1: New BD and CCE limits with high SCS (480KHz and 960KHz) for NR from 52.6 GHz to 71 GHz is needed.

Observation 2: PDCCH monitoring burden is high due to short TTI at high SCS (480KHz and 960KHz).

Proposal 1: Support multi-slot span based PDCCH monitoring based on combination (X, Y), where the minimum PDCCH monitoring gap X is larger than one slot, and the maximum PDCCH monitoring span Y is one or more slots, for SCS of 480KHz and 960KHz.

Proposal 2: Support adaptation and UE assistance information report for X and/or Y when UE supports multiple combinations (X, Y).

Proposal 3: Support maximum number MPDCCHmax,(X,Y), μ of PDCCH candidates per multi-slot span for combination (X, Y), where X >1 slots, Y>=1 slots, and μ=5, or 6.

Proposal 4: Support maximum number CPDCCHmax,(X,Y),μ of non-overlapped CCEs per multi-slot span for combination (X, Y), where X >1 slots, Y>=1 slots, and μ=5, or 6.

Proposal 5: For multi-slot span based PDCCH monitoring based on combination (X, Y), support limitations on search space set configurations, including

- PDCCH monitoring periodicity, ks,

- PDCCH monitoring duration, Ts.

Proposal 6: Support PDCCH candidates allocation/dropping per a span over multiple slots.

# 5. Reference

[1] RP-202925  WID: Extending current NR operation to 71GHz

[2] 3GPP TS 38.213 v16.4.0 Physical layer procedure for control