3GPP TSG RAN WG1 #104-e	R1-2101200

e-Meeting, January 25th – February 5th, 2021

Agenda item:	8.2.7

Source:	Samsung

Title:	Discussion on the synchronization raster for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

# Introduction

This contribution discusses synchronization raster and its related issues for NR from 52.6 GHz to 71 GHz, and the number of synchronization raster entries for a given band will further impact the decision on the default subcarrier spacing(s) used by the UE for initial cell search.

# Rel-15 and Rel-16 Synchronization Raster

There have been two types of synchronization and channel raster design in Rel-15 and Rel-16, for licensed spectrum and unlicensed spectrum, respectively.

Figure 1 Illustration of synchronization and channel raster in Rel-15 and Rel-16.

For Type 1 synchronization and channel raster design, channel raster interval is as small as subcarrier spacing level to provide full flexibility for operators to allocate the channels, and the synchronization raster is designed to guarantee at least one SS/PBCH block being implemented within the minimum channel bandwidth (MCBW). To achieve this purpose, the synchronization raster interval is chosen as Δsync=BWCH-BWSSB, wherein BWCH is the minimum channel bandwidth to include the corresponding SS/PBCH block with respect to the targeted SCS (not including the guard band), and BWSSB is the SS/PBCH block bandwidth with respect to the targeted SCS. For example, Type 1 synchronization and channel raster design is applicable to Rel-15 FR2. When the default SCS of SS/PBCH block is 120 kHz, BWCH=46.08 MHz, BWSSB=28.80 MHz, such that Δsync=17.28 MHz; when the default SCS of SS/PBCH block is 240 kHz, BWCH=92.16 MHz, BWSSB=57.60 MHz, such that Δsync=34.56 MHz.

For Type 2 synchronization and channel raster design, channel raster and synchronization raster are both fixed as single location for a given channel. There is one and only one synchronization raster entry within a channel with minimum channel bandwidth, and the synchronization raster interval is approximately same as minimum carrier bandwidth, subject to some rounding to integer number of RBs to be compatible with GSCN values. For example, Type 2 synchronization and channel raster design is applicable to Rel-16 NR-U, wherein Δsync=18.72 or 20.16 MHz.

# Synchronization Raster for 52.6 to 71 GHz

For NR operated from 52.6 GHz to 71 GHz, both licensed and unlicensed bands could be included in this carrier frequency range, hence, both types of synchronization and channel raster design are being considered in RAN4. This section analyzes the complexity of each type of synchronization and channel raster design, and especially its impact to the selection of default SCS for SS/PBCH block for initial cell search.

Table 1 summarizes the synchronization raster interval for combinations of SCS of SS/PBCH block and minimum channel bandwidth, for both Type 1 and Type 2 synchronization and channel raster design. Note that the value of synchronization raster interval for Type 2 synchronization and channel raster design is an approximation, and it can be further subject to changes due to different rounding to integers. The following observation can be made from Table 1.

Observation 1: Synchronization raster interval can be larger when supporting a larger minimum channel bandwidth.

Table 1 Synchronization raster interval for NR 52.6 to 71 GHz.

| Sync Raster Interval (GSCN) |  |  | Minimum Channel BW (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 | 100 | 200 | 400 |
| Type 1 Sync and Channel Raster Design | SCS of SSB (kHz) | 120 | 1 | 3 | 9 | 20 |
|  |  | 240 | 2 | 2 | 7 | 18 |
|  |  | 480 | 4 | 4 | 4 | 15 |
|  |  | 960 | 8 | 8 | 8 | 8 |
| Type 2 Sync and Channel Raster Design | SCS of SSB (kHz) | 120 | 3 | 6 | 12 | 24 |
|  |  | 240 | 6 | 6 | 12 | 24 |
|  |  | 480 | 12 | 12 | 12 | 24 |
|  |  | 960 | 24 | 24 | 24 | 24 |

Table 2 summarizes the estimated number of synchronization raster entries for band from 57 to 66 GHz. Note that the number does include the impact from guard band yet, but the trend should be clear regardless. The following observation can be made based on Table 2.

Observation 2: The number of synchronization raster entries can be smaller when supporting a larger minimum channel bandwidth.

Table 2 Number of synchronization raster entries for NR 57 to 66 GHz.

| Number of Sync Raster Entries |  |  | Minimum Channel BW (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 | 100 | 200 | 400 |
| Type 1 Sync and Channel Raster Design | SCS of SSB (kHz) | 120 | 520 | 173 | 57 | 26 |
|  |  | 120 + 240 | 780 | 433 | 131 | 54 |
|  |  | 120 + 480 | 650 | 303 | 187 | 60 |
|  |  | 120 + 960 | 585 | 238 | 122 | 91 |
|  |  | 120 + 240 + 480 + 960 | 975 | 628 | 326 | 153 |
| Type 2 Sync and Channel Raster Design | SCS of SSB (kHz) | 120 | 180 | 90 | 45 | 22 |
|  |  | 120 + 240 | 270 | 180 | 90 | 44 |
|  |  | 120 + 480 | 225 | 135 | 90 | 44 |
|  |  | 120 + 960 | 202 | 112 | 67 | 44 |
|  |  | 120 + 240 + 480 + 960 | 337 | 247 | 157 | 88 |

We also note that in NR FR2, at most 344 synchronization raster entries were supported for a given band (please refer to Appendix for details), then in order to keeping the same or similar UE complexity in initial searching a band between 52.6 GHz to 71 GHz, we should carefully choose the value of minimum channel bandwidth and the value of default SCS for SS/PBCH block. Also note that the determination of the minimum channel bandwidth should also take into account other aspects, including the assignment/auction of the spectrum utilization, flexibility in frequency resource allocation, and the link coverage with power constraint, which will be discussed in RAN4.

Proposal: RAN1 shall coordinate with RAN4 on determining the minimum channel bandwidth and extra default SCS for SS/PBCH block in initial cell search.

# Conclusion

The observations and proposal made in this contribution are summarized below:

Observation 1: Synchronization raster interval can be larger when supporting a larger minimum channel bandwidth.

Observation 2: The number of synchronization raster entries can be smaller when supporting a larger minimum channel bandwidth.

Proposal: RAN1 shall coordinate with RAN4 on determining the minimum channel bandwidth and extra default SCS for SS/PBCH block in initial cell search.

# Appendix

Table 3 summarizes the number of synchronization raster entries in FR2, based on Table 5.4.3.3-1 in TS 38.101-2 (also quoted below).

Table 3 Summary for the number of synchronization raster entries in FR2.

| NR operating band | SCS of SSB | Number of sync raster entries per SCS | Total number of sync raster entries per band |
| --- | --- | --- | --- |
| n257 | 120 | 171 | 255 |
|  | 240 | 84 |  |
| n258 | 120 | 187 | 280 |
|  | 240 | 93 |  |
| n259 | 120 | 230 | 344 |
|  | 240 | 114 |  |
| n260 | 120 | 172 | 257 |
|  | 240 | 85 |  |
| n261 | 120 | 47 | 70 |
|  | 240 | 23 |  |

========================= Quoted from TS 38.101-2 Section 5.4.3.3 ==============================

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n257 | 120 kHz | Case D | 22388 - <1> - 22558 |
|  | 240 kHz | Case E | 22390 - <2> - 22556 |
| n258 | 120 kHz | Case D | 22257 - <1> - 22443 |
|  | 240 kHz | Case E | 22258 - <2> - 22442 |
| n259 | 120 kHz | Case D | 23140 – <1> – 23369 |
|  | 240 kHz | Case E | 23142 – <2> – 23368 |
| n260 | 120 kHz | Case D | 22995 - <1> - 23166 |
|  | 240 kHz | Case E | 22996 - <2> - 23164 |
| n261 | 120 kHz | Case D | 22446 - <1> - 22492 |
|  | 240 kHz | Case E | 22446 - <2> - 22490 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. |  |  |  |

========================= Quoted from TS 38.101-2 Section 5.4.3.3 ==============================