3GPP TSG RAN WG1 #104-e	R1-2101205

# e-Meeting, January 25th – February 5th, 2021

Agenda Item:		8.3.4

Source:		Samsung

Title:		Discussion for propagation delay compensation enhancements

Document for:		Discussion and decision

# Introduction

In RAN 1 # 103e-meeting, some evaluation assumptions for propagation delay compensation were discussed and agreed:

Agreements:

- Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control.

- Asymmetry between downlink and uplink channel for smart grid scenario is not considered.

- TA adjustment accuracy is not considered for the evaluation of time synchronization error.

- errorBS,DL,TXerrorBS, DL, TX is included in the equation for calculating the overall time synchronization error.

Agreements:

TA adjustment accuracy is not considered for the evaluation of time synchronization error.

Agreements:

For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error:

- Option 1: 200 ns

- Option 2: 65 ns

Moreover, the two main solutions to be further studied were identified in RAN 1 #102e-meeting as:

Agreements: The following options for propagation delay compensation are further studied in RAN1 - Option 1: TA-based propagation delay - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity). - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te) - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected) - Option 2: RTT based delay compensation: - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

In additional, it was agreed in RAN 2 that:

=> Discuss by email the delay components and understand the requirements with each component and agree on what needs to be addressed

=>	Introduce propagation delay compensation for the improved synchronisation accuracy requirement in case of in UL Time Synchronization

Agreements

1: RAN2 should consider the following three scenarios, with a focus on Scenario 2 and 3:

•	Scenario 1: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the CN. The 5GS introduced error is caused by the relative time-stamping inaccuracy at the NW-TT and the DS-TTs.

•	Scenario 2: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the UE. The 5GS introduced error is caused by the relative time-stamping inaccuracies at the involved DS-TTs.

•	Scenario 3: In the smart grid use case, where the TSC devices behind a target UE are synchronized to the 5G GM TD. The 5GS introduced error is caused by the synchronization of the 5G clock to the DS-TT.

2	RAN2 should evaluate the synchronicity budget by dividing the 5GS E2E path into three parts: Network, Device, and Uu interface. Where the Uu interface is understood as the maximum 5GS time synchronization error between the UE and the gNB-DU (i.e. DU-CU interface error is not included)

3 RAN2 assumes the two Uu interfaces in Scenario 2 have the same time synchronization error budget.

4 The Uu interface budget for Scenario 1, 2 and 3 are respectively calculated as following:

•	Scenario 1: Uu budget = 900ns – Device – Network scenario1

•	Scenario 2: Uu budget = (900ns – 2xDevice – 2xNetwork scenario2)/2 (assumption is based on GPTP)

•	Scenario 3: Uu budget = 1000ns – Device – Networkscenario3 (baseline assumption that this is based on GNSS)

5  The Device part time synchronization accuracy budget is assumed to be in the range ±50 to ±100ns, this applies to all three scenarios

6  The error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is to be included in the network part budget, and RAN1 should be informed not to include this error in Uu interface.

7  The Network part time synchronization accuracy budget for Scenario 1, 2, and 3 are assumed to be the following:

•	Scenario 1: ±120 to ±200ns (NetworkScenario1) (assuming 3-5 hops worst case scenario

•	Scenario 2: ±240 to ±400ns (2xNetworkScenario2) (assuming 6-10hops worst case scenario)

•	Scenario 3: ±100ns (NetworkScenario3)

8	Based on Proposal 4, 5, 6 and 7, the per Uu interface time synchronization accuracy for Scenario 1, 2 and 3 are as following:

•	Scenario 1: ±595ns to ±725ns

•	Scenario 2: ±145ns to ±275ns

•	Scenario 3: ±795ns to ±845ns

9	LS to RAN1 providing the scenarios and values.  Indicate to RAN1 that they should aim to meet the most stringest requirements, but a number within the range is also acceptable

10	 It is up to RAN1 to decide which PDC options should be supported for Scenario 1, 2 and 3 in Release-17.

This contribution discusses propagation delay compensation from RAN 1 perspective.

# Propagation delay compensation

In RAN 2 #111-emeeting, it was agreed to introduce propagation delay compensation for the improved synchronization accuracy requirement. Two main options were discussed in RAN 1: TA-based propagation delay and RTT based delay compensation. The accuracy and pros/cons of the two methods were discussed below.

2.1 TA-based propagation delay

In general, for TA-based method, gNB estimation propagation delay and signals or pre-compensate the propagation delay to the UE. Based on the study in Rel-16 and the agreement, the estimation errors are related to subcarrier spacing and mainly consist of the following parts shown as figure 1:

- gNB transmit error (errorBS, DL, TX): 65ns was agreed to use for control-to-control cases in previous RAN 1 meeting. For smart grid cases, ±200ns or 82.5ns are two options for further study. The gNB transmission error mainly caused by the hardware of gNB. We don’t expect difference for different cases. Therefore, 82.5ns is preferred for the evaluation.

- Downlink frame timing error (errorUE, DL, RX): In previous RAN 1 meeting, there are some discussion on whether to consider downlink frame timing error. In our understanding, in RAN 5 test case, it is assumed to be 0 to test the Te.

- UE transmit timing error (Te): As defined in TS 38.133 Table 7.1.1-2, this error Te is about as ±12*64*Tc , ±8*64*Tc and ± 7*64*Tc for 15KHz, 30KHz and 60KHz respectively.

- BS detecting error (errorBS, UL, RX): This error is introduced when gNB estimates UL channel/signals to obtain propagation delay. PRACH may not able to provide enough accuracy of time estimation. SRS/UL DMRS or PUSCH can be used. The estimation accuracy depends on occupied BW. The error about ~100ns can be assumed with 20MHz bandwidth.

- TA granularity error (errorTA_indication): If using current TA granularity, i.e., 16∙64∙Tc/2μ, and the error is  ±8∙64∙Tc/2μ.

Since gNB received the signal from UE which covers two propagation delay, and indicates UE to adjust propagation delay. Therefore, we proposal to use the following equation to calculate the error:

errortotal≤errorBS, DL, TX2+errorBS, UL,RX+errorTA_indication2+Te2

Proposal 1: The following equation is used to calculate the error for TA-based method:

errortotal≤errorBS, DL, TX2+errorBS, UL,RX+errorTA_indication2+Te2

2.2 RTT based propagation delay

RTT based propagation delay was briefly discussed. The method is from positioning.  The main idea is that, the RTT time (double of propagation delay) is equal to ∆T1 +∆T2, where, ∆T1=tUERx-tgNBTx and ∆T2=tgNBRx-tUETx. Therefore, ∆T1 +∆T2 = (tUERx-tUETx)+ (tgNBRx-tgNBTx). In this method, it is also gNB to estimate the propagation delay. And gNB needs to know the offset at UE side between transmission and reception time. In addition, gNB needs to know when to receive the uplink transmission. In positioning, the information is from SRS-Pos from the LMF.

The main error comes from the following parts:

- gNB transmit error (errorBS, DL, TX): It happens when gNB transmit DL signaling at tgNBTx. This error is the offset between on actual gNB transmission time and the assumed gNB transmission time at gNB side.

- UE transmit timing error (Te): When UE estimate the signaling from gNB, it has estimation error. In addition, UE will transmit based on the indication from higher layer. It will additional cause transmit error. With this Te, there is no need to double count downlink frame timing error errorUE, DL,RX.

- BS detecting error (errorBS, UL, RX): This error is introduced when gNB estimates UL channel/signals at tgNBRx. Similarly,  the accuracy depends on the occupied BW. Assuming similar occupied BW as TA-based method, the error can be also assumed to be ~100ns.

- UE Rx-Tx difference granularity error (errorRxTxDiff, report): Because gNB need to estimate propagation delay with gNB Rx-Tx offset and UE Rx-Tx offset. In TS 38.133, the reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx)is defined with the resolution step of 2kTc, where: k =0,…,5 based on different configurations, with the estimation of PRS.

- Propagation delay indication granularity error (errorindication): gNB eventually need to signaling to UE about the propagation delay. Therefore, an additionally signaling to indicate propagation delay cannot be avoided. The granularity of propagation delay indication will also affect the total error.

Similarly, since RTT time if two times of propagation delay, the error for propagation delay estimation is half of the sum of the above errorBS, DL, TX, Te, errorBS, UL, RX, errorindication. And when UE estimate time, gNB transmit error errorBS, DL, TX and the granularity error of propagation delay errorindication needs to be additionally added. Therefore, the following equation can be used to calculate total error for RTT-based method:

errortotal≤errorBS, DL, TX+errorBS, UL,RX+errorRxTxDiff, report+Te+errorindication2

Proposal 2: The following equation is used to calculate the error for RTT-based method:

errortotal≤errorBS, DL, TX+errorBS, UL,RX+errorRxTxDiff, report+Te+errorindication2

Figure 1

2.3 Comparison of TA based method and RTT based method

From the above analysis on TA based method and RTT based method, we can see that, the fundamental of the two methods are the same. For TA based method, gNB estimate TA assuming UE Tx-Rx difference = 0, and gNB calculate gNB Tx-Rx different based on a virtual  t'gNBTx of the configured PRACH resource. Table 1 summarizes the total error from different method. With current procedure of TA and RTT method for positioning, the main different comes from the indication granularity.

For TA based method, there are three solutions.  Current TA procedure is to adjust uplink transmission time. Therefore, the granularity of current TA considered the required accuracy of UL reception at gNB side. However, in order to reduce the timing error for TNS, the granularity for propagation delay is much smaller. The finer granularity is not very useful for TA adjustment considering the hardware limited. On the other hand, finer TA granularity will cost a lot of overhead since it may happen more frequently for uplink transmission. Therefore, it is preferred to introduce a separate procedure to with a new dedicated signalling for propagation delay compensation.

Observation 1: Current TA procedure is to align the uplink signal arriving time at gNB sides, to achieve the target error of time estimation, big overhead is redundant for a normal TA procedure.

Table 1

| TA based method |  |  | Potential enhancement | RTT based method |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz |  | 15kHz | 30kHz |  |
| gNB transmit error (errorBS, DL, TX): | 65ns | 65ns | - | 65ns | 65ns | gNB transmit error (errorBS, DL, TX): |
| UE transmit timing error (Te) | 391ns | 260ns | - | 391ns | 260ns | UE transmit timing error (Te) |
| gNB estimation error (errorBS, UL, RX) | 100ns | 100ns | UL signalling, e.g. SRS | 100ns | 100ns | gNB estimation error (errorBS, UL, RX) |
| TA granularity (errorTA_indication) | 260ns | 130ns | Finer granularity | 0.5~16 ns | 0.5~16 ns | UE Rx-Tx difference granularity error (errorRxTxDiff, report) |
| - | - | - | UL signaling with a pre-known TA. | TBD | TBD | Propagation delay indication granularity error (errorindication) |
| Total is 1/2 of above all | 408ns | 277.5ns |  | （275.25~286）ns+TBD/2 | （213~221）ns+TBD/2 | Total is 1/2 of above all |

* For RTT based method, the transmission time needs to be further clarified.

For position RTT based method, most of the configuration are from LMF other than gNB. In order to support RTT based solution, a whole new procedure and configurations needs to be introduced. In addition, to support current RTT based method, gNB and UE need to support PRS transmission and estimation. The specification impact of both RAN 1/2/3 is large and it may also have impacts on higher layer.

Observation 2: RTT-based solution requires PRS and positioning functionality. More impact in higher layer is expected while the gain compared with TA based solution is not clear.

Therefore, considering the above analysis and observation, a new procedure for propagation delay estimation can be considered.

Proposal 3: Introduce a new procedure for propagation delay estimation.

In order to improve the accuracy of time estimation, the following aspects can be considered:

- To introduce finer granularity to reduce the error TTAe caused by quantization, especial for smaller subcarrier space case, e.g., 15kHz and 30kHz.

- To reduce the estimation error Tge on UL reception time. In the analysis, SRS/UL DMRS or PUSCH is assumed for propagation delay estimation other than PRACH.

- To avoid TA adjustment error Tadj by transmit UL signal for propagation delay estimation with pre-known TA.

Other errors either are limited by hardware, or depend on UE or gNB detection performance.

Proposal 4: Introduce a finer granularity to indicate propagation delay.

Proposal 5: Consider to use SRS, UL DMRS or PUSCH for propagation delay estimation. A predefined TA can be used for transmission of the UL signalling for propagation delay estimation.

# Conclusions

This contribution discusses propagation delay compensation from RAN 1 perspective. The following observations are made:

Observation 1: Current TA procedure is to align the uplink signal arriving time at gNB sides, to achieve the target error of time estimation, big overhead is redundant for a normal TA procedure.

Observation 2: RTT-based solution requires PRS and positioning functionality. More impact in higher layer is expected while the gain compared with TA based solution is not clear.

Based on the observations, the following proposals are made:

Proposal 1: The following equation is used to calculate the error for TA-based method:

errortotal≤errorBS, DL, TX2+errorBS, UL,RX+errorTA_indication2+Te2

Proposal 2: The following equation is used to calculate the error for RTT-based method:

errortotal≤errorBS, DL, TX+errorBS, UL,RX+errorRxTxDiff, report+Te+errorindication2

Proposal 3: Introduce a new procedure for propagation delay estimation.

Proposal 4: Introduce a finer granularity to indicate propagation delay.

Proposal 5: Consider to use SRS, UL DMRS or PUSCH for propagation delay estimation. A predefined TA can be used for transmission of the UL signalling for propagation delay estimation.

# References

- RP- 201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”

- TR 38.825 Study on NR Industrial Internet of Things (IoT)

- TS 38.133 Requirements for support of radio resource management v16.5.0