3GPP TSG RAN WG1 #104-e	R1-2101232

# e-Meeting, January 25th – February 5th, 2021

Agenda item:	8.11.1.2

Source: 	Samsung

Title: 	On Feasibility and Benefits for Mode2 Enhancements

Document for:	Discussion and Decision

- Introduction

In RAN1#103-e meeting [1], the following conclusions were made for resource allocation enhancement on Rel-17 NR sidelink as:

- The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

  - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

    - e.g., based on its sensing result

  - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

    - e.g., based on its sensing result and/or expected/potential resource conflict

  - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

  - FFS: details of sensing operation at UE-A side

  - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

  - Note: these different types may be used in combination with each other

- From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

- Send an LS to RAN plenary

- For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

  - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

  - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

  - How UE-A and UE-B are determined

  - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

  - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

  - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

In this contribution, we discuss about the schemes of inter-UE coordination and its feasibility and possible benefits based on the conclusions above.

- Discussion on inter-UE coordination

Inter-UE coordination in Rel-17 NR sidelink mode2 is to share resource allocation information among the UEs communicating with each other. In Rel-16 NR sidelink mode2, only TX UE performs resource allocation by sensing and resource selection procedure. On the other hand, in Rel-17 NR sidelink mode2, other UE(s) may provide resource selection assistance information (RSAI) to TX UE by inter-UE coordination if its feasibility and benefit are verified. In this section, we discuss about the inter-UE coordination in the regards of performance benefits, supported scenarios, RSAI details, and other issues.

| - Periodic Traffic | - Aperiodic Traffic |
| --- | --- |

Figure 1: The performance of inter-UE coordination in sidelink unicast mode

Performance benefits

In this subsection, we provide system level evaluation results on inter-UE coordination in order to see possible gain. At first, we consider a scenario where RX UE provides RSAI to TX UE by inter-UE coordination. Unlike TX UE based mode 2 resource allocation, RX UE based mode2 can provide the performance advantages for the following aspects:

- Hidden-node problem

- Exposed-node problem

- Half duplex problem

Basically, it is well known that TX UE based mode 2 resource allocation can experience the above problems. Specifically, TX UE may fail to monitor the hidden nodes and select a resource with high interference which lead to reception failure of RX UE. Also, TX UE may exclude the resources by exposed nodes which have low interference and receive successively in RX UE perspective. Moreover, even though TX UE selects a resource properly, half duplex problem can cause transmission failure when RX UE performs transmission and cannot receive simultaneously. If RX UE performs the sensing and resource selection procedure instead of TX UE, above hidden-node and exposed node problems can be resolved. Moreover, RX UE can exclude resources which its transmissions occurs and avoid half duplex problem.

In order to observe the performance benefits from RX UE based mode 2 resource allocation, we applied the following evaluation assumptions in our study:

- Baseline scheme is the Rel-16 mode 2 which TX UE performs sensing and resource selection procedure.

- The same Rel-16 mode 2 sensing and resource selection procedure is applied for inter-UE coordination scheme but it is performed by RX UE instead of TX UE.

- RX UE provides RSAI to TX UE but there is no delay and signalling overhead for sharing RSAI.

The performance evaluation results under these assumptions are shown in Figure 1. More detailed parameters used in this evaluation are given in the Appendix. From Figure 1, the performance benefits from the inter-UE coordination are observed for both periodic and aperiodic traffics. This can be explained by the fact that RX based resource allocation can address the issues on hidden-node, exposed-node, and half duplex problems. Since the delay and signalling overhead of RSAI was not considered, Figure 1 can be considered as an upper bound performance for RX UE based resource allocation and RSAI sharing between UEs.

Observation 1: RX UE based resource allocation and inter-UE coordination provides performance benefits in mode 2 by addressing hidden-node, exposed-node, and half duplex problems.

| - Scenario A | - Scenario B |
| --- | --- |

Figure 2 Two different scenarios describing a UE who provides coordinating information

Supported scenarios

Besides possible performance benefits, inter-UE coordination in Rel-17 NR sidelink mode2 can be utilized in a scenario where power saving of the UEs is necessary. As explained, other UE(s) may provide RSAI to TX UE by inter-UE coordination. Here, the other UE(s) who provides RSAI can be high energy UE (HE-UE) and the TX UE can be low energy UE (LE-UE). In the help of inter-UE coordination, LE-UE can use RSAI from other UE(s) and does not perform sensing which requires power consumption. Of course, in order to reduce the power consumption for receiving RSAI, corresponding mechanism needs to be specified. For instance, we can consider applying inter-UE coordination with sidelink DRX mechanism, or restrict the slots used for RSAI reception as a subset of resource pool. Different from LE-UE in LTE who does not expect sidelink reception, LE-UE in Rel-17 NR sidelink may be allowed to monitor partial slots and receive some essential message, consequently the power consumption of RSAI reception can be acceptable.

Observation 2: Inter-UE coordination can be utilized to reduce power consumption when low energy UE uses RSAI from high energy UE without sensing operation.

Proposal 1: Study an application of inter-UE coordination for a scenario of power consumption reduction.

Since cast types of broadcast, groupcast, and unicast are supported in NR sidelink, the most important issue is to clarify the reasonable scenarios that inter-UE coordination is applied. According to applied cast type, feasibility and possible solutions for inter-UE coordination would be different. Unlike unicast and groupcast, RX UEs in broadcast are unspecified and can be any UE around the TX UE. Thus, it is difficult to achieve the performance benefits from RX UE based mode 2 resource allocation in the broadcast scenario. Therefore, inter-UE coordination may not fit for sidelink broadcast scenario even though this can used to reduce power consumption.

Proposal 2: Inter-UE coordination is supported in unicast and groupcast.

For inter-UE coordination, the next question is that other UE(s) who provides RSAI to TX UE becomes any UE or it should be RX UE(s) which is expected to receive signal from TX UE. In Figure 2, the former case corresponds to Scenario A while the latter case corresponds to Scenario B. As depicted in Figure 2 (a): Scenario A, other than RX UE can be the coordinating UE which provides RSAI to TX UE. On the other hand, in Figure 2 (b): Scenario B, only RX UE becomes the coordinating UE which provides RSAI to TX UE. For Scenario B, we already presented the performance benefits by Figure 1. Unlike Scenario B, the possible performance benefits from Scenario A may come from which the coordinator UE has more scheduling information than TX UE and provide RSAI to TX UE as described in Figure 2 (a). We need to further study on performance advantages for each scenarios and possible RAN2 impacts if any in order to decide supported scenario for inter-UE coordination.

Proposal 3: Consider the following scenarios for inter-UE coordination and decide whether to support both or one of them

- Scenario A: Other than RX UE can be the coordinating UE

- Scenario B: RX UE only is the coordinating UE

RSAI details

In this contribution, we have defined “a set of resources” as RSAI (Resource Selection Assistance Information). According to the conclusion in the last meeting, the following options are identified as

- Option 1: UE-A sends RSAI to UE-B based on its sensing results where RSAI is the preferred resource(s) for UE-B’s transmission.

- Option 2: UE-A sends RSAI to UE-B based on its sensing results where RSAI is the non-preferred resource(s) for UE-B’s transmission.

- Option 3: UE-A sends RSAI to UE-B without sensing where RSAI is the non-preferred resource(s) for UE-B’s transmission.

If there is no limitation on signaling overhead and there are plenty of candidates in the list of selected resource(s) for RSAI, above Option 1 and Option 2 can provide similar performance. However, with limited RSAI overhead, Option 2 may not provide useful coordinating information for UE-B’s resource selection. Note that Option 1 was considered in our evaluation (See Figure 1). For Option 3, details for deciding RSAI was not identified yet but one possible approach is by using decoding results for UE-B’s transmission. However, HARQ feedback is supported already in Rel-16 sidelink for unicast and groupcast. Therefore, it is not clear that additional RSAI is necessary. Considering these aspects, we prefer Option 1.

Proposal 4: UE-A sends RSAI to UE-B based on its sensing results where RSAI is the preferred resource(s) for UE-B’s transmission.

Next, we need to define a condition for UE-A to provide RSAI into UE-B and the following two options can be considered.

- Option 1: By pre-defined time instance on RSAI

- Option 2: By request signaling on RSAI

The Option 1 is that UE-A transmits RSAI to UE-B in the pre-defined time instances. The time instance for RSAI transmission can be (pre-)configured in a pool. According to configuration, time domain behaviour for transmitting RSAI in Option 1 can be periodic or semi-persistent. The Option 2 is that UE-A transmit RSAI to UE-B when UE-A receives request signalling on RSAI. Therefore, time domain behaviour for transmitting RSAI in Option 2 can be aperiodic. For Option 2, request signaling on RSAI to UE-A can be from TX UE or other UE. This feature should be supported in order to enable the inter-UE coordination in all coverage scenarios of in-coverage, partial coverage, and out-of-coverage. In addition, if UE-A is in-coverage, gNB can request to UE-A to transmit RSAI. To avoid unnecessary RSAI transmissions, we prefer Option 2. If Option 2 is supported, then we need to decide further details on RSAI request signaling.

Proposal 5: Define request signaling for RSAI and RSAI can be requested

- By TX UE or other UE

- By gNB when UE-A is in-coverage

In addition, as a container carrying RSAI, the following options can be considered:

- Option 1: MAC message

- Option 2: PC5-RRC signaling

- Option 3: 1st stage SCI

- Option 4: 2nd stage SCI

- Option 5: PSFCH

If high layer signaling is used (Option 1 or Option 2), more delay is expected to share RSAI compared to physical layer signaling. Considering possible RSAI overhead, 1st stage SCI (Option 3) seems not desirable but 2nd stage SCI (Option 4) can be a good alternative with small spec impact. PSFCH (Option 5) can be considered as another alternative depending on RSAI contents. However, if RSAI content is more than one bit, further spec impact on Option 5 is expected than Option 3. Therefore,

Proposal 6: 2nd SCI is used as a container carrying RSAI

Then, if UE-B (TX UE) receives RSAI from UE-A, how does UE-B take RSAI into account? The following two options can be considered:

- Option 1: UE-B should use a list of selected resource(s) in RSAI for its transmission

- Option 2: It is up to UE-B how to use a list of selected resource(s) in RSAI for its transmission

In case of Option 1, UE-B should apply RSAI from UE-A for its transmission. One possible interpretation of Option 1 is that UE-B consider RSAI as scheduling information which is provided by UE-A. On the other hand, for Option 2, UE-B can decide how to apply RSAI for its transmission. There can be a case where UE-B does not have other choice to select resource(s) without RSAI. Also, there can be another case that UE-B can perform mode2 operation but RSAI is regarded as invalid. Therefore, we propose

Proposal 7: It is up to UE-B how to use a list of selected resource(s) in RSAI for its transmission

Other issues

In order to support inter-UE coordination, there would be other issues which need to be specified further. At first, resource pool information needs to be shared between UEs. For example, when UE-A determines a list of selected resource which is preferred UE-B’s transmission, the UE-A may not have information on TX pool(s) configured for UE-B. Thus, TX pool(s) configured for UE-B can be shared to UE-A or separate pool(s) used for inter-UE coordination can be configured between UEs. In addition, we need to specify mode2 procedure to determine RSAI. According to Proposal 4, RSAI can include a list of selected resource(s). In this case, the number of selected resource(s) for RSAI should be decided. Furthermore, we may need to specify validation of RSAI. For example, we need to consider latency bound for inter-UE coordination. Depending on the latency, RSAI may or may not be valid. If RSAI is from far way UE, this information may be invalid. If we consider inter-UE coordination in groupcast and RSAI is from other group, this information may be invalid. Therefore, we need to study UE behaviour if received invalid RSAI. For instance, UE may re-trigger new RSAI. In this regard, we propose:

Proposal 8: The following issues should be considered for inter-UE coordination

- Resource pool sharing for inter-UE coordination

- Detailed mode2 procedure to determine RSAI

- Validation of RSAI and UE behaviour if received invalid RSAI

- Conclusions

This contribution discusses on feasibility and possible benefits for Mode 2 enhancement in Rel-17 NR sidelink. Based on the discussion, the following observations and proposals are provided:

Observation 1: RX UE based resource allocation and inter-UE coordination provides performance benefits in mode 2 by addressing hidden-node, exposed-node, and half duplex problems.

Observation 2: Inter-UE coordination can be utilized to reduce power consumption when low energy UE uses RSAI from high energy UE without sensing operation.

Proposal 1: Study an application of inter-UE coordination for a scenario of power consumption reduction.

Proposal 2: Inter-UE coordination is supported in unicast and groupcast.

Proposal 3: Consider the following scenarios for inter-UE coordination and decide whether to support both or one of them

- Scenario A: Other than RX UE can be the coordinating UE

- Scenario B: RX UE only is the coordinating UE

Proposal 4: UE-A sends RSAI to UE-B based on its sensing results where RSAI is the preferred resource(s) for UE-B’s transmission.

Proposal 5: Define request signaling for RSAI and RSAI can be requested

- By TX UE or other UE

- By gNB when UE-A is in-coverage

Proposal 6: 2nd SCI is used as a container carrying RSAI

Proposal 7: It is up to UE-B how to use a list of selected resource(s) in RSAI for its transmission

Proposal 8: The following issues should be considered for inter-UE coordination

- Resource pool sharing for inter-UE coordination

- Detailed mode2 procedure to determine RSAI

- Validation of RSAI and UE behaviour if received invalid RSAI

- Appendix

In this section, we provide the list of evaluation assumptions used for analysis on inter-UE coordination.

Table 1 Simulation parameters used for evaluations of inter-UE coordination

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario from NR V2X methodology - Vehicle speed = 140 km/h |
| Channel model | TR 37.885, NR V2X Channel Model |
| Number of antennas | 1TX and 2RX |
| Spectrum allocation | Carrier frequency: 6GHz Simulated Bandwidth:20 MHz |
| Subcarrier spacing | 15 kHz |
| Communication mode | Unicast |
| Traffic model | Periodic traffic Model 2(TR 37.885): - Inter-packet arrival time: 30 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 30 ms Aperiodic traffic Model 1(TR 37.885): - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Packet size: Uniformly random in the range between 200 bytes and 2000 bytes with the quantization step of 200 bytes - Latency requirement: 50 ms |
| TTI structure | NR Slot TTI: 10 Symbols for Data, 4 Symbols for overhead |
| MCS | The lowest MCS index |

References

3GPP TSG RAN1#103-e, Final Report of 3GPP TSG RAN.