3GPP TSG RAN WG1 Meeting #104-e	R1-2101268

E-meeting, January 25 –February 5, 2021

Agenda Item:	8.2.7

Source:	Huawei, HiSilicon

Title:	Link level and System level evaluation results for 52-71 GHz

Document for:	Discussion and Decision

Introduction

In RAN1#103-e and RAN #90-e, subcarrier spacing of 120 kHz with NCP is recommended to be supported and PTRS enhancement needs to be studied in WI. For unlicensed spectrum, channel access mechanism assuming beam based operation is also recommend to be studied [1][2]. Following the agreed objectives, we provide new results to study the PTRS pattern enhancement for 120 kHz SCS for PDSCH/PUSCH in link level evaluation and channel access mechanism under indoor scenario in system level evaluation.

Assumptions and results for Link level evaluation

Simulation assumptions

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 60 GHz |
| Duplexing | TDD |
| Waveform | CP-OFDM for DL/UL, DFT-s-OFDM for UL |
| Numerology | [120 kHz] |
| Antenna Configuration (Mg,Ng,M,N,P) | - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) |
| PN model | Example 2 phase noise model scaling to 60 GHz in 38.803 |
| Channel coding | LDPC code |
| frame structure | 2(PDCCH)/1(DMRS)/11(data+PTRS) |
| System BW | 400MHz：4096 FFT size |
| Transmission BW | 64 RBs for CP-OFDM 256 RBs for DFT-s-OFDM |
| TTI | 1 slot |
| DMRS | 1/2 in frequency in each port |
| PTRS | For CP-OFDM: K=2 in frequency domain, L=1 in time domain For DFT-s-OFDM: 8*4 or 16*2 |
| Receiver | ICI compensation |
| Channel estimation | Realistic |

Simulation results

PDSCH

To analyze the necessity of ICI compensation for different SCS, BLER performance comparison for CPE compensation and ICI compensation are evaluated. The results are shown in Figure 1 from which we can observe that the ICI compensation provides sufficient benefits for BLER performance in 960 kHz SCS and reduces the error floor to at least lower than 1% under a high MCS, where the equivalent power of ICI caused by phase noise is comparable with or larger than the noise power in a high SNR region.

Figure 1. BLER performance comparison between CPE and ICI compensation

For ICI compensation, the use of enhanced block PTRS is an intuitive solution. BLER performance and SE performance comparisons of block PTRS and Rel-15 PTRS are provided in Figure 2 and Figure 3 respectively, where every K-th (K=4) RB is used to map Rel-15 PTRS, CN means block number per OFDM symbol, and CS means block size per PTRS block. By simply putting PTRS REs together in a block, the BLER performance is slightly worse than that of the distributed Rel-15 PTRS: about 0.6dB loss at BLER of 1% is observed. However, when a special sequence (denoted as S-Seq in the figure legend) which has a constant module in both time domain and frequency domain is used with block PTRS, the performance of block PTRS is significantly improved and becomes about 0.3dB better than that of Rel-15 PTRS. Similar observations can be made for SE performance.

Figure 2. BLER performance comparison between block PTRS and distributed PTRS

Figure 3. SE performance comparison between block PTRS and distributed PTRS

As analyzed in our companion contribution [3], power boosting cannot work effectively for the distributed PTRS as most of the received signal used to estimate ICI coefficients are from unknown data signal. However, this problem can be avoid by block PTRS, where most of the received signal used to estimate ICI coefficients are from PTRS with power boosting. Additional simulations were performed to verify the effectiveness of power boosting for both distributed PTRS and block PTRS. The results are shown in Figure 4 and Figure 5. As can be observed in Figure 4 and Figure 5, performance of power-boosted block PTRS is improved significantly and the demodulation SNR at BLER of 1% is decreased about 0.6~0.7dB when compared with no power boosting (shown in Figure 1). The result is also slightly better than that of Rel-15 PTRS even when the sequence defined in Rel-15 is used for block PTRS.

Figure 4. BLER performance comparison between block PTRS and distributed PTRS with 3dB power boosting

Figure 5. SE performance comparison between block PTRS and distributed PTRS with 3dB power boosting

Observation 1: Block PTRS with the sequence defined in Rel-15 performs a little worse than distributed PTRS defined in Rel-15 without power boosting. However, if sequence with constant modulus in time domain is used, both BLER and SE performance are better than that of distributed PTRS.

Observation 2: If power boosting is used, block PTRS with the sequence defined in Rel-15 performs a little better than distributed PTRS defined in Rel-15. The performance gap can be increased further by using a sequence with constant modulus in time domain for block PTRS.

PUSCH

The performance of block PTRS can be extended to PUSCH of CP-OFDM directly. Therefore, only the evaluation results of PTRS for DFT-s-OFDM are shown in details in this section.

As discussed before[4], the PTRS pattern of DFT-s-OFDM defined in Rel-15 is insufficient when 120 kHz is used with a wide scheduled bandwidth and a high scheduled MCS due to the long interpolation range between two adjacent PTRS groups. As a solution to this problem, a new pattern with the same PTRS overhead as (CN, CS) = 8*4 is evaluated which has more PTRS groups and fewer PTRS samples per PTRS group where CN means NgroupPT-RS, CS means Nsamplegroup.

Figure 6. BLER performance comparison of different PTRS pattern for 120 kHz

As the results in Figure 5 for different delay spreads show, the new pattern provides significant gains for high MCS: With half of the interpolation range of pattern (CN, CS) = 8*4, the new pattern improves the BLER floor at least one order of magnitude. For medium MCS, its performance is almost the same as that of pattern (CN, CS) = 8*4 due to a worse noise reduction performance with only 2 PTRS samples per PTRS group at medium SNR region.

Observation 3: Simulation results for DFT-s-OFDM with different PTRS patterns show that, if the pattern defined in Rel-15 is used, BLER performance of 120 kHz for 64QAM reaches a floor above 10-2 due to a longer interpolation range. BLER performance can be improved by using a new pattern with more PTRS groups.

Assumptions and results for system level evaluation

In RAN1 #103-e and RAN #90-e, it was agreed that both LBT mode and no-LBT mode are recommended to be supported to cover a wide range of use cases and regulatory requirements. For LBT mode, the channel access mechanism defined in EN302 567[5] has been identified as the baseline and enhancements have been identified for further discussion. In the revised WID, objectives were set to specify both LBT and No-LBT related procedure, and to study, and if needed specify, omni-direction LBT, direction LBT and receiver assistance in channel access.  In this section, we mainly study the receiver-assisted/receiver-only LBT and provided some simulation results for indoor scenario A.

Simulation assumptions for indoor scenario A

| Parameters | Assumptions |
| --- | --- |
| Layout | Indoor scenario-A: Office box 120m x 50m，12 BS per operator, 2 operator.BS randomly deployed within 10m x 10m virtual box. BS mounting on the ceiling. |  |
| Inter-BS distance | Random with minimum distance of 2m for scenario-A |  |
| UE distribution | 100% Indoor, randomly distributed within the office box, 5 users per BS |  |
| Carrier bandwidth | 2000MHz |  |
| Numerology | 960kHz for 2000MHz |  |
| Carrier frequency | 60GHz |  |
| Channel model | InH open channel model in 38.901 |  |
| Max. allowed BS Tx power(EIRP) | 40dBm |  |
| Max. allowed UE Tx Power(EIRP) | 25dBm |  |
| BS antenna configurations | See 38.802 Table A.2.1-7 for ceiling mount(antenna gain = 5dB) |  |
| UE antenna configuration | See 38.802 Table A.2.1-8, boresight parallel to ground(antenna gain = 5dB) |  |
| BS antenna height | 3m |  |  |
| UE antenna height | 1.5m |  |  |
| BS receiver noise figure | 7dB |  |  |
| UE receiver noise figure | 10dB |  |
| BS antenna array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ or (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ |  |
| UE antenna array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ or (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ |  |
| MCOT | 5ms |  |
| Traffic model | FTP3 with packet size of 27 Mbytes. |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| Feedback assumption | Realistic |  |
| Channel estimation | Realistic/ideal |  |
| Traffic type | 50% DL, 50% UL |  |

Observations regarding Receiver-assisted/receiver-only LBT performance

Impact of number of gNB/UE antennas on Receiver-assisted/receiver-only LBT performance

Table 1. Antenna configuration for indoor scenario

| gNB/UE antenna configuration | gNB antenna configuration | UE antenna configuration |
| --- | --- | --- |
| Baseline assumption in TR38.808 | M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2) |
| Assumption of low-cost devices | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2) |

In TR38.808, 64 antenna elements and 8 antenna elements are configured for gNB and UE, respectively, as the baseline SLS assumption in the indoor scenario. The probability of experiencing significant interference levels form hidden nodes is low under the current baseline assumption, mainly due to the narrow analog transmission beamwidth. However, in practice, a low-cost device may be equipped with a smaller number of antennas and wider transmission beams would have to be used. The performance of different antenna configuration assumptions as illustrated in Table 1 are thus evaluated.

Figure 7 shows the average and fifth percentile DL and UL user perceived throughput (UPT) under the baseline SLS assumption for No-LBT/Omni-directional LBT/Directional LBT/Received-assisted LBT/Receiver-only LBT in indoor scenario A with InH open channel models for BS to UE links. Table 2 shows the related results and gains at high traffic load. More detailed results could be found in Appendix B, Tables 04 to 06, respectively. More details of the receiver-assisted and receiver-only LBT mechanisms could be found in our companion contribution [6].We assume that the CW is set to a fixed value of 5 observation slots and MCOT equals 5ms. We also assume that the EDT used for the Rx-side LBT procedure before sending CTS with interference level feedback in receiver-assisted/receiver-only LBT is the same as the baseline EDT (-47dBm). Antenna directional gain is also considered in the EDT calculation.

Figure 8 illustrates the average and fifth percentile DL and UL UPT under the assumption of a reduced antenna elements for No LBT/Omni-directional LBT/Directional LBT/received-assisted LBT/receiver-only directional LBT in indoor scenario A with InH open channel models for BS to UE links. Table 3 shows the related results and gains at high traffic load. More detailed results could be found in Appendix B, Tables 07 to 09, respectively.

According to the above evaluation results, receiver-only LBT has the best performance compared to other channel access types in fifth percentile DL and UL UPT. The gain of receiver-only LBT compared to No LBT could be 177% in DL and 75% in UL for cell edge at high traffic load under the baseline assumption. In turn, the gain of receiver-only LBT compared to No LBT could be  291% in DL and 103% in UL for cell edge at high traffic load when reduced number of antenna elements are configured.

Table 2. Performance of various channel access type at high traffic load (64 antenna elements for gNB, 8 antenna elements for UE)

| Channel access type/User throughput(Mbps) | DL mean UPT | UL mean UPT | DL 5% UPT | UL 5% UPT | Gain of DL mean UPT w.r.t No-LBT | Gain of UL mean UPT w.r.t No-LBT | Gain of 5% DL UPT w.r.t No-LBT | Gain of 5% UL UPT w.r.t No-LBT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No-LBT | 2388.7 | 1521.8 | 85.7 | 86.6 |  |  |  |  |
| Directional LBT | 2446.7 | 1521.1 | 101.3 | 82.3 | 2% | 0% | 18% | -5% |
| Omni-dir LBT | 2452.9 | 1538 | 95.9 | 89.8 | 3% | 1% | 12% | 4% |
| Receiver-assisted LBT | 2494.2 | 1607.9 | 191.7 | 145.6 | 4% | 6% | 123% | 68% |
| Receiver-only LBT | 2715.7 | 1637.6 | 237.6 | 151.5 | 14% | 8% | 177% | 75% |

Table 3. Performance of various channel access type at high traffic load (32 antenna elements for gNB, 4 antenna elements for UE)

| Channel access type/User throughput(Mbps) | DL mean UPT | UL mean UPT | DL 5% UPT | UL 5% UPT | Gain of DL mean UPT | Gain of UL mean UPT | Gain of 5% DL UPT | Gain of 5% UL UPT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No-LBT | 1842.4 | 1208.4 | 34.7 | 44.5 |  |  |  |  |
| Directional LBT | 1852.7 | 1193.5 | 39.3 | 16.6 | 1% | -1% | 13% | -37% |
| Omni-dir LBT | 1964.1 | 1218.8 | 52.4 | 40.9 | 7% | 1% | 51% | -8% |
| Receiver-assisted LBT | 2033.6 | 1299.1 | 88.9 | 76.4 | 10% | 8% | 156% | 72% |
| Receiver-only LBT | 2215.7 | 1340 | 135.6 | 90.4 | 20% | 11% | 291% | 103% |

Figure 7. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A(64 antenna elements for gNB, 8 antenna elements for UE)

Figure 8. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A (32 antenna elements for gNB, 4 antenna elements for UE)

Observation 4: Reducing the number of antenna elements by 50 percent (32 for gNB and 4 for UE) compared to the baseline assumption in TR38.808 for the indoor scenario increases the probability of hidden nodes. This results in significantly more gains at DL cell-edge in Receiver-assisted/Receiver-only LBT compared to No-LBT.

Impact of EDT of Rx-side LBT on Receiver-assisted LBT performance

For receiver-assisted/receiver-only LBT,  EDT_Rx = -71dBm + 10log10 (BW/2GHz) + offset_dB was assumed at the UE side for LBT before assistance information reporting or at gNB for directional sensing in direction of potential PUSCH transmission from the corresponding UE.  The intuition is that, for the DL for instance, the maximum tolerable interference at a receiving UE could be defined as offset_dB above the minimum received power level from the serving gNB, i.e., the minimum association threshold agreed for SLS assumptions -71dBm + 10log10 (BW/2GHz).  Figure 9 illustrates the average and fifth percentile DL and UL user throughput for different EDT_Rx threshold at receiver side in indoor scenario A at medium traffic load. The performances of both mean and fifth percentile do not degrade when EDT_Rx becomes lower. In this evaluation, we assume that the interference level is reported to the transmitter.

Figure 9. Simulation results with different EDT_Rx threshold in receiver assisted LBT in scenario-A

Observation 5: For Receiver-assisted LBT, if the interference level at the receiver is reported to gNB  (for DL) or assessed by gNB (for UL) when the corresponding LBT procedure is successful, reducing EDT results in more LBT failures and less number of interference assessment/report. However, this does not considerably change the UPT performance if gNB uses the reported/assessed interference levels to prioritize UE scheduling.

Impact of receiver assistance information on Receiver-assisted LBT performance in DL

Table 4 shows the performance of the average and fifth percentile DL UPT if only CTS/idle indication is fed back when the interference level sensed by the UE in certain directional beam is lower than the EDT_Rx. Table 3 also compares these results with the corresponding results where the actual interference levels are also fed back to the gNB along with the one-bit CTS indication. As can be observed from the table, sending CTS/idle indication along with actual interference level would be beneficial since gNB can use the reported interference levels to prioritize scheduling the UEs. The gain of reporting actual interference level increases when a higher EDT_Rx is used. It is due to the fact that when a higher EDT_Rx is used, more UEs pass the CCA. In such a case, gNB has more potential UEs to schedule. However, if only one-bit CTS information is fed back from the UEs, gNB has no means to properly prioritize scheduling the UEs among the UE candidates based on their interference level and, as the number of candidates for scheduling increases, the used scheduling scheme based only on one-bit feedback CTS becomes increasing sub-optimal.

Table 4. Simulation results with different EDT_Rx and feedback information in DL receiver assisted LBT in indoor scenario-A

| EDT_Rx(dBm) | -47 |  | -51 |  | -61 |  |
| --- | --- | --- | --- | --- | --- | --- |
| User perceived throughput(Mbps) | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT |
| CTS/idle indication with actual interference level reporting | 3873.5 | 559.5 | 3875.8 | 566.6 | 3832.6 | 563.7 |
| Only CTS/idle indication reporting | 3528.2 | 377.4 | 3551.1 | 419.5 | 3771.9 | 550.1 |
| Relative Loss | -9% | -33% | -8% | -26% | -2% | -2% |

Observation 6: If the EDT_Rx threshold used at the receiver in Receiver-assisted LBT is increased, the DL cell-edge performance degrades when only CTS/idle indication (and not actual interference level) is fed back to the gNB after CCA assessment.

Conclusions

In this contribution, we have provided link level and system level simulation results for both licensed and unlicensed bands. Based on the discussion, we have the following proposals and observations:

Observation 1: Block PTRS with the sequence defined in Rel-15 performs a little worse than distributed PTRS defined in Rel-15 without power boosting. However, if sequence with constant modulus in time domain is used, both BLER and SE performance are better than that of distributed PTRS.

Observation 2: If power boosting is used, block PTRS with the sequence defined in Rel-15 performs a little better than distributed PTRS defined in Rel-15. The performance gap can be increased further by using a sequence with constant modulus in time domain for block PTRS.

Observation 3: Simulation results for DFT-s-OFDM with different PTRS patterns show that, if the pattern defined in Rel-15 is used, BLER performance of 120 kHz for 64QAM reaches a floor above 10-2 due to a longer interpolation range. BLER performance can be improved by using a new pattern with more PTRS groups.

Observation 4: Reducing the number of antenna elements by 50 percent (32 for gNB and 4 for UE) compared to the baseline assumption in TR38.808 for the indoor scenario increases the probability of hidden nodes. This results in significantly more gains at DL cell-edge in Receiver-assisted/Receiver-only LBT compared to No-LBT.

Observation 5: For Receiver-assisted LBT, if the interference level at the receiver is reported to gNB  (for DL) or assessed by gNB (for UL) when the corresponding LBT procedure is successful, reducing EDT results in more LBT failures and less number of interference assessment/report. However, this does not considerably change the UPT performance if gNB uses the reported/assessed interference levels to prioritize UE scheduling.

Observation 6: If the EDT_Rx threshold used at the receiver in Receiver-assisted LBT is increased, the DL cell-edge performance degrades when only CTS/idle indication (and not actual interference level) is fed back to the gNB after CCA assessment.

- References

[1] Chairman’s Notes RAN1#103-e

[2] RP-202925. Revised WID: on Extending current NR operation to 71 GHz

[3] R1-2100201. PDSCH/PUSCH enhancements for 52-71GHz band. Huawei, HiSilicon.

[4] R1-2007604 PHY design in 52.6-71 GHz using NR waveform, Huawei, HiSilicon.

[5] ETSI EN 302 567 Multiple-Gigabit/s radio equipment operating in the 60 GHz band; V2.1.20

[6] R1-2100202 Channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon.

- Appendix A.

Table 01 SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM and different phase noise compensation method at MCS28 for 960 kHz

| Channel | CPE compensation | ICI compensation |
| --- | --- | --- |
| CDL-B, 20ns | NAN/NAN | 25.2/30.5 |
| CDL-D, 20ns | 26/NAN | 19.7/21.3 |

Table 02: SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM and different PTRS patterns at MCS22

| Channel | Power boosting value | Rel-15 PTRS | Block-PTRS | Block PTRS with special sequence |
| --- | --- | --- | --- | --- |
| CDL-B, 20ns | Without power boosting | 17.7/21.9 | 17.8/22.6 | 17.2/21.5 |
|  | Power boosting 3dB | 17.7/22.0 | 17.1/21.3 | 16.7/20.7 |
| CDL-D, 20ns | Without power boosting | 13.9/14.8 | 14.2/15.4 | 13.6/14.5 |
|  | Power boosting 3dB | 13.9/14.8 | 13.6/14.6 | 13.2/14.1 |

Table 03: SINR in dB achieving PUSCH BLER of 10% /1% for DFT-s-OFDM and different PTRS patterns

| MCS | Channel | 120 kHz@400 MHz |  |
| --- | --- | --- | --- |
|  |  | (CN, CS) = 8*4 | (CN, CS) = 16*2 |
| 16 | CDL-B, 20ns | 12.1/13.7 | 12.5/14.1 |
|  | CDL-B, 50ns | 12.1/13.2 | 12.3/13.6 |
| 22 | CDL-B, 20ns | 20.9/NAN | 18.5/20.5 |
|  | CDL-B, 50ns | 21.4/NAN | 18.1/20.0 |

- Appendix B.

Table 04. System level evaluation results for indoor scenario A (no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1570.8 | 456.8 | 85.7 | 1506.8 | 490.0 | 95.9 |
|  | 50%ile | 5647.9 | 3130.5 | 1630.7 | 5656.8 | 3153.5 | 1712.7 |
|  | 95%ile | 8913.9 | 8901.2 | 8155.6 | 8929.5 | 8914.1 | 8080.0 |
|  | mean | 5424.9 | 3894.8 | 2388.7 | 5407.0 | 3938.5 | 2452.9 |
| DL delay (s) | 5%ile | 0.0242 | 0.0243 | 0.0264 | 0.0242 | 0.0242 | 0.0258 |
|  | 50%ile | 0.0380 | 0.0682 | 0.1298 | 0.0381 | 0.0676 | 0.1236 |
|  | 95%ile | 0.1415 | 0.4434 | 1.3619 | 0.1433 | 0.4190 | 1.2857 |
|  | mean | 0.0571 | 0.1383 | 0.4710 | 0.0570 | 0.1318 | 0.4465 |
| UL UPT (Mbps) | 5%ile | 915.6 | 310.5 | 86.7 | 899.0 | 329.8 | 89.8 |
|  | 50%ile | 2740.6 | 2070.7 | 1329.6 | 2700.5 | 2060.4 | 1372.9 |
|  | 95%ile | 4500.4 | 4235.8 | 3676.1 | 4429.1 | 4175.5 | 3678.3 |
|  | mean | 2680.1 | 2106.7 | 1521.8 | 2635.4 | 2090.3 | 1538.0 |
| UL delay (s) | 5%ile | 0.0480 | 0.0498 | 0.0565 | 0.0487 | 0.0505 | 0.0572 |
|  | 50%ile | 0.0783 | 0.1034 | 0.1549 | 0.0798 | 0.1038 | 0.1527 |
|  | 95%ile | 0.2326 | 0.6326 | 1.4036 | 0.2360 | 0.6097 | 1.3439 |
|  | mean | 0.1013 | 0.2031 | 0.4990 | 0.1027 | 0.2028 | 0.4878 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.2 | 0.4 | 0.8 | 1.2 |
| 𝜌DL |  | 99.60% | 98.78% | 92.27% | 99.59% | 98.90% | 92.45% |
| 𝜌UL |  | 99.04% | 98.39% | 91.80% | 99.07% | 98.40% | 92.02% |
| BO |  | 22% | 48% | 70% | 22% | 48% | 69% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 5, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 05. System level evaluation results for indoor scenario A (directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1536.6 | 486.6 | 101.3 | 1572.3 | 559.5 | 191.7 |
|  | 50%ile | 5728.0 | 3148.4 | 1715.4 | 5610.2 | 3186.3 | 1880.3 |
|  | 95%ile | 8929.2 | 8912.7 | 8057.0 | 8359.5 | 8337.7 | 7802.4 |
|  | mean | 5432.0 | 3928.0 | 2446.7 | 5304.3 | 3873.5 | 2494.2 |
| DL delay (s) | 5%ile | 0.0242 | 0.0242 | 0.0257 | 0.0258 | 0.0259 | 0.0269 |
|  | 50%ile | 0.0375 | 0.0676 | 0.1225 | 0.0383 | 0.0669 | 0.1129 |
|  | 95%ile | 0.1403 | 0.4190 | 1.1880 | 0.1367 | 0.3723 | 0.8499 |
|  | mean | 0.0564 | 0.1322 | 0.4309 | 0.0557 | 0.1228 | 0.3813 |
| UL UPT (Mbps) | 5%ile | 896.6 | 318.9 | 82.0 | 961.9 | 376.6 | 145.6 |
|  | 50%ile | 2703.0 | 2053.8 | 1350.8 | 2741.9 | 2125.2 | 1435.6 |
|  | 95%ile | 4427.9 | 4156.9 | 3674.3 | 4429.3 | 4331.7 | 3756.6 |
|  | mean | 2639.0 | 2086.1 | 1521.1 | 2698.9 | 2157.0 | 1607.9 |
| UL delay (s) | 5%ile | 0.0487 | 0.0504 | 0.0570 | 0.0487 | 0.0493 | 0.0565 |
|  | 50%ile | 0.0795 | 0.1042 | 0.1550 | 0.0781 | 0.1008 | 0.1463 |
|  | 95%ile | 0.2359 | 0.6222 | 1.3580 | 0.2160 | 0.5311 | 1.0102 |
|  | mean | 0.1025 | 0.2022 | 0.4940 | 0.0989 | 0.1838 | 0.4269 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.2 | 0.4 | 0.8 | 1.2 |
| 𝜌DL |  | 99.60% | 98.89% | 92.80% | 99.59% | 98.98% | 93.54% |
| 𝜌UL |  | 99.07% | 98.44% | 91.67% | 99.10% | 98.64% | 93.06% |
| BO |  | 22% | 48% | 70% | 23% | 48% | 70% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 06. System level evaluation results for indoor scenario A (receiver-only directional LBT)

| Cases |  | receiver-only directional LBT |  |  |
| --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1694.6 | 624.5 | 237.6 |
|  | 50%ile | 6044.3 | 3499.3 | 2035.2 |
|  | 95%ile | 8957.8 | 8858.4 | 8566.0 |
|  | mean | 5675.1 | 4170.1 | 2715.7 |
| DL delay (s) | 5%ile | 0.0241 | 0.0244 | 0.0248 |
|  | 50%ile | 0.0356 | 0.0609 | 0.1104 |
|  | 95%ile | 0.1273 | 0.3372 | 0.7293 |
|  | mean | 0.0521 | 0.1135 | 0.3369 |
| UL UPT (Mbps) | 5%ile | 973.0 | 383.6 | 151.5 |
|  | 50%ile | 2762.1 | 2179.3 | 1472.7 |
|  | 95%ile | 4431.0 | 4335.5 | 3787.7 |
|  | mean | 2708.9 | 2186.9 | 1637.6 |
| UL delay (s) | 5%ile | 0.0487 | 0.0492 | 0.0559 |
|  | 50%ile | 0.0776 | 0.0986 | 0.1429 |
|  | 95%ile | 0.2122 | 0.5235 | 0.9657 |
|  | mean | 0.0983 | 0.1802 | 0.4157 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.2 |
| 𝜌DL |  | 99.63% | 99.15% | 94.43% |
| 𝜌UL |  | 99.10% | 98.67% | 93.34% |
| BO |  | 22% | 47% | 68% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |

Table 07. System level evaluation results for indoor scenario A (no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1521.7 | 441.2 | 34.7 | 1583.4 | 456.9 | 52.4 |
|  | 50%ile | 5043.8 | 2906.9 | 1145.7 | 5053.4 | 3015.4 | 1223.9 |
|  | 95%ile | 8910.1 | 8626.6 | 6789.3 | 8927.6 | 8851.4 | 6963.3 |
|  | mean | 5083.6 | 3517.4 | 1842.4 | 5149.1 | 3590.2 | 1964.1 |
| DL delay (s) | 5%ile | 0.0242 | 0.0249 | 0.0313 | 0.0242 | 0.0243 | 0.0304 |
|  | 50%ile | 0.0427 | 0.0732 | 0.1813 | 0.0424 | 0.0707 | 0.1684 |
|  | 95%ile | 0.1399 | 0.4582 | 1.8838 | 0.1346 | 0.4381 | 1.7086 |
|  | mean | 0.0581 | 0.1387 | 0.6160 | 0.0569 | 0.1359 | 0.5564 |
| UL UPT (Mbps) | 5%ile | 923.9 | 301.5 | 44.5 | 902.6 | 334.5 | 40.9 |
|  | 50%ile | 2410.6 | 1716.5 | 946.9 | 2379.6 | 1784.9 | 970.7 |
|  | 95%ile | 4447.3 | 3841.6 | 3240 | 4383.4 | 3804.6 | 3236.9 |
|  | mean | 2474.2 | 1882.6 | 1208.4 | 2436.5 | 1884.3 | 1218.8 |
| UL delay (s) | 5%ile | 0.0481 | 0.0557 | 0.0650 | 0.0491 | 0.0557 | 0.0648 |
|  | 50%ile | 0.0890 | 0.1206 | 0.2171 | 0.0898 | 0.1199 | 0.2120 |
|  | 95%ile | 0.2269 | 0.6403 | 2.002 | 0.2325 | 0.5775 | 2.0632 |
|  | mean | 0.1066 | 0.2054 | 0.6635 | 0.1082 | 0.1986 | 0.6692 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 1.0 | 0.25 | 0.6 | 1.0 |
| 𝜌DL |  | 99.70% | 98.67% | 89.27% | 99.68% | 98.43% | 89.98% |
| 𝜌UL |  | 98.79% | 98.26% | 89.66% | 98.79% | 98.08% | 89.49% |
| BO |  | 16% | 45% | 74% | 16% | 44% | 72% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 5, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 08. System level evaluation results for indoor scenario A (directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1581.3 | 328.9 | 39.3 | 1591.0 | 506.1 | 88.9 |
|  | 50%ile | 4862.9 | 2696.7 | 1178.4 | 5149.8 | 3037.9 | 1400 |
|  | 95%ile | 8923.7 | 8841.6 | 6820.2 | 8348.1 | 8284.5 | 6685.9 |
|  | mean | 5070.8 | 3423.3 | 1852.7 | 5044.8 | 3568.9 | 2033.6 |
| DL delay (s) | 5%ile | 0.0242 | 0.0243 | 0.0313 | 0.0259 | 0.0260 | 0.0317 |
|  | 50%ile | 0.0439 | 0.0794 | 0.1754 | 0.0418 | 0.0698 | 0.1489 |
|  | 95%ile | 0.1337 | 0.5506 | 1.7758 | 0.1335 | 0.3909 | 1.2342 |
|  | mean | 0.0569 | 0.1625 | 0.6028 | 0.0563 | 0.1259 | 0.4852 |
| UL UPT (Mbps) | 5%ile | 913.6 | 282.4 | 16.6 | 987.4 | 366.3 | 76.4 |
|  | 50%ile | 2359.1 | 1695.8 | 950.9 | 2427.1 | 1861.2 | 1067.5 |
|  | 95%ile | 4346.9 | 3780.7 | 3142.6 | 4388.0 | 3823.8 | 3347.9 |
|  | mean | 2410.8 | 1829.2 | 1193.5 | 2485.1 | 1942.1 | 1299.1 |
| UL delay (s) | 5%ile | 0.0492 | 0.0568 | 0.0663 | 0.0491 | 0.0548 | 0.0629 |
|  | 50%ile | 0.0913 | 0.1250 | 0.2177 | 0.0870 | 0.1148 | 0.1953 |
|  | 95%ile | 0.2320 | 0.6401 | 2.1766 | 0.2158 | 0.5371 | 1.5271 |
|  | mean | 0.1092 | 0.2203 | 0.6956 | 0.1047 | 0.1871 | 0.5598 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 1.0 | 0.25 | 0.6 | 1.0 |
| 𝜌DL |  | 99.70% | 98.34% | 89.19% | 99.68% | 98.60% | 91.22% |
| 𝜌UL |  | 99.00% | 97.86% | 88.13% | 98.79% | 98.37% | 91.28% |
| BO |  | 16% | 46% | 73% | 16% | 44% | 72% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 09. System level evaluation results for indoor scenario A (receiver-only directional LBT)

| Cases |  | receiver-only directional LBT |  |  |
| --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1782.7 | 586.0 | 135.6 |
|  | 50%ile | 5568.2 | 3326.4 | 1514.8 |
|  | 95%ile | 8863.0 | 8813.6 | 7150.0 |
|  | mean | 5402.7 | 3857.1 | 2215.7 |
| DL delay (s) | 5%ile | 0.0244 | 0.0245 | 0.0297 |
|  | 50%ile | 0.0386 | 0.0634 | 0.1372 |
|  | 95%ile | 0.1195 | 0.3514 | 1.0153 |
|  | mean | 0.0517 | 0.1148 | 0.4364 |
| UL UPT (Mbps) | 5%ile | 999.6 | 397.9 | 90.4 |
|  | 50%ile | 2462.5 | 1869.8 | 1105.4 |
|  | 95%ile | 4386.4 | 3822.1 | 3385.8 |
|  | mean | 2494.4 | 1967.4 | 1340.0 |
| UL delay (s) | 5%ile | 0.0491 | 0.0543 | 0.0618 |
|  | 50%ile | 0.0866 | 0.1143 | 0.1875 |
|  | 95%ile | 0.2096 | 0.4987 | 1.3874 |
|  | mean | 0.1038 | 0.1805 | 0.5289 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 1.0 |
| 𝜌DL |  | 99.70% | 98.84% | 92.39% |
| 𝜌UL |  | 98.79% | 98.43% | 91.78% |
| BO |  | 16% | 43% | 70% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |