3GPP TSG-RAN WG1 Meeting #104-e	R1-2101271

# E-meeting,  January 25th - February 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev | x | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Type II CSI reporting in Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For CSI reporting of eType II codebook, the reported elements are divided into three groups with different priority levels. For example, in group 1, the KNZ/2-υ highest priority elements of i2,4,l and and the KNZ/2-υ highest priority elements of i2,5,l are reported. However, the value of KNZ/2-υ may be negative in some parameter settings, aslo for the value of KNZ-v=KNZ2+KNZ2-υ. For example, if paramCombination-r16=1, in which L,pv={1,2},pv={3,4}, β=(2,14,18,14), and R=1, υ=4, for different values of N3, the values of K0 are listed in the table below. N3; M1=p1N3; K0=β2LM1; KNZ≤2K0; KNZ/2-υ; KNZ-υ. 3; 1; 1; ≤2; ≤-3; ≤-2. 4; 1; 1; ≤2; ≤-3; ≤-2. 5; 2; 2; ≤4; ≤-2; ≤0. 6; 2; 2; ≤4; ≤-2; ≤0. 7; 2; 2; ≤4; ≤-2; ≤0. 8; 2; 2; ≤4; ≤-2; ≤0. 9; 3; 3; ≤6; ≤-1; ≤2. 10; 3; 3; ≤6; ≤-1; ≤2. 11; 3; 3; ≤6; ≤-1; ≤2. 12; 3; 3; ≤6; ≤-1; ≤2. If paramCombination-r16=2, in which L,p{1,2},p{3,4}, β=(2,14,18,12), and R=1, υ=4, for different values of N3, the values of K0 are listed in the table below. N3; M1=p1N3; K0=β2LM1; KNZ≤2K0; KNZ/2-υ; KNZ-υ. 3; 1; 2; ≤4; ≤-2; ≤0. 4; 1; 2; ≤4; ≤-2; ≤0. If paramCombination-r16=3, in which L,p{1,2},p{3,4}, β=(4,14,18,14), and R=1, υ=4, for different values of N3, the values of K0 are listed in the table below. N3; M1=p1N3; K0=β2LM1; KNZ≤2K0; KNZ/2-υ; KNZ-υ. 3; 1; 2; ≤4; ≤-2; ≤0. 4; 1; 2; ≤4; ≤-2; ≤0. |
| Summary of change: |  | - Clarify the highest priority elements of i2,4,l and i2,5,l in priority group 1 to be reported, if KNZ/2-υ can be negative. - Clarify the lowest priority elements (or remaining elements) of i2,4,l and i2,5,l in priority group 2 to be reported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reported elements in priority groups 1 and 2 can be misleading if KNZ/2-υ is negative so that potential gNB/UE can have different interpretation of given CSI reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of the text proposal >

5.2.3	CSI reporting using PUSCH

< Unchanged part omitted>

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where  is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority  is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the  CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in increasing order with the lowest subband of csi-ReportingBand as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level.

-	For Enhanced Type II reports, for a given CSI report n, each reported element of indices  i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1, i1,2 and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the  max(0, KNZ2-υ)KNZ/2-υ highest priority elements of i2,4,l and the  max(0, KNZ2-υ)KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the max(0,KNZ-υ-max0, KNZ2-υ)KNZ/2 lowest priority elements of i2,4,l and the max(0,KNZ-υ-max0, KNZ2-υ)KNZ/2 lowest priority elements of i2,5,l (l=1,…,υ).

< End of the text proposal >