3GPP TSG RAN WG1 Meeting #104-e	R1-2101274

E-meeting, January 25th – February 5th, 2021

Agenda Item:	8.1.5

Source:	Huawei, HiSilicon, China Unicom

Title:	Further details on CSI Enhancements for Rel-17

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss CSI enhancements for Rel-17 to provide supplementary information for the main paper [1]. More simulation results and details are discussed here.

Simulation Results of Rel-17 PS CB enhancement based on angle and delay reciprocity

Based on the discussion of RAN1#103e, in this section, we compared the performance of Rel-17 PS CB and R16 regular enhanced type II codebook by SLS based on agreements in RAN1#102e.

For Rel-17 PS CB, 8 ports are used to convey 32 SD-FD beamforming bases by utilizing mechanism conveying SD-FD beamforming bases on CSI-RS ports in Section 3.1. In order to focus on main design principle of the scheme and verify the performance, we consider a specific case for Alt 5 of Rel-17, where is W1 is an identity matrix and Mv= N=1 for Wf , so there is no UCI payload to report W1 and Wf. In addition, binomial coefficient is used to indicate y coefficients selected from x coefficients, which is denoted “Coefficient = {xSy} and x=PK=32” in legends in simulation results figures. In the simulation, both non-DFT bases and DFT bases are simulated based on the second order statistics of FDD uplink channel. More simulation assumption can be found in Table A-1 of Appendix A.

Figure 1-(1) gives the performance of R16 regular CB, R16 PS CB with Eigen bases for SD and Rel-17 PS CB with Eigen bases for both SD and FD under Rank adaptation (up to Rank 2 per UE) with SRS error and calibration error. It can be observed that with help of Eigen bases for SD, R16 PS CB is slightly better than R16 regular CB. R17 PS CB with Eigen bases for both SD-FD can provide about 17.6% performance gain over R16 regular CB at low overhead if the same reporting overhead is assumed; at high overhead, about 15.0% performance gain can be achieved for the same reporting overhead.

Note that by utilizing angle and delay reciprocity, free bases selection and non-DFT-based channel decomposition especially in frequency domain can be achieved at gNB sides for R17 PS CB, which could provide significant performance gain. Furthermore, R17 PS CB can achieve finer precoding granularity for PMI (larger R, numberOfPMISubbandsPerCQISubband), enable finer precoding granularity and provide better performance. In the simulation, R of R16 PS CB equals to 2 and R of R17 PS CB equals to 4. In addition, for Rank adaptation up to Rank2, by taking advantage of more accurate CSI quantization, R17 PS CB can eventually increase scheduling possibility of higher rank after MU-MIMO scheduling from SLS, e.g., the percentage of rank 2 transmission have increased by at least 5% within following evaluations compared to the baseline.

In addition, Figure 1-(2) gives the performance of R16 regular CB, R16 PS CB with DFT bases for SD and Rel-17 PS CB with DFT bases for both SD-FD under Rank adaptation (up to Rank 2 per UE) with SRS error and calibration error. It can be observed that R16 PS CB with DFT bases for SD is worse than R16 regular CB, however R17 PS CB with DFT bases for both SD-FD can still provide performance gain than R16 regular CB, e.g. about 5% performance gain at low overhead and about 8% performance gain at high overhead if the same reporting overhead is assumed. This is because though DFT bases is used for SD and FD, by utilizing angle and delay reciprocity, free bases selection and finer precoding granularity for PMI can be achieved for R17 PS CB, which could provide significant performance gain.

- Non-DFT bases

- DFT bases

Figure 1. Performance between R16 regular CB and Rel-17 PS CB without and with SRS error and calibration error

Observation 1: Compared to Rel-16 regular codebook,

- Rel-17 PS CB with Eigen bases for both SD and FD can provide 18% mean UPT gain at low overhead and 15% mean UPT gain at high overhead if the same reporting overhead and R=4 is assumed;

- Rel-17 PS CB with DFT bases for both SD and FD can provide 5% mean UPT gain at low overhead and 8% mean UPT gain at high overhead if the same reporting overhead and R=4 is assumed.

Details on mechanism to convey SD-FD bases using CSI-RS ports

As discussed in [1], multiple SD-FD beamforming bases per CSI-RS port can provide lower CSI-RS overhead and more configurable CSI-RS resources per slot, which should be supported by R17 codebook structure. In this section, we will more details on the procedure related to convey multiple beamforming bases over one CSI-RS port so that the definition of the CSI-RS port remain unchanged. Before descripting the details, we have the following notations: the CSI-RS beamforming bases obtained from the 2nd order statistics are denoted as B=[b1,…bq,…,bQ]∈CNtxNf×Q , where bq∈CNtxNf×1 is the SD and FD beamforming base corresponding to the q-th angle-delay pair, Ntx is the number of antenna ports at gNB, Nf is the number of frequency unit, which is related to number of RBs (NRB) for CSI-RS transmission, number of RBs (NRBCQI) per CQI subband and R (numberOfPMISubbandsPerCQISubband), specifically, R=NRBCQI(NRB/Nf).

FDM-like mechanism to convey SD-FD bases using CSI-RS ports

FDM-like mechanism to convey SD-FD bases using CSI-RS ports is to divide frequency units of each CSI-RS port into multiple groups and each group corresponds to a sub-sampled CSI-RS beamforming vector. As illustrated in Figure 2, for example, it is assumed that there are two CSI-RS beamforming vectors b1∈CNtxNf×1 and b2∈CNtxNf×1. And Nf frequency units of the p-th port are assumed to be divided into two groups and each group contains Nf2 frequency units. The CSI-RS beamforming vector sampled from b1 in frequency domain can be applied to the first frequency unit group (red blocks in Figure 2). And the CSI-RS beamforming vector sampled from b2 in frequency domain can be applied to the second frequency unit group (blue blocks in Figure 2). Therefore for given CSI-RS port p-th, it eventually can accommodate two sampled CSI-RS SD-FD beamforming vectors.

Figure 2. Illustration of frequency unit division corresponding to two SD-FD beamforming vectors

Generally, suppose that one CSI-RS port can carry K sampled CSI-RS beamforming vectors so that Nf frequency units of each CSI-RS port can be divided into K groups, and the each group contains NfK=N frequency units whose index sequence is Γk∈NN×1. In addition, the index sequence Γk of the k-th group can be a subsequence of Γ={1,2,…,Nf}∈NNf×1 with an offset of k and an interval of K, which can be expressed as Γk={k,…,n×K+k,…,N-1×K+k}∈NN×1, k={1,2…,K}.

Based on given grouping pattern, a total of P=QK CSI-RS ports are required. Denote the n-th frequency unit of the k-th sampled CSI-RS beamforming vector on the p-th port as bp,n,k∈CNtx×1, which can be obtained by sampling bpk∈CNtxNf×1 in frequency domain according to the index sequence Γk. Specifically, bp,n,k= bpkΓk(n)-1×Ntx+1:Γk(n)×Ntx)∈CNtx×1, where bpk is the pk-th CSI-RS beamforming vector of B and the pk can be indexed as pk=k-1×P+p. Then the K sampled CSI-RS beamforming vectors are multiplexed on the p-th CSI-RS port, which can be shown in Figure 3.

Figure 3. Illustration of K sampled CSI-RS beamforming vectors multiplexed on each CSI-RS port

Furthermore, the precoder bp'∈CNtxNf×1 of p-th CSI-RS port can be expressed equivalently as bp'=bp,1,1T,…,bp,n,kT,…,bp,N,KTT=k=1Kbpk⨀dk1, where ⨀ and ⨂ are Hadamard product and Kronecker product respectively, 1∈NNtx×1 is a vector in which all elements are one, and dk∈NNf×1 is denoted as the frequency domain sampling vector of the k-th group, which is determined by the index sequence Γk, that is, dkΓk(n)=1, n∈{1,…,N} and the remaining Nf-N elements are set to zero.

When the enhanced beamforming scheme mentioned above is adopted, the UE can determine the combination coefficients by summing interleaved beamforming CSI-RS ports in a CSI-RS resource based on above grouping pattern. Taking grouping pattern shown in Figure 2 as an example, the calculation of two combination coefficients at the p-th CSI-RS port corresponding to two beamforming vectors b1 and b2 is shown in Figure 4.

Figure 4. Illustration of UE calculating combination coefficients based on the grouping pattern given in Figure 2

From UE perspective, the UE should calculate and feedback combination coefficients in the following two steps.

Step1: According to one CSI-RS port corresponding to K sampled CSI-RS beamforming vectors shown in Figure 3, UE will receive beamformed downlink channel matrix HrDL∈CP×Nf for given r-th receiving antenna of UE. The coefficient cr,p,k of the k-th sampled CSI-RS beamforming vector of the p-th CSI-RS port for the r-th UE receiving antenna can be obtained by summing interleaved the p-th beamforming CSI-RS port as shown in (1).

cr,p,k=HrDLp,:dk  (1)

where HrDLp,:∈C1×Nf is the beamformed downlink channel matrix of the p-th CSI-RS port for the r-th UE antenna port, dk∈NNf×1 is the frequency domain sampling vector of the k-th frequency units group. Note that, for the r-th UE receiving antenna, the coefficient cr,p,k is an approximation of the coefficient corresponding to the k-th non-sampled CSI-RS beamforming vector of the p-th CSI-RS port.

Then the coefficient vector cr consisted of the coefficients of P×K beamforming vectors on r-th UE antenna port can be obtained as cr=cr,1,1,…,cr,1,K,…,cr,P,1,…,cr,P,KT∈CPK×1. Of course, other order of the P×K coefficients can also be further studied and determined by a predefined rule in specification when defining/reporting the final PMI by the UE.

Step2: For Nrx receiving antenna ports, UE constructs the coefficient matrix C=c1,⋯,cNrxT∈CNrx×PK. And denote the l-th eigenvector of CHC as el=c1,1,…,c1,K,…,cP,1,…,cP,KT∈CPK×1. For layer l, the UE shall select and report some coefficients from el based on the codebook structure W=W1W2WfH.More specific details of W1, Wf and W2 can be found in [1].

Based on W1, Wf and W2 reported by UE, gNB can re-construct the best precoder vector V for the first layer as follows

W=W1W2WfH∈CP×K

V=p=1Pk=1KWp,kbpk*  	(2)

where V=[v1T, …,vnT,…,vNfT]T∈CNtxNf×1, vn∈CNtx×1 is the precoder vector at frequency unit n, Wp,k is the p-th row, k-th column element of W, which is the coefficient corresponding to the k-th beamforming vector on the p-th CSI-RS port. And bpk is the pk-th CSI-RS beamforming vector of B, which is known by the gNB only, with DFT, SVD, or other SD/FD precoding bases preferred by NW implementation.

Observation 2: Dividing frequency units into multiple groups carrying multiple sampled SD-FD beamforming vectors can effectively improve the utilization of UE-specific CSI-RS ports/resources.

CDM-like mechanism to convey SD-FD bases using CSI-RS ports

In this section, another enhanced beamforming scheme that can effectively improve CSI-RS port utilization will be presented in detail.

In accordance with the above, assumed that B=[b1,…bq,…,bQ]∈CNtxNf×Q  is consisted of Q CSI-RS beamforming vectors obtained from the 2nd order statistics, where bq∈CNtxNf×1 is q-th beamforming vector of B, Ntx is the number of antenna ports at gNB, Nf is the number of frequency unit. And the Q CSI-RS beamforming vectors can be sorted in descending order based on their corresponding eigenvalues if the SVD bases are adopted. The the antenna port number at UE can be assumed as Nrx.

Based on design principles of CSI acquisition described in [2], when one CSI-RS port can carry multiple CSI-RS beamforming vectors, the gNB can shift the delay of the corresponding multiple angle-delay pairs  to multiple delay positions. As shown in Figure 5, for example, it is also assumed that there are two CSI-RS beamforming vectors b1∈CNtxNf×1 and b2∈CNtxNf×1, which corresponds to the two estimated angle-delay pairs respectively. And denote b1∈CNtxNf×1 and b2∈CNtxNf×1 are two weighted CSI-RS beamforming vectors that can be multiplexed on one CSI-RS port. To be more specific, b1=b1⨀d11, where ⨀ and ⨂ are Hadamard product and Kronecker product respectively, 1∈NNtx×1 is a vector in which all elements are one, and d1∈CNf×1 can be a DFT vector where d1=[1,…,e-j2πnNfn1,…,e-j2πNf-1Nfn1]T. Similarly, b2=b2⨀d21, where d2=[1,…,e-j2πnNfn2,…,e-j2πNf-1Nfn2]T∈CNf×1 is a DFT vector. Then b1 and b2 are multiplexed on one CSI-RS port, which is equivalent to shifting the delay of the two estimated angle-delay pairs to the delay positions n1 and n2 respectively. UE is restricted to calculate the combination coefficients corresponding to the two estimated angle-delay pairs at the delay positions n1 and n2.

Moreover, similar procedures can be applied to all CSI-RS ports and all CSI-RS ports can correspond to the same delay positions n1 and n2

Figure 5 Illustration of 3.2	CDM-like mechanism

Generally, when one CSI-RS port carries K CSI-RS beamforming vectors, there are totally P=QK CSI-RS ports associated with Q CSI-RS beamforming vectors. Denote dk∈CNf×1 as the DFT vector corresponding to the delay position nk , where dk=[1,…,e-j2πnNfnk,…,e-j2πNf-1Nfnk]T, k ϵ {1,2,…,K}. Moreover, the K delay positions can be uniformly selected among Nf delay positions. And the k-th delay position can be expressed as nk=NfKk-1. For example, when K=4, the four delay positions are 0, Nf4, 2Nf4 and 3Nf4, respectively.

Then the K weighted CSI-RS beamforming vectors multiplexed on the p-th CSI-RS port are shown in Figure 6, where bp,n,k∈CNtx×1  is denoted as the n-th frequency unit of the k-th beamforming vector on the p-th CSI-RS port, which can be represented as bp,n,k=bpkn-1×Ntx+1:(n×Ntx), bpk∈CNtxNf×1 is the pk-th beamforming vectors of B and the pk can be indexed as pk=k-1×P+p. And dk(n) is the n-th element of dk.

Figure 6 Illustration of K sampled CSI-RS beamforming vectors multiplexed on each CSI-RS port

Based on the beamforming scheme shown in Figure 6, the coefficient cr,p,k of the k-th beamforming vector of the p-th CSI-RS port of the r-th UE receiving antenna port of UE can be obtained by weighted sum of the beamformed downlink channel.

cr,p,k=HrDLp,:dk*

where HrDLp,:∈C1×Nf is the beamformed downlink channel matrix of the p-th CSI-RS port of the r-th UE antenna port, dk∈CNf×1 is the DFT vector corresponds to the k-th delay position.

Similarly, the coefficient vector of the r-th UE antenna port can be obtained as cr=cr,1,1,…,cr,1,K,…,cr,P,1,…,cr,P,KT∈CPK×1. And UE feedbacks the coefficients in the same way as described in Step 2 of Section 3.1

Following the same reconstruction process as described in Section 3.1, the gNB can reconstruct the precoder based on the W1, W2, and Wf.

Observation 3: Applying multiple beamforming vectors over single CSI-RS port, with FDM or CDM multiplexing manners, can effectively improve utilization of CSI-RS.

Conclusions

This contribution provides further details on CSI enhancements for Rel-17. In summary, the following observations are made.

Observation 1: Compared to Rel-16 regular codebook,

- Rel-17 PS CB with Eigen bases for both SD and FD can provide 18% mean UPT gain at low overhead and 15% mean UPT gain at high overhead if the same reporting overhead and R=4 is assumed;

Rel-17 PS CB with DFT bases for both SD and FD can provide 5% mean UPT gain at low overhead and 8% mean UPT gain at high overhead if the same reporting overhead and R=4 is assumed.

Observation 2: Dividing frequency units into multiple groups carrying multiple sampled SD-FD beamforming vectors can effectively improve the utilization of UE-specific CSI-RS ports/resources.

Observation 3: Applying multiple beamforming vectors over single CSI-RS port, with FDM or CDM multiplexing manners, can effectively improve utilization of CSI-RS.

- References

[1] R1-2100214, CSI Enhancements for Rel-17, Huawei, HiSilicon, RAN1#104e, E-meeting, January 25th – February 5th, 2021.

[2] R1-2005248, Enhancements on CSI for Rel-17, Huawei, RAN1#102e, E-meeting, 17th –28th August, 2020.

- Appendix A.

Table A-1 SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Urban Macro |
| Frequency Range |  | FR1 only, 2.1GHz, with duplexing gap of 200MHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ Other configuration is not precluded. |
| BS Tx power |  | 44dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with rank adaptation |
| CSI feedback |  | Feedback assumption: - CSI feedback periodicity (full CSI feedback) : 5ms - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| SRS Configuration |  | - SRS periodicity with 10ms - Comb: 2 - Number of OFDM symbols: 2 |
| SRS Error Model |  | SRS error Modelling in Table A.1-2 in 36.897. Δ=9dB and detailed derivation of Δ can be found in R1-144943 |
| Calibration error model at gNB |  | HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN amplitude error (expressed in decibels) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation for simulation bandwidth , respectively |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI reporting overhead |