3GPP TSG-RAN WG1 Meeting #104-e	R1-2101303

# e-Meeting, January 25th – February 5th, 2021

Title:	[DRAFT] Reply LS on the use of simultaneous CSI-RS resources and ports

Response to: 	R2-2011274, LS on the use of simultaneous CSI-RS resources and ports

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson

To:	RAN WG2

Contact Person:

#### Name:	Stephen Grant

E-mail Address:	stephen.grant@ericsson.com

Attachments:	None

1. Overall Description:

In R1-200022(R2-2011274), RAN2 has discussed the use of simultaneous CSI-RS resources and ports, and has asked RAN1 for clarification on the following aspect:

In RAN2's understanding, the field maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC is supposed to indicate a restriction on the number of active CSI-RS resources. Furthermore, the field totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC is supposed to indicate a restriction on the total number of ports to be used in active CSI-RS resources.

Question to RAN1:

RAN2 has asked RAN1 whether the fields maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC and totalNumberPortsSimultaneousNZP-CSI-RS are supposed to indicate: the number of active CSI-RS resources and ports, respectively; or the number of configured CSI-RS resources and ports (including active CSI-RS resources and ports), respectively?

The reason for the question is that the current field description for maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC in TS 38.306 includes “This parameter limits the total number of NZP-CSI-RS resources that the NW may configure across all CCs, and across MCG and SCG in case of NR-DC (irrespective of the associated codebook type).”; and the current field description for totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC  in TS 38.306 includes “This parameter limits the total number of ports that the NW may configure across all NZP-CSI-RS resources across all CCs”.

Response to Question:

In short, it is RAN1's understanding that the the two capability parameters refer to the number of active CSI-RS resources and ports, respectively.

In more detail, it is RAN1's understanding that the key word "simultaneous" in the field description of the capability parameters refers to CSI-RS resources and ports on which the UE is currently measuring, and thus "active" refers to CSI-RS resources/ports that fufill the following:

- Ports of a periodic CSI-RS resource

- Ports of a semi-persistent CSI-RS resource that has been activated by MAC-CE

- Ports of an aperiodic CSI-RS resource that has been triggered by DCI and on which the UE has not completed its measurement

Semi-persistent CSI-RS resources that have not yet been activated or have been deactivated as well as aperiodic CSI-RS resources for which the UE has completed its measurement do not count toward "active." In this sense, the UE capability parameters do not refer to the number of configured CSI-RS resources/ports. Hence the sentences in the the current field description for maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC in TS 38.306 that refer to the total number of resources/ports that the NW may configure is not needed and could be removed by RAN2, especially since it can create confusion.

One possibility approach for resolving potential ambiguity could be to make the following change to TS 38.306:

-     maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC indicates the maximum number of simultaneous CSI-RS resources (irrespective of the associated codebook type) in active BWPs across all CCs, and across MCG and SCG in case of NR-DC. This parameter limits the total number of NZP-CSI-RS resources that the NW may configure across all CCs, and across MCG and SCG in case of NR-DC (irrespective of the associated codebook type). The network applies this limit in addition to the limits signalled in MIMO-ParametersPerBand-> maxNumberSimultaneousNZP-CSI-RS-PerCC and in Phy-ParametersFRX-Diff-> maxNumberSimultaneousNZP-CSI-RS-PerCC;

-     totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC indicates the total number of CSI-RS ports (irrespective of the associated codebook type) in simultaneous CSI-RS resources in active BWPs across all CCs, and across MCG and SCG in case of NR-DC. This parameter limits the total number of ports that the NW may configure across all NZP-CSI-RS resources across all CCs, and across MCG and SCG in case of NR-DC (irrespective of the associated codebook type). The network applies this limit in addition to the limits signalled in MIMO-ParametersPerBand-> totalNumberPortsSimultaneousNZP-CSI-RS-PerCC and in Phy-ParametersFRX-Diff-> totalNumberPortsSimultaneousNZP-CSI-RS-PerCC.

2. Actions:

To RAN2:

ACTION:  RAN1 respectfully asks RAN2 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #104-bis-e 	12 Apr. – 20 Apr. 2021		Electronic Meeting

TSG-RAN WG1 Meeting #105-e 	19 – 27 May 2021		Electronic Meeting