3GPP TSG RAN WG1 #104-e 								R1-2101327

e-Meeting, January 25th – February 5th, 2021

Agenda item:	8.8.1.1 Enhancements on PUSCH repetition type A

Source: 	Sierra Wireless

Title: 	Design considerations for PUSCH repetition Type A Enhancements

Document for: 	Discussion and decision

# Introduction

This tdoc discusses design considerations for PUSCH repetition Type A Enhancements for the following work item [1] objectives:

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

# Number of Repeats based on available UL Slots

This section discusses this objective stated in the WID[1]:

The number of repetitions counted on the basis of available UL slots.

With the legacy specification, repeats are counted whether the slot is available for UL transmission or not (i.e. repeats are punctured). This leads to a very low number (e.g. ~ 1/5th) of actual slots of transmission for TDD cases when repeats are used since so many repeats get punctured.  To fix this issue, repeats should be postponed instead of punctured for slots which are not available for UL transmission.  For example, postponed repetitions of 8 would look like this for a TDD pattern of DDDSU:

|  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subframe # | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 |
| TDD Slot | D | D | D | S | U | D | D | D | S | U | D | D | D | S | U | D | D | D | S | U | D | D | D | S | U | D | D | D | S | U | D | D | D | S | U | D | D | D | S | U | D |
| PDCCH | UG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PUSCH |  |  |  |  | U1 |  |  |  |  | U2 |  |  |  |  | U3 |  |  |  |  | U4 |  |  |  |  | U5 |  |  |  |  | U6 |  |  |  |  | U7 |  |  |  |  | U8 |  |

Figure 1: Example of Postponed Repeats for TDD

A new repeat type is specified “postponed repeats” where repeats are postponed for invalid UL slots.

# Increasing the maximum number of repetitions

This section discusses this objective stated in the WID[1]:

Increasing the maximum number of repetitions up to a number to be determined during the course of the work

As stated in the WID, the maximum number of repetitions is FFS. There are many factors to consider when deciding how much to increase the maximum number of repeats.

The TR 38.830[3] gave guidance that repeats should be increased to a maximum of 32:

“Increasing the maximum number of repetitions, e.g., up to 32.”

TR 38.830 gave guidance as a conclusion that repeats should be increased up to 32

eMBB Use Case:

The minimum data rate for the eMBB use case was agreed to be 100 kbps and is also documented in the WID. The figure below shows the LLS result for repeats [8 and 16] and TBS [888 and 1800] which corresponds to a data rate of 100kbps (detailed simulation assumptions are in appendix A):

Figure 2: LLS Result for 100kbps, Repeats=8 and Repeats=16

As seen from the above figure, 16 repeats does not provide coverage gain compared to 8 repeats. This is expected since the energy per bit is the same since the data rate is kept constant at 100kbps. In fact, there is a small loss in coverage with 16 repeats. This is due to the loss of coding gain when using the larger TBS = 1800 since a higher code rate is needed. Some of this loss is recovered at Fdop=25Hz and 50Hz due to the increase in time diversity with 16 repeats but this is not enough to make up for the code gain loss.

For a data rate of >=100kbps (eMBB), increasing repeats beyond 8 does not provide any coverage gain

VoIP Use Case:

The following FR1 simulation assumption for VoIP were agreed:

Vocoder generate one 320 bits TB every 20ms

Maximum latency 50/100ms

FR1 TDD frame structure: DDDSU and DDDDDDDSUU with SCS = 30 kHz

FR1 FDD  SCS = 15 kHz

Since coverage is generally proportional to the time and power of transmission, we can use these factors to determine the maximum number of repeats needed.  If latency beyond 20ms is acceptable, TBs need to be aggregated (i.e. a larger TBS is used) to avoid the need for the UE transmit two TBs simultaneously which would split the UE’s power and decrease coverage. To avoid TB transmissions overlapping, the transmission time needs to be less than or equal to the vocoder packet aggregation time. Given 100ms was agreed to be the maximum latency, the maximum number of aggregations of the 20ms  vocoder packets is 3. The figure below shows an example of  3 aggregated vocoder packets which result in 60ms of transmission time and a latency of 100ms:

| 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aggregated TB #1 TBS = 960 bits |  |  | Aggregated TB #2 TBS = 960 bits |  |  | Aggregated TB #3 TBS = 960 bits |  |  |
|  |  |  | Max Transmit time =60ms #1 TBS = 960 bits |  |  | Max Transmit time =60ms #2 TBS = 960 bits |  |  |

Figure 3: Example of 3 aggregated vocoder packets resulting in 100ms latency

Given the above the following observation can be made:

Assuming a maximum latency of 100ms for VoIP, the maximum transmission time is 60ms

Given the above observation, and given the TDD formats of DDDSU and DDDDDDDSUU, the following observation can be made for TDD:

For FR1 TDD VoIP with 100ms latency, the maximum number of repeats that can be used is 24.

The FDD case is more complex because HARQ can be effectively be used to increase transmissions since there are no scheduling restrictions and the time for re-scheduling is small (e.g. 2 slot) compared to the maximum number of repeats (e.g. 32).  Since only 2 slots are needed to schedule a retransmission, two HARQs of 29 repeats each can be scheduled within 60ms which will provide very similar coverage as one with 60 repeats.

For FDD VoIP with 100ms latency, HARQ can be effectively used to transmit more repeats when needed thus no need to increase maximum number of repeats beyond 29.

The study item[2] states:  "LPWA services and scenarios are not included”  thus 3GPP should not specify repeats similar to or beyond that of LTE-M and NB-IOT.  LTE-M uses a maximum of 32 repeats for CE mode A which is largely commercialized so no more than 32 repeats should be specified during this NR WI.

The SID states “LPWA services are not included” thus limiting the maximum number of repeats

Given the above observations, the following proposal is made:

The maximum number of inter-slots repeats shall be increased to 32 slots.

# Conclusions

How to increase number of repeats for TDD?

- A new repeat type is specified “postponed repeats” where repeats are postponed for invalid UL slots.

How much to Increasing the maximum number of repetitions?

- TR 38.830 gave guidance as a conclusion that repeats should be increased up to 32

For a data rate of >=100kbps (eMBB), increasing repeats beyond 8 does not provide any coverage gain

Assuming a maximum latency of 100ms for VoIP, the maximum transmission time is 60ms

For FR1 TDD VoIP with 100ms latency, the maximum number of repeats that can be used is 24.

For FDD VoIP with 100ms latency, HARQ can be effectively used to transmit more repeats when needed thus no need to increase maximum number of repeats beyond 29.

The SID states “LPWA services are not included” thus limiting the maximum number of repeats

The maximum number of inter-slots repeats shall be increased to 32 slots.

# References

RP-202928  China Telecom, “New WID on NR coverage enhancements”

RP-200861 China Telecom, “Study on NR coverage enhancements”

TR 38830 “Study on NR coverage enhancements”

# Appendix A: LLS Simulation Assumptions:

| Parameter | Value |
| --- | --- |
| System bandwidth | 20 MHz |
| Configuration | FDD |
| Carrier frequency | 700MHz |
| Antenna configuration | NA |
| RX Antenna ports | 2 |
| Channel model | TDL-C 37ns |
| Doppler frequency | 2, 25, 50, 100 Hz |
| Number of PRBs | 4 |
| Frequency tracking error | 0Hz |
| Channel Estimation | Perfect |
| Frequency Hopping | None |
| DMRS Configuration | 2 DMRS per slot |
| Transform Pre-coding | No |
| SCS | 15 kHz |
| Modulation | QPSK |
| TBS | 888 and 1800 bits |
| Repeats | 8 and 16 |