3GPP TSG RAN WG1 #104-e			R1-2101368

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.15.1

Source:	Apple

Title:	On Link Budget of IoT NTN

Document for:	Discussion/Decision

# Introduction

The study item description of NB-IoT/eMTC support for NTN was provided in [1]. One objective of this study item is to identify scenarios applicable to NB-IoT/eMTC. The following agreements have been made in RAN1 #103-e meeting [2].

Agreement:

IoT NTN scenarios A, B, and C are included in the study as shown below:

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |

Agreement:

The following IoT NTN reference scenario parameters are agreed:

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C |
| --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth |
| Altitude | 35,786 km | 600 km; 1,200 km |
| Frequency Range (service link) | < 6 GHz (e.g. 2 GHz in S band) |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations , including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |
| Payload | Transparent type | Transparent Type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |

NOTE 1:    Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite.

NOTE 2:   This beam size refers to the Nadir pointing of the satellite.

NOTE 3: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam.

NOTE 4: The use of a Circular polarized antenna is optional.

NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888 for device can be assumed for link budget. The noise figure is device vendor implementation specific.

NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link.

NOTE 7: System bandwidth is FFS

In this contribution, we provide link budget analysis based on the agreed IoT NTN reference scenario parameters.

# Discussion

Three different IoT NTN scenarios have been agreed [2]. The reference parameters of these IoT NTN scenarios have been agreed [2]. More details can be found in [3].These parameters are used in our link budget analysis.

The set-2 satellite parameters in [4] are used in our link budget analysis. The set-2 satellite parameters are more restrictive than the set-1 satellite parameters, which is preferrable to be used in link budget analysis. Furthermore, we assume the IoT device maximum transmit power as 23 dBm, shadowing is 3 dB, atmospheric path loss is 0.1 dB, scintillation loss is 2.2 dB and no additional loss in our analysis.

The link budget evaluations for downlink are provided in Table 1 and the link budget evaluations for uplink is provided in Table 2.

As mentioned in [3], the alignment of link budget analysis from different companies is preferred. Hence, we have the following proposal:

Proposal 1: RAN1 to align the link budget analysis for IoT NTN.

Table 1: DL NB-IoT/eMTC link budget based on set 2 satellite parameters in [4]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| Satellite EIRP density (dBW/MHz) | 53.5 |  | 34 |  | 28 |  |
| Channel bandwidth (MHz) | 0.18 | 1.08 | 0.18 | 1.08 | 0.18 | 1.08 |
| Satellite EIRP (dBm) | 76.05 | 83.83 | 56.55 | 64.33 | 50.55 | 58.33 |
| Max. distance between satellite and IoT device (km) | 40581 |  | 3131 |  | 1932 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.64 |  | 168.38 |  | 164.19 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.1 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Additional loss (dB) | 0 |  | 0 |  | 0 |  |
| IoT antenna temperature (K) | 290 |  | 290 |  | 290 |  |
| Thermal noise (dBW/Hz) | -174 |  | -174 |  | -174 |  |
| Noise floor (dBm) | -121.45 | -113.67 | -121.45 | -113.67 | -121.45 | -113.67 |
| IoT noise figure (dB) | 7 |  | 7 |  | 7 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| CNR (dB) | -5.44 | -5.44 | -2.68 | -2.68 | -4.49 | -4.49 |

Table 2: UL NB-IoT/eMTC link budget based on set 2 satellite parameters in [4]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| IoT device max Tx power (dBm) | 23 |  | 23 |  | 23 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| IoT device EIRP (dBm) | 23 |  | 23 |  | 23 |  |
| Max. distance between satellite and IoT device (km) | 40581 |  | 3131 |  | 1932 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.64 |  | 168.38 |  | 164.19 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.1 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Additional loss (dB) | 0 |  | 0 |  | 0 |  |
| Satellite Rx max gain (dBi) | 45.5 |  | 24 |  | 24 |  |
| Channel bandwidth (MHz) | 0.015 | 0.18 | 0.015 | 0.18 | 0.015 | 0.18 |
| CNR (dB) | -5.20 | -5.99 | -4.45 | -5.24 | -0.25 | -1.04 |

# Conclusion

In this contribution, we provided our link budget analysis on IoT over NTN. Our proposal is as follows:

Proposal 1: RAN1 to align the link budget analysis for IoT NTN.

# References

- RP-202689, “New study WID on NB-IoT/eMTC support for NTN,” Dec. 2020.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #103-e Meeting, Nov. 2020.

- R1-2008868, “Email summary discussion in scenario applicable to NB-IoT/eMTC,” Nov. 2020.

- 3GPP TR 38.821, Solution for NR to support non terrestrial networks (NTN), v. 16.2.0, Dec. 2019.