| 3GPP TSG-RAN WG1 Meeting #104 e-Meeting, January 25th – February 5th, 2021 | R1-2101430 |
| --- | --- |

Title: 		[Draft] Reply to RAN2 LS on SDT Resource Configuration

Response to:		R2-2010841

Release:           			NR R17

Work Item:       		NR_SmallData_INACTIVE-Core

Source:		Qualcomm [RAN1]

To:		RAN2

Send any reply LS to:	  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		  None

# Overall Description

RAN1 would like to thank RAN2 for the LS on physical layer aspects of small data transmission.

From RAN1 perspective, UE shall monitor PDCCH addressed to C-RNTI in initial DL BWP or active DL BWP after successful completion of the RACH procedure during RA-SDT. The active DL BWP (if different from initial DL BWP), CORESET and search space configurations for the UE can be included in the RRCResume or RRCReconfiguration message.

From RAN1 perspective, gNB can explicitly indicate the SSB index(es) as well as the CG resources (i.e. PUSCH/DMRS occasions) associated with the SSB index(es)  in the RRCRelease message. Each CG resource includes a PUSCH occasion and associated DMRS resource. Similar to higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant, the PUSCH occasion can be defined by a time and frequency resource and the DMRS can be defined by antennaPort, cg-DMRS-Configuration and dmrs-SeqInitialization. Before CG-SDT, UE needs to validate the PUSCH occasion(s) and the TA. The PUSCH validation procedures specified in Section 8.1A of TS 38.213 can be re-used for the validation of CG-SDT resources.

# Actions

To 3GPP RAN2

ACTION: RAN1 respectfully asks RAN2 to take RAN1’s feedback on physical layer aspects of small data transmission into account in their specification work.

# Dates of Next TSG-RAN1 Meetings:

# RAN1 #104bis-e			12 April – 20 April 2021			Electronic

# RAN1 #105-e			19 May – 27 May 2021			Electronic

# References

- RP-201305, “Work Item on NR small data transmissions in INACTIVE state, ” 3GPP TSG RAN#88e.

- R2-2010841, “LS on physical layer aspects of small data transmission, ” 3GPP TSG RAN2#112e, November, 2020.