3GPP TSG RAN WG1 #104-e 	R1-2101431

e-Meeting, January 25th – February 8th

Source:              Qualcomm

Title:                   Discussion on CSI-RS ambiguities

Agenda Item:     7.1

Document for:   Discussion and Decision

# Introduction

In this paper we describe a set of potential ambiguities that may arise with respect to following topics in NR Rel-15:

- Interpretation of the Frequency-domain Occupation for ZP-CSIRS

- NZP CSI-RS resource association to a CSI-ResourceConfig

- Rate match NZP CSI-RS resources of the active BWP only

- Semi-persistent NZP CSI-RS resources are rate matched only if active

# Frequency-domain Occupation for ZP-CSIRS

In NR Rel-15 specification, the The CSI-FrequencyOccupation IE of 38.331 is used for both NZP and ZP-CSI frequency domain configuration. In 38.331, in the description of the “startingRB” field, it is described that the reference point is CRB#0.

The maximum value of “maxNPRBs” is 275. It is generally known that CRB0 can be farther away from the BWP of the UE than 275 PRBs, and this is why,  with regards to the NZP CSI-RS, the following text appears in 38.214 was agreed to clarify the scenarios for which the startingRB<NBWPstart.

5.2.2.3 Reference signal (CSI-RS) 5.2.2.3.1 NZP CSI-RS … The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP, as defined in Clause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize).

We also observe that this text is within a section called “NZP CSI-RS”, whereas a corresponding text does not appear for ZP-CSIRS. Therefore, one may get confused on how to interpret the CSI-FrequencyOccupation field when it is used for a ZP-CSIRS resource.

It is reasonable to assume that the intention of the spec is to apply the same interpretation in both the NZP-CSIRS and ZP-CSI-RS resources. Based on this, we make the following proposal:

Proposal 1: Clarify that the CSI-FrequencyOccupation IE of TS 38.331 is interpreted the same way for both NZP-CSI-RS and ZP-CSI-RS resources.

# NZP CSI-RS resource association to a CSI-ResourceConfig

An NZP CSI-RS Resource IE does not contain the BWP ID nor a “resourceType”, but rather it is included in the CSI-ResourceConfig. This was done to enable sharing of basic configuration parameters across BWPs (e.g. have one underlying NZP CSI-RS resource config, which is connected to multiple CSI-ResourceConfigs, each one associated to a different BWP).

CSI-ResourceConfig information element -- ASN1START -- TAG-CSI-RESOURCECONFIG-START CSI-ResourceConfig ::= SEQUENCE { csi-ResourceConfigId CSI-ResourceConfigId, csi-RS-ResourceSetList CHOICE { nzp-CSI-RS-SSB SEQUENCE { nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId OPTIONAL -- Need R }, csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId }, bwp-Id BWP-Id, resourceType ENUMERATED { aperiodic, semiPersistent, periodic }, ... } -- TAG-CSI-RESOURCECONFIG-STOP -- ASN1STOP

However, the ambiguity with the current solution is the following: If there is a NZP CSI-RS resource configured that is not associated with any CSI-ResourceConfig, how can the UE know the BWP ID for which this NZP CSI-RS resource is applicable, and how can it know whether it is aperiodic semi-persistent or periodic?

We believe the intention was that the network would associated each NZP CSI-RS resource with at least a single CSI-ResourceConfig, otherwise there would be unknown configuration parameters which are important for the UE to know and function properly. Therefore, we make the following proposal:

Proposal 2: Clarify that a UE expects that each NZP CSI-RS resource belongs in at least one CSI-ResourceConfig.

# NZP CSI-RS resource rate matching Ambiguities

# Rate match NZP CSI-RS resources of the active BWP only

Rate matching of NZP CSI-RS resources is written in 38.211 Section 7.3.1.5, as shown below.

The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource;

Should a UE should rate match NZP CSI-RS resources associated with a CSI-ResourceConfig of the active BWP only, or should the UE rate match all the NZP CSI-RS resources independent of whether they are associated with the active BWP? We believe the intention is the UE to rate match CSI-RS resources of the active BWP only.

Proposal 3: Clarify that a UE is not expected to rate match NZP CSI-RS resources associated with a BWP other than the active BWP.

# Semi-persistent NZP CSI-RS resources are rate matched only if active

Should a UE should rate match Semi-persistent NZP CSI-RS resources that are activated or should it rate match resources that are configured but not active? We believe that the UE is expected to rate match only active Semi-persistent resources.

Proposal 4: Clarify that a UE is not expected to rate match semi-persistent NZP CSI-RS resources which are not active.

# Summary Of Proposals

We make the following proposals:

Proposal 1: Clarify that the CSI-FrequencyOccupation IE of TS 38.331 is interpreted the same way for both NZP-CSI-RS and ZP-CSI-RS resources.

Proposal 2: Clarify that a UE expects that each NZP CSI-RS resource belongs in at least one CSI-ResourceConfig.

Proposal 3: Clarify that a UE is not expected to rate match NZP CSI-RS resources associated with a BWP other than the active BWP.

Proposal 4: Clarify that a UE is not expected to rate match semi-persistent NZP CSI-RS resources which are not active.